3GPP TSG-RAN WG2 Meeting #113 bis electronic									R2-2102769

Online, April 12 – April 20, 2021

Agenda item:	5.4.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification on sCellFrequenciesSN-NR and scellFrequenciesSN-EUTRA

WID/SID:	NR_newRAT-Core

Document for:	Discussion and Decision

# 1	Introduction

There seems to be an inconsistency related to the field sCellFrequenciesSN-NR and scellFrequenciesSN-EUTRA in the inter-node message in section 11.2.3 of TS 38.331. We discuss the options to try to see if there is a simple way to fix this.

# 2	Discussion

In the case of EN-DC, upon releasing SCell(s), the SN is supposed to inform the MN using the scellFrequenciesSN-NR field. See parts in CYAN below. The PSCell is separately indicated.

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration.

Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                           CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1540-IEs ::=             SEQUENCE {

pSCellFrequency                     ARFCN-ValueNR                                   OPTIONAL,

reportCGI-RequestNR                 SEQUENCE {

requestedCellInfo                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR,

cellForWhichToReportCGI             PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

ph-InfoSCG                          PH-TypeListSCG                                  OPTIONAL,

nonCriticalExtension                CG-Config-v1560-IEs                             OPTIONAL

}

CG-Config-v1560-IEs ::=             SEQUENCE {

pSCellFrequencyEUTRA                ARFCN-ValueEUTRA                                OPTIONAL,

scg-CellGroupConfigEUTRA            OCTET STRING                                    OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                    OPTIONAL,

candidateServingFreqListEUTRA       CandidateServingFreqListEUTRA                   OPTIONAL,

needForGaps                         ENUMERATED {true}                               OPTIONAL,

drx-ConfigSCG                       DRX-Config                                      OPTIONAL,

reportCGI-RequestEUTRA              SEQUENCE {

requestedCellInfoEUTRA          SEQUENCE {

eutraFrequency                             ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA              EUTRA-PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

nonCriticalExtension                CG-Config-v1590-IEs                             OPTIONAL

}

CG-Config-v1590-IEs ::=             SEQUENCE {

scellFrequenciesSN-NR               SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueNR          OPTIONAL,

scellFrequenciesSN-EUTRA            SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueEUTRA       OPTIONAL,

nonCriticalExtension                CG-Config-v1610-IEs                                                    OPTIONAL

}

CG-Config-v1610-IEs ::=             SEQUENCE {

drx-InfoSCG2                        DRX-Info2                                       OPTIONAL,

nonCriticalExtension                CG-Config-v1620-IEs                             OPTIONAL

}

CG-Config-v1620-IEs ::=             SEQUENCE {

ueAssistanceInformationSCG-r16      OCTET STRING (CONTAINING UEAssistanceInformation)  OPTIONAL,

nonCriticalExtension                CG-Config-v1630-IEs                                OPTIONAL

}

CG-Config-v1630-IEs ::=             SEQUENCE {

selectedToffset-r16                 T-Offset-r16                                       OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

PH-TypeListSCG ::=                  SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoSCG

PH-InfoSCG ::=                      SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierSCG,

ph-SupplementaryUplink              PH-UplinkCarrierSCG                             OPTIONAL,

...

}

PH-UplinkCarrierSCG ::=             SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

MeasConfigSN ::=                    SEQUENCE {

measuredFrequenciesSN               SEQUENCE (SIZE (1..maxMeasFreqsSN)) OF NR-FreqInfo  OPTIONAL,

...

}

NR-FreqInfo ::=                     SEQUENCE {

measuredFrequency                   ARFCN-ValueNR                                       OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                               OPTIONAL,

requestedP-MaxFR1                   P-Max                                               OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                     OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                               OPTIONAL

]],

[[

requestedP-MaxFR2-r16               P-Max                                               OPTIONAL,

requestedMaxInterFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                     OPTIONAL,

requestedMaxIntraFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                     OPTIONAL,

requestedToffset-r16                T-Offset-r16                                        OPTIONAL

]]

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=           SEQUENCE {

bandCombinationIndex                BandCombinationIndex,

requestedFeatureSets                FeatureSetEntryIndex

}

FR-InfoList ::= SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::= SEQUENCE {

servCellIndex       ServCellIndex,

fr-Type             ENUMERATED {fr1, fr2}

}

CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR

CandidateServingFreqListEUTRA ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueEUTRA

T-Offset-r16 ::= ENUMERATED {ms0dot5, ms0dot75, ms1, ms1dot5, ms2, ms2dot5, ms3, spare1}

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

Given the following in TS 38.331 section 11.2.3, it would make sense to send the scellFrequenciesSN-NR upon SCell add/release. See below the snippet of the procedural text from TS 38.331. See part in YELLOW.

For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise. As an exception to this general rule, the absence of the below listed fields means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted: - configRestrictInfo; - gapPurpose; - measGapConfig (for which delta signaling applies); - measGapConfigFR2 (for which delta signaling applies); - measResultCellListSFTD; - measResultSFTD-EUTRA; - sftdFrequencyList-EUTRA; - sftdFrequencyList-NR; - ue-CapabilityInfo; - servFrequenciesMN-NR; - scellFrequenciesSN-EUTRA; - scellFrequenciesSN-NR.

Based on current field description, scellFrequenciesSN-NR does not include the PSCell seeing the part in GREEN in the field description.

scellFrequenciesSN-EUTRA, scellFrequenciesSN-NR Indicates the frequency of all SCells configured in SCG. The field scellFrequenciesSN-EUTRA is used in NE-DC; the field scellFrequenciesSN-NR is used in (NG)EN-DC and NR-DC. In (NG)EN-DC, the field is optionally provided to the MN.

Observation 1: It seems that if only the PSCell remains, the scellFrequenciesSN-NR shall be signalled as empty but there is no way to current signal it.

Observation 2: If the scellFrequenciesSN-NR is omitted (ASN.1 optional field) the procedural text in 11.2.3 will be broken as delta signalling applies for this field and MN will wrongly assume that the SN still has the SCell(s).

Observation 3: Same problem exists also for NE-DC, i.e. for the field scellFrequenciesSN-EUTRA.

In most of the practical scenarios today in the field there is one SCell in addition to the PSCell. That is the PSCell mostly remains with one SCell being added or removed. In that sense, one could argue that the SCell does not have to be removed but just deactivated but this is sub-optimal from the UE point of view and hence is not a solution option.

# 3	Potential solutions

There are several ways to fix the issue above:

Solution 1: Add a field (starting Rel-15) indicating that all the SCells are released (one field for NE-DC and other for rest of the MR-DC options).

- This proposal impacts both MN and SN and changes current behavior but is not strictly NBC

- We could also start from Rel-16 if late non-critical extensions are not possible to be signalled in Rel-15 ASN.1 due to known limitations

Solution 2:  Change the cardinality of the fields to start from 0 rather than 1 (in this case ASN.1 encoding will not change)

- This proposal impacts both MN and SN and changes current behavior but is strictly NBC

Solution 3: Use some other field (e.g.  fr-InfoListSCG) and maybe clarify in the field description that this could be used instead.

- This impacts the MN only and changes the current behavior

fr-InfoListSCG Contains information of FR information of serving cells that include PScell and SCells configured in SCG.

Solution 4: Remove the fields scellFrequenciesSN-NR and scellFrequenciesSN-EUTRA from delta configuration.

- This proposal impacts both MN and SN and changes current behavior but is strictly NBC

Solution 5: Dummify scellFrequenciesSN-NR and scellFrequenciesSN-EUTRA and add the fields with the correct cardinality i.e. starting from 0.

- This proposal impacts both MN and SN and changes current behavior but is backward compatible but would require network IOT to ensure both MN and SN are compatible with the CR

Note that all the above solutions need some specification change. Even Solution 3 which apparently does not require any specification changes, but it would be cleaner to update the field description to indicate the relationship for clarity with the other field to check for consistency.

| Solution option | MN impact | SN impact | ASN.1 NBC | Impact |
| --- | --- | --- | --- | --- |
| Solution 1 | YES | YES | Partly | Medium (affects other scenarios but simpler?) |
| Solution 2 | YES | YES | Strictly | Low |
| Solution 3 | YES | NO | No | Lowest |
| Solution 4 | YES | YES | Strictly | High (affects other scenarios?) |
| Solution 5 | YES | YES | No | Medium (affects other scenarios but simpler?) |

Table 2-1: List of all potential solutions with impact analysis

Proposal: RAN2 to discuss the potential solution options and correct the issue starting Rel-15.

# 3	Conclusion

Observation 1: It seems that if only the PSCell remains, the scellFrequenciesSN-NR shall be signalled as empty but there is no way to current signal it.

Observation 2: If the scellFrequenciesSN-NR is omitted (ASN.1 optional field) the procedural text in 11.2.3 will be broken as delta signalling applies for this field and MN will wrongly assume that the SN still has the SCell(s).

Observation 3: Same problem exists also for NE-DC, i.e. for the field scellFrequenciesSN-EUTRA.

| Solution option | MN impact | SN impact | ASN.1 NBC | Impact |
| --- | --- | --- | --- | --- |
| Solution 1 | YES | YES | Partly | Medium (affects other scenarios but simpler?) |
| Solution 2 | YES | YES | Strictly | Low |
| Solution 3 | YES | NO | No | Lowest |
| Solution 4 | YES | YES | Strictly | High (affects other scenarios?) |
| Solution 5 | YES | YES | No | Medium (take new fields into account) |

Table 2-1: List of all potential solutions with impact analysis

Proposal: RAN2 to discuss the potential solution options and correct the issue starting Rel-15.