3GPP TSG-RAN WG2 Meeting #113 electronic                                             　    　　　　　　 R2-2102773

Online, April 12 – April 20, 2021

Source: 	KDDI Corporation

Title:	Considerations on contents of slice related cell selection info

Tdoc Type:	Discussion

Agenda Item:	8.8.2	Cell Reselection

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In last RAN #91, the slice WID was agreed. The WI has mainly two objectives, slice based cell reselection and slice based RACH Configuration. This paper discusses Slice based cell reselection

- Support slice based cell reselection, specify mechanisms and signalling including [RAN2] - To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message. - To assist cell reselection, include slice info (with similar information as in SI message) in RRCRelease message. - Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2] - Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group - Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group - Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs).

Discussion

SST/SD

With regard to how to indicate the supported slice info of the current cell and neighbor cells, and cell reselection priority per slice, RAN2 identified the large size of the relevant information as an issue to be addressed. One solution is using SST(service slice type) instead of using S-NSSAI to reduce the size. Operators can organize the SST values while considering the CellReselectionPriority(0…7) needed for each service identified with the SST value. Addition to SST value, if necessary, SD(slice differentiator) value can be optionally used to indicate CellReselectionSubPriority (oDot2, oDot4, oDot6, oDot8) to configure the priority of the cell supporting the relevant slice service with more granularity.

For example, a gNB indicate

- freq1, SST=1, priority=7

- freq1, SST=2, priority=5

- freq1, SD=1, subpriority=oDot2

- freq1, SD=2, subpriority=oDot4

then the UE

- is configured intended slice, SST=1 and SD=1

  - the UE overwrites the priority with a new priority(6→7.2)

- is configured intended slice, SST=1 and SD=2

  - the UE overwrites the priority with a new priority(6→7.4)

- is configured intended slice, SST=2 and SD=1

  - the UE overwrites the priority with a new priority(6→5.2)

- is configured intended slice, SST=2 and SD=2

  - the UE overwrites the priority with a new priority(6→5.4)

S-NSSAI ::= CHOICE{ sst BIT STRING (SIZE (8)), sst-SD BIT STRING (SIZE (32)) }

Fig1. S-NSSAI information element

CellReselectionPriority ::= INTEGER (0..7) CellReselectionSubPriority ::= ENUMERATED {oDot2, oDot4, oDot6, oDot8}

Fig2. CellReselectionPriority/CellReselectionSubPriority information element

So, we made the following proposals.

Proposal1: RAN2 agree that the gNB broadcasts the supported slice service type(s)(SST) of the current cell and neighbor cells, and CellReselectionPriority per SST in system information message.

Proposal2: RAN2 agree that gNB optionally broadcast the supported slice differentiator(s)(SD) of the current cell and neighbor cells, and cellReselectionSubPriority per SD in system information message.

Table1 : Examples of changing or not reselection priority and subpriority before and after considering NSSAI (Proposal1,2)

| System Information Message | UE supported NSSAI(SST+SD) list | Reselection Priority Before considering NSSAI | Reselection Priority After considering NSSAI |
| --- | --- | --- | --- |
| SST 1, Freq 1, cellReselectionPriority 7 SST 2, Freq 1, cellReselectionPriority 5 SD 1,Freq 1, cellReselectionSubPriority oDot2 SD 2,Freq 1, cellReselectionSubPriority oDot4 | SST 1 SD 1 | 6 | 7.2 |
|  | SST 1 SD 2 | 6 | 7.4 |
|  | SST 2 SD 1 | 6 | 5.2 |
|  | SST 2 SD 2 | 6 | 5.4 |

Multiple intended slices

In case where UE has multiple intended slices, UE has to consider multiple cell reselection priorities and decide which priority is adopted. In our opinion, the gNB indicate priority order which slices are prioritized, so that the UE can decide the cell reselection priority to be adopted.

For example, if a UE is configured intended slice, SST=1 and SST=2, and a gNB indicate (freq1, SST=1, priority=7), (freq1, SST=2, priority=5), then the UE configures freq1 with priority 7. If the gNB indicate (freq1, SST=2, priority=5), (freq1, SST=1, priority=7) (the other way around), then the UE configures freq1 with priority 5.

Table2 : Examples of changing or not reselection priority before and after considering SST in case of multiple intended slices (Proposal3)

| System Information Message | UE supported SST list | Reselection Priority Before considering SST | Reselection Priority After considering SST |
| --- | --- | --- | --- |
| SST 1, Freq 1, cellReselectionPriority 7 SST 2, Freq 1, cellReselectionPriority 5 | SST 1,2 | 6 | 7 |
| SST 2, Freq 1, cellReselectionPriority 5 SST 1, Freq 1, cellReselectionPriority 7 | SST 1,2 | 6 | 5 |

So, we made the following proposals.

Proposal3: RAN2 agree that the UE adopts the first entry of the cell priority of the intended slices, if the UE has multiple intended slices

Conclusion

In this conclusion, we make the following proposals.

Proposal1: RAN2 agree that the gNB broadcasts the supported slice service type(s)(SST) of the current cell and neighbor cells, and CellReselectionPriority per SST in system information message.

Proposal2: RAN2 agree that gNB optionally broadcast the supported slice differentiator(s)(SD) of the current cell and neighbor cells, and cellReselectionSubPriority per SD in system information message.

Proposal3: RAN2 agree that the UE adopts the first entry of the cell priority of the intended slices, if the UE has multiple intended slices

References

[1] RAN Meeting #91e RP-210912 RP WID_RAN Slicing