3GPP TSG RAN WG2 Meeting #113b-e                                                        	   R2-2102837

E-Meeting, 12th – 20th April, 2021

Agenda Item:	8.16.3

Source: 	Intel Corporation

Title:	RAN2 impact on support UE onboarding and provisioning for NPN

Document for: 	Discussion/Decision

# Introduction

In the last meeting, RAN2 agrees to the following:

- Broadcast a 1-bit indication for onboarding per O-SNPN.

- R2 assumes that the 1-bit indication for onboarding is in SIB1.

- The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

In this contribution, we analysed the RAN2 impact for support UE onboarding and provisioning for NPN and proposed the changes needed from the RAN2 side and areas of study/discussion.

# Discussion

UE onboarding is a process by which the UE obtains 3GPP connectivity to perform remote provisioning of the SO-SNPN (Subscription owner- SNPN) credentials.

For network selection, the onboarding indication needs to be provided in the SIB for each SNPN and forward it to UE upper layers for UE to decide whether onboarding and provisioning can be performed for an SNPN. In the last meeting, RAN2 agreed to signal the onboarding indication in SIB1.

With this information, the UE can decide to select the onboarding SNPN to perform onboarding to an SNPN during network selection. During the registration to the SNPN for onboarding, the UE needs to indicate that it is performing onboarding to the NG-RAN. This can be done either via RRC Setup Request message (Msg3) or via NR RRC Setup Complete message (Msg5).  Our understanding is that the purpose of this indication is to allow the gNB to select the appropriate AMF for onboarding as extracted from SA2 TR conclusion [1] as follow

Upon registration to an SNPN for Onboarding, the UE provides an indication at RRC level that the RRC connectionis for onboarding. This information will be specified only for SNPN and allows NG-RAN to select an appropriate AMF that supports onboarding procedures.

Observation#1: Per SA2 conclusion, the indication on the RRC level is to allow the gNB to select the appropriate AMF for onboarding

Hence including this in Msg5 will be sufficient.

Proposal#1: Include an indication in the RRC Setup Complete message (Msg5) during the initial registration for selecting the appropriate AMF by the gNB for onboarding.

There is a note below checking with RAN2 on RAN level congestion control:

NOTE 3:	RAN WGs can work with SA2 to decide whether handling of RAN-level congestion is feasible.

In our view, the gNB just have to turn off the onboarding indication during congestion period.  There is no need for further enhancement (e.g. enhancing the UAC by introducing an access category) on top of adding the indication in the SIB since on-boarding is a one-time event for a UE where UE attempts to obtain its 3GPP credentials via remote provisioning.

Observation#2: On-boarding is a one-time event for a UE where UE attempts to obtain its 3GPP credentials via remote provisioning

Proposal#2: The enabling and disabling of the onboarding indication per SNPN can be used also for RAN level congestion control.  No additional mechanism or enhancement to UAC (e.g. introducing an access category for onboarding) is needed.

From the conclusion in the TR, there is no requirement to change the existing procedures on cell selection/reselection and the connected mode mobility.  The UE is just performing an initial registration to an onboarding SNPN based on manual selection. If cell selection/reselection occurs while UE is performing such initial registration for onboarding, it will have to abort such procedure and perform the network selection and the onboarding NAS procedure again as like any initial registration.

While UE is performing onboarding with an O-SNPN, normal cell reselection procedure with the O-SNPN is performed during this on-boarding process. After successful completion of on-boarding, the UE initiates de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network initiates the de-registration after successful completion of onboarding. Subsequently, the UE performs normal registration as before to a SNPN for normal access. Hence the normal cell reselection procedure and suitability definition is not affected by the onboarding process.

SA2 has also responded as follow [2]:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

[SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection.

Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

Observation#3: Per SA2 conclusion, there is no requirement to change the existing procedures on cell selection/reselection and the connected mode mobility. It is further confirmed by the SA2 response that “no impact to mobility procedures as remote provisioning can continue in the target cell”.

Proposal#3: No impact to cell selection/reselection or suitability definition for onboarding to SNPN.

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Observation#1: Per SA2 conclusion, the indication on the RRC level is to allow the gNB to select the appropriate AMF for onboarding

Proposal#1: Include an indication in the RRC Setup Complete message (Msg5) during the initial registration for selecting the appropriate AMF by the gNB for onboarding.

Observation#2: On-boarding is a one-time event for a UE where UE attempts to obtain its 3GPP credentials via remote provisioning

Proposal#2: The enabling and disabling of the onboarding indication per SNPN can be used also for RAN level congestion control.  No additional mechanism or enhancement to UAC (e.g. introducing an access category for onboarding) is needed.

Observation#3: Per SA2 conclusion, there is no requirement to change the existing procedures on cell selection/reselection and the connected mode mobility. It is further confirmed by the SA2 response that “no impact to mobility procedures as remote provisioning can continue in the target cell”.

Proposal#3: No impact to cell selection/reselection or suitability definition for onboarding to SNPN.

# References

[1] TR23.700-07 v1.2.0 Study on enhanced support of non-public networks

[2] S2-2101076 Reply LS on clarification request for eNPN features