3GPP TSG-RAN WG2 Meeting #113 bis electronic				R2-2102851

Online, April 12 – April 20, 2021

Agenda item:		8.11.4

Source:			Intel Corporation

Title:		Support of on demand PRS

Document for:	 	Discussion and decision

# Introduction

At the RAN#91, RAN2 completed the work on “Study Item on NR Positioning Enhancements” [1], and the WID [2] was revised in [3] with following objectives:

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

In this contribution, we discuss how to support on demand PRS.

# Discussion

As captured in the TR “-	The exact parameters that can be dynamically changed and necessary measurement and/or assistance information for LMF/UE initiated on demand DL-PRS are expected to be decided by RAN1 and RAN2 during WI phase.”, therefore we first discuss what parameters can be changed.

As discussed in RAN2, following parameters could be dynamic changed:

Switch on/off of PRS transmission or beam transmission:

Efficiency: On-demand DL-PRS avoids unnecessary overhead, waste of energy, etc. in the case that no UE positioning is required during a particular time or in a particular area of a network. In case of beamformed DL-PRS, DL-PRS transmission in all beam sweeping directions may result in an unnecessary transmission of DL-PRSs.

The motivation to support on-demand DL-PRS is to optimize usage of the network resources and make DL-PRS configuration more UE-centric. For example, UE may not be able to process all TRPs and beams and therefore transmission on the whole DL-PRS resource set (associated with TRP in a positioning frequency layer) or subset of DL-PRS resources (representing beams) within DL PRS resource set can be disabled. To support this functionality UE may need to indicate the list of recommended TRPs and associated DL PRS resources favorable for positioning measurements. UE reporting may be used to facilitate construction of UE-or group centric DL-PRS transmissions targeting specific UE or group of UEs (i.e. UE centric or group common DL PRS configuration).

Occasion/periodicity of DL-PRS transmission:

Latency: The current DL-PRS configuration may not be sufficient to meet the response time requirements of the LCS client; e.g., may have a too large periodicity.

To meet latency requirements, UE may indicate recommended periodicity of DL PRS transmission. Network may allocate DL PRS resources without actual transmission using small periodicity value. The actual DL PRS transmission on specific occasion of DL PRS allocation may be separately controlled and indicated towards UEs. Such approach if it is supported can balance DL PRS latency/spectrum efficiency.

Bandwidth, repetitions:

Accuracy: The current DL-PRS configuration may not be sufficient to meet the accuracy requirements of the LCS client; e.g., may have a too small bandwidth, too few repetitions, etc.

To meet accuracy requirements UE may need to provide recommendation on required bandwidth DL-PRS frequency layer and DL-PRS resource repetition.

For construction of UE/group centric DL-PRS configurations UE may need to report results of cell-centric DL PRS configuration (targeting all UEs) or other DL reference signals processing, e.g. SSB or CSI-RS or provide a priory known coordinate.

In summary, the following on-demand feedback information and parameters can be  recommended by UEs for on-demand operation assuming that the UE is aware of the “cell/system-centric reference” DL PRS:

- List of recommended TRPs / frequency layers / DL-PRS resource set IDs for positioning measurements

- Recommended DL-PRS resource set configuration parameters, DL-PRS muting patterns

- DL-PRS resource configuration parameters (e.g. recommended DL PRS resource IDs, number of symbols, resource repetitions, comb-factor)

- Recommended DL-PRS periodicity of allocation

- Recommended bandwidth of DL-PRS frequency layer

Proposal 1: To confirm, UE or LMF can use on-demand DL-PRS procedure to request/recommend the following changes:

- Switch on/off of DL-PRS transmission or beam transmission;

- Occasion/periodicity of DL-PRS transmission;

- DL-PRS Bandwidth, repetitions;

Proposal 2: RAN2 to agree the following on-demand feedback information and parameters can be  recommended by UEs for on-demand operation

- List of recommended TRPs / frequency layers / DL-PRS resource set IDs for positioning measurements

- Recommended DL-PRS resource set configuration parameters, DL-PRS muting patterns

- DL-PRS resource configuration parameters (e.g. recommended DL PRS resource IDs, number of symbols, resource repetitions, comb-factor)

- Recommended DL-PRS periodicity of allocation

- Recommended bandwidth of DL-PRS frequency layer

To support the request from the UE:

1 the UE should have pre-configured PRS configuration or the lists (via LPP or broadcast signalling);

2 the UE sends on-demand PRS request to the LMF when PRS configuration needs to be changed, e.g. on/off, change pattern.

3 when the LMF receives the request from the UE, it is LMF decision on whether PRS should be changed or not, e.g. based on the information from multiple UEs (the request from UE, accuracy requirement or measurements from the UE), etc.

4/5 If yes, the LMF should contact gNBs to update the PRS configuration;

6 if needed, the LMF sends the updated PRS configuration back to the UE; It may be done via broadcast signalling or LPP message;

To support the request from the LMF:

3 the LMF decision on whether PRS should be changed or not. For LMF initiated request, the LMF may decide to change PRS configuration based on the accuracy requirement or measurements from the UE, etc.

4/5 the LMF should contact gNBs to update the PRS configuration;

6 if needed, the LMF sends the updated PRS configuration back to the UE; It may be done via broadcast signalling or LPP message;

Proposal 3: For UE initiated on-demand PRS and LMF initiated on-demand PRS, common procedure is used for LMF to update the PRS configuration. The only difference between UE initiated on-demand PRS and LMF initiated on-demand PRS is whether the LMF receives the request from the UE.

Proposal 4: UE initiated on-demand PRS request is sent to LMF via LPP message.

The procedure is shown as below:

Proposal 5:  To capture the on demand PRS procedure in the stage 2 specification.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: To confirm, UE or LMF can use on-demand DL-PRS procedure to request/recommend the following changes:

- Switch on/off of DL-PRS transmission or beam transmission;

- Occasion/periodicity of DL-PRS transmission;

- DL-PRS bandwidth, repetitions;

Proposal 2: RAN2 to agree the following on-demand feedback information and parameters can be  recommended by UEs for on-demand operation

- List of recommended TRPs / frequency layers / DL-PRS resource set IDs for positioning measurements

- Recommended DL-PRS resource set configuration parameters, DL-PRS muting patterns

- DL-PRS resource configuration parameters (e.g. recommended DL PRS resource IDs, number of symbols, resource repetitions, comb-factor)

- Recommended DL-PRS periodicity of allocation

- Recommended bandwidth of DL-PRS frequency layer

Proposal 3: For UE initiated on-demand PRS and LMF initiated on-demand PRS, common procedure is used for LMF to update the PRS configuration. The only difference between UE initiated on-demand PRS and LMF initiated on-demand PRS is whether the LMF receives the request from the UE.

Proposal 4: UE initiated on-demand PRS request is sent to LMF via LPP message.

Proposal 5:  To capture the on demand PRS procedure in the stage 2 specification.

# Reference

- RP-202094 “Revised SID: Study on NR Positioning Enhancements”, CATT, Intel Corporation

- RP-202900 “New WID on NR Positioning Enhancements” CATT, Intel Corporation, Ericsson

- RP-210903 “Revised WID on NR Positioning Enhancements” Intel Corporation, CATT