3GPP TSG-RAN WG2 Meeting #113bis-e                                               R2-2102951

Electronic meeting, 12th - 20th April, 2021

Source:	CATT

Title:	Discussion on UL Scheduling Enhancements in NR NTN

Agenda Item:	8.10.2.2

Document for:	Discussion and Decision

Introduction

For the UL Scheduling enhancements, the following agreements were achieved in RAN2 #113-e meeting [1]:

- Both Type 1 and Type 2 configured grant are feasible in NTN. - From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN. - No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN. - UE in NTN can have both 2-step RACH and configured grant configurations at the same time.

This document will discuss the following aspects:

- The Restrictions of using BSR over 2-step RACH

- Co-existence issue of BSR over CG and BSR over 2-step RACH

Discussion

The Restrictions of using BSR over 2-step RACH

For the agenda item 6.11.2, the CR R2-2102079 clarifying available UL-SCH resource when sending BSR was agreed in RAN2 #113-e meeting. This CR indicated that the note of available UL-SCH resource in TS38.321may be misunderstood that MSGA PUSCH is not considered as available UL-SCH resource. The CRclarfied UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant when sending BSR. For this definition, it can deduce that the UL grant of 2-step RA can be used to send BSR. That means BSR can trigger 2-step RA procedure in TN. The following text is the correction of the CR R2-2102079:

5.4.5 Buffer Status Reporting … NOTE 2: UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time. 5.22.1.6 Buffer Status Reporting … NOTE 1: UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

Observation 1: The BSR can be sent on MSGA PUSCH of 2-step RACH.

The propagation delays in terrestrial mobile systems are usually less than 1 ms. In contrast, the propagation delays in NTN are much longer, ranging from several milliseconds to hundreds of milliseconds depending on the altitudes of the satellite in NTN. When the BSR is triggered for the new data transmissions, using 2-step RACH resources to send BSR can reduce the delay.

Proposal 1: RAN2 confirm that BSR over 2-step RACH is supported in NTN.

According to the Performance requirements for satellite access of Table 7.4.2-1 in the TS22.261, the required overall user density is [100]/km2 [1]. According to the Table 7.2.1.1.1.1-2 in the TR38.821, for LEO with 26000 km2 of coverage, the supported UE density is [101]/km2 when the one RACH per 10 min per UE [3]. It means that a UE need to guarantee random access at most one time per 10 mins. The Table 7.2.1.1.1.1-2 in the TR38.821 is as following:

Table 7.2.1.1.1.1-2: Supported UE density for typical GEO and LEO cell

|  | Coverage (km2) | RACH per second per UE | Supported UE density |
| --- | --- | --- | --- |
| GEO | 650000 (hex with r=500km) | 1.157 * 10-5 (= 1 time per day per UE) | ~596 UE/km2 |
|  | 650000 | 2.78 * 10-4 (= 1 time per hour per UE) | ~25 UE/km2 |
|  | 650000 | 0.0017 (= 1 time per 10 min per UE) | ~4 UE/km2 |
|  | 162500 (hex with r=250km) | 1.157 * 10-5 (= 1 time per day per UE) | ~2383 UE/km2 |
|  | 162500 | 2.78 * 10-4 (= 1 time per hour per UE) | ~99 UE/km2 |
|  | 162500 | 0.0017 (= 1 time per 10 min per UE) | ~16 UE/km2 |
| LEO | 26000 (hex with r=100km) | 1.157 * 10-5 (= 1 time per day per UE) | ~14893 UE/km2 |
|  | 26000 | 2.78 * 10-4 (= 1 time per hour per UE) | ~620 UE/km2 |
|  | 26000 | 0.0017 (= 1 time per 10 min per UE) | ~101 UE/km2 |
|  | 6500 (hex with r=50km) | 1.157 * 10-5 (= 1 time per day per UE) | ~59571 UE/km2 |
|  | 6500 | 2.78 * 10-4 (= 1 time per hour per UE) | ~2479 UE/km2 |
|  | 6500 | 0.0017 (= 1 time per 10 min per UE) | ~405 UE/km2 |

Observation 2: For LEO with 26000 km2 of coverage, a UE need to guarantee random access at most one time per 10 mins.

The number of UE served by each NTN cell is higher than that served by the TN cell. Meanwhile, the probability of UEs using random access resources may be increased because of the mobility of some satelite types, e.g. LEO, the RACH resources may be shortage in NTN.

In NTN, if the BSR is triggered, the BSR can be sent over 2-step RACH can decrease delay for UL scheduling. However, the 2-step RACH can also be used for BFR/HO or RA procedure etc. Using the MSGA related resource to send BSR in NTN may increase collision probability and usage frequency of 2-step RACH resources and cause the delay increases. Therefore, the restrictions of using BSR over 2-step RACH may be considered for reducing the collision probability and usage frequency of 2-step RACH resource.

Observation 3: The increase in the number of serving UE and the mobility of satellites may cause shortage of RACH resources in NTN.

Proposal 2: In NTN, the restrictions of using BSR over 2-step RACH may be considered for reducing the collision probability and usage frequency of 2-step RACH resource.

Co-existence issue of BSR over CG and BSR over 2-step RACH

As BSR over 2-step RACH and BSR over CG are both supported in NTN, the specification impacts on how to use these two methods properly may be studied first. On the current specification, if the BSR is triggered, the UE confirm that whether the UL-SCH resources are available. If there are UL-SCH resources to send BSR, the BSR will be transmitted over the UL-SCH resources. If the configured uplink grants and 2-step RACH resources are both available for the UE, there are two options for UE to decide which resources the UE may choose:

Option 1: The UE may be expected to select the configured grants to send the BSR since using the 2-step RACH resource to send BSR may introduce the collision of 2-step RACH resources.

Option 2: Introduce a timer to control when to use the configured grant and when to use the 2-step RACH resource.

For the option 1, the specification impact is relatively small, like the current specification. The UE determines if there are available configured grants at first, then only when there are no configured grants or the configured grants do not meet the LCP mapping restrictions, the BSR can be sent over 2-step RACH.

For the option 2, if the configured grants are arrived when the timer is running, the UE will send the BSR over CG. If the configured grants are not arrived when the timer is running, the UE will use the 2-step RA to send BSR. The benefit of option2 is to reduce delay by avoiding waiting for the long periodicity of CG.

Proposal 3: After the BSR is triggered, if the configured uplink grants and 2-step RACH resources are both available for the UE, RAN2 can discuss the feasibility of the following two options for UE to decide which resources the UE may choose:

Option 1: The UE may be expected to select the configured grants to send the BSR since using the 2-step RACH resource to send BSR may introduce the collision of 2-step RACH resources.

Option 2: Introduce a new timer to control when to use the configured grant and when to use the 2-step RACH resource.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: The BSR can be sent on MSGA PUSCH of 2-step RACH.

Proposal 1: RAN2 confirm that BSR over 2-step RACH is supported in NTN.

Observation 2: For LEO with 26000 km2 of coverage, a UE need to guarantee random access at most one time per 10 mins.

Observation 3: The increase in the number of serving UE and the mobility of satellites may cause shortage of RACH resources in NTN.

Proposal 2: In NTN, the restrictions of using BSR over 2-step RACH may be considered for reducing the collision probability and usage frequency of 2-step RACH resource.

Proposal 3: After the BSR is triggered, if the configured uplink grants and 2-step RACH resources are both available for the UE, RAN2 can discuss the feasibility of the following two options for UE to decide which resources the UE may choose:

Option 1: The UE may be expected to select the configured grants to send the BSR since using the 2-step RACH resource to send BSR may introduce the collision of 2-step RACH resources.

Option 2: Introduce a new timer to control when to use the configured grant and when to use the 2-step RACH resource.

Reference

- RAN2 #113-e Chairman notes

- TS22.261 Service requirements for the 5G system V17.5.0

- TR38.821 Solutions for NR to support non-terrestrial networks (NTN) V16.0.0