3GPP TSG-RAN WG2 #113bis-e	Tdoc R2-2102966

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.12.3.2

Source:	Ericsson

Title:	Mechanisms for RRM relaxation for RedCap

Document for:	Discussion, Decision

# 1	Introduction

There are several scenarios in which a less capable device can be deployed. Such less capable devices have been studied in the now completed SI on RedCap, the outcome of which is captured in a technical report report [4]. A WI [1] has been approved in RAN#91-e. The WID lists e.g. the following as one of the generic requirements:

Generic requirements: - Device complexity: Main motivation for the new device type is to lower the device cost and complexity as compared to high-end eMBB and URLLC devices of Rel-15/Rel-16. This is especially the case for industrial sensors.

In this paper we focus on one of the objectives of the WI:

- RRM relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected, considering the alternatives identified in the RedCap SI: - Study until RAN#92e, and, if agreed, specify RRM measurement relaxation criteria (where, for RRC_Idle/Inactive the Rel-16 mechanism is the baseline, and for RRC_Connected the mechanism reuses the Rel-16 RRM relaxation criteria from RRC_Idle/Inactive so as to maximize the commonality with Idle/Inactive UEs) [RAN2] - Enabling/disabling of RRM relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM relaxation. - After RAN#92e, if agreed in RAN2, specify RRM measurement relaxation [RAN4] - No RRM relaxations are specified for the serving cell.

The plan in the first two meetings of the WI is to continue discussing the open issues from the SI trying to agree on a way forward. This contribution provides a brief look at the results of the SI and suggestions on how  and on what subjects to focus efforts in the WI.

# 2	RRM relaxation for stationary UEs

During the SI-phase a number of aspects of RRM relaxation have been discussed as possible mechanisms to reduce the device power consumption; most solutions are for RRC_IDLE/INACTIVE devices but there are also some solutions for RRC_CONNECTED device. The outcome of the discussion are often captured in the form of proposed ideas for how to reduce power consumption even further (than what is possible for Rel-16 NR). Before resuming the discussions of enhancements, it is good to once again recapitulate the baseline and what hurdles need to be overcome (for proposed enhancements).

## 2.1.	Relaxation for a device in RRC_IDLE/INACTIVE

When an NR device is in RRC_IDLE/INACTIVE the main contributors to power consumption are activities related to loop convergence (automatic gain control (AGC)/automatic frequency control (AFC)/Timing), RRM measurements, paging monitoring, and potential cell reselections. Part of this loop is up to implementation and therefore not specified in detail. There are contributors not included here, such as periodical connections to the NW for the sake of registration updates, as they are quite infrequent.

In-between occasions for the abovementioned activities, the device can enjoy various levels of power saving. The figure below depicts an example of typical RRC_IDLE/INACTIVE device activities and accommodated level of power consumption when camping on a cell.

Figure 1. Principle of loop convergence measurements (without neighbor RRM, SI reading, registration updates, etc).

The loop convergence related measurements, i.e. AGC, time and frequency tracking loops (TTL & FTL), are necessary for the sake of paging reception. This means one SSB for AGC (depending on time since last AGC) every now and then (say every 160..300 ms) and one for combined AFC/measurements. Note that, only serving cell related activities are depicted, i.e. no neighbor RRM, no SI reading, no registration updates. This scenario is the best that can be achieved (i.e. minimum amount of activities) in terms of power saving, i.e. if the device can refrain from neighbor cell RRM measurements.

### 2.1.1	Baseline power saving from RRM relaxation

RRM relaxation is possible for NR devices in Rel-16. Two main RRM relaxation criteria are defined for detecting scenarios in which

- Devices are in low mobility

- Devices are not at cell edge

These criteria are based on serving cell quality only, i.e. they are not beam-specific or based on neighbor cell conditions. Only broadcast configuration is possible, i.e. the configuration is not device specific. A new device capability is defined for the relaxed measurement feature without signaling to the gNB, i.e. gNB will not know whether a UE supports this feature (see section 5, TS 38.306 [5]).

Perhaps the most interesting criterion for this WI is the one on low mobility. This criterion is fulfilled if Srxlev < SrxlevRef – SsearchDeltaP for a time TSearchDeltaP, i.e. the device is low mobility. Thus, TSearchDeltaP is the time period over which the Srxlev (i.e. RSRP) variation is evaluated to determine if the device is in a mobility state that allows relaxed measurements. SSearchDeltaP is the maximum allowed drop in the serving cell’s Srxlev variation during this time period.

The other criterion involves comparisons of measurements and thresholds to determine if the device is near the cell edge or not. If (Srxlev > SSearchThresholdP and Squal > SSearchThresholdQ), the UE may relax RRM measurements on intra frequency and inter-frequency/RAT cells of equal/lower priority neighbors, since the device is not near  the cell edge.

In Rel-16 “low mobility” and “not at cell edge” can be configured together or separately. Further, if both criteria are configured there are situations where only one criterion is met, then existing requirements may be relaxed by a factor 3. The maximum relaxation of one hour is allowed for a low-mobility device not at the cell edge, i.e. when both criteria are fulfilled or when only low-mobility criteria is configured and it is met.

The maximum relaxation of one hour is allowed in Rel-16 for a low-mobility device not at cell edge and provides a significant reduction of relative power consumption.

The abovementioned thresholds, time period and maximum variation are optionally signaled via SIB2. If SIB2 includes these values the device that reads SIB2 is allowed to apply relaxation.

Figure 2. Relative power consumption as a function of interval between DRX cycles [2].

Figure 2 [2] shows how the relative power consumption decreases as the interval between RRM measurements increases. When relaxation is configured so that measurements only occur every 4th DRX cycle the relative power consumption decreases with 31%; measurements every 20th DRX cycle results in 39.3% reduction and so on. One important observation from Figure 2 is that at some point any further increase of relaxation provides very little power saving. The dashed line represents the one hour relaxation possible for NR Rel-16 devices. There is really no difference in relative power consumption between 1 hour and 25 hours relaxation. The evaluation is based on the UE energy consumption model provided in TR 38.840 [3].

An RRM measurement relaxation of 25 h, compared to the maximum relaxation of 1 h as of Rel-16 UE power saving, provides a negligible gain.

Figure 3. Illustration of power consumption model [3].

The model records the time a device is active in some way, see principle in Figure 3. There are different types of activities that all represent a level of power consumption. Periods when the device is inactive and allowed to sleep the power consumption is the lowest. The model also considers power consumption when switching between states.

### 2.1.2	Proposed improvements in the TR

The proposed improvements for RRM relaxation are, according to TR 38.875, clause 8.4, are, in short:

Trigger conditions that help detect when the device is “stationary”:

- Introduce an additional SsearchDeltaP_stationary threshold to support 2-level speed evaluation (i.e. stationary and low mobility)

- Introduce an additional TSearchDeltaP_stationary to support 2-level speed evaluation (i.e. stationary and low mobility).

- Take into account changes in beam measurements in serving cell

- UE determines its stationary property based on subscription information

- Introduce an additional SsearchDeltaP_correction threshold and configure the UE to use it if it observes higher received signal power variation that does not violate stationary property, e.g. stationary rotating device

- UE determines its confined mobility property based on subscription information

RRM relaxations that are the actions proposed in order to reduce power consumption:

- Stop measurements on neighbour cells for T (T>>1) hours

- Further relaxation by reducing the number of the monitored reference signals (RS). UE only needs to measure specific beams.

- Only perform measurements on a number of dedicated intra-frequency, inter-frequency cells.

- Minimize the number of measured frequencies

- Expand the Rel-16 scenario of performing "stop measurements for 1 hour" for stationary UEs

- Upon UE fulfils the criterion (i.e. RSRP threshold evaluation), UE can trigger measurement relaxation on part of configured frequencies

#### 2.1.2.1	Trigger conditions for applying relaxed RRM measurements

In TR 38.875 [3] there are different enhancements suggesting adding other triggers than those specified for RRM relaxation in NR Rel-16. For example, in Enhancement 1, the proposal is to add one additional “mobility state”, i.e. more than the 3 defined in TS 38.304 [5]. Such additional states come at a cost and the benefit of introducing additional states needs to be carefully evaluated:

- For each mobility state the device has to measure for a while to be able to determine that the device really is in the assumed mobility state, or rely on older measurements. Using old measurements may result in inaccurate determination of the state.

- RSRP measurements may change not only because device mobility but due to changes in radio conditions, and depending on the configuration, reliably determining whether the UE is “stationary” may not be possible.

- Increasing the number of states may mean the mobility state is changed more often, and as a result the UE moving in and out of relaxed measurements more frequently, possibly leading to increased amount of necessary measurements and evaluations of the conditions.

- Additional states increase the implementation complexity and testing requirements.

- In practice it may be difficult to properly configure RSRP change thresholds for multiple states to e.g. reliably differentiate between “low mobility” and “stationary” state.

Additionally, the existing “low mobility” threshold is configurable, thus it is not clear what additional gain would be achieved by adding yet another mobility state. In worst case if the finer-grained mobility state is not correctly evaluated, the UE might see increased power consumption, contrary to what is the intention with RRM relaxation.

It is already possible to configure low mobility criterion in Rel-16. Addition of more “mobility-states” risk increasing number of mobility state changes and UE moving in and out from relaxed monitoring. Addition of more mobility states needs to be carefully considered.

Another proposal in the TR, Enhancement 2, is to add a new mobility state (stationary) and another timer for the measurements (TSearchDeltaP_stationary), only to be used in low mobility state to determine that a device is stationary.

A shorter time for classification of mobility state may result in misclassifications or to more frequent changes of states, while a longer time may lead to increased power consumption. Also, as mentioned above, finer defined mobility states may result in that the device being moved in and out of relaxation more often. This may in worst case increase the measurements time (on neigboring cells).

Adding a new timer and either reducing or increasing evaluation time (TSearchDeltaP) may lead to increased power consumption if the change reduces the time the device may sleep.

Enhancement 3 proposes to use beams to evaluate mobility. This is potentially a good enhancement since, e.g. the rate of beam switches provides information on mobility. For this to work there has to be many beams, i.e. assume FR2 frequencies, and it works best when the device travels in a direction across the beam.

Beam-related methods may provide faster determination UE’s mobility and potentially help reduce power consumption (but only in FR2).

Enhancement 4 where the gNB receives subscription information stating that the device is stationary may sound like a reasonable way forward. However, even if there is subscription information it is likely that the device would have to provide some sort of proof that it is really stationary. This determination would require measurements which may cost as much, from a power consumption perspective, as any of the others.

Subscription information is not a realiable method to obtain the location and mobility status of a device, since it is likely that the device must provide some proof that it is in fact stationary.

Consider enhancements that may result in faster determination of whether the UE is stationary, e.g. enhancement 3.

#### 2.1.2.2	How to relax RRM measurements

One proposed solution that supposedly reduces power consumption is to allow longer time for relaxation than what is standardized in NR Rel-16. The evaluations provided in Figure 2 which suggests there is little reduction of power consumption by extending the relaxation beyond one hour. However, one of the results provided in the TR [4] actually sees some gain (probably for FR2) compared to Rel-16 by configuring measurements so that not all SSBs are included and in that way reducing the time needed.

Relaxation beyond what is possible in Rel-16 seems to provide little additional power saving.

Enhancements that propose limiting the scope of measurements, e.g. B, D, E and F, may provide some gain compared to the baseline, however, some of the proposals risk affecting the accuracy of measurements. A gain vs complexity analysis of these enhancements should provide significant gain in order to be of any use for a real implementation.

Reducing the scope of RRM measurements, e.g. limiting frequencies to measure on, while keeping Rel-16 configuration, may help reduce power consumption.

Analyze the different proposals on reducing the scope of measurements, e.g. enhancements b, d, e and f, to determine if any solution for RRC_IDLE/INACTIVE provides large enough gain compared to Rel-16 while neither adding too much complexity nor affecting the performance.

## 2.2.	Relaxation for devices in RRC_CONNECTED.

There is one set of results in the TR with an attempt to analyze how relaxation affects power consumption and it shows that by relaxing measurements, in a similar manner as in the RRC_IDLE/INACTIVE case, power consumption is reduced. However, there is also a slight increase in handover failures. These results seem reasonable, in general. This since, in RRC_CONNECTED  there are more frequent PDCCH and PDSCH receptions than in RRC_IDLE/INACTIVE. Power consumption is dominated by PDCCH and PDSCH procedures. This means that there will be power saving from RRM measurements on neighbor cells, however, it is comparatively limited. Furthermore, relaxing RRM measurements in RRC_CONNECTED may lead to DL performance degradation etc. if the best neighbor cells  are not selected on time. This is one of the reasons that the solutions that RAN2 agree on needs to be checked by RAN4.

RRM Relaxation of devices in RRC_CONNECTED was discussed briefly in the SI  and the outcome of the discussion is presented in the TR  [4].

“RRM relaxation has been studied for all the RRC states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED) and both for neighbour cell and for serving cell measurements.

-	For RRC_CONNECTED, it is recommended that UEs which are fixed or immobile are considered with higher priority compared to UEs which are slightly moving.”

The discussion also resulted in a number of solutions that trigger relaxed RRM:

Solution 1: The UE reports "stationary" status to network in Msg5.

Solution 2: The network provides (e.g. low mobility, not-at-cell-edge) evaluation parameters to the UE via dedicated signalling.

Solution 3: The AMF sends a "stationary" indication to the gNB (based on user subscription).

Solution 4: The UE reports "stationary" in UE Assistance Information to the network.

Solution 5: The network enables measurement relaxation based on the UE's measurement report.

The idea with trigger solutions provided is similar to the ones discussed for low mobility devices in RRC_IDLE/INACTIVE, namely quick determination of when RRM relaxation can be applied in the device. In general, the solutions for RRC_CONNECTED devices focusses less on measured mobility states.

An important difference from RRC/IDLE/INACTIVE, not limited to the one on solutions, is that for RRC_CONNECTED mobility status and trigger criteria can be exchanged with dedicated signaling, and not only via broadcast. Thus there is much more possibilitites for the gNB to control the use of RRM relaxation.

Out of the presented solutions, solution 4 provides some promise. The network can estimate the device mobility state based on UL measurements e.g. from RNA change rate in idle, or from CSI reports, and channel information, as well as the device location. However, the UE typically has more information about its mobility status. Therefore, it may be beneficial to have the device provide this information to the network. Further, the device may indicate a preference for a certain measurement configuration, e.g. less frequent measurements. Note that the network should remain in control of both starting and stoping relaxation as well as the measurement configuration.

If a connected mode solution is deemed necessary, investigate if solution 4 can provide relevant (e.g. up-to-date) and reliable information useful for the network to decide if the gNB should configure relaxed RRM measurements.

Any relaxation for RRC_CONNECTED devices and the device behavior during relaxation should be directly controlled by the network, in order for the NW to maintain/guarantee acceptable system performance.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The maximum relaxation of one hour is allowed in Rel-16 for a low-mobility device not at cell edge and provides a significant reduction of relative power consumption.

Observation 2	An RRM measurement relaxation of 25 h, compared to the maximum relaxation of 1 h as of Rel-16 UE power saving, provides a negligible gain.

Observation 3	It is already possible to configure low mobility criterion in Rel-16. Addition of more “mobility-states” risk increasing number of mobility state changes and UE moving in and out from relaxed monitoring. Addition of more mobility states needs to be carefully considered.

Observation 4	Adding a new timer and either reducing or increasing evaluation time (TSearchDeltaP) may lead to increased power consumption if the change reduces the time the device may sleep.

Observation 5	Beam-related methods may provide faster determination UE’s mobility and potentially help reduce power consumption (but only in FR2).

Observation 6	Subscription information is not a realiable method to obtain the location and mobility status of a device, since it is likely that the device must provide some proof that it is in fact stationary.

Observation 7	Relaxation beyond what is possible in Rel-16 seems to provide little additional power saving.

Observation 8	Reducing the scope of RRM measurements, e.g. limiting frequencies to measure on, while keeping Rel-16 configuration, may help reduce power consumption.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider enhancements that may result in faster determination of whether the UE is stationary, e.g. enhancement 3.

Proposal 2	Analyze the different proposals on reducing the scope of measurements, e.g. enhancements b, d, e and f, to determine if any solution for RRC_IDLE/INACTIVE provides large enough gain compared to Rel-16 while neither adding too much complexity nor affecting the performance.

Proposal 3	If a connected mode solution is deemed necessary, investigate if solution 4 can provide relevant (e.g. up-to-date) and reliable information useful for the network to decide if the gNB should configure relaxed RRM measurements.

Proposal 4	Any relaxation for RRC_CONNECTED devices and the device behavior during relaxation should be directly controlled by the network, in order for the NW to maintain/guarantee acceptable system performance.

# 4	References

RP-210918, “Revised WID on support of reduced capability NR devices”, Nokia, Ericsson, RAN#91e, Online, March 2021.

R2-2009620: "RedCap power saving enhancements", Ericsson, RAN2#112-e.

TR 38.840 Study on User Equipment (UE) power saving in NR (Release 16). 3rd Generation Partnership Project;Technical Specification Group Radio Access Network. V16.0.0 (2019-06).

TR 38.875 Study on support of reduced capability NR devices (Release 17). 3rd Generation Partnership Project;Technical Specification Group Radio Access Network. V2.0.0 (2021-03).

TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 16). 3rd Generation Partnership Project;Technical Specification Group Radio Access Network. V16.4.0 (2021-03).

R1-2009202, “High-level UE energy consumption profiling”, Ericsson, RAN1#103-e.