3GPP TSG-RAN WG2 Meeting #113Bis Electronic	R2-2103017

Online, April 12 – April 20, 2021

Agenda Item:	8.3.3

Source:	CableLabs

Title:	Discussions on various ways to support various of leave scenarios and procedures for Multi-SIM UEs

Document for:	Discussion, Decision

# Introduction

One of the objectives of Multi-SIM WID [1] is:

- Mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose): - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

On this objective, recent RAN2 113e had reached the following agreements.

Agreements

1	Switching procedure can be used to notify network A that the UE has a preference to leave RRC_CONNECTED state in network A.

2	The switching procedure can be used to notify network A that the UE has a preference to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

These agreements, however, still have more details to be worked out and agreed. This contribution reviews various of idle/inactive procedures the UE executes, provides observations and proposes some way forward.

# The Various Needs to Leave and Their Procedures

Under the applicable architecture in the WID, if the UE is in active state on network A, the state of the UE in network B can be either idle or inactive. While the UE is in either idle or inactive state with network B, there are several idle/inactive procedures the UE needs to leave network A and execute with network B. These are

- Measurement of the current cell RF conditions and execute cell-reselection when re-selection conditions are met. The cell re-selection could also result in TAU/RNA update procedure.

- Page reception and respond to page if necessary

- Reception of system information change indication and respond to system information change indication(System information re-acquisition).

- Periodic Tracking Area Update,/RNA update

# Characteristics of These Idle/Inactive Mode Procedures

The current cell measurement, system information change indication reception, and paging reception procedure are largely periodic in nature. For TAU, RNAU, however, they have both periodic and event-drive flavors. The need for system information re-acquisition is event-drive which only be needed when the system information change indication indicates update is required. The event-drive aspect of the TAU/RNAU is that the condition for update is met, e.g. different tracking area; therefore, triggers TAU/RNAU procedure. Another aspect of the event driven procedure is that the UE could be responding to a page message, which leads to procedures, e.g. voice session setup, with network B.

# Support for the Periodic Procedures

The nature of the periodic procedure is that the procedure be executed in periodically fashion and that the state be ideally maintained once the procedure is completed. As such, the system should be able to configure the period, the execution duration, and expect the state be maintained once the periodic procedure completes its execution.

The period of the paging reception, current cell measurement, system information change indication reception, and periodic TAU/RNAU can largely accommodated via the periodicity of the paging reception. However, the time needed for execute of these procedures varies, i.e. the time needed to execute paging reception is different from the time needed for executing TAU/RNAU, for example. Also, even though the periodicity of the TAU/RNAU can be covered by the periodicity of the paging reception, they are really on different magnitude in that the paging periodicity are mostly in the neighborhood of seconds while the periodicity of TAU/RNAU are in the neighborhood of minutes.

To be able to support periodic procedures with different execution duration requirements with network B while optimize the network A performance, the network A could support multiple periodic “leaves”.   From the connection state’s perspective, since the execution of these periodic procedure can be as long as 10 ms or slightly longer[4], it would be advantageous that the network A still keeps the UE in the connected state while the UE executes these periodic procedures with network B to minimize the performance impact to the network A.

The execution duration varies between different periodic idle/inactive procedures.

Multiple periodic leave configurations to accommodate these idle/inactive procedures can optimize network performance.

The longest of these periodic procedures could take nominally 10 ms or slightly more to complete.

it might be advantageous for the network A to keep UE in connected state while UE executes these periodic procedures with network B.

From a different perspective, these periodic procedures could trigger either an event-driven periodic procedure, as in system information re-acquisition case, or an one-shot event-driven procedure, as in event-driven TAU/RNAU case or event-driven voice session case, for example.

# Support for the Event-Driven Procedures

To satisfy event-driven system information re-acquisition procedure, since the periodicity of the system information acquisition procedure is much different and smaller from the paging procedure, it would be advantageous to configure another temporary periodic procedure to support such need.

As for the event-driven TAU/RNAU procedure, which is a one-time procedure and takes on the order of 10 ms, to be executed with network B, it would be advantageous to support a one time leave procedure which transitions the UE connection to inactive state in network A to facilitate quick transit back to connected state when UE returns. Also, as discussed in the previous section, the periodicity of the TAU/RANU is much longer than the paging periodicity, if configurating the measurement gap type of leave is not desirable for the periodic TAU/RANU, the alternative would be to support these periodic TAU/RANU via one time leave procedure and transition the UE state to inactive while the UE is absent.

The other event-driven procedure, such as voice paging from network B, could result in setting up a voice session on network B or sending busy indication on network B. For the voice session setup on network B case, it could take much longer time to complete. Therefore, it might be advantageous for network A to support one time procedure which result in UE transition to idle state. Optionally, the network A could transition the UE to inactive state to support the leave. Transition the UE state to idle could take in a matter of few ms longer than transition the UE state to inactive. The delay difference, in comparison to the timer (T3513) the AMF uses to wait for service request in response to paging, is insignificant. In fact, the delay difference is even much smaller compared the paging cycle delay which would nominally be on the order of hundreds of ms.

Transition the UE state to idle can be achieved via NAS procedure and the procedure is more widely supported. Transition the UE state to inactive state can be achieved through RRC procedure in 5G if further optimization over NAS procedure is desired. Furthermore, in order to avoid either core network or RAN to run into UE non-reachable status, the UE can either inform the core network via NAS or the RAN via RRC to suspend the non-reachable procedure. Alternative, the UE could setup “measurement gap” like  periodic leave to come back to network A to receive page while in network B.

There might be advantages when it comes to restore the UE state to connected from idle versus inactive state(~10 ms versus few ms).

As for the busy indication case, it should be treated as a one time leave much like the one time TAU/RANU procedure.

Event-driven periodic procedure may not be adequately supported by the paging cycle.

Network A can support temporary periodic leave while keeping the UE in connected state. Network A can cancel the temporary periodic leave once the task is completed.

There are event-driven one time procedures that can take around 10 ms to complete

For the event-driven one-time procedure that take around 10 ms to complete, the network A should support a one time leave procedure and transition the UE to inactive state.

If supporting periodic TAU/RNAU via proposal 1 is not desirable, these periodic TAU/RNAU can be supported via proposal 4.

There are event-driven procedure that could take on the order of minutes to complete.

For event-driven one-time procedure which is expected to take longer time to complete, as a baseline, an one time NAS procedure that transition the UE to idle state should be supported by network A. Optionally, the network A could transition the UE to inactive state via 5G RRC procedure if the optimization is desirable.

The core network or RAN could run into UE non-reachable after UE switches to other network(network B).

The UE should inform core network, if NAS procedure was used to transition UE state to idle, or RAN, if RRC procedure was use to transition the UE to inactive state, that the non-reachable procedure be suspended. The UE should re-enable the non-reachable procedure when it comes back to network A. The core network or RAN should also have error handling procedure should the UE fails to come back to re-enable the non-reachable procedure.

Alternative to suspend non-reachable procedure, the UE could setup periodic leave on network B to come back to network A for paging reception.

# Conclusion

This contribution provides the following observations and proposals.  The author invites discussions and conclusions on these observations and proposals.

Observations 1.The execution duration varies between different periodic idle/inactive procedures.

Observations 2.The longest of these periodic procedures could take nominally 10 ms or slightly more to complete.

Observations 3.Event-driven periodic procedure may not be adequately supported by the paging cycle.

Observations 4.There are event-driven one time procedures that can take around 10 ms to complete

Observations 5.There are event-driven procedure that could take on the order of minutes to complete.

Observations 6.The core network or RAN could run into UE non-reachable after UE switches to other network(network B).

Proposals 1.Multiple periodic leave configurations to accommodate these idle/inactive procedures can optimize network performance.

Proposals 2.it might be advantageous for the network A to keep UE in connected state while UE executes these periodic procedures with network B.

Proposals 3.Network A can support temporary periodic leave while keeping the UE in connected state. Network A can cancel the temporary periodic leave once the task is completed.

Proposals 4.For the event-driven one-time procedure that take around 10 ms to complete, the network A should support a one time leave procedure and transition the UE to inactive state.

Proposals 5.If supporting periodic TAU/RNAU via proposal 1 is not desirable, these periodic TAU/RNAU can be supported via proposal 4.

Proposals 6.For event-driven one-time procedure which is expected to take longer time to complete, as a baseline, an one time NAS procedure that transition the UE to idle state should be supported by network A. Optionally, the network A could transition the UE to inactive state via 5G RRC procedure if the optimization is desirable.

Proposals 7.The UE should inform core network, if NAS procedure was used to transition UE state to idle, or RAN, if RRC procedure was use to transition the UE to inactive state, that the non-reachable procedure be suspended. The UE should re-enable the non-reachable procedure when it comes back to network A. The core network or RAN should also have error handling procedure should the UE fails to come back to re-enable the non-reachable procedure.

Proposals 8.Alternative to suspend non-reachable procedure, the UE could setup periodic leave on network B to come back to network A for paging reception.

.

# References

- RP-201309	Support for Multi-SIM devices for LTE/NR

- R2-2101950	Report from session on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

- TR 23.761 	Study on system enablers for devices having MUSIM

- R2-2011241 Reply LS on System support for Multi-USIM devices