3GPP TSG-RAN WG2 Meeting #113bis electronic                     R2-2103035

e-meeting, 12th – 20th April 2021

Source: 			ZTE Corporation, Sanechips,

Title: 	Activation and deactivation of SCG

Agenda item:		8.2.2.3

Document for: 	Discussion and Decision

- Introduction

After RAN2_113e meeting, RAN2 has made following agreements on procedure of SCG activation and deactivation:

Agreements

1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

1b 	SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

3 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

In this document, we further discussed the FFS points of SCG activation and deactivation procedures, and provided our views.

- Progress in RAN3

In the past RAN3 meetings, RAN3 mainly focus on the discussion of MN-initiated SCG deactivation/activation procedures. According to previous RAN2 agreements, MN can trigger SCG (de)activation during both “SN Addition” and “SN Modification” procedures.

In addition, RAN3 agreed to reuse existing “SN Addition Request/Response” and “SN Modification Request/Response” messages with some updates, in order to support MN triggered SCG (de)activation. The detail agreements are pasted below:

RAN3_111e agreements: Xn interface: MN initiated SN addition procedure: Add a new IE in the SN addition request message to indicate at least the de-activation, while the detail code of this new IE is FFS. Add a new IE in the SN addition response message to indicate at least the de-activation result, while the detail code of this new IE is FFS. MN initiated SN modification procedure: Add a new IE, e.g., “SCG activation requested” with two codepoints in the SN modification request message in order to indicate the SCG is requested to activate or de-activate. Add a new IE, e.g., “SCG activation result” with two codepoints in the SN modification response message in order to indicate the SCG is activated or de-activated.

As we see, in response message, explicit IEs will be defined for SN to indicate the result of SCG (de)activation. Take SN Addition procedure as an example, if MN indicates “SCG deactivation” in SN Addition Request message, but the SN cannot accommodate the SCG deactivation request, the SN is not required to reject the entire SN addition procedure. Instead, the SN can finish establishing the SCG, and inform the MN that SCG is activated in SN addition response message. The motivation is to avoid SN addition/modification failure just because of undesired SCG (de)activation. Therefore, for MN triggered SCG (de)activation, partial rejection is supported.

Observation 1: Based on RAN3’s agreements, no matter MN triggers SCG (de)activation through SN Addition or SN Modification procedure, SN can reject the SCG (de)activation request without rejecting the whole procedure (e.g. partial rejection is supported for MN triggered SCG (de)activation).

However, SN triggered or UE triggered SCG (de)activation haven’t been discussed in RAN3, mainly because RAN3 is waiting for RAN2’s guidance. Although it is agreed in RAN3 that MN can trigger SCG (de)activation based on the Activity Notification sent by SN, so far, it hasn’t been regarded as “SN triggered SCG (de)activation” procedure, because this message is mainly used to indicate the data activity in SN node. After receiving the message, MN can do a variety of operations: e.g. SN release, SCG suspend and release UE to RRC_INACTIVE, bearer type change, not only SCG deactivation.

Observation 2: RAN3 haven’t conclude the signalling procedure for SN-triggered SCG (de)activation, because they’re waiting for RAN2’s guidance.

- Discussion

As mentioned above, for MN triggered SCG (de)activation, RAN3 have agreed it can be done during SN addition or MN-initiated SN Modification procedures, and SN can reject the SCG (de)activation request without rejecting the whole procedure.

Therefore, in this document, we mainly discussed the SN triggered and UE triggered SCG (de)activation procedures. And we understand RAN2 can focus on the mechanism of UE triggered SCG (de)activation, because Uu interface is involved. For SN triggered SCG (de)activation, it involves MN/SN interaction. So in our understanding, RAN2 can discuss the solutions, e.g. whether/how SN can trigger the SCG (de)activation, whether MN can reject it, but the detail design can be left to RAN3.

Considering RAN3’s next meeting is in May 2021, it is necessary to send RAN2’s conclusions to RAN3 after this meeting.

Observation 3: RAN2 can focus on discussing and defining UE triggered SCG (de)activation. For MN/SN triggered SCG (de)activation procedure, RAN2 can discuss and provide guidance to RAN3, but it is up to RAN3 to make decision on the signalling details (e.g. how to reject, code point of IE, need of new cause value).

  - Activation of SCG triggered by SN

SN may trigger SCG activation when DL data arrives for SN terminated SCG bearer, the throughput for SN terminated split bearer increases. As we analyzed in [1], if SN needs to first send Activity Notification message to MN, then let MN to decide SCG activation and send SN Modification Request message to SN, the latency can be 20ms more than SN directly triggers SN Modification Required message.

There was a discussion in RAN3 on whether to enhance Activity Notification message (i.e. adding SCG activation/deactivation indications). However, from RAN2 point of view, SCG activation/deactivation may be performed together with radio configuration update (e.g. allocate PUCCH/SRS resources), so simple enhancement of Activity Notification is not sufficient for this purpose. In addition, there is no response message defined for Activity Notification, in case SN uses Activity Notification to indicate SCG activation request, it is unclear how does SN know if the request is accepted by SN or not, unless new procedure is defined.

Observation 4: For SN triggered SCG activation procedure, using Activity Notification method will cause more latency (e.g. 20ms) than using SN-initiated SN modification procedure.

In our understanding, to ensure user experience, low latency should be the key point when discussing the transition between SCG deactivation and SCG activation. Thus, we suggest to allow SN to trigger SCG activation via SN-initiated procedure, an example of signalling flow is shown in below figure:

Figure 1 SN triggered SCG activation

Proposal 1: 	SN can trigger SCG activation via SN initiated SN Modification procedure, and MN can accept or reject it.

From MN perspective, after receiving then SN Modification Required message contains SCG activation request. Seems there is no convincing reason for MN to reject the request, because the SN may initiate the procedure due to data arrives at SN terminated PDCP. However, MN is main control node in MR-DC. Based on RAN3 spec, so far, MN is allowed to reject any procedure initiated by SN, except SN-initiated SN release.

So, for consistency, we see no harm to allow MN to reject the SCG activation initiated by SN. As MN can reject the request, and triggers bearer type change instead. However, different from MN triggered SCG (de)activation procedure, we think there is no need to support “partial rejection”, because the SN RRCReconfiguration container may be generated for SCG activation purpose (e.g. including new configured radio resources or RRM configuration), delivering the SN RRC message to UE but keeping UE in SCG deactivation may cause resource waste and increased UE power consumption.

Therefore, if MN cannot accommodate the SCG activation request, MN should reject the whole procedure (sends SN Modification Refuse). But considering signalling interaction is mainly RAN3’s work, we suggest to capture it as a “RAN2’s understanding”, so the final decision is still up to RAN3.

Proposal 2: 	RAN2 understand if MN cannot accept the SCG activation request from SN, MN should trigger SN Modification Refuse (i.e. partial rejection is not supported). It is up to RAN3 to make final decision.

  - Deactivation of SCG triggered by SN

SN may trigger SCG deactivation upon low data activity of SN terminated bearer, compare to transition from SCG deactivation to activation, the need of low latency for SCG deactivation is not so urgent. Which means the SN may reuse Activity Notification to inform MN the low data activity in SCG. However, this implies it is up to MN to decide whether SCG deactivation or SN release will be performed, regardless of the expectation from SN.

If SN-initiated SN modification procedure is adopted for SN triggered SCG activation, it is quite easy to extend it to support SN initiated SCG deactivation. In any case, the latency of SCG deactivation can be reduced, so UE can enter “power saving” mode earlier.

Thus we suggest to adopt the same signalling design for SN triggered SCG activation and deactivation. And same as in SCG activation, MN can accept or reject the SCG deactivation request, and MN should reject the whole procedure if cannot accept the request.

Proposal 3: 	SN can trigger SCG deactivation via SN initiated SN Modification procedure, and MN can accept or reject it.

Proposal 4: 	RAN2 understand if MN cannot accept the SCG deactivation request from SN, MN should response SN Modification Refuse (i.e. partial rejection is not supported). It is up to RAN3 to make final decision.

  - Activation of SCG triggered by UE

After RAN2_113e, RAN2 agreed that SCG activation can be requested by UE, but there is no conclusion on the signalling procedure, e.g. whether the “request” will be sent to MN or SN? In general, the UE may trigger SCG activation when there is UL data arrives on SCG bearer, since RAN2 concluded there is no UL transmission over SCG when SCG is deactivated, so UE has to request network to re-activate SCG.

During the offline discussion last meeting, companies discussed the support of SCG bearer in SCG deactivation state, and following remaining issue is listed in RAN2 agreements:

8	Further discuss the comparison between

-  define a mechanism for SCG activation upon UL data arrival on SCG bearers

-	use split bearer with primary path on MCG (network sees UL data and can initiate activation)

The second bullet means we will not support SCG bearer when SCG is deactivated. If a UE was configured with SCG bearers (no matter the PDCP anchor is located at MN or SN), then upon SCG deactivation, network should trigger bearer type change, in order to change the SCG bearer(s) into split bearers, and set MCG path as primary path. When UE has UL data, the UE can transmit the data via MCG leg. As long as network (MN) receives UL data via MCG path of the split bearer, network (MN) can trigger SCG activation.

In our view, at least using split bearer to replace SCG bearer has following drawbacks:

- Additional restriction to network implementation.

  - Usually network determines bearer type based on service type and the resource in network. Disallow SCG bearer in SCG deactivation implies network has to trigger bearer type change before SCG deactivation and after SCG activation.

- Frequent PDCP reconfiguration/recovery:

  - According to TS 37.340, bearer type change from(to) SCG bearer to(from) split bearer, requires PDCP reconfigure/recovery, thus if SCG deactivatipn/activation is triggered frequently, UE also need to do PDCP reconfigure/recovery frequently, causes complexity in UE.

- Complex MN implementation:

  - From UE perspective, UE may already be configured with split bearer (before SCG deactivation). For these “regular” split bearer, MN does not need to trigger SCG activation when receives the UL data is received via MCG path. So when SCG is deactivated, MN has to “remember” which split bearer(s) are regular split bearer(s), which are transformed from SCG bearer due to SCG deactivation, and treat them differently. Not only this complicate MN’s implementation, but also such information will be lost after inter-MN handover.

Observation 5: Reply on “split bearer” (disallow SCG bearer) when SCG is deactivated have several drawbacks (see above).

So, we suggest to support UE be configured with SCG bearer when SCG is deactivated.

Proposal 5: 	UE can be configured with SCG bearer when SCG is deactivated.

If SCG bearer is supported, RAN2 needs to define mechanism for SCG activation for UL data arrives on SCG bearer. During offline discussion [2], there are two alternative solutions:

- Solution 1: The UE sends an indication via the MCG then the network can indicate SCG activation via the MCG;

- Solution 2: The UE sends an indication to the SCG (random access or PUCCH)

For solution 1, as we analyzed in [1], if UE has to first send indication to MN, let MN to request SN to activate SCG, and then sends SCG activation command to UE after receiving SN’s response, the latency could be even larger than blind SN addition. So, although solution 1 looks straightforward, and probably with less specification effort, it does not fulfill the requirement of fast SCG activation. While for solution 2, since UE can inform SN about SCG activation directly, the latency of SCG activation can be largely reduced. So we suggest to further study and specify it. In our understanding, the simplest way is to perform RACH (CBRA or CFRA) towards PSCell. In addition, it can be configurable by network, once network enable this function then network cannot reject the request.

Proposal 6: 	Support UE to trigger SCG activation by performing RACH towards PSCell. It is configurable and network cannot reject the procedure if configured.

  - Deactivation of SCG triggered by UE

During offline discussion last meeting, companies also discussed the mechanism of UE triggered SCG deactivation, and following FFS is added to the chairman notes.

11	It is FFS whether the UE can provide some assistance information for deactivation of the SCG (but there is no proposal so far).

In our understanding, UE may want to deactivate SCG for multiple reasons: 1) high power consumption or overheating; 2) short battery life. So the motivation is similar to UAI mechanism.

Looking into the definition of UE Assistance Information defined in TS 38.331, currently, following indication are introduced:

# reduced bandwidth#

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

# reduced CC number#

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

# preferred RRC state#

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

In case of NR-DC, since UAI is sent to MN, and MN is responsible to do “split” between MCG and SCG, it seems impossible for UE to clearly indicate the preference of SCG deactivation through above fields. In addition, for (NG)EN-DC, although UE can transmit an independent UAI for SCG (e.g. contains 0MHz bandwidth, or zero CC), it is hard to say SN will trigger SN deactivation instead of SN release.

If we want to support UE to indicate network the preference of SCG deactivation, the simplest way is to introduce one explicit indication in UAI message, similar to preferredRRC-State-r16.

Proposal 7: 	Enhance UE Assistance Information message, to support UE to inform network the preference of SCG deactivation.

  - Other remaining issues

For SCG deactivation, based on the agreement made in RAN2_112e: “SCG activation state (activated/deactivated) can be configured at PSCell addition/change, RRC resume or HO”. However, there are two remaining issues related to the signalling of SCG deactivation:

- Issue 1: When network triggers PSCell change while the SCG is deactivated, whether the UE does RACH towards target cell?

- Issue 2: Whether SCell can be added/reconfigured/released while the SCG is deactivated?

For issue 1, it is up to network whether to keep the SCG in deactivated state upon PSCell change. In case UE’s data throughput does not change much, the network may expect UE to keep SCG in deactivated state. Considering there is no date transmission requirement when SCG is deactivated state, RACH procedure seems not necessary. From UE perspective, it can simply store the received SCG configuration, and perform RACH procedure only when SCG is activated.

Proposal 8: 	When UE is configured to keep SCG in deactivation state upon PSCell change, UE can just store the configured SCG configuration without performing RACH procedure towards target PSCell.

For issue 2, based on RAN2 agreements, UE can continue performing SN configured RRM measurement when SCG is deactivated. If the UE is already configured with SCG SCell, then at least network can trigger PSCell change when UE moves outside the coverage of PSCell, the motivation is to ensure UE can resume as soon as possible. Based on this, it is nature to also allow network to update the configuration of SCG SCell. For instance, when UE moves outside the coverage of one SCG SCell, network can release that SCell, so when UE triggers RRC Resume, network won’t activate an invalid SCell for data transmission.

So it make sense to support updating the SCG SCell configuration when SCG is deactivated. And this behaviour should not cause any state change on SCell.

Proposal 9: 	SCG SCell can be added/reconfigured/released when SCG is in deactivated. In this case, the SCell maintains its state (remains in deactivated state).

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation 1: Based on RAN3’s agreements, no matter MN triggers SCG (de)activation through SN Addition or SN Modification procedure, SN can reject the SCG (de)activation request without rejecting the whole procedure (e.g. partial rejection is supported for MN triggered SCG (de)activation).

Observation 2: RAN3 haven’t conclude the signalling procedure for SN-triggered SCG (de)activation, because they’re waiting for RAN2’s guidance.

Observation 3: RAN2 can focus on discussing and defining UE triggered SCG (de)activation. For MN/SN triggered SCG (de)activation procedure, RAN2 can discuss and provide guidance to RAN3, but it is up to RAN3 to make decision on the signalling details (e.g. how to reject, code point of IE, need of new cause value).

Observation 4: For SN triggered SCG activation procedure, using Activity Notification method will cause more latency (e.g. 20ms) than using SN-initiated SN modification procedure.

Observation 5: Reply on “split bearer” (disallow SCG bearer) when SCG is deactivated have several drawbacks (see above).

Proposal 1: 	SN can trigger SCG activation via SN initiated SN Modification procedure, and MN can accept or reject it.

Proposal 2:	RAN2 understand if MN cannot accept the SCG activation request from SN, MN should trigger SN Modification Refuse (i.e. partial rejection is not supported). It is up to RAN3 to make final decision.

Proposal 3: 	SN can trigger SCG deactivation via SN initiated SN Modification procedure, and MN can accept or reject it.

Proposal 4: 	RAN2 understand if MN cannot accept the SCG deactivation request from SN, MN should response SN Modification Refuse (i.e. partial rejection is not supported). It is up to RAN3 to make final decision.

Proposal 5: 	UE can be configured with SCG bearer when SCG is deactivated.

Proposal 6: 	Support UE to trigger SCG activation by performing RACH towards PSCell. It is configurable and network cannot reject the procedure if configured.

Proposal 7: 	Enhance UE Assistance Information message, to support UE to inform network the preference of SCG deactivation.

Proposal 8: 	When UE is configured to keep SCG in deactivation state upon PSCell change, UE can just store the configured SCG configuration without performing RACH procedure towards target PSCell.

Proposal 9: 	SCG SCell can be added/reconfigured/released when SCG is in deactivated. In this case, the SCell maintains its state (remains in deactivated state).

- Reference

[1] R2-2100568    Further consideration on SCG deactivation and activation   ZTE Corporation, Sanechips

[2] R2-2101969	Summary of [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation (Huawei)	Huawei	discussion	Rel-17	LTE_NR_DC_enh2-Core