3GPP TSG RAN WG2#113bis-e                                                               R2-2103100

Electronic, 12th April – 20th April, 2021

Source:	CATT

Title:	Further Consideration on Immediate MDT Enhancements

Agenda Item:	8.13.3.1

Document for:	Discussion and Decision

Introduction

For immediate MDT measurement, the M5/M6/M7 measurements following proposals were summarized in [1]:

Summary Proposal 1: It is proposed RAN2 to discuss whether to adopt option 1. If not, option 2 or 3 can be considered. Summary Proposal 2: For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer, configures the configuration to UE. Summary proposal 3: For the accuracy of the result, the M6 result with data marker (duplication indicator) could be sent to the OAM by CU.

In this contribution, we’d like to discuss the enhancements for M5/M6/M7 of immediate MDT in DC scenario.

Discussion

General Discussion

For the immediate MDT of M5/M6/M7, the recorded nodes and the entities are different:

⁻	M5: Average UE throughout measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.552

⁻	M6: Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.552 and TS 38.314

⁻	M7: Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.552 and TS 38.314

If the DC is applied, the configuration of immediate MDT of M5/M6/M7 should always be received by the MN and then send to the SN, therefore the coordination is needed for the configuration. After receiving the configuration, the gNB/UE will perform the measurement and collect the measurement result until each end of the measurement collection period. Then the measurement result could be reported to the TCE. If there is no QoS monitoring requirement, we consider the measurement result of different nodes does not to be merged, the measurement result could be reported directly from the network node in which collected it.

Based on the discussion in [1], we discuss the remaining issues for M6/M5/M7, respectively.

M6

The summarized options in [1] for split bearer (i.e. MN terminated split bearer, SN terminated split bearer) for D1are as following:

- Option 1: No differentiation and UE reports a single D1 to network. - Need differenation between MN and SN: Option 2: D1 measurement for MN terminated bearers is configured by and reported to MN. Vice versa, i.e. only the node hosting the PDCP entity configures the D1 measurement. UE reports two D1s to the node hosting the PDCP entity in one RRC message. Option 3: D1 measurement is configured by and reported to the node with lower layer configurations, i.e. MN and SN can independently configure the UE with D1 measurements in the split bearer. UE reports the D1 to each node.

For the D1 measurement, report a single D1 to the network cannot reflect the real UL packet delay of MN and SN for split bearer without duplication. The protocol architecture for split bearers from UE perspective is shown as following:

Figure 1 protocol architecture for split bearers from UE perspective

Assume the PDCP entity send M number of packets to MN and N number of packets to SN during time period 𝑇. The UL PDCP average delay of MN is calculated by dividing the total delay of all M number of packets by the M, similarly, D1 of SN side can be obtained.

MN: D1=i=0i=MdiM, di is the UL PDCP delay of MN packet i       (1)

SN: D1=j=0j=NdjN, dj is the UL PDCP delay of SN packet j       (2)

If a single D1 is used to report to network, the calculation of D1 may be as following:

D1= i=0i=Mdi+j=0j=NdjM+N           (3)

For an example to calculate the single D1 and D1 separately for MN and SN,

| The number of packets were sent to MN | M=10 |
| --- | --- |
| The number of packets were sent to SN | N=50 |
| The total delay of all M number of packets | 15ms |
| The total delay of all N number of packets | 10ms |

Using the equation (3) to calculate single D1, the D1 is 0.42ms. Using the equation (1) and equation (2) to calculate D1 respectively for MN and SN, the results are respectively 1.5ms and 0.2ms. The PDCP packet average delay for MN and SN are different, based on QoS verification of MDT requirement for SA5, the delay of multiple legs should be reflected in D1. Therefore, reporting the single D1 to the network cannot reflect the real UL PDCP average delay of MN and SN for split bearer without duplication. In order to correctly reflect the situation of MN and SN, the D1 respectively for MN and SN should be reported to the network.

Since the MN and the SN are both allowed to perform RRM measurement, the MN and SN can respectively configure the D1 measurements for different bearers. And also based on the RRM measurement, the UE will report the D1 result to the node which configured the measurement.

Observation 1: Report a single D1 to the network cannot reflect the real UL PDCP average delay of MN and SN for split bearer without duplication.

Proposal 1: UE reports M6 UL D1 results respectively to MN and SN based on the measurement configured by each node.

M5

M5 is Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, the reference is only TS 28.552.

From the current specification, the M5 measurement is calculated in RLC level. For the split bearer and MN/SN terminated SCG/MCG bearer, the PDCP layer cannot have the knowledge of the data volumes that are acknowledged by the UE. Therefore, PDCP layer cannot obtain the accurate UE throughput, the current M5 measurement calculation is enough for split bearer or for MN/SN terminated SCG/MCG bearer.

Observation 2: PDCP layer cannot obtain the accurate UE throughout measurement.

Proposal 2: It is proposed not to introduce M5 measurement at PDCP layer for split bearer or for MN/SN terminated SCG/MCG bearer.

For split bearer or for MN/SN terminated SCG/MCG bearer, the RLC throughput respectively of MN and SN can be reported to the TCE for calculating the overall throughput measurements.  Therefore, M5 will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

Proposal 3: Measurement of M5 will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

M7

M7 is Packet loss rate measurement separately for DL and UL, per DRB per UE, the reference of Packet loss rate about DL Uu transmission is TS 38.314, and the reference of Packet loss rate about UL/DL F1-U and about UL Uu transmission is TS 28.552. A table below illustrates all the possible Packet Loss Rates:

Table 1 Packet Loss Rate in UL/DL Uu or F1-U

| Packet Direction | Sub-Name of M7 | Involved packet loss Measurement | Involved Specification |
| --- | --- | --- | --- |
| DL | Packet Uu Loss Rate in the DL per DRB per UE | in Uu transmission | TS38.314 |
|  | DL F1-U Packet Loss Rate | Only in F1-U | TS28.552 |
| UL | UL PDCP SDU Loss Rate | in Uu transmission, in the gNB-CU and on the F1-U |  |
|  | UL F1-U Packet Loss Rate | Only in F1-U |  |

DL Packet Uu  Loss Rate in the DL per DRB per UE

For the DL packet Uu Loss Rate, the collection entity is RLC. For the split bearer or for MN/SN terminated SCG/MCG bearer, the PDCP layer cannot have the knowledge of which packets were lost and which packets were successfully transmitted. Therefore, PDCP layer cannot obtain the accurate DL Packet Uu Loss Rate

Observation 3: For the split bearer or for MN/SN terminated SCG/MCG bearer, PDCP layer cannot obtain the accurate DL Packet Uu Loss Rate.

Proposal 4: It is proposed not to introduce M7 measurement at PDCP layer for split bearer or for MN/SN terminated SCG/MCG bearer.

The DL packet Uu loss rate only related to the RLC SDU of RLC layer, it is described as the ratio of the lost RLC SDU and the total RLC SDU which have been transmitted over the air. The results of M7 respectively for MN and SN can be sent to TCE to obtain total DL Packet Uu Loss Rate. Since the related RLC SDUs to calculate M7 have been transmitted over the air, there is no difference between the split bearer and non-split bearer. Therefore, measurement of M7 packet loss rate of DL Uu transmission will not be impacted by DC scenario.

Proposal 5: Measurement of M7 packet loss rate of DL Uu transmission will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

UL PDCP SDU Packet Loss Rate

For the UL PDCP SDU Loss Rate, single leg bearer which also includes MN/SN terminated SCG/MCG bearer, the result can only be impacted by the channel quality of the specific leg; But

- For split bearer, the result maybe impact by the channel quality of the two paths. If one leg is much better than the other, the packet loss rate will be different compared with the only one leg scenario;

- For duplication case, the packet loss rate may be lower since the different legs could transmit the same packet which reduces the possibility of packet losing.

Therefore, for split bearer or for duplication case, a data marker could be introduced by CU to indicate whether the measurement result is collected in DC scenario or by packet duplication.

Proposal 6: For the accuracy of the result, measurement of M7 packet loss rate of UL PDCP SDU with data marker (like DC with split bearer indicator and/or duplication indicator) could be sent to the OAM by CU.

DL/UL F1-U Packet Loss Rate

For the DL/UL F1-U Packet Loss Rate, it only collects the packet loss of F1-U. And since this packet loss rates is the requirement of SA5, the results could be reported to the TCE by CU directly. DC scenario will have no impact on the recording and reporting of this measurement result.

Proposal 7: Measurement of M7 packet loss rate of DL/UL F1-U is performed by CU and it will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

Conclusion

According to the discussion in section 2, we propose:

For M6 measurement:

Observation 1: Report a single D1 to the network cannot reflect the real UL PDCP average delay of MN and SN for split bearer without duplication.

Proposal 1: UE reports M6 UL D1 results respectively to MN and SN based on the measurement configured by each node.

For M5 measurement:

Observation 2: PDCP layer cannot obtain the accurate UE throughout measurement.

Proposal 2: It is proposed not to introduce M5 measurement at PDCP layer for split bearer or for MN/SN terminated SCG/MCG bearer.

Proposal 3: Measurement of M5 will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

For M7 measurement:

DL Packet Uu  Loss Rate in the DL per DRB per UE

Observation 3: For the split bearer or for MN/SN terminated SCG/MCG bearer, PDCP layer cannot obtain the accurate DL Packet Uu Loss Rate.

Proposal 4: It is proposed not to introduce M7 measurement at PDCP layer for split bearer or for MN/SN terminated SCG/MCG bearer.

Proposal 5: Measurement of M7 packet loss rate of DL Uu transmission will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

UL PDCP SDU Packet Loss Rate

Proposal 6: For the accuracy of the result, measurement of M7 packet loss rate of UL PDCP SDU with data marker (like DC with split bearer indicator and/or duplication indicator) could be sent to the OAM by CU.

DL/UL F1-U Packet Loss Rate

Proposal 7: Measurement of M7 packet loss rate of DL/UL F1-U is performed by CU and it will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

Reference

[1] Report of [Post113-e][853][NR17 SON/MDT]  IMM MDT