3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103173

Electronic, 12th Apr – 20th Apr, 2021

Agenda Item:	8.15.3

Source: 	Huawei, HiSilicon

Title: 	On resource allocation enhancement in Rel-17 NR eSL

Document for:	Discussion and Decision

- Introduction

In this paper, we provide an initial discussion on the RAN2 related issues for mode 2 enhancement, including both inter-UE coordination part and power-saved resource allocation parts, and discuss the potential impacts from RAN2 perspective based on the latest RAN1 agreements reached in [1][2].

- Inter-UE coordination

In the RAN1#103-e meeting, the following agreements were achieved with respect to inter-UE coordination [1]:

Conclusion: - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B: - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission - e.g., based on its sensing result - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission - e.g., based on its sensing result and/or expected/potential resource conflict - UE-A sends to UE-B the set of resource where the resource conflict is detected - FFS: details of resource conflict, e.g., including type of resource conflict - FFS: details of sensing operation at UE-A side - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s) - Note: these different types may be used in combination with each other - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required - Send an LS to RAN plenary Conclusion: - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed. - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it - How UE-A and UE-B are determined - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

Moreover, in the RAN1#104-e meeting, the following agreements were further achieved [2]:

Conclusion: - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature. - The detailed observations can be found in the attachment of the LS Further discuss the detailed observations (starting from the FL’s summary) Agreements: Enclose following contents as an attachment of LS ============================================================================= RAN1 has studied and evaluated schemes of inter-UE coordination in the following categories: - Type A: UE-A sends to UE-B the set of resources preferred for UE-B’s transmission - e.g., based on its sensing result - Type B: UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission - e.g., based on its sensing result and/or expected/potential resource conflict - Type C: UE-A sends to UE-B the set of resources where the resource conflict is detected

Basically, the above RAN1 agreements mainly consist of some conclusions on what specific issues are to be further looked into by RAN1 and a confirmation on the feasibility/benefit of specification work, with regard to this topic. The rest of this section is discussed on top of the above RAN1 conclusions from a RAN2 perspective.

Supported scenarios

It is seen from the summary in RAN1 discussion [3] that the scenario to be supported for inter-UE coordination is an aspect that should be first determined and concluded. Although there has been no crystal-clear agreed scenarios so far, the key points under RAN1 discussions for the supported scenarios can be observed from [3]. As a generic framework shown in Fig.1, UE A signals “a set of resources” to UE B, where UE A is the coordinating UE and UE B is the coordinated UE. If we support inter-UE coordination to all cast types, additional signaling overhead for coordination message transfer may have to be introduced for groupcast or broadcast. This issue has been raised by some companies in RAN1 [3], as it is an important aspect related to the whole scheme design. Also, there were some discussions in RAN1 on whether the coordinating UE should be one of the two peer UEs involving in SL unicast communication, or it can be a third-party UE that exclusively performs coordinating functionalities w/o participation in the actual data communication in the unicast connection.

Since the supported scenario discussion has already been under the discussion of RAN1 w/o final conclusions draw so far, we think this discussion on what scenarios to be actually supported should be continued and eventually determined by RAN1. The aspects in terms of the supported scenarios can include, e.g. for what cast type(s) it is supported, whether the coordination is done by a UE as the third party, etc.

Figure 1: Framework for inter-UE coordination schemes

Proposal 1: The scenarios to be supported for inter-UE coordination is up to RAN1 (e.g. which cast type inter-UE coordination applies, whether there is a third-party coordinating UE, etc.).

Despite the supported scenarios remaining under RAN1 discussion, there seems one aspect that obviously need to be discussed from RAN2 perspective. Specifically, it was agreed by RAN1 that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 resource allocation. However, the UE B in above Figure 1 can be either in Mode 1 or Mode 2, whereas the “a set of resources” seems meaningless for UE B in Mode 1 as in this case the resource allocation of UE B is fully controlled by NW, w/o need for other UEs’ assistance from sidelink. Therefore, it seems necessary for UE A to know whether UE B is in Mode 1 or Mode 2, in particular for unicast case, so that UE A will only send “a set of resources” to UE B when UE B is in Mode 2.

Since Mode control for SL communication is a topic mainly governed by RAN2, we think this issue needs to be resolved by RAN2 as well. Specifically, RAN2 needs to discuss how the coordinating UE (i.e. the UE performing coordinating functionality) decides if another UE is in Mode 1 or Mode 2, thus deciding whether it can/need send the coordination information (e.g. “a set of resources” info) for Mode-2 to that UE.

Figure 2: Mode info negotiation for inter-UE coordinationProposal 2: RAN2 discusses how a coordinating UE (i.e. the UE performing coordinating functionality) decides whether another UE is in Mode-1 or Mode-2, in order to decide whether it can/need signal the Inter-UE coordination information (e.g. “a set of resources” info) to that UE.

Coordinating UE identification

From this subclause on, we discuss some more aspects that need potential RAN2 work, after RAN1 reaches substantial progress and firm conclusions on the inter-UE coordination design.

Different solutions are now under the RAN1 discussion as seen in [3]. However, irrespective of what specific scheme(s) RAN1 finally decides to support, a fundamental aspect that may need higher layer impacts is how to identify the coordinating UE, as it is straightforward that not every UE can be a coordinating UE. In general, the capability and authorization info may need to be considered for such identification. It is also seen that this aspect is included in the FFS list in RAN1 conclusions (e.g. “	How UE-A and UE-B are determined”) reached in RAN1#103e. Therefore, RAN2 can discuss this aspect based on the progress RAN1 is going to make.

Proposal 3: RAN2 may need to discuss how to identify a coordinating UE by taking into account the factors, e.g. capability, authorization info, etc., based on the supported inter-UE coordination scheme to be concluded by RAN1.

Signaling aspect on “a set of resources” information

Regarding which kind of signaling to carry such “a set of resources” info, RAN1 discussed this issue in the previous meetings, where at least the following ways can be considered:

- MAC message

- PC5-RRC signaling

- New 2nd-stage SCI format

Also, some companies proposed to use some other forms of PHY signaling [3], e.g. PSCCH, PSFCH, new PHY channel, etc. Technically speaking, this (which kink of signaling is used) should be decided by RAN1, as this depends on how timely such inter-UE resource allocation information exchange is expected in SL, as well as the specific contents of the signaling for such “a set of resources” info transfer. The signaling related discussion is also within the FFS list in above RAN1 conclusions (e.g. “	How UE-A sends "A set of resources" to UE-B, including container used for carrying it, implicitly or explicitly or both”). To this end, RAN2 should follow RAN1 conclusions to be made, and only starts the discussion on the further signaling impacts, after RAN1 decides to use L2/RRC signaling. This discussion, when it is really carried out in RAN2, obviously also needs to be based on the specific information that is determined as needed by RAN1.

Proposal 4: It is up to RAN1 to decide what kind of signaling is used to carry such “a set of resources” information in SL (e.g. L1 signaling or higher layer signaling).

Proposal 5: If L2/RRC signaling is used to carry such “a set of resources” information, RAN2 to discuss the detailed signaling design based on the specific information that is determined as needed by RAN1.

Conditions when coordinating UE sends “a set of resources” info

As per the latest proposals in [3], for the conditions on when a coordinating UE sends “a set of resources” information to the coordinated UE, at least the following options can be considered:

- Option 1: Based on a signaling of triggering or requesting

- Option 2: Based on a pre-defined or (pre)configured triggering condition(s)

For option 2, some criteria checked in the AS can be considered as the (pre)configured triggering condition(s). For example, when the CBR at the coordinating UE side is excessively high, the resource collision probability is high, and thus the coordinating UE may signal “a set of resources” information to the coordinated UE to help reduce the resource collision. By contrast, for option 1, the signaling that triggers/requests the coordinating UE to signal this “a set of resources” information should be defined, although the trigger of this “a set of resources” information itself is not needed.

However, which WG to discuss the trigger of signaling transmission should depend on which layer’s signaling is eventually used. If MAC CE or PC5 RRC message is used, the initiation condition should be handled by RAN2. By contrast, if SCI is used, RAN1 should be responsible for discussing this issue. Note that the conditions/triggers for the signaling exchange for such “a set of resources” info are also included in the FFS list in RAN1 (e.g. “	When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it”). Therefore, this discussion should also be carried out after RAN1 has the firm conclusion on the related discussions.

Proposal 6: If RAN1 determines to use MAC CE or PC5 RRC message to convey the “as set of resources” information, RAN2 to discuss the initiating condition for the coordinating UE to signal this “a set of resources” information (e.g. when the CBR is excessive high, etc.).

- Resource allocation for power saving

Impacts on resource pool configuration

Regarding resource pool configuration on resource allocation for power saving, the latest RAN1 conclusion was reached as “In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof” in RAN1 #103e [1].

Agreements: […] - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc. […]

Observation 1: RAN1 reached the agreement that R17 resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combinations(s) thereof.

Based on the above observation, RAN2 needs to work out the signaling for the resource pool configuration that support any combinations of the above resource allocation mechanisms. On the other hand, however, other potential details on the configuration aspects are also included in the FFS list in RAN1 (e.g. “o	FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.”). Therefore, RAN2 needs to wait for more RAN1 agreements (or even directly Rel-17 L1 parameter sheet) in finalizing the signaling aspects for resource pool configuration in R17 eSL.

Proposal 7: RAN2 to wait for more RAN1 progress in finalizing the signaling aspect on how to indicate the supported mode-2 mechanism in the resource pool configuration in R17 eSL.

Partial sensing

RAN1 reached a couple of agreements in RAN1 #104e meeting [2] for partial sensing. Basically, the details on how the partial sensing in NR SL should be performed should be up to RAN1, as in LTE V2X, because sensing is basically a physical layer procedure. RAN2 does not need to touch the details for that.

Proposal 8: The details for how the UE performs partial sensing (e.g. time length for partial sensing) is up to RAN1.

Random selection

In RAN1 #104e, limited agreements were made for random selection design:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection

In LTE V2X, there was a similar discussion for P2X. Specifically, there was a conclusion that in a P2X specific pool which allows both partial sensing and random selection, random selection is carried out in a resource reservation way, i.e. random selection with multiple MAC PDU transmissions. This is specified based on a RAN1 conclusion which motivated from the protection of the performance of partial sensing that shares the same pool with random selection. However, since this is from the performance perspective, whether such a mechanism should be inherited also in Rel-17 NR SL should still be up to RAN1. RAN2 needs to wait for RAN1 conclusion before starting any further related discussion.

Proposal 9: How a UE performs random selection in a resource pool (e.g. one-shot based or resource reservation based) is up to RAN1.

Others

In LTE V2X, a P-UE which needs power saving for resource allocation optionally supports CBR measurement and reporting and zone-based resource allocation. In Rel-17, perhaps a UE that needs to perform power-reduced resource allocation also optionally supports such features. RAN2 may be able to discuss these aspects which are more RAN2 related.

Proposal 10: RAN2 may discuss whether a UE needing power-reduced resource allocation in NR SL:

- optionally supports CBR measurement and reporting;

- optionally supports zone-based operation.

- Conclusion

This contribution further resource allocation enhancement for NR SL. The observation and proposals are as follows:

Observation 1: RAN1 reached the agreement that R17 resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combinations(s) thereof.

Proposal 1: The scenarios to be supported for inter-UE coordination is up to RAN1 (e.g. which cast type inter-UE coordination applies, whether there is a third-party coordinating UE, etc.).

Proposal 2: RAN2 discusses how a coordinating UE (i.e. the UE performing coordinating functionality) decides whether another UE is in Mode-1 or Mode-2, in order to decide whether it can/need signal the Inter-UE coordination information (e.g. “a set of resources” info) to that UE.

Proposal 3: RAN2 may need to discuss how to identify a coordinating UE by taking into account the factors, e.g. capability, authorization info, etc., based on the supported inter-UE coordination scheme to be concluded by RAN1.

Proposal 4: It is up to RAN1 to decide what kind of signaling is used to carry such “a set of resources” information in SL (e.g. L1 signaling or higher layer signaling).

Proposal 5: If L2/RRC signaling is used to carry such “a set of resources” information, RAN2 to discuss the detailed signaling design based on the specific information that is determined as needed by RAN1.

Proposal 6: If RAN1 determines to use MAC CE or PC5 RRC message to convey the “as set of resources” information, RAN2 to discuss the initiating condition for the coordinating UE to signal this “a set of resources” information (e.g. when the CBR is excessive high, etc.).

Proposal 7: RAN2 to wait for more RAN1 progress in finalizing the signaling aspect on how to indicate the supported mode-2 mechanism in the resource pool configuration in R17 eSL.

Proposal 8: The details for how the UE performs partial sensing (e.g. time length for partial sensing) is up to RAN1.

Proposal 9: How a UE performs random selection in a resource pool (e.g. one-shot based or resource reservation based) is up to RAN1.

Proposal 10: RAN2 may discuss whether a UE needing power-reduced resource allocation in NR SL:

- optionally supports CBR measurement and reporting;

- optionally supports zone-based operation.

- Reference

- Draft Report of 3GPP TSG RAN WG1 #103-e v0.2.0

- Draft Report of 3GPP TSG RAN1 WG1 #104-e v0.3.0

- R1-2009788, “Feature lead summary for AI 8.11.2.2 Feasibility and benefits for mode 2 enhancements”, LG Electronics, RAN1#103e, October 26-November 13, 2020