3GPP TSG-RAN WG2 Meeting #113 bis electronic				R2-2103262
Online, April 12 – April 20, 2021

Agenda item:		8.10.2.2

Source:			Asia Pacific Telecom, FGI

Title:		HARQ retransmission schemes in NTN

Document for:	 	Discussion and decision

Introduction

In RAN2, several agreements for HARQ aspects have been reached as below:

Agreement in RAN2#111e - From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL Agreement in RAN2#112e - For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset) - From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded Agreement in RAN2#113e - For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started. - FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled. - From RAN2 per spective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation). - For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

In RAN1, some agreements for HARQ aspects, which may have RAN2 impacts, are shown below:

Agreement in RAN1#102e Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling Agreement in RAN1#104e For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 - Note: The TB of the two PDSCHs can be either same or different

For DL HARQ, we have agreed that the DL HARQ feedback can be enabled/disabled per HARQ process via RRC configuration, and the drx-HARQ-RTT-TimerDL is not started if the DL HARQ feedback of the corresponding HARQ process is disabled. Besides, an FFS has been left for the blind retransmission issue. Regarding the blind retransmission, RAN1 agreed that the UE is not expected to receive another PDSCH for the given HARQ process that starts until X after the end of the reception of the last PDSCH for that HARQ process.

For UL HARQ, so far, no further enhancement has been achieved. We merely confirmed that the gNB can send a UL grant without waiting for decoding result of previous PUSCH transmission based on NW implementation, which may be the legacy behavior.

In this contribution, we further share our perspectives on both DL and UL HARQ retransmission schemes on top of the current status.

Discussion

DL HARQ retransmission scheme in NTN

Figure 1: DL HARQ retransmission scheme in NTN

In Figure 1, an illustration for the DL HARQ retransmission scheme is shown. It has been agreed that the HARQ feedback of a HARQ process can be disabled by RRC. If the HARQ feedback of the HARQ process has been disabled, the corresponding drx-HARQ-RTT-TimerDL is not started. It is known that the drx-HARQ-RTT-TimerDL is used for triggering the drx-RetransmissionTimerDL for the corresponding DL HARQ process. Since the drx-HARQ-RTT-TimerDL is not started for the disabling HARQ process, the UE will not start the drx-RetransmissionTimerDL for the disabling HARQ process based on the current specification. As a result, the UE will not expect any DL retransmission scheduling.

Observation 1: The HARQ feedback of a DL HARQ process can be enabled/disabled by a RRC configuration.

Observation 2: The UE does not start the drx-HARQ-RTT-TimerDL for a DL HARQ process if the HARQ feedback of the HARQ process is disabled.

However, RAN1 has identified that DL blind retransmission can be considered for enhancing the performance and reliability of DL transmission when the HARQ feedback is disabled. The DL blind retransmission implies that the PDSCH (re-)transmissions of the same TB can be scheduled without waiting for the HARQ feedback. Comparing to the aggregated (e.g., repetition) transmission scheme, the blind retransmission scheme further provides the diversity with a larger time-domain factor, which is not sub-ordinary to aggregated transmission. Furthermore, the blind retransmission scheme can reduce the significant TB error rate, and reduce resource waste by dynamically scheduling. Hence, for the disabling DL HARQ feedback, the DL blind retransmission can be supported. Moreover, in our view, it seems not all kinds of DL packets should be reliable. In some cases, there is no need to enable the functionality of the DL blind retransmission, e.g., if the DL packet is not quite important. The scheduling aspect always relies on the gNB implementation, thus the gNB should have the power to decide whether the DL blind retransmission is needed or not.

Proposal 1: NW can send a DL blind retransmission indication to UE to enable/disable the DL blind retransmission for a HARQ process when the DL HARQ feedback of the HARQ process is disabled.

In the case that the DL blind retransmission is enabled, the UE should wake-up to monitor the PDCCH in a time duration that the UE expects to receive a DL blind retransmission scheduling. Some companies proposed that the current DRX timers can be used, e.g., drx-InactivityTimer or drx-RetransmissionTimerDL. The main difference between these two timers is that the drx-InactivityTimer is applied per MAC entity while the drx-RetransmissionTimerDL is applied per HARQ process. Since the transmission is associated with a HARQ process, the corresponding blind retransmission should also be expected to receive for that HARQ process. Therefore, drx-RetransmissionTimerDL is preferred to be applied for DL blind retransmission. Then, since there is no drx-HARQ-RTT-TimerDL in HARQ feedback disabling case, we have to discuss the exact timing that the UE should start the drx-RetransmissionTimerDL.

In RAN1#104, there is an agreement shows that the UE is not expected to receive another PDSCH scheduled for the given HARQ process that starts until X (which can be referred to the Scheduling restriction X in Figure 1) after the end of the reception of the last PDSCH for that HARQ process. The TB of the two PDSCHs can be the same, so this rule can apply to retransmission cases. (Note that the definition of X has not been decided by RAN1, which may be T_proc,1, max(T_proc,1, K1), etc.)

Agreement in RAN1#104e For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 Note: The TB of the two PDSCHs can be either same or different

Since there is a scheduling restriction between the two PDSCHs, an offset should be needed to start the drx-RetransmissionTimerDL after the end of the PDSCH reception. How to specify the offset can wait for RAN1 progress.

Proposal 2: If the DL blind retransmission is enabled, the UE should start the drx-RetransmissionTimerDL after the end of the PDSCH reception, else if the DL blind retransmission is disabled, the UE should not start the drx-RetransmissionTimerDL.

Proposal 3: FFS on the exact timing, based on an offset, to start the drx-RetransmissionTimerDL after the end of the PDSCH reception. The offset depends on the scheduling restriction X for the two consecutive PDSCHs, from RAN1.

UL HARQ retransmission scheme in NTN

For UL HARQ, RAN2 only agreed that the gNB can send UL grant without waiting for decoding result of previous PUSCH transmission based on NW implementation. In our view, we think that the UL HARQ retransmission scheme should be similar to DL HARQ retransmission scheme as much as possible to reduce confusion.

The UL HARQ retransmission scheme can be categorized into three types, which reflect the different behaviors for UL HARQ retransmission, as summarized in Table 1.

Table 1: UL HARQ retransmission types

| Types | Enable/Disable UL HARQ retransmission | Expected behaviors for UL HARQ retransmission | drx-HARQ-RTT-TimerUL drx-RetransmissionTimerUL |
| --- | --- | --- | --- |
| Type 1 | Enabling UL HARQ retransmission | UL blind retransmission is disabled: gNB sends UL grant based on decoding result of previous PUSCH transmission (i.e., Two consecutive PUSCHs should > 1 RTT) | - drx-HARQ-RTT-TimerUL is started as legacy - drx-RetransmissionTimerUL is started as legacy |
| Type 2 |  | UL blind retransmission is enabled: gNB can send UL grant without waiting for decoding result of previous PUSCH transmission (i.e., Two consecutive PUSCHs may < 1 RTT) | - drx-HARQ-RTT-TimerUL is not started - drx-RetransmissionTimerUL is started after the end of the PUSCH transmission |
| Type 3 | Disabling UL HARQ retransmission | UL HARQ with no retransmission | - drx-HARQ-RTT-TimerUL is not started - drx-RetransmissionTimerUL is not started |

In Figure 2, an UL HARQ retransmission scheme is illustrated.

Figure 2: UL HARQ retransmission scheme in NTN

In DL, based on the agreement, an RRC indication can be used to disable the HARQ feedback of a HARQ process. If the HARQ feedback is disabled for the HARQ process, the UE does not provide the HARQ feedback for this HARQ process. Similarly, in UL, an RRC indication can also be used to enable/disable the UL HARQ retransmission. That is, if the UL HARQ retransmission is disabled, the UE does not expect the scheduling for UL HARQ retransmission would be received. Then the UE does not start the drx-HARQ-RTT-TimerUL in this case.

Proposal 4: The UL HARQ retransmission of a UL HARQ process can be enabled/disabled by a RRC configuration.

Proposal 5: The UE does not start the drx-HARQ-RTT-TimerUL for a UL HARQ process if the UL HARQ retransmission of the UL HARQ process is disabled.

To align with DL, the blind retransmission for UL can also be supported. The definition of UL bind retransmission is a littile bit different from the DL. As shown in Table 1, in type 1, if the UL HARQ retransmission is enabled, and the UL blind retransmission is disabled, it means that the gNB sends UL grant based on decoding result of previous PUSCH transmission, i.e., two consecutive PUSCHs should > 1 RTT, which is the legacy behavior.  In type 2, if the UL HARQ retransmission is enabled, and the UL blind retransmission is enabled, gNB can send UL grant without waiting for decoding result of previous PUSCH transmission, i.e., two consecutive PUSCHs may < 1 RTT, which is an enhancement for the UL scheduling. In type 3, if the UL HARQ retrretransmissiondisabled, no UL retransmission is expected, so the blind retransmission can be void. Thus, the combination of the enabling/disabling UL HARQ retransmission indication as well as the UL blind retransmission indication can be used to control the UE behaviors on both the drx-HARQ-RTT-TimerUL and the drx-RetransmissionTimerUL.

Proposal 6: NW can send a UL blind retransmission indication to UE to enable/disable the UL blind retransmission for a HARQ process when the UL HARQ retransmission of the HARQ process is enabled.

Proposal 7: If the UL HARQ retransmission of the HARQ process is enabled and the UL blind retransmission is enabled, the UE should start the drx-RetransmissionTimerUL after the end of the PUSCH transmission but not start the drx-HARQ-RTT-TimerUL.

Proposal 8: FFS on the exact timing, based on an offset, to start the drx-RetransmissionTimerUL after the end of the PUSCH transmission.  The offset depends on the scheduling restriction X for the two consecutive PUSCHs, from RAN1.

Conclusion

In summary, it’s beneficial to align with the DL HARQ and the UL HARQ as much as possible, thus we have the following observations and proposals based on the above anslysis.

Observation 1: The HARQ feedback of a DL HARQ process can be enabled/disabled by a RRC configuration.

Observation 2: The UE does not start the drx-HARQ-RTT-TimerDL for a DL HARQ process if the HARQ feedback of the HARQ process is disabled.

Proposals for DL HARQ

Proposal 1: NW can send a DL blind retransmission indication to UE to enable/disable the DL blind retransmission for a HARQ process when the DL HARQ feedback of the HARQ process is disabled.

Proposal 2: If the DL blind retransmission is enabled, the UE should start the drx-RetransmissionTimerDL after the end of the PDSCH reception, else if the DL blind retransmission is disabled, the UE should not start the drx-RetransmissionTimerDL.

Proposal 3: FFS on the exact timing, based on an offset, to start the drx-RetransmissionTimerDL after the end of the PDSCH reception. The offset depends on the scheduling restriction X for the two consecutive PDSCHs, from RAN1.

Proposals for UL HARQ

Proposal 4: The UL HARQ retransmission of a UL HARQ process can be enabled/disabled by a RRC configuration.

Proposal 5: The UE does not start the drx-HARQ-RTT-TimerUL for a UL HARQ process if the UL HARQ retransmission of the UL HARQ process is disabled.

Proposal 6: NW can send a UL blind retransmission indication to UE to enable/disable the UL blind retransmission for a HARQ process when the UL HARQ retransmission of the HARQ process is enabled.

Proposal 7: If the UL HARQ retransmission of the HARQ process is enabled and the UL blind retransmission is enabled, the UE should start the drx-RetransmissionTimerUL after the end of the PUSCH transmission but not start the drx-HARQ-RTT-TimerUL.

Proposal 8: FFS on the exact timing, based on an offset, to start the drx-RetransmissionTimerUL after the end of the PUSCH transmission.  The offset depends on the scheduling restriction X for the two consecutive PUSCHs, from RAN1.

Reference

[1] Report of 3GPP TSG RAN WG2 meeting #113-e

[2] R2-2102043 [AT113-e][103][NTN] HARQ aspects Phase 2 (InterDigital)

[3] R1-2102143 Summary#4 of AI 8.4.3 for HARQ for NTN (ZTE)