3GPP TSG-RAN WG2 Meeting #113 bis electronic				R2-2103266
Online, April 12 – April 20, 2021

Agenda item:		8.9.2

Source:			Asia Pacific Telecom, FGI

Title:		Discussion on indications for UE power saving

Document for:	 	Discussion and decision

Introduction

In RAN2, the following agreements were reached on indications for power saving in RRC_IDLE/INACTIVE:

Agreement in RAN2#112 - Confirm that UE grouping is considered a candidate of paging enhancement for UE power saving - RAN2 have discussed and considered “paging indication for UE subgroups using paging DCI”, “paging early indication or wake-up signal (WUS) for UE subgroups”, “cross-slot scheduling of paging for UE subgroups”. Agreement in RAN2#113 - There is support to have UE ID based enhancement - There is still significant interest to have other additional methods (but also some concerns). The approach to have a single mechanism that can take several aspects into account can be a way forward. There are still questions on the details, e.g. whether CN or RAN would provide a parameter.

In RAN1, the following agreements were reached on indications for power saving in RRC_IDLE/INACTIVE:

Agreements in RAN1#102 For potential paging enhancements, RAN1 to study the following candidate schemes: - Paging early indication before a target PO to indicate UE whether to monitor PDCCH scrambled with P-RNTI at the PO. Potential candidate indication methods include - DCI-based indication, e.g., based on - Extending existing DCI format 1_0 or 2_6 - New DCI format - RS-based or sequence-based indication, e.g., based on TRS/CSI-RS or SSS - Sub-grouping for paging, based on - Legacy paging DCI - Paging early indication - Additional reception occasions in time/frequency domain - Multiple P-RNTIs - Cross-slot scheduling for paging PDSCH - Other proposal is not precluded Agreements in RAN1#103 For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective - FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS - Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact Agreements in RAN1#104 - Carrying UE subgroups information is considered in physical layer design for paging enhancement - Further study the design on how to provide the indications for UE subgroups over PEI and/or paging PDCCH, subject to the metrics agreed in RAN1 102e.

Based on the achieved agreements, we share our prospect on the indications for UE power saving in RRC_IDLE/INACTIVE.

Discussion

Background of paging mechanism

To reduce power consumption in RRC_IDLE/RRC_INACTIVE, a DRX mechanism is applied for paging monitoring, which implies that the RRC_IDLE/RRC_INACTIVE UE does not need to monitor the PDCCH continuously. As shown in Figure 1, a UE can be configured with DRX cycle(s) and several parameters for the determination of paging occasion (PO). It is noted that the UE may only monitor one PO per DRX cycle. But in some cases (e.g., multi-beam operation, operation with shared spectrum channel access), the UE would monitor multiple PDCCH monitoring occasions in one PO. In addition, a PO may be a set of ‘S’ consecutive PDCCH monitoring occasions, where ‘S’ is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

Figure 1: DRX mechanism for paging monitoring

Currently, paging allows the NW to reach UEs through Paging message and notify UEs of system information change and ETWS/CMAS indications through Short Message. As shown in Figure 2, a UE monitors the PDCCH to receive a Paging DCI which is transmitted by a DCI format 1_0 with CRC scrambled by P-RNTI. Upon the UE receives the paging DCI, the UE checks the Short Message Indicator to know if there is a short message carried in the DCI and check if there is a scheduling information for the paging message. If the short message indicator indicates there is a paging message, the UE further receives the paging message on the PDSCH based on the scheduling information indicated by the DCI. Upon the UE receives the paging message while in RRC_IDLE/RRC_INACTIVE, the UE further checks if the UE ID included in the paging record matches the UE’s ID (allocated by upper layer or UE’s stored RNTI). If so, it means the NW would like to reach the UE.

Figure 2: Paging DCI, short message, and a paging message

Indications for UE subgroups

It is noted that RAN1 has identified that the indications to carry the UE subgroups information are beneficial for power saving in RRC_IDLE/RRC_INACTIVE based on the evaluation. According to the last RAN2 agreement, the UE subgrouping based on UE ID is supported as a baseline, and other schemes are FFS. Moreover, RAN2 confirms that the following candidate indications for UE subgroups information can be considered.

- Paging DCI

- PEI

- Cross-slot scheduling

Paging DCI

As shown in Figure 3, if the UE subgroup information can be included in paging DCI, NW can indicate which subgroup(s) of UEs should further receive the paging message via PDSCH. In comparison with the legacy paging DCI, the benefit is to reduce the false alarm rate for receiving the paging message, e.g., some group(s) of UEs can skip receiving the paging message, which can save the power on PDSCH decoding. Another advantage is that minimum specification impact would be expected since the reserved bits in paging DCI can be used, i.e., no need to specify a new DCI format or new occasion for monitoring a new indication/signal. However, it is noted that only limited reserved bits in the current paging DCI can be used. For example, in a short message indicator, there is only one reserved index. In a short message, only five reserved indexes can be used. On the other hand, since paging DCI should be indicated closed to paging message in the time domain, the UE still needs to perform the SSB measurement for pre-synchronization before receiving the paging DCI. In other words, there is no power saving gain on “pre-synchronization” and “PDCCH monitoring”.

Figure 3: Paging DCI

Paging Early Indication (PEI)

As shown in Figure 4, if the UE subgroup information can be included in PEI, NW can indicate which subgroup(s) of UEs should or should not wake-up to monitor paging DCI in advance. For example, the PEI can be indicated before one to three SSB bursts before the PO. When the PEI indicates that the UE(s) don’t need to monitor the upcoming PO, the UE(s) can skip the following SSB measurements and skip monitoring the PO (also the paging PDSCH). The benefits are to save not only the power on paging monitoring but also the “pre-synchronization” step. However, the drawback is the complexity because RAN1 needs to discuss how to model the PEI (e.g., PDCCH-based as DCP (DCI with CRC scrambled by PS-RNTI) or sequence-based as I-WUS), how to define a new DCI format as well as new monitoring occasion(s) for PEI, etc.

Figure 4: Paging Early Indication (PEI)

Cross-slot scheduling (CSS)

Regarding cross-slot scheduling (CSS), if NW could give UE some hints to let UE know a-priori the range/offset of symbols not carrying data transmissions that the UE expects to receive or transmit, the UE could be able to put its RF, portion of the front-end hardware and potentially other modem hardware into power saving mode (e.g., micro-sleep mode) for those symbols.

In our views, there are two possible alternatives for indicating CSS in RRC_IDLE/RRC_INACTIVE.

Alt 1, as shown in Figure 5, is to introduce a new DCI format that can not only indicate which subgroup(s) of UEs should further monitor the PO, but also indicate the time and frequency resources of the PDSCH in advance. This alternative is very similar to the function of PEI. Although the specification impact may be less compared to PEI, the shortage is the limitation on the range between the new DCI with CSS and PO (e.g., up to 32 slots). The power saving gain is not significant for “pre-synchronization” compared to PEI.

Alt 2, as shown in Figure 6, is to indicate the CSS information via paging DCI. The paging DCI and the corresponding paging PDSCH can be transmitted in different slots. The power saving gain comes from the that UE can avoid unnecessary DL signal buffering if there is no paging PDSCH transmitted at all. However, it is challenging for NW to decide the resource allocation and MCS for paging PDSCH when the scheduling offset between the paging DCI and the paging PDSCH is very large for power saving benefit.

Figure 5: Alt 1- New DCI with CSS

Figure 6: Alt 2- Paging DCI with CSS

Conclusion

Based on the above analysis, we summarize the pros and cons of these three different indications in Table 1. From our perspective, we think the method of Paging DCI leads to the least power saving gain but the smallest specification impact.  From the point of specification impact, paging DCI is a good approach. On the other hand, the method of PEI leads to the highest power saving gain but also the highest complexity. From a power saving perspective, PEI is another attractive approach. Thus, we think the compromise way is to support both mechanisms. Then NW can decide which method is better to use by NW implementation.  As for CSS, we think the method of CSS was discussed in RAN1 for RRC_CONNECTED in Rel-16, so it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE in RAN1 first. Whether to support CSS in RRC_IDLE/RRC_INACTIVE should be determined by RAN1.

Observation 1: Paging DCI for UE subgroups incurs the least power saving gain but the smallest specification impact.

Observation 2: PEI for UE subgroups leads to the highest power saving gain but the highest complexity.

Observation 3: CSS was discussed in RAN1 for RRC_CONNECTED UEs in Rel-16, so it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE UEs in RAN1.

Table 1: Pros and Cons of the indications for UE subgroups

|  |  | Pros | Cons |
| --- | --- | --- | --- |
| Paging DCI |  | - Reserved bits can be used - Minimum specification impact | - Limited reserved bits support less UE groups - No power saving gain on “pre-synchronization” and “PDCCH monitoring” |
| Paging Early Indication (PEI) |  | - Power saving gain on both PDCCH monitoring and skipping “pre-synchronization” step | - The complexity of PEI modeling |
| Cross-Slot Scheduling (CSS) | Alt 1- New DCI with CSS | - Less specification impact than PEI | - Limitation on the range between the new DCI with CSS and PO. - Less power saving gain on “pre-synchronization” than PEI |
|  | Alt 2- Paging DCI with CSS | - Minimum specification impact | - Limitation on the offset between paging DCI and paging PDSCH - No power saving gain on “pre-synchronization” and “PDCCH monitoring” |

Proposal 1: From RAN2 perspective, both paging DCI and paging early indication (PEI) for UE subgroups are supported.

Proposal 2: Whether cross-slot scheduling for RRC_IDLE/RRC_INACTIVE UEs can be supported is determined by RAN1.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: Paging DCI for UE subgroups incurs the least power saving gain but the smallest specification impact.

Observation 2: PEI for UE subgroups leads to the highest power saving gain but the highest complexity.

Observation 3: CSS was discussed in RAN1 for RRC_CONNECTED UEs in Rel-16, so it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE UEs in RAN1.

Proposal 1: From RAN2 perspective, both paging DCI and paging early indication (PEI) for UE subgroups are supported.

Proposal 2: Whether cross-slot scheduling for RRC_IDLE/RRC_INACTIVE UEs can be supported is determined by RAN1.