3GPP TSG RAN WG2#113bis-e                                	R2-2103267

Online, April 12th – April 20th, 2021

Agenda item:	8.1.2.1

Source: 	Asia Pacific Telecom, FGI

Title: 	HARQ modelling for supporting retransmission in MBS

Document for:	Discussion and Decision

Introduction

In RAN1#103e [1], the following agreements were achieved.

Agreements: For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following: - ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources. - FFS details including conditions for it to be used - NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, further down-select between: - FFS: PUCCH resource configuration for NACK only feedback. - FFS details including conditions for it to be used

And in RAN1#104e [2], the following agreements were achieved.

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

According to the agreements above, from RAN1 perspective, HARQ mechanism has been introduced to increase the reliability of MBS data transmission. Moreover, it may also support data retransmission via different transmission modes (i.e., PTM scheme 1 and PTP) relying on UE HARQ-ACK feedback. The HARQ modelling includes MAC layer’s involvement which needs RAN 2 to discuss with.

Discussion

HARQ operation in legacy

The HARQ operation for unicast data reception in legacy is described: once a downlink assignment is received on the PDCCH, for example, a DCI with CRC bits scrambled by C-RNTI is decoded on PDCCH, UE’s MAC entity will be indicated by PHY about the reception of the downlink assignment on the PDCCH. Moreover, the DCI indicates UE to perform corresponding PDSCH reception by a specific HARQ process. A new data indicator (NDI) carried by the DCI will also be delivered from the PHY to the MAC entity. Then, the MAC entity evaluates whether the NDI for the HARQ process of the corresponding PDSCH have been toggled or not (a.k.a. determine whether the received TB is an initial transmission or a retransmission).

MBS data transmission modes

According to the progress made in RAN1, MBS supports transmission modes of PTP, PTM scheme I and PTM scheme II. In RAN1, PTP and PTM scheme I are defined as the following:

- PTP transmission: For RRC_CONNECTED UEs, use DCI with CRC bits scrambled by UE-specific RNTI (e.g., C-RNTI) and the DCI is transmitted on UE-specific PDCCH to schedule UE-specific PDSCH reception which is scrambled with the same UE-specific RNTI.

- PTM transmission scheme I: For RRC_CONNECTED UEs in the same MBS group, the gNB apply a DCI with CRC bits scrambled by group-common RNTI (e.g., G-RNTI) and the DCI is transmitted on group-common PDCCH to schedule group-common PDSCH reception which is scrambled with the same group-common RNTI.

MBS data retransmission crossing PTP and PTM scheme I

Turning back to the HARQ process for MBS. Obviously, since MBS data delivery via PTM transmission mode could be received by a group of UEs, it is possible that not all the UEs within the group can receive the data successfully. Depending on radio condition for each UE and some other situations, some UEs within the group could not successfully receive the MBS data, whilst the other UEs could. According to RAN1’s discussion and progress, based on HARQ-ACK feedback, the gNB could understand the MBS data reception situation (i.e., whether all of the UE receives data well) in real time.

Based on the HARQ-ACK feedback the gNB could properly adjust modulation coding rate and/or performing corresponding retransmission via different transmission schemes. For an example, once a UE within the group fails on an MBS data reception via PTM scheme I, the gNB could schedule a DL assignment for corresponding MBS data retransmission via PTP. Aligning with RAN1, the gNB could let the UE handle the retransmitted data by a same HARQ process as which handled the initial transmitted data, that is, the gNB indicates UE a same HARQ process ID to the UE for both of the initial transmission via PTM scheme I and the retransmission via PTP. This approach allows UE to perform soft combine with same HARQ process.

Observation 1 	For a retransmitted data, gNB indicates UE to handle it by a HARQ process same as for initial transmission.

Observation 2 	Continue with observation 1, NDI may be applied by gNB for indicating UE whether the data is an initial transmission or a retransmission.

However, in case of MBS service with PTP and PTM scheme I as mentioned above, there exist some cases which may bring challenges to gNB on indicating initial transmission or retransmission via the NDI. For example, as shown in figure 1, a gNB performs a 1st DL assignment with PTP and a 2nd DL assignment with PTM scheme I to UE A. Both of 1st and 2nd DL assignments are indicated to be handled by a HARQ process X. But since the 2nd DL assignment is with PTM scheme I, the NDI of the 2nd DL assignment may be received by some other UEs (e.g., UE B) subscribing same MBS service. It is noted, the process X of the UE B may be indicated for handling data with PTM scheme I previously, which is different from UE A. Hence, it is difficult for the gNB to indicate the initial transmission or retransmission via the NDI to UE A and UE B because the NDI corresponding to HARQ process X of UE B and UE A may not sync.

Figure 1 MBS data transmission via different transmission modes

Observation 3 	Once gNB dynamically switches the transmission mode for MBS data transmission, it brings challenge on how to indicate, by NDI, UE each scheduled DL data reception is an initial transmission or is a retransmission.

Proposal 1	Support initial transmission and retransmission of an MBS data with the same HARQ process.

Proposal 2	RAN2 to discuss HARQ modelling for MBS data reception based on identical HARQ process for both initial transmission and retransmission.

Conclusion

In this contribution, we have the following observations

Observation 1 	For a retransmitted data, gNB indicates UE to handle it by a HARQ process same as for initial transmission.

Observation 2 	Continue with observation 1, NDI may be applied by gNB for indicating UE whether the data is an initial transmission or a retransmission.

Observation 3 	Once gNB dynamically switches the transmission mode for MBS data transmission, it brings challenge on how to indicate, by NDI, UE each scheduled DL data reception is an initial transmission or is a retransmission.

Based on observations, the following proposals are made

Proposal 1	Support initial transmission and retransmission of an MBS data with the same HARQ process.

Proposal 2	RAN2 to discuss HARQ modelling for MBS data reception based on identical HARQ process for both initial transmission and retransmission.

Reference

[1]	Chairman’s Notes, 3GPP TSG RAN WG1 #103 e-meeting

[2]	Chairman’s Notes, 3GPP TSG RAN WG2 #104 e-meeting