3GPP TSG-RAN WG2 Meeting #113-bis electronic	R2-2103328

E-Meeting, 12th April – 20th April, 2021

Source:	vivo

Title:	Discussion on L2 and L3 relay co-existence

Agenda Item:	8.7.1

Document for:	Discussion and Decision

# Introduction

At RAN#91 meeting, a New WID on NR Sidelink Relay has been approved, with the following objective [1]:

The objective of this work item is to specify solutions to enable single-hop, sidelink-based, L2 and L3 based UE-to-Network (U2N) relaying. Work Item objectives on aspects common to both L2 and L3: - Specify mechanisms for U2N relay discovery and (re)selection for L3 and L2 relaying [RAN2, RAN4] - Re-use LTE relay discovery and (re)selection as baseline - Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3] - Re-use LTE as baseline Work Item objectives specific to Layer-2 (L2) relaying: - Specify mechanisms for E2E, i.e. PC5 and Uu, QoS management [RAN2]: - Specify mechanisms for service continuity - Limited to intra-gNB cases [RAN2] - Specify mechanisms for U2N Adaptation layer design [RAN2] - For bearer mapping and Remote UE identification, incl. RAN related security aspects if any - Specify Control Plane procedures for U2N, including RRC connection management, system information delivery, paging mechanism and access control for Remote UE [RAN2, RAN3] NOTE 1: RAN requests RAN2 to strive for completion of the common parts (objective 1) by RAN#92 (June). RAN understands that RAN2 will also initially work on other aspects that have cross-group dependencies. NOTE 2: For L2 UE-to-Network Relay, it is assumed that the Remote UE has a single active connection towards gNB via only a single Relay UE at a given time in this release. NOTE 3: Only NR Uu interface, i.e. gNB, and 5GC is considered, and it is limited to NR SA scenario in this release. NOTE 4: Work specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”, and the group mobility is not supported in this release.

Based on the WID, both L2 relay architecture and L3 relay architecture would be specified in this WI. Based on relay deployment, L2 relay architecture and L3 relay architecture could co-exist within a same area.

This contribution discusses remote UE relaying in co-existent L2 relay and L3 relay architecture.

# Discussion

Based on remote UE and relay UE capabilities, and the network support of relay architecture relaying for remote UE may supported or not.

## Network relay deloyment and UE relay capability

## Network side support of L2 and L3 relay architecture

As L2 and L3 would specified, network deployment may support both L2 and L3. This kind of relay architecture support may come at least in threefold:

- Option 1: Relay architecture support per PLMN, i.e., L2 or L3 support per area or PLMN

- Option 2: Each cell supports only one kind of relay architecture L2 or L3, but not both.

- Option 3: Each cell supports both L2 and L3 architecture.

Thus,

- Relay architecture support can be deployed:

- Per area or PLMN, i.e., each cell support only one relay architecture

- One relay architecture per cell, i.e., cell supports only one kind of relay architecture L2 or L3

- Full relay architecture per cell, i.e., cell supports both L2 and L3 architecture

- Option 1 is simple from network and UE point of view. Based on PLMN and relay architecture support UE can select and establish relay connection with network. But, Inter-PLMN mobility may be problematic if the PLMNs do not support the same relay architecture.

- For Option 2, within the cell the relay architecture support is simple. From SIB information UE can know whether it can select an establish relay connection with the concerned cell. But, for UE to fully support relay selection and establish relay connection with the network, UE has to have capability support for both L2 and L3 relay architecture.

For Option 2, the following issues need to be considered:

- Issue 1: In case L2 relay, if the serving cell, which support only L3 architecture relay, of the remote UE is different with the serving cell of the relay (Figure 1), how can the remote UE select and establish relay connection with network?

- Issue 2: In case L3 relay, if the serving cell, which support only L2 architecture relay, of the remote UE is different with the serving cell of the relay (Figure 2), can the remote UE select and establish relay connection with network?

Additionally, in case of remote UE HO between L2 relay and L3 relay, as shown in Figure 3, remote UE service continuity cannot be guaranteed.

Option 3 may be complicated deployment scenario. But it provides some flexibility for UE support and establishment of relay connection with network, as UE do not have to support both L2 and L3 architecture relay.

Additionally, UE has the possibility to select relay at ease, for example relay section per service, if a certain service is deployed only on one relay architecture.

Therefore,

- RAN2 to discuss which relay architecture co-existence to support, i.e.:

- Per area or PLMN, i.e., each cell support only one relay architecture

- One relay architecture per cell, i.e., cell supports only one kind of relay architecture L2 or L3

- Full relay architecture per cell, i.e., cell supports both L2 and L3 architecture

## UE relay capability support

Based on how and where relay is deployed, UE has to support relay. UE support relay capability can would be:

- L3 support only

- L2 support only

- Both L2 and L3 support

L3 relay has less RAN impact, if any. And basic L3 relay without N3N3IWF deployment is also simple from RAN and CN perspective. This deployment can address most relay service requirements such as public safety. Thus,

- L3 Relay architecture support can be achieved with less specification and hardware impact with less impact from CN and RAN perspective.

As L2 relay would also be specified ij this release. UE L2 relay support capability should also be considered. But, UE only supporting L2 relay architecture is not an optimal relay deployment, as:

L2 relay deployment significant specification effort and hardware upgrade at RAN.

Using L2 relay to achieve address relay scenario, such relaying between smart phone and smart watch, is not necessary. Or using L2 relay to relay service with less service continuity requirement would also be unnecessary. Thus, such kind of L2 relay deployment usage is not useful and just costly. Thus,

- UE with only L2 relay architecture support capability is NOT supported.

For full support of relay to address all relay deployment scenarios and service requirements, including service with less service continuity requirement or service with high continuity requirement, the UE can have L2 and L3 relay support capability. Thus,

- UE with both L2 and L3 relay support capability would address all relay deployment scenarios and service requirements, including service with less service continuity requirement or service with high continuity requirement

But, UE both with L2 and L3 relay support capability would increase UE complexity. Therefore,

- The following UE relay support capabilities should be considered:

- UE with only L3 relay architecture support capability

- UE with both L2 and L3 architecture support capability

## Remote UE and Relay UE relaying capability

## Scenario of Remote and Relay UE with same relaying capability

## Support L3 architecture only

In case the remote UE and relay UE support only L3 architecture relaying, based network relay architecture support, there is three deployment alternatives that may be foreseen (see Figure 4):

- Alt 1: Network supports L3 relay architecture only

- Alt 2: Network supports L2 relay architecture only

- Alt 3: Network supports both L2 and L3 relay architectures

For Alt 1, as all remote UE, relay UE and network have L3 capability support, remote UE is able to select relay and establish L3 relaying connectivity with network. Thus, such deployment is a viable scenario for L3 relaying.

For Alt 2, as remote UE and relay both support L3 rely architecture, UE may be able to select relay UE. But, as network only support L2 relay architecture, remote UE would not be able to establish relaying connection with network. Thus, such deployment is not a viable scenario for L3 relaying.

For Alt 3, as remote UE and relay both support L3 rely architecture, UE may be able to select relay UE. And as network also has support capability for L3 relay architecture, remote UE can be able to establish relaying connection with network. Thus, such deployment is also a viable scenario for L3 relaying.

In case the relay UE also support L2, the relaying possibilities are similar to above three Alt.

- In case Remote UE supports L3 only and Relay UE supports L3 relay architecture relaying scenarios can be considered as follows:

- Network supporting L3 architecture is a viable L3 relaying scenario

- Network supporting L2 architecture is NOT a viable L3 relaying scenario

- Network supporting both L2 and L3 architectures is a viable L3 relaying scenario

## Support L2 and L3 architectures

In case both remote UE and relay UE support L2 and L3 architecture relaying, based network relay architecture support, there is three deployment alternatives that may be foreseen:

- Alt 1: Network supports L3 relay architecture only

- Alt 2: Network supports L2 relay architecture only

- Alt 3: Network supports both L2 and L3 relay architectures

For Alt 1, as remote UE and relay both support L2 and L3 rely architecture, as network has L3 relay architecture support, based on system information of the serving cell the remote UE would be able to select the relay using L3 support capability and establish relaying connection with network. Thus, such deployment is a viable scenario for L3 relaying.

For Alt 2, as remote UE and relay both support L2 and L3 rely architecture, as network has L2 relay architecture support, based on system information of the serving cell the remote UE would be able to select the relay using L2 support capability and establish relaying connection with network. Thus, such deployment is a viable scenario for L2 relaying.

For Alt 3, as remote UE and relay both support L2 rely architecture, two sub-alternatives can be considered:

- Alt 3-1: The serving cell broadcast only L2 or only L3 relaying support, based on system information of the serving cell the remote UE would be able to select the relay using L2 or L3 support capability and establish L2 or L3 relaying connection with network. Thus, such deployment is a viable scenario for L2 or L3 relaying.

- Alt 3-2: The serving cell broadcast both L2 and L3 relaying support, based on remote UE preference the remote UE would be able to select the relay using either L2 or L3 support capability and establish L2 or L3 relaying connection with network. Thus, such deployment is a viable scenario for L2 or L3 relaying.

- In case Remote UE and Relay UE support both L2 and relay architecture relaying scenarios can be considered as follows:

- Network supporting L3 architecture is a viable L3 relaying scenario

- Network supporting L2 architecture is a viable L2 relaying scenario

- Network supporting both L2 and L3 architectures is a viable L2 or L3 relaying scenario

## Scenario of Remote and Relay UE with different relaying capability

In case the remote UE and relay UE have different relaying capability support, the following the deployment scenarios can be considered:

- Remote UE supports L2 architecture only, the Relay supports L3 architecture only and network supports L2 architecture only

- Remote UE supports L3 architecture only, the Relay supports L2 architecture only and network supports L2 architecture only

- Remote UE supports L2 architecture only, the Relay supports L3 architecture only and network supports L3 architecture only

- Remote UE supports L3 architecture only, the Relay supports L2 architecture only and network supports L3 architecture only

- Remote UE supports L2 architecture only, the Relay supports L3 architecture only and network supports both L2 and L3 architecture

- Remote UE supports L3 architecture only, the Relay supports L2 architecture only and network supports both L2 and L3 architecture

- Remote UE supports L2 architecture only, the Relay supports L3 architecture only and network supports both L2 and L3 architecture

- Remote UE supports L3 architecture only, the Relay supports L2 architecture only and network supports both L2 and L3 architecture

In all above scenarios, as the remote UE and relay UE support different relay architecture support, no matter the network relay architecture support, the remote UE would not be able to support the relay and establish relaying connection with network. Thus,

- The case where Remote UE and Relay UE have different relay architecture support capability is NOT viable L2 and/or L2 relaying scenario

## Summary

Based on discussion in section 2.1 and section 2.2, based on remote UE, relay UE and network support of relay architecture, viable relaying scenarios can be summarized as in the table 2.3-1

Table 2.3-1: Relaying possibility based on remote UE, relay UE and RAN support of relay architecture

| RAN support Relay UE support. | L2 only | L3 only | Both L2 and L3 |
| --- | --- | --- | --- |
| Remote UE support Relay UE L3 Remote UE L3 only | No | Yes | Yes |
| Relay UE both L2 and L3 Remote UE both L2 and L3 | Yes | Yes | Yes (select L2 or L3) |

Therefore,

- From remote UE, relay UE and RAN capability support of relay architecture type, the following scenarios can be further considered in this WI:

- Remote UE supports L3 only and (Relay UE and RAN support L3)

- All Remote UE, Relay UE and RAN support both L2 and L3

# Conclusion

This paper further discussed L2 and L3 relay co-existence related scenarios and issues. The paper concludes with:

- Relay architecture support can be deployed:

- Per area or PLMN, i.e., each cell support only one relay architecture

- One relay architecture per cell, i.e., cell supports only one kind of relay architecture L2 or L3

- Full relay architecture per cell, i.e., cell supports both L2 and L3 architecture

- L3 Relay architecture support can be achieved with less specification and hardware impact with less impact from CN and RAN perspective.

- UE with both L2 and L3 relay support capability would address all relay deployment scenarios and service requirements, including service with less service continuity requirement or service with high continuity requirement

- In case Remote UE supports L3 only and Relay UE supports L3 relay architecture relaying scenarios can be considered as follows:

- Network supporting L3 architecture is a viable L3 relaying scenario

- Network supporting L2 architecture is NOT a viable L3 relaying scenario

- Network supporting both L2 and L3 architectures is a viable L3 relaying scenario

- In case Remote UE and Relay UE support both L2 and relay architecture relaying scenarios can be considered as follows:

- Network supporting L3 architecture is a viable L3 relaying scenario

- Network supporting L2 architecture is a viable L2 relaying scenario

- Network supporting both L2 and L3 architectures is a viable L2 or L3 relaying scenario

- The case where Remote UE and Relay UE have different relay architecture support capability is NOT viable L2 and/or L2 relaying scenario

- RAN2 to discuss which relay architecture co-existence to support, i.e.:

- Per area or PLMN, i.e., each cell support only one relay architecture

- One relay architecture per cell, i.e., cell supports only one kind of relay architecture L2 or L3

- Full relay architecture per cell, i.e., cell supports both L2 and L3 architecture

- UE with only L2 relay architecture support capability is NOT supported.

- The following UE relay support capabilities should be considered:

- UE with only L3 relay architecture support capability

- UE with both L2 and L3 architecture support capability

- From remote UE, relay UE and RAN capability support of relay architecture type, the following scenarios can be further considered in this WI:

- Remote UE supports L3 only and (Relay UE and RAN support L3)

- All Remote UE, Relay UE and RAN support both L2 and L3

# References

- RP-210904, New WID on NR Sidelink Relay