3GPP TSG-RAN WG2 Meeting #113 bis electronic                      	             R2-2103349

Online, April 12 – April 20, 2021

Agenda Item:	8.4.2

Source: 	vivo

Title:         	Discussion on miscellaneous issues in eIAB

Document for: 	Discussion and Decision

- Introduction

RAN2 discussed the issues to be concerned for Rel-17 eIAB, including fairness, latency and congestion aspects. The following issues on Enhancements to improve topology-wide fairness multi-hop latency and congestion mitigation were agreed at RAN2#113-e meeting [1]:

On topology-wide fairness - IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes) - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement. - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load) On multi-hop latency - IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16 - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free - IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel On congestion - IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion. - Both IC-1 and CI-7 are related to RAN3. RAN3 seems to also work on this, so to what extent R2 shall work on this is currently not clear.

From our perspective, fairness issues (IF-1 to IF-4) can be solved by implementation and do not require further specification, the RAN2 progress on congestion issues (IC-1 to IC-7) are pending RAN3’s agreements, therefore in this paper, we mainly focus on the latency issues, e.g., latency reference for scheduling and LCG extension, and some other issues (behaviours upon the reception of Type-2/3 RLF indication, flow control feedback enhancements).

- Discussion

  - latency reference for scheduling (IL-1)

A packet failed to arrive at the destination IAB-node or UE shall be discarded, or, alternatively shall be prioritized for transmission so that the latency can be fulfilled. Currently the intermediate IAB-nodes are not informed of the experienced latency of the packet during transmission, this issue was also identified as IL-1 and gained a significant interest for further studies.

To solve IL-1, two sub-issues should be considered:

- sub-issue 1: to which sublayer the experienced latency is indicated;

- sub-issue 2: from which information the experienced latency to be derived.

- sub-issue 1: to which sublayer the experienced latency is indicated

To enable the prioritization for scheduling with respect to latency, the latency related information should be indicated in the header of the PDU. In an IAB topology network, packets are routed between IAB-donor-CU and UE, traversing one or more intermediate IAB-node(s). An intermediate IAB-node is able to decode the received packet up to the level of BAP sublayer. Thus the latency of a packet could possibly be indicated in the header of BAP or RLC or MAC PDU.

- The latency of a packet could possibly be indicated in the header of BAP/RLC/MAC PDU.

One of the functionalities of BAP sublayer is to perform routing selection (including the selection of both BAP Path and BH RLC Channel), which in our understanding, can be closely associated with prioritization transmission strategy. For example, an intermediate IAB-node can preferentially transmit the packets with fewer PDB/hops (suppose this information is known to IAB-node) by selecting a more appropriate BAP Path ID and BH RLC channel ID, to ensure the packets arrive on time.

If the latency indicator is placed in the header of a BAP PDU, it should be much simpler and straight-forward compared to other options (in the header of RLC PDU or MAC PDU) as no inter-layer interaction is required.

- One of the functionalities of BAP sublayer is to perform routing selection (including the selection of both BAP Path and BH RLC Channel), which is closely associated with prioritization transmission strategy.

- If the latency indicator is placed in the header of a BAP PDU, it should be much simpler and straight-forward compared to other options (RLC PDU or MAC PDU header) as no inter-layer interaction is required.

As a result of all the above reasons combined, we propose:

The latency indicator should be placed in the header of BAP PDU.

- sub-issue 2: from which information the experienced latency to be derived

Subsequently, we need to consider what kind of latency indicator should be carried in IAB network, with the intention of assisting the IAB-node to perform dynamic scheduling, so that the overall or remaining PDU can be met for a packet. The most intuitive solution on the table is to indicate the remaining/overall PDB along with the remaining hops per packet.

Admittedly, the variable remaining hops of a packet to be transmitted plays a crucial role in adjusting the scheduling strategy for prioritization, and should be absolutely signaled per packet. However, we doubt if this variable really needs to be included in the header of BAP PDU, as 1) the field remaining hops adds extra overhead to each BAP PDU, in addition to the ones that can be used to derive experienced latency; and 2) the filed remaining hops requires constant update/modification at each intermediate IAB-node upon the reception of the to-be-transmitted BAP PDU (since the variable should be decreased by one at every transmission), which further introduces complexity to IAB-node operation.

- If the variable remaining hops of a packet is included in the header of BAP PDU, extra overhead to each BAP PDU is inevitable, additionally constant update/modification at each intermediate IAB-node also introduces complexity to IAB-node operation.

The remaining hops of a packet is actually relevant to both the destination IAB-node and the PATH ID, which is identical to BAP routing ID. With the per BAP routing ID granularity of remaining hops indication in mind, we spotted that it might work perfectly well if we extend the current BH Routing Configuration to include this variable, which is to add one more optional field (see Annex) for each corresponding BAP routing ID field for the indication of remaining hops, as this does NOT add any overhead to the BAP PDU and NOT require frequent update/modification to the BAP header at each intermediate IAB-node. With such an enhancement, the intermediate IAB-node receives a BAP PDU that does not belong to this IAB-node can read the BAP routing ID of the packet, and refers to the BH Routing Configuration to find out the corresponding remaining hops of this packet without much effort.

- The remaining hops of a packet transferred to an intermediate IAB-node can be derived based on the BAP routing ID carried within the BAP PDU.

- The solution that to extend the legacy BH Routing Configuration to include remaining hops as an optional field for each corresponding BAP routing ID field for the indication of remaining hops overrides the solution of including the information in the BAP header.

Nevertheless, the stage-3 design can be further discussed, we may first achieve the following understandings:

RAN2 to discuss whether the remaining hops of a packet should be indicated per BAP routing ID per IAB-node via BH Routing Configuration.

As we mentioned earlier, the most intuitive solution on the table is to indicate the remaining/overall PDB along with the remaining hops per packet. But from our point of view, the straight-forward indication on remaining/overall PDB is not an easy-to-achieve solution. Let’s say we decide to indicate the remaining/overall PDB of a packet in the BAP header of each BAP PDU, the intermediate IAB-nodes have to estimate the one-hop latency over the prior-hop transmission so that the nodes can derive the remaining PDB of this packet, we are concerned of the estimation accuracy of the delay over each BH link and the signaling cost intended for this use.

- Intermediate IAB-nodes have to estimate the one-hop latency over the prior-hop transmission to derive the remaining PDB of this packet, how to derive the experienced latency of a BAP PDU and the signalling cost intended for this use should be concerned.

Note that IIoT WI adopts a global time domain for the whole network to calculate the elapsed time of a transmitted packet over 5GS, and the elapsed time can be derived based on the timestamp (reference time) that the packet was delivered to 5GS and the time point that the packet is about to leave 5GS. In light of reference time indication introduced in IIoT WI, we may also consider a time domain mastered by IAB-donor-CU applies for the whole IAB network, and to include the variable reference time in the BAP header of a BAP SDU that comes from upper layers to represent the time point where this BAP PDU was generated. Since all IAB-nodes are under the same time domain, the local time of each node is closely synchronized. Thus, the intermediate IAB-node can obtain the experienced latency by subtracting the reference time that carries over the BAP header from the local time point that it receives the BAP PDU.

RAN2 to discuss whether a reference time (in a time domain mastered by the IAB-donor-CU) should be introduced for the experienced latency derivation for a BAP PDU.

  - LCG extension (IL-2)

LCG (logical channel group), consists of a group of logical channels configured by RRC, is used as the finest granularity for buffer status reporting. The number of LCHs/LCGs in LTE is 16/4, whereas the number of LCHs/LCGs in NR is 64/8. The length of LCID in IAB has been extended to an additional 16 bits, with the number of LCHs increased to 65535 at most, though, the number of LCGs remains unchanged at the level of 8. This undoubtedly will impact the scheduling granularity of IAB as now each LCG includes more logical channels that to be reported, which leaves more uncertainty to the parent IAB-node for LCH scheduling.

- Though the number of LCHs in increased from 64 to 65536, the maximum number of LCGs is still 8 in Rel-16 IAB.

- The small number of LCGs used for IAB node’s buffer status report granularity limits the scheduling performance improvement.

- Follow the same design on the number of LCHs/LCGs in LTE and NR, the number of LCGs for IAB-MT can be increased to 256. This large number of LCGs will, on the other hand, burdens the overhead of BSR. Consider the use of LCHs might not always approach the boundary (to always occupy 65535 LCHs), the number of LCGs shall be designed to be configurable by RRC, correspondingly, the BSR format for the configured number of LCGs shall also be pre-defined by the specification with a different one-octet eLCID.

The maximum number of LCGs for IAB-MT can be up to 256.

New BSR format(s) should be introduced if the maximum number of LCGs is increased.

The BSR format used by the IAB-MT is determined based on the number of LCGs configured.

  - behaviors upon reception of Type-2/3 RLF indication

It was also agreed that the behaviors of an IAB-MT upon reception of type-2/3 RLF indication should be specified, and one possible action the IAB-MT will perform is to trigger deactivation or reduction of SR and/or BSR transmissions once it receives Type-2 RLF indication.

On RLF indication - RAN2 to support type-2/3 RLF indication (FFS specified behavior(s) TS impact, FFS details). - Type-2 RLF indication may be used to trigger deactivation or reduction of SR and/or BSR transmissions

Since Pre-emptive BSR is also applicable to IAB-MT, we think the agreement that applies to BSR can be ported to Pre-emptive BSR as well.

Type-2 RLF indication may be used to trigger deactivation or reduction of Pre-emptive BSR transmissions.

Type-3 RLF indication represents that the previously failed BH link has been recovered, in case the SR/BSR/Pre-emptive BSR transmissions are deactivated rather than being cancelled directly, we believe the reception of type-3 RLF indication can also be used as a signal to activate the deactivated SR/BSR/Pre-emptive BSR.

Type-3 RLF indication can be used to activate/reactivate SR/BSR/Pre-emptive BSR procedure.

  - flow control feedback enhancements

There are two different feedback mechanisms specified for DL hop-by-hop FC in Rel-16, i.e., polling-based FC feedback and event-triggered FC feedback. If event-triggered FC is configured by IAB-donor-CU, and the buffer load of the IAB keeps growing or remains at high level after triggering FC feedback, as there is no prohibition period to avoid intensive feedbacks, subsequent FC feedback messages will be constructed and sent.

- If event-triggered FC is configured, the feedback can be triggered frequently once the buffer load of the IAB-node is above a pre-configured threshold for a period of time, as there is no restriction to avoid intensive feedbacks caused by buffer fluctuations.

Intensive FC feedback does not help the congestion resolution but to increase the FC feedback overhead. To avoid such intensive feedbacks, we propose:

RAN2 to support enhancements to avoid intensive feedbacks of event-triggered flow control scheme.

For both polling-based FC and event-triggered FC, the buffer status can be reported per BH RLC channel and or per routing ID via BAP control PDU. When a FC message is generated, the buffer status of all BH RLC channels and (or) BAP routing IDs should be included and sent to parent IAB-node.

- Once the FC feedback is triggered, the buffer status of all BH RLC CHs or BAP routing IDs are reported.

In the case that the FC feedback is reported per BH RLC channel, a total of 5 bytes (BH RLC channel ID & available buffer size) is required for each BH RLC channel. Likewise, a total of 6 bytes (Routing ID & available buffer size) are needed for each BAP route if the FC feedback is configured per BAP routing ID. RAN2 perceived that the number of BH RLC channels per backhaul link can be up to 216, straight-forwardly the size of the FC feedback will at most be 216×5 bytes (per BH RLC channel) or 220×6 bytes (per BAP routing) plus 1 byte BAP header.

The initial intention of constructing a FC feedback is to inform the parent IAB node of the congestion risk of the child IAB-node. With this in mind, it is quite counterproductive to carry the BH RLC CHs showing no congestion risk in FC feedback, as there are no obvious benefits but unnecessary overhead increase.

- Even if a large number of BH routes or BH RLC channels are congested, some enhancements are still needed to limit the FC message size to an acceptable level. Otherwise, a FC message with a large size might cause congestion in the upstream BH link.

- The size of FC feedback can be very large as the number of report fields is proportional to the number of configured BH RLC CHs and/or routing IDs.

- Based on the above discussion, enhancements on the formats of FC feedbacks should be discussed to reduce the unnecessary overhead caused by the inclusion of the congestion-risk-free BH RLC CHs or BAP routings.

For DL Hop by Hop FC, RAN2 consider enhancements to reduce the FC feedback message size.

- Conclusion

The observations and proposals are the following:

Observation 1	The latency of a packet could possibly be indicated in the header of BAP/RLC/MAC PDU.

Observation 2	One of the functionalities of BAP sublayer is to perform routing selection (including the selection of both BAP Path and BH RLC Channel), which is closely associated with prioritization transmission strategy.

Observation 3	If the latency indicator is placed in the header of a BAP PDU, it should be much simpler and straight-forward compared to other options (RLC PDU or MAC PDU header) as no inter-layer interaction is required.

Observation 4	If the variable remaining hops of a packet is included in the header of BAP PDU, extra overhead to each BAP PDU is inevitable, additionally constant update/modification at each intermediate IAB-node also introduces complexity to IAB-node operation.

Observation 5	The remaining hops of a packet transferred to an intermediate IAB-node can be derived based on the BAP routing ID carried within the BAP PDU.

Observation 6	The solution that to extend the legacy BH Routing Configuration to include remaining hops as an optional field for each corresponding BAP routing ID field for the indication of remaining hops overrides the solution of including the information in the BAP header.

Observation 7	Intermediate IAB-nodes have to estimate the one-hop latency over the prior-hop transmission to derive the remaining PDB of this packet, how to derive the experienced latency of a BAP PDU and the signalling cost intended for this use should be concerned.

Observation 8	Though the number of LCHs in increased from 64 to 65536, the maximum number of LCGs is still 8 in Rel-16 IAB.

Observation 9	The small number of LCGs used for IAB node’s buffer status report granularity limits the scheduling performance improvement.

Observation 10	If event-triggered FC is configured, the feedback can be triggered frequently once the buffer load of the IAB-node is above a pre-configured threshold for a period of time, as there is no restriction to avoid intensive feedbacks caused by buffer fluctuations.

Observation 11	Once the FC feedback is triggered, the buffer status of all BH RLC CHs or BAP routing IDs are reported.

Observation 12	The size of FC feedback can be very large as the number of report fields is proportional to the number of configured BH RLC CHs and/or routing IDs.

Proposal 1	The latency indicator should be placed in the header of BAP PDU.

Proposal 2	RAN2 to discuss whether the remaining hops of a packet should be indicated per BAP routing ID per IAB-node via BH Routing Configuration.

Proposal 3	RAN2 to discuss whether a reference time (in a time domain mastered by the IAB-donor-CU) should be introduced for the experienced latency derivation for a BAP PDU.

Proposal 4	The maximum number of LCGs for IAB-MT can be up to 256.

Proposal 5	New BSR format(s) should be introduced if the maximum number of LCGs is increased.

Proposal 6	The BSR format used by the IAB-MT is determined based on the number of LCGs configured.

Proposal 7	Type-2 RLF indication may be used to trigger deactivation or reduction of Pre-emptive BSR transmissions.

Proposal 8	Type-3 RLF indication can be used to activate/reactivate SR/BSR/Pre-emptive BSR procedure.

Proposal 9	RAN2 to support enhancements to avoid intensive feedbacks of event-triggered flow control scheme.

Proposal 10	For DL Hop by Hop FC, RAN2 consider enhancements to reduce the FC feedback message size.

- References

- Chairman notes, 3GPP WG2 Meeting #113-e, Online, Jan 25 – Feb 5, 2021

- Annex (extension on BH Routing Configuration)

The following is one example of how to indicate the remaining hops of a packet to each intermediate IAB-node.

#### 9.2.9.1	BAP MAPPING CONFIGURATION (TS 38.473 V16.4.0)

This message is sent by the gNB-CU to provide the backhaul routing information and/or traffic mapping information to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | - YES | - reject |
| Transaction ID | M |  | 9.3.1.23 |  | - YES | - reject |
| BH Routing Information Added List |  | 0...1 |  |  | - YES | - ignore |
| >BH Routing Information Added List Item |  | 1.. <maxnoofRoutingEntries> |  |  | - EACH | - ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - - |  |
| >>Next-Hop BAP Address | M |  | 9.3.1.111 | Indicates the BAP address of the next hop IAB-node or IAB-donor-DU. | - - |  |
| >>remaining hops | O |  |  | Indicates the remaining hops of the packet to be transferred with the BAP Routing ID. |  |  |
| >BH Routing Information Removed List Item |  | 1.. <maxnoofRoutingEntries> |  |  | - EACH | - ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - - |  |
| Traffic Mapping Information | O |  | 9.3.1.95 |  | - YES | - ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRoutingEntries | Maximum no. of routing entries, the maximum value is 1024. |