3GPP TSG RAN WG2 Meeting #113-bis-e       	                                            R2-2103383
Online, 12–20 April 2021

Agenda item:	8.11.3

Source:	Lenovo, Motorola Mobility

Title:	On Positioning in RRC_INACTIVE state

Document for:	Discussion and Decision

Introduction

The revised positioning enhancements WID was approved during the RAN#91-e meeting [1] and the following set of objectives in relation to RRC_INACTIVE positioning were introduced:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3, RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

This contribution provides further discussion points on enabling UE-based and UE-assisted positioning methods in RRC_INACTIVE state.

Positioning Methods and Measurements in RRC_INACTIVE state

## DL-only UE-Assisted/UE-Based Positioning Methods

The initial focus should be the support of both UE-assisted and UE-based positioning methods. The supported positioning methods could be differentiated based on DL-only RAT-dependent and RAT-independent positioning methods.

In the case of RAT-dependent methods at least DL-TDOA and DL-AoD positioning techniques should be supported for both UE-based and UE-assisted methods in RRC_INACTIVE state.  UE-assisted DL NR E-CID are also supported to a certain degree since SS-RSRP and SS-RSRQ measurements can already be measured in RRC_INACTIVE state.

Observation 1: UE-assisted DL NR E-CID measurements are supported in RRC_INACTIVE state at least in the case of SS-RSRP and SS-RSRQ measurements.

Proposal 1: Support at least both UE-assisted and UE-based measurements relating to DL-TDOA and DL-AoD positioning methods. Confirm existing UE-assisted support of DL NR E-CID measurements.

In the case of RAT-independent positioning methods, A-GNSS, Motion Sensor, WLAN and TBS are supported for both UE-based and UE-assisted operations and such support should also be extended to positioning in RRC_INACTIVE state. In addition, UE-assisted Bluetooth positioning can also be supported in RRC_INACTIVE state.

Proposal 2: Support the following the RAT-independent UE-assisted and UE-based positioning methods in RRC_INACTIVE state:  A-GNSS, Motion Sensor, WLAN, TBS and Bluetooth (UE-assisted only).

Regarding UL and UL+DL methods, it is not clear how the transmission of SRS for positioning will meet the SDT requirements in RRC_INACTIVE state due to the varying SRS configuration and therefore requires further study. The 2nd priority work for UL and UL+DL measurements can be maintained as outlined in the WID pending the completion of DL-only positioning support in RRC_INACTIVE.

## Support of DL-only Measurements

According to 38.215 [2], the following DL-only measurements have been specified to support DL-AoD and DL-TDOA methods in Rel-16:

Table 1: DL PRS reference signal received power (DL PRS-RSRP)

| Definition | DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

Table 2: DL reference signal time difference (DL RSTD)

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the positioning node j and the reference positioning node i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from positioning node j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from positioning node i that is closest in time to the subframe received from positioning node j. Multiple DL PRS resources can be used to determine the start of one subframe from a positioning node. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency |

Currently, these intra-frequency and inter-frequency positioning measurements are only supported by the UE in RRC_CONNECTED state. Ideally, these DL-only measurement requirements should also be equally extended to UEs operating in RRC_INACTIVE state. This is to maintain the location accuracy requirements set by the internal/external LCS client at the LMF/UE with respect to the positioning measurements, however there is a potential trade-off in terms of the duration and periodicity of these measurements in RRC_INACTIVE state and the location estimate accuracy, which may be different from the RRC_CONNECTED measurement requirements due to the energy consumption constraints. More specifically, the LMF may need to limit the number, duration and periodicity of DL-PRS occasions transmitted in RRC_INACTIVE state due to the energy consumption constraints.

Observation 2: DL-only measurement durations and periods may be different between RRC_CONNECTED and RRC_INACTIVE states due to energy consumption constraints, which may affect the overall location accuracy.

The LMF should also be cognizant or aware of the operating state of the UE (e.g. RRC_CONNECTED or RRC_INACTIVE state) by adapting the associated DL-PRS configuration (e.g. periodicity, bandwidth, number of occasions, comb pattern). This implies a certain degree of RRC state awareness by the LMF and moves away from the notion that the LMF should be state transparent. The serving gNB and LMF may share such UE RRC state information to further optimize the DL-PRS configuration based on the operating state of the UE, which may have RAN3 impacts.

Proposal 3: LMF should adapt the DL-only positioning assistance data based on measurement requirements of the UE depending on if the UE positioning measurements are performed in RRC_CONNECTED or RRC_INACTIVE state.

However, it is up to RAN4 to decide whether the RRC_INACTIVE measurement requirements are similarly extended based on the existing measurement requirements in RRC_CONNECTED state. The measurement requirement would have an impact on the achievable accuracy.

Proposal 4: Send an LS to RAN4, Cc RAN1 indicating at least the need for support and measurement requirements for the following DL-only positioning measurements in RRC_INACTIVE state:

- DL PRS-RSRP

- DL Reference Signal Time difference (DL RSTD)

Support of enhanced LPP Procedures in RRC_INACTIVE State

## SDT and Measurement/Location estimate response

The support for RRC_INACTIVE state positioning, especially measurement reporting can be enabled through close coordination with the concurrent SDT WID [3], which has been updated to include specification support for the transfer of UL NAS messages via SRB2 as described by the following extract in the updated WID: “Specify configuring of SRB1 and SRB2 for small data transmission in RRC_INACTIVE state by reusing the framework for DRBs.”. Small UL data transmissions are to be specified based on the following key objectives of the SDT WI:

- UL small data transmissions for RACH-based schemes (i.e. 2-step and 4-step RACH)

- Transmission of UL data on pre-configured PUSCH resources (i.e. reusing the configured grant type 1) – when TA is valid

This opens up the possibility of utilizing the SDT framework to transmit the positioning measurement report or location estimate via UL NAS messages. This would be based on the agreements made during the SDT WI session.

During the RAN2#113-e meeting [4] the following agreements were made, which confirms the support of existing procedures to support RRC_INACTIVE positioning in terms of assistance data delivery and measurement/location estimate request LPP messages:

Proposal4: For DL positioning in IDLE/INACTIVE, the followings are already supported for the current spec and can be reused:

	Current stage3 spec has already supported assistance data delivery for DL positioning during RRC_CONNECTED and on-demand SI request in RRC_IDLE/ INACITVE for IDLE/INACTIVE positioning. (14/14)

	Current stage3 spec already supports the transfer of RequestLocationInformation in RRC_CONNECTED for PRS measurement in IDLE/INACTIVE. (14/14)

Proposal5: Support RAT-Independent positioning in RRC_IDLE/INACTIVE. FFS the procedures that can be supported. (13/14)

Therefore, based on the above agreements a partial set of LPP procedures are already supported to enable the configuration and appropriate location measurement or location estimate request to a certain degree:

- RequestLocationInformation message received in RRC_CONNECTED state, which may be valid once the UE switches to RRC_INACTIVE state.

- Broadcast and On-demand SI request for a posSIB is supported for RRC_INACTIVE state.

The remaining component is ProvideLocationInformation LPP message (shown in Figure 1), can include the UE transmission of measurement reports (UE-assisted methods) based on the configured positioning technique or UE’s location estimate (UE-based methods). It can be observed that the measurement report can be quite large (approximately > 1000 bits) depending on the whether RAT-dependent and RAT-independent positioning techniques are to be provided and which measurements are configured.

-- ASN1START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideLocationInformation-r9	ProvideLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

...,

[[

sensor-ProvideLocationInformation-r13

Sensor-ProvideLocationInformation-r13

OPTIONAL,

tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

]],

[[	nr-ECID-ProvideLocationInformation-r16

NR-ECID-ProvideLocationInformation-r16		OPTIONAL,

nr-Multi-RTT-ProvideLocationInformation-r16

NR-Multi-RTT-ProvideLocationInformation-r16 OPTIONAL,

nr-DL-AoD-ProvideLocationInformation-r16

NR-DL-AoD-ProvideLocationInformation-r16	OPTIONAL,

nr-DL-TDOA-ProvideLocationInformation-r16

NR-DL-TDOA-ProvideLocationInformation-r16	OPTIONAL

]]

}

-- ASN1STOP

Figure 1: ProvideLocationInformation Message [5]

Observation 3: In certain cases, the configured measurement report to the network may exceed the data volume threshold to support SDT RACH-based or CG-based transmissions in RRC_INACTIVE state.

A key issue is the size limitation (data volume threshold) of the SDT UL transmission for both RACH-based and pre-configured PUSCH resources (which is intended for small infrequent user data transmissions) in relation to the size of the positioning measurement reports normally transmitted in RRC_CONNECTED state. To overcome this issue and provide flexibility for UE transmission of the measurement report in RRC_INACTIVE state, LPP segmentation of the measurement report can be considered to segment the measurement report into manageable segments that fulfill the data volume threshold for an SDT transmission using SRB2. The measurement report segments are to be self-decodable at the LMF, while fulfilling the size requirements of the SDT RACH-based or CG-transmissions UL transmissions.  The segmentation criteria can for example, depend on the type of configured positioning techniques, e.g. RAT-dependent or RAT-independent positioning techniques required by the LMF in RRC_INACTIVE state, which is inline with the configuration format presented in the ProvideLocationInformation message.

Proposal 5: RAN2 supports LPP segmentation of the measurement report for large measurement report size exceeding the data volume threshold size to support SDT transmissions in RRC_INACTIVE state based on a set of criteria. FFS threshold and segmentation criteria for the measurement report.

## Capability Aspects

An issue which was raised during the [Post112-e][609] email discussion [6], was related to the exchange of the PRS capability between the LMF and UE during RRC_INACTIVE state and whether the LMF can store the UE’s capability based on an existing LPP session.

The differing positioning capabilities may be distinct among different UEs supporting RRC_CONNECTED positioning and/or RRC_INACTIVE positioning and thus a differentiation should be also be made in the capability indication between UEs supporting the two types of positioning capabilities. For example, a high-end positioning capable UE with relaxed energy requirements may perform most positioning procedures in RRC_CONNECTED state while a lower-end/reduced capability UE may be designed to exploit positioning procedures while operating in RRC_INACTIVE state.

Proposal 6: RAN2 to confirm if the capability information among UEs performing RRC_CONNECTED or RRC_INACTIVE positioning will be varying and whether special capability indication is necessary.

## Low Latency posSIB Updates

Currently, a UE/group of UEs performing positioning may receive the DL-PRS configuration via posSIBs in the RRC_IDLE/RRC_INACTIVE state. The current value tag indicating posSIB change is limited in terms of the delay in notifying the UE of a posSIB change. The UE may also acquire low latency posSIB updates while in RRC_IDLE/RRC_INACTIVE state using a paging DCI received from the gNB. To enable this, the gNB directly informs the UE using the Paging DCI if there’s an update available for the posSIB(s). Also, the gNB can indicate if the existing scheduling for such a posSIB remains valid or if the same has also been modified (based on signalling exchanges with the LMF using NRPPa).

Proposal 7: RAN2 to consider low latency change notifications of posSIBs for UEs performing RRC_INACTIVE/RRC_IDLE positioning, e.g. using paging and SI change notification messages.

For measurements and/or reporting performed in the RRC_CONNECTED state, it would be beneficial for the UE to autonomously release it RRC connection, depending on if the UE is required to perform measurements in RRC_INACTIVE state. The NG-RAN and LMF may require coordination in order to enable the UE to autonomously release the RRC connection for the purposes of positioning-related procedures, e.g. performing measurements and reporting in RRC_INACTIVE state.

Proposal 8: Support UE autonomous RRC release indication for UEs performing RRC_INACTIVE state positioning. FFS RAN3 impacts to NRPPa.

Conclusions

The observations can be summarized as follows:

Observation 1: UE-assisted DL NR E-CID measurements are supported in RRC_INACTIVE state at least in the case of SS-RSRP and SS-RSRQ measurements.

Observation 2: DL-only measurement durations and periods may be different between RRC_CONNECTED and RRC_INACTIVE states due to energy consumption constraints, which may affect the overall location accuracy.

Observation 3: In certain cases, the configured measurement report to the network may exceed the data volume threshold to support SDT RACH-based or CG-based transmissions in RRC_INACTIVE state.

The following proposals are presented in relation to supporting RRC_INACTIVE positioning:

Proposal 1: Support at least both UE-assisted and UE-based measurements relating to DL-TDOA and DL-AoD positioning methods. Confirm existing UE-assisted support of DL NR E-CID measurements.

Proposal 2: Support the following the RAT-independent UE-assisted and UE-based positioning methods in RRC_INACTIVE state:  A-GNSS, Motion Sensor, WLAN, TBS and Bluetooth (UE-assisted only).

Proposal 3: Proposal 3: LMF should adapt the DL-only positioning assistance data based on measurement requirements of the UE depending on if the UE positioning measurements are performed in RRC_CONNECTED or RRC_INACTIVE state.

Proposal 4: Send an LS to RAN4, Cc RAN1 indicating at least the need for support and measurement requirements for the following DL-only positioning measurements in RRC_INACTIVE state:

- DL PRS-RSRP

- DL Reference Signal Time difference (DL RSTD)

Proposal 5: Proposal 5: RAN2 supports LPP segmentation of the measurement report for large measurement report size exceeding the data volume threshold size to support SDT transmissions in RRC_INACTIVE state based on a set of criteria. FFS threshold and segmentation criteria for the measurement report.

Proposal 6: RAN2 to confirm if the capability information among UEs performing RRC_CONNECTED or RRC_INACTIVE positioning will be varying and whether special capability indication is necessary.

Proposal 7: Study signalling mechanisms for low latency notifications of posSIB changes/updates for UEs performing RRC_INACTIVE/RRC_IDLE positioning.

Proposal 8: Support UE autonomous RRC release indication for UEs performing RRC_INACTIVE state positioning. FFS RAN3 impacts to NRPPa.

References

- RP-210903, “Revised WID on NR Positioning Enhancements”, CATT, Intel Corporation, 3GPP RAN#91-e, March 2021.

- TS 38.215, “Physical layer measurements (Release 16)”, V16.1.0, March 2020.

- RP-210870, “Updated Work Item on NR small data transmissions in INACTIVE state”, ZTE, 3GPP RAN#91-e, March 2021.

- R2-2101955, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), Jan./Feb. 2021.

- TS 37.355, “LTE Positioning Protocol (LPP)”, V16.3.0, December 2020.

- R2-2101230, “[Post112-e][609][POS] Positioning support in RRC_IDLE and  RRC_INACTIVE (Huawei)”, RAN2#113-e, Jan./Feb. 2021.