3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103385

Online, April 12 – April 20, 2021

Agenda item:	 8.13.2.1

Source:		Lenovo, Motorola Mobility

Title:		SON Enhancements for CHO

Document for:		Discussion and Decision

1	Introduction

In RAN2#112e [1], the SON for Conditional Handover (CHO) was discussed, and the following agreements were achieved:

Agreements:

The following time information is as part of the UE RLF report:

Time between the first CHO execution and the corresponding CHO command received at UE at least in the CHO failure case.

Agreements:

The following cells’ related cell and beam measurements are included in the RLF report associated to CHO failure:

a.	Source cell of the CHO. FFS the detail on cell ID. Try our best to reuse the existing information.

b.	The target cell towards which the CHO was executed, if CHO related condition was satisfied. FFS the detail on cell ID. Try our best to reuse the existing information.

c.	The cell in which the re-establishment is performed after the CHO failure or source RLF. Try our best to reuse the existing information. FFS on the related measurements.

Agreements:

RLF-report shall contain information to differentiate an ordinary HO failure from the CHO failure and CHO recovery failure. FFS: implicit indication vs explicit indication.

Focused scenarios:

In case of successive CHO related failures, the UE stores and reports both RLF related information in the RLF report. The successive failure referred above, includes at least the following scenarios.

a.	A UE that has CHO configuration declares RLF in the source cell. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell.

b.	A UE that has CHO configuration executes the CHO towards the target cell upon fulfilling the configured condition and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell.

c.	A UE that has CHO configuration executes the normal HO towards the target cell and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE fails to re-establish to the selected CHO candidate cell using CHO procedure.

Note: other scenarios still can be discussed.

In RAN2#113e [2], further agreements were achieved:

Agreements:

1	Include in the RLF report the “Time elapsed since CHO execution until connection failure”. How to convey this information is FFS.

2	Reuse the following legacy timers in the RLF report also for CHO: timeUntilReconnection, timeSinceFailure.

3	In the RLF report for CHO, the UE includes of the latest radio measurement results. FFS: to indicate whether or not it is candidate target cell.

Signalling model for RLF report:

FFS:	Separate IEs/fields within the existing RLF-report are used to represent the second HOF. Also consider the second HO is successful case together. What measurements also need to be considered.

Agreements:

UE reports "Time elapsed since CHO execution until connection failure" implicitly or explicitly, i.e. UE either explicitly provides the aforementioned timing information or provides sufficient information for the network to compute it.

=>	Continue the discussion ”UE shall include the latest radio measurement results of the candidate target cells in the RLF-report.” through email. (Ericsson)

=>	Before agreeing on including an indication indicating whether a neighbor cell, included as part of neighbor cell measurement result, is associated to a CHO candidate target cell or not, RAN2 waits RAN3 to confirm whether the source cell can keep the UE context, at least up to the point the RLF-report is received by the source cell. Draft LS to RAN3 for this.

In this paper, we would further discuss the details of MRO for CHO.

2	Discussion

In R16, CHO recovery procedure for RLF/HO failure/CHO failure was introduced, i.e. when RLF occurs in the source gNB or initial CHO execution fails or normal HO fails, the UE performs cell selection for re-establishment, and if the selected cell is a target candidate cell and if network configured the UE to try CHO recovery after RLF/HO failure/CHO failure, then the UE attempts a second CHO execution, if the selected cell is not a target candidate cell or the second CHO execution fails, re-establishment procedure is performed.

  - 2.1 Time related information reporting

RAN3#110e meeting [3] has agreed to report the time elapsed since CHO execution until connection failure to network. RAN2#113e meeting [2] has agreed that UE reports "Time elapsed since CHO execution until connection failure" implicitly or explicitly, i.e. UE either explicitly provides the aforementioned timing information or provides sufficient information for the network to compute it.

For implicit solution, the network needs to derive "Time elapsed since CHO execution until connection failure" via “Time between connection failure and the corresponding CHO command received at UE” and “Time between the first CHO execution and the corresponding CHO command received at UE”, RAN2#112e meeting [1] has agreed that UE reports “Time between the first CHO execution and the corresponding CHO command received at UE”, but how to represent “Time between connection failure and the corresponding CHO command received at UE” is not clear.

The key point is how to understand the TimeConnFailure IE if it is reused for CHO, e.g. for implicit way it can be reused to represent “Time between connection failure and the corresponding CHO command received at UE”, for explicit solution it can be reused to represent “Time elapsed since CHO execution until connection failure”. Currently, the existing TimeConnFailure IE in legacy is used to indicate the time elapsed since the last HO initialization until connection failure. In CHO, handover procedure is initialized/performed when CHO execution condition is fulfilled. So, the straightforward and simple way is to reuse the TimeConnFailure IE to indicate the time elapsed since initial CHO execution until connection failure with some field description updates.

Proposal 1: Reuse the existing IE i.e. TimeConnFailure to indicate the time elapsed since initial CHO execution until connection failure with updates for field description if necessary.

CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.Since CHO execution condition(s) can be modified before the first CHO execution, whether CHO execution condition(s) is modified or not will impact the timing calculation. For example, the time of receiving the initial CHO configuration including candidate cell#1 configuration and the corresponding execution condition is noted as T0. Then, the UE may receive the second CHO configuration to only modify the execution condition associated with this candidate cell#1. we note this timing as T1. We should further discuss whether the UE reports the time between the first CHO execution and the corresponding initial CHO configuration (T0), or the time between the first CHO execution and the corresponding latest CHO configuration with the modified execution condition (T1), or both. Since it is the latest CHO execution condition determines the CHO execution, it is reasonable to report the time between the first CHO execution and the corresponding latest CHO configuration.

Furthermore, there is no existing IE can be reused for this time information, so a new IE can be introduced in the RLF report to indicate the time between the first CHO execution and the corresponding latest CHO configuration.

Besides CHO failure case, for CHO success case, the time elapsed since receiving the CHO configuration until the CHO execution is also useful for the source gNB to decide the suitable moment to provide the CHO configuration to the UE, because if the CHO configuration is provided too early to the UE, it makes unnecessary radio resource reservation in the target gNB.

Proposal 2: The UE reports the time between the first CHO execution and the corresponding latest CHO configuration received for the selected target cell received at UE.

  - 2.2 Candidate Cell List

In RAN3#110e meeting [3], it was agreed that the source node needs to know the candidate cell list, since it is beneficial for the source gNB decide whether the failure is due to improper CHO execution condition(s) or unsuitable target candidate cell(s). There is a possibility that the UE context in the source node is already released when RLF-report is received by the source node, e.g. an RLF occurs shortly after the successful handover. If so, how the source node achieves the candidate cell list needs to be solved. There are two alternatives on table:

Alternative 1: UE includes candidate cell list in RLF Report.

Alternative 2: Source nodes sends the info to the target node.

Since CHO candidate cell list can be modified during CHO preparation phase, alternative 2 would cause Xn signalling/resources waste if CHO candidate cell list is updated frequently, also it would impact Xn interface no matter the candidate cell list is transferred via the existing or new defined Xn message. So Alternative 1 is preferred.

Observation 1: The UE context in the source node may be released when RLF-report is received by the source node.

Proposal 3: The UE reports the CHO candidate cell list to the network in the RLF-Report.

  - 2.3 CHO execution condition(s)

Similar as CHO candidate cell list, it was also agreed that the source node needs to know the CHO execution condition(s) especially when the UE context is released. The two alternatives are as below:

Alternative 1: UE includes CHO execution condition(s) in RLF Report.

Alternative 2: Source nodes sends the info to the target node.

In CHO, it is the source gNB which decides the CHO execution condition(s), and obviously it is the source gNB to judge whether CHO execution condition(s) are set properly or not. Sending CHO execution condition(s) to the target gNB by the source gNB seems not necessary, and if so Xn signalling would be impacted to transfer this information. Reporting the CHO execution condition(s) for all candidate target cells including the target cell whose CHO execution is fulfilled and the non-target cell(s) is preferred, thus the source gNB can get the CHO execution condition(s) included in the RLF report.

Proposal 4: The UE reports the CHO execution condition(s) of all the configured candidate target cells including the target cell whose CHO execution is fulfilled and the non-target cell(s), e.g. A3 and/or A5 Event configuration and corresponding TTT value, to the network in the RLF-Report.

Besides CHO execution condition(s) for the configured candidate target cell(s), latest radio measurement results are also useful for the network to optimize CHO configuration(s). Thus, the UE can report the latest radio measurement results of the configured candidate target cell(s) in the RLF-Report.

Proposal 5: The UE reports the latest radio measurement results of the candidate target cell(s) in the RLF-Report.

If it is agreed that the UE reports CHO candidate cell list in the RLF-Report, it is redundant to report an indication of whether a measured cell in the RLF-Report is a CHO candidate cell or not since the source node can decide based on the received CHO candidate cell list.

  - 2.4 CHO indication

The issue on whether CHO failure indication is implicit or explicit is FFS. Even though candidate cell list, CHO execution condition(s) or CHO specific time information, e.g. time between the initial CHO execution and the corresponding CHO configuration, can implicitly indicate CHO failure. To enable the network know it is CHO failure clearly and distinguish it from normal HO /DAPS HO failure, an explicit CHO failure indication is needed.

Proposal 6: The UE can report an explicit CHO failure indicator to the network in the RLF-Report.

Since CHO recovery is performed after CHO failure or normal HO failure or RLF, to make the network configure suitable candidate target cell(s), an indication of whether the cell after CHO failure or normal HO failure or RLF is a CHO candidate target cell or not can be reported. For example, if the cell for CHO recovery is a CHO candidate target cell, the network can configure this cell as a CHO candidate target cell.

Proposal 7: The UE can report an indication of whether the cell after CHO failure or normal HO failure or RLF is a CHO candidate target cell or not.

  - 2.5 Signalling model

RAN3 #110e meeting [3] agreed that the UE reports information related with the two failures if UE has experienced failure twice. Also RAN2#112e meeting [1] agreed the UE stores and reports both two RLF related information in the RLF report. The issue is how to report these two failures’ related information. The potential solutions are summarized in [4]:

- A: Two separate entries in the RLF report are used, i.e. one entry is used to represent measurements/parameters related to the first HOF, the second one is used to represent measurements/parameters related to the second HOF.

- B: Separate IEs within the existing RLF-report are used to represent the second HOF. The first HOF can be represented by reusing as much as possible existing IEs.

- C: Two separate RLF reports are introduced, one containing IEs related to the first HOF, the other one containing IEs related to the second HOF.

- D: Too early to decide.

Option B has less spec impact, and we can make the final decision when the state for CHO specific parameters to be reported are stable.

Proposal 8: Use separate IEs within the existing RLF-report to represent the second failure, and the first failure can be represented by reusing as much as possible existing IEs.

  - 2.6 other scenarios

- Besides the scenarios agreed in RAN2#112e meeting [1], there are more cases which should also be considered:

RLF/HO Failure/CHO Failure with CHO Recovery Success

As shown in the Figure 1, a UE that has CHO configuration declares RLF in the source cell or executes the CHO towards the target cell upon fulfilling the configured condition and experiences a HO failure or executes the normal HO towards the target cell and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE re-establishes to the selected CHO candidate cell using CHO procedure successfully. In this case, the UE indicates rlf-InfoAvailable in the RRCReconfigurationComplete message. Then, the UE may send the RLF-Report IE corresponding to the RLF/HO Failure/CHO Failure to the network. The RLF-Report should include the RLF/HO Failure/CHO Failure related information e.g. failed cell information, time information. Also, the UE may send successful CHO recovery related information to the network.

Figure 1 CHO Failure with CHO Recovery Success

Proposal 9: In case of RLF/HO Failure/CHO Failure with CHO Recovery Success, besides RLF/HO Failure/ CHO Failure related information, UE also stores and reports successful CHO recovery related information to the network.

- For CHO recovery, if the cell for CHO recovery is a CHO candidate target cell, this cell may only meet the S-criteria rather than the CHO execution condition. That means handover is performed even the execution condition is not met. It could be helpful for network to be aware whether the execution condition is met or not in the case that UE successfully performs CHO recovery. Therefore, the information of the successful CHO recovery should be reported to the network.

Proposal 10: Whether the execution condition associated with CHO recovery cell is met or not should be reported in the case that UE successfully performs CHO recovery.

RLF/HO Failure/CHO Failure with CHO Recovery Success followed by an RLF

As shown in the Figure 2, a UE that has CHO configuration declares RLF in the source cell or executes the CHO towards the target cell upon fulfilling the configured condition and experiences a HO failure or executes the normal HO towards the target cell and experiences a HO failure. The UE selects for connection re-establishment a configured candidate CHO target cell. The UE re-establishes to the selected CHO candidate cell using CHO procedure successfully. And then the UE declares RLF shortly and triggers normal RRC Re-establishment procedure.

Figure 2 CHO Failure with CHO Recovery Success followed by an RLF

- MRO aims at detecting and enabling correction of connection failure due to mobility. It makes sense to report the all failure cases during mobility e.g. to report the first CHO failure so that the network can adjust the CHO trigger conditions for the first failed cell and report the second RLF failure so that the network can adjust the CHO trigger conditions for the recovery cell.

Proposal 11: In case of RLF/HO Failure/CHO Failure with CHO Recovery Success followed by an RLF, the UE stores/reports both failure information of the first failure (initial RLF/HOF/initial CHO execution failure) and another failure (RLF failure) related information to the network.

3	Conclusion

In this contribution, the issues on SON enhancements for CHO are discussed. We have the following observation and proposals:

Observation 1: The UE context in the source node may be released when RLF-report is received by the source node.

Proposal 1: Reuse the existing IE i.e. TimeConnFailure to indicate the time elapsed since initial CHO execution until connection failure with updates for field description if necessary.

Proposal 2: The UE reports the time between the first CHO execution and the corresponding latest CHO configuration received for the selected target cell received at UE.

Proposal 3: The UE reports the CHO candidate cell list to the network in the RLF-Report.

Proposal 4: The UE reports the CHO execution condition(s) of all the configured candidate target cells including the target cell whose CHO execution is fulfilled and the non-target cell(s), e.g. A3 and/or A5 Event configuration and corresponding TTT value, to the network in the RLF-Report.

Proposal 5: The UE reports the latest radio measurement results of the candidate target cell(s) in the RLF-Report.

Proposal 6: The UE can report an explicit CHO failure indicator to the network in the RLF-Report.

Proposal 7: The UE can report an indication of whether the cell after CHO failure or normal HO failure or RLF is a CHO candidate target cell or not.

Proposal 8: Use separate IEs within the existing RLF-report to represent the second failure, and the first failure can be represented by reusing as much as possible existing IEs.

Proposal 9: In case of RLF/HO Failure/CHO Failure with CHO Recovery Success, besides RLF/HO Failure/ CHO Failure related information, UE also stores and reports successful CHO recovery related information to the network.

Proposal 10: Whether the execution condition associated with CHO recovery cell is met or not should be reported in the case that UE successfully performs CHO recovery.

Proposal 11: In case of RLF/HO Failure/CHO Failure with CHO Recovery Success followed by an RLF, the UE stores/reports both failure information of the first failure (initial RLF/HOF/initial CHO execution failure) and another failure (RLF failure) related information to the network.

Reference

[1] RAN2#112e chairman notes.

[2] RAN2#113e chairman notes.

[3] RAN3#110e chairman notes.

[4] R2-2101451, [Post112-e][853][NR R17 SON/MDT] R17 Information needed in UE report for CHO cases. Ericsson