3GPP TSG-RAN WG2 Meeting #113bis-e			     R2-2103418

Electronic meeting, 12 April – 20 April, 2021

Agenda Item:	8.4.2 (NR_IAB_enh-Core)

Source: 	LG Electronics Inc.

Title:         	Further consideration on identified issues for fairness, latency and congestion

Document for: 	Discussion and Decision

Introduction

In the RAN2#113e meeting, the identified issues are finally agreed to further study. This document discusses the identified issues for fairness, latency and congestion and presents our views on each issue.

Discussion

Topology-wide fairness

The below three issues were identified and agreed for topology-wide fairness at the last RAN2 meeting.

ISSUES: eIAB work on topology-wide fairness will focus on the following issues - IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes) - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement. - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load)

In IF-1, the scheduler of the IAB node wants to use the overall route link quality for making appropriate scheduling decisions. The proponents think topology-wide fairness may be improved by using the overall route link quality because every scheduling decision can be adjusted by the link quality across multiple hops. However, even though an IAB node takes into account the overall route link quality to make proper scheduling decision, this scheduling decision may turn out a bad decision when the scheduled data is actually arrived at multiple hops away descendent or ascendant IAB node because the link quality can be changed a lot compared to the link quality which was used for scheduling decision at the multiple hops away IAB node. In addition, since link quality is frequently changed in general, if link quality of multiple hops is taken into account, every scheduling decision may need up to date information of multiple hops link quality to reflect the frequently changed link quality of multiple hops and this causes inevitable high signaling overhead.

Observation 1. Since link quality across multiple hops is frequently changed, scheduling decision based on link quality across multiple hops may make a bad scheduling decision and cause inevitable high signalling overhead.

Someone may argue that the link quality may not be changed frequently compared to legacy UE scenarios because the IAB network is well deployed and managed by the operator. If so, the link quality across multiple hops may not be changed a lot. This means that the additional information for overall route link quality would be almost same in most case. Thus, we doubt whether this overall route link quality is actually needed to make optimized scheduling decision. In this case, it is sufficient to use per hop QoS parameters on each BH RLC/LCH as in Rel-16 rather than overall route link quality for scheduling decision.

Observation 2. If the link quality across multiple hops may not be changed frequently due to well deployed IAB network, link quality across multiple hops may be useless and per hop QoS parameters on each BH RLC/LCH as in Rel-16 may be sufficient for scheduling decision.

For IF-2, given that Rel-16 IAB is based on centralized configuration by the IAB donor CU, when UEs want to receive a service, the IAB donor CU can know where each UE is attached to the IAB network and how many hops are required to reach each UE. Then, the IAB donor CU can figure out how to satisfy QoS for each UE’s service all over the path. Based on this understanding, since LCH (logical channel) priority is related to QoS on each hop, the IAB donor CU can configure/allocate proper LCH priority at each node on the path depending on the location of a UE and required QoS of the service. Furthermore, the IAB donor CU may also consider the number of DRBs allocated on one BH RLC CH (i.e., LCH) and the number of ascendant or descendant IAB nodes when configuring LCH priority at each node. We think this is network configuration and it is already possible in Rel-16 IAB.

Observation 3. Rel-16 IAB network configuration can consider the number of hops, DRB’s QoS, the number of DRBs on one BH RLC channel, and the number of descendant IAB nodes, when per hop QoS parameter (e.g., LCH priority) is determined and configured to the IAB node.

With observation 3, BH RLC channels carrying UE bearers with same or similar QoS requirements would be balanced and may not be congested in normal case. If there is a BH RLF or a beam blockage, this may cause congestion, but this kind of events will block all transmission of BH RLC channels, not a specific BH RLC channel. Thus, we think that if there is congestion on specific BH RLC channels carrying UE bearers with same or similar QoS requirement, it would be caused by bad IAB implementation or configuration.

Observation 4. If there is a BH RLF or a beam blockage, this may cause congestion, but this kind of events will block all transmission of BH RLC channels, not a specific BH RLC channel.

Observation 5. In normal case, BH RLC channels carrying UE bearers with same or similar QoS requirements would be balanced and may not be congested. However, if bad IAB implementation/configuration may cause congestion on specific BH RLC channels carrying UE bearers with same or similar QoS requirement.

For IF-4, we actually have a strong concern on this issue and want to know what the problem is here. As addressed in observation 3, normally when a DRB is established, the IAB donor CU can know traffic load of DRB including required QoS and then configures higher LCH priority and parameters to give more resource to those BH RLC CHs that aggregate more bearers, if needed. For example, suppose that traffic load of all bearers are same and BH RLC CH 1 has 2 bearers and BH RLC CH 2 has 4 bearers, the IAB donor CU will configure higher priority on BH RLC CH 2 to give more resource than BH RLC CH 1. As another example, when traffic load of each bearer is different and BH RLC CH 1 has 2 bearers and BH RLC CH 2 has 4 bearers, but if total traffic load of BH RLC CH 1 and 2 are same, in this case, even though BH RLC CH 2 has more bearer than BH RLC CH 1, the IAB donor CU does not need to configure higher priority on BH RLC CH 2 and same priority would be enough to support same traffic load on each BH RLC CH. In the same assumption that BH RLC CH 1 has 2 bearers and BH RLC CH 2 has 4 bearers. If total traffic load of BH RLC CH 1 is two time heavier than BH RLC CH 2, even though BH RLC CH 2 has more bearer than BH RLC CH 1, the IAB donor CU needs to configure higher priority on BH RLC CH 1 than BH RLC CH 2. Thus, we think that just giving more resource to those BH RLC CHs that aggregate more bearers may be bad impact to topology-wide fairness and the current Rel-16 configuration would already consider fairness aspect for all traffic loads of each BH RLC CH when they are configured.

Observation 6. Just giving more resource to those BH RLC CHs that aggregate more bearers may be bad impact to topology-wide fairness and the current Rel-16 configuration would already consider fairness aspect for all traffic loads of each BH RLC CH when they are configured.

Based on observation 1 through 6, we hesitate to further study IF-1, IF-2, and IF-4 and think that Rel-16 IAB configuration can resolve all these issues.

Proposal 1. RAN2 confirms that existing Rel-16 IAB configuration can handle IF-1, IF-2, and IF-4 and provide topology-wide fairness.

Multi-hop latency

The followings were made for focusing on issues on multi-hop latency at the last RAN2 meeting.

ISSUES: In the first instance, eIAB work on multi-hop latency will focus on the following issues: - IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16 - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free - IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel

For IL-1, when a DRB is established over the IAB network, the IAB donor CU can know whole topology of IAB network and how many hops are required for this DRB. Hence, the IAB donor CU can estimate the required PDB for each hop and configure BH RLC channel for this DRB according to the estimated one hop PDB. Of course, the experienced one hop PDB may vary with IAB node circumstance, but variance of the experienced one hop PDB would be limited in the normal operation. We think that this variance would be large only when unexpected events occurs such as BH RLF and data congestion. However, there are already Rel-16 solutions to handle this unexpected events and further enhancement in Rel-17 IAB work is on-going to recover from this unexpected events as soon as possible. With this enhancements, the experienced one hop PDB may be acceptable even after an unexpected event occurs and the accumulated PDB for a packet can be managed without the proposed enhancement for IL-1. Thus, we think that PDB requirement of the packet can be managed in most cases and doubt whether RAN2 needs to develop a complicated mechanism to satisfy PDB requirement of each packet even for the rare case.

Observation 7. In normal situation, proper network configuration can make accumulated PDB of the packet meet with PDB requirement of the packet. Rel-16 and enhanced Rel-17 solutions can recover from unexpected events (e.g., BH RLF and data congestion) as soon as possible and PDB requirement of the packet can be managed in most cases.

Proposal 2. RAN2 confirm PDB management by intermediate IAB nodes is not enhanced in Rel-17 IAB.

For IL-2, in legacy NR, up to 8 LCGs can be configured to support 32 LCIDs (logical channel identity). In Rel-16 IAB, the maximum LCID space is extended to 65855, but maximum LCG space is not extended and still remained in 8. This means that one LCG may have over 32 logical channels (maybe over hundreds in some cases) and there is a possibility that logical channels which have different QoS requirements may be included in the same LCG due to lack of available LCG spaces. Considering that the buffer status of each LCG is reported by BSR MAC CE, the network may have trouble to manage QoS requirement and provide appropriate scheduling between two IAB nodes. In addition, if logical channels having same or similar QoS are included into the same LCG, the buffer size of each LCG in the BSR can indicate exact total amount of data available for each QoS level and this would be helpful to support finer QoS provision. Thus, we think that LCG space extension should be considered in Rel-17 IAB and also LCG space can be extended to at least 16 or even larger value.

Observation 8. If maximum LCG space is not extended to support 65855 LCID space, the network may have trouble to manage QoS requirement and provide appropriate scheduling between two IAB nodes.

Proposal 3. The maximum LCG space is extended at least up to 16. FFS whether to support larger LCG value.

For IL-3, buffer size calculation for pre-emptive BSR is left to implementation in Rel-16. However, since there is no aligned principle for buffer size calculation, some IAB nodes may report the pre-emptive BSR with a larger buffer size value than actually required data volume, in order to reduce UL transmission latency and enhance UL transmission efficiency. We think that this may cause bad impact to scheduling decision at the parent node and competition between IAB nodes to request a larger size of UL grant as early as possible. Thus, it would be good to define buffer size calculation for pre-emptive BSR to avoid this problem.

When RAN2 discussed Pre-emptive BSR in Rel-16 IAB, the biggest concern is waste of radio resource because the node may receive UL grants before arriving actual data from the child node. Given this, buffer size calculation for Pre-emptive BSR should consider this concern to avoid it as much as possible. We think that one clear way to achieve this is that the total amount of expected data for the Pre-emptive BSR is determined based on the total amount of UL grant is provided to child IAB node or UE. Note that Pre-emptive BSR is separate procedure from the legacy BSR procedure and the already available data which can be reported by a legacy BSR should not be considered for buffer size calculation for Pre-emptive BSR.

Proposal 4. RAN2 confirm that buffer size calculation for Pre-emptive BSR is defined in Rel-17 IAB.

Proposal 5. The expected data volume in the Pre-emptive BSR is determined based on the total amount of UL grant is provided to child IAB node or UE.

For IL-5 and IL-6, those two issues make the IAB donor CU know some information. We think that basically if the IAB donor CU needs to know some information from the IAB node, RAN3 should study first to make sure that this information is really needed and helpful for the IAB donor node to manage the whole IAB network. However, it seems to go opposite way of work. Now, RAN2 tries to study RAN3 issue first without any input or LS from the RAN3. Thus, we think that RAN2 needs to check first those two issues are actual issues in RAN3 perspective.

Proposal 6. The issue IL-5 and IL-6 related to IAB donor CU should not be discussed in RAN2 without RAN3 inputs.

Congestion mitigation

This section discusses RAN2 agreements for congestion mitigation. As addressed in the below agreement, it is not clear what RAN2 do for these two issues, but it could be worthwhile to show why these two issues are not relevant to RAN2 for now.

R2 has concluded that there is sufficient interest among companies to address the following two issues: - IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion. Both IC-1 and CI-7 are related to RAN3. RAN3 seems to also work on this, so to what extent R2 shall work on this is currently not clear.

The issue IC-1 is technically correct and it would be good to fix this problem. However, we are reluctant to say that this is a RAN2 issue because the best way to handle long-term downstream congestion is to make the IAB donor CU-UP throttle down downstream traffics toward long-term congested IAB node. In other words, end-to-end (E2E) flow control would be a better option than hop-by-hop flow control because the E2E flow control can be reached to IAB donor CU-UP, but the hop-by-hop flow control cannot be reached to IAB donor CU-UP.

One problem to resolve long-term downstream congestion using the current E2E flow control is that the current E2E flow control can be transmitted only from access IAB node to IAB donor CU, but the long-term congested IAB node would be an intermediate IAB node. Thus, in our view, the required enhancement is that the intermediate IAB node can transmit the E2E flow control to the IAB donor CU-UP. Having said that, this is RAN3 scope and the related discussion is already started in RAN3. Given this situation, we think RAN2 can start discussion on this issue after seeing the result of RAN3 discussion or receiving any input from RAN3.

Observation 9. Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, but this can be resolved by E2E flow control which is under discussion in RAN3.

Proposal 7. RAN2 does not start to discuss the issue IC-1 (i.e.,“Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets”) until RAN3 input is received.

For IC-7, this is also the IAB donor CU related issue as in IL-5 and IL-6. We think that same argument and reason can be also applicable to the issue IC-7. That is, RAN3 should study first to make sure that this information is really needed and helpful for the IAB donor node. Then, if RAN3 concludes that it is really helpful and needed, RAN2 can start a discussion on this issue based on the LS from RAN3.

Proposal 8. RAN2 does not start to discuss the issue IC-7 (i.e., “CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion”) until RAN3 input is received.

Conclusion

Based on the above discussions, we present the following observations:

Observation 1. Since link quality across multiple hops is frequently changed, scheduling decision based on link quality across multiple hops may make a bad scheduling decision and cause inevitable high signalling overhead.

Observation 2. If the link quality across multiple hops may not be changed frequently due to well deployed IAB network, link quality across multiple hops may be useless and per hop QoS parameters on each BH RLC/LCH as in Rel-16 may be sufficient for scheduling decision.

Observation 3. Rel-16 IAB network configuration can consider the number of hops, DRB’s QoS, the number of DRBs on one BH RLC channel, and the number of descendant IAB nodes, when per hop QoS parameter (e.g., LCH priority) is determined and configured to the IAB node.

Observation 4. If there is a BH RLF or a beam blockage, this may cause congestion, but this kind of events will block all transmission of BH RLC channels, not a specific BH RLC channel.

Observation 5. In normal case, BH RLC channels carrying UE bearers with same or similar QoS requirements would be balanced and may not be congested. However, if bad IAB implementation/configuration may cause congestion on specific BH RLC channels carrying UE bearers with same or similar QoS requirement.

Observation 6. Just giving more resource to those BH RLC CHs that aggregate more bearers may be bad impact to topology-wide fairness and the current Rel-16 configuration would already consider fairness aspect for all traffic loads of each BH RLC CH when they are configured.

Observation 7. In normal situation, proper network configuration can make accumulated PDB of the packet meet with PDB requirement of the packet. In unexpected events such as BH RLF and data congestion, Rel-16 and enhanced Rel-17 solutions can reduce transmission interruption and only rare case may not satisfy PDB requirement of packet.

Observation 8. If maximum LCG space is not extended to support 65855 LCID space, the network may have trouble to manage QoS requirement and provide appropriate scheduling between two IAB nodes.

Observation 9. Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, but this can be resolved by E2E flow control which is under discussion in RAN3.

Based on the observations, we have following proposals:

Proposal 1. RAN2 confirms that existing Rel-16 IAB configuration can handle IF-1, IF-2, and IF-4 and provide topology-wide fairness.

Proposal 2. RAN2 confirm PDB management by intermediate IAB nodes is not enhanced in Rel-17 IAB.

Proposal 3. The maximum LCG space is extended at least up to 16.

Proposal 4. RAN2 confirm that buffer size calculation for Pre-emptive BSR is defined in Rel-17 IAB.

Proposal 5. The expected data volume in the Pre-emptive BSR is determined based on the total amount of UL grant is provided to child IAB node or UE.

Proposal 6. The issue IL-5 and IL-6 related to IAB donor CU should not be discussed in RAN2 without RAN3 inputs.

Proposal 7. RAN2 does not start to discuss the issue IC-1 (i.e.,“Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets”) until RAN3 input is received.

Proposal 8. RAN2 does not start to discuss the issue IC-7 (i.e., “CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion”) until RAN3 input is received.