3GPP TSG-RAN WG2 Meeting #113bis Electronic	R2-2103496

Online, April 12 – April 20, 2021

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Potential TRS/CSI-RS occasion(s)

WID/SID:	NR_UE_pow_sav_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

WID on UE power saving was approved in RP-200938

In this contribution we discuss potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs as per the following study item objective:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In RAN2#113 meeting the following was agreed:

[041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - SIB signalling is the baseline; - Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - Option 2: Existing SIB, other than SIB1; - Option 3: New SIB type, e.g. SIB-x;

# 2	Discussion

The aspects related to presence indication or guarantee for the IDLE/Inactive UEs for potential TRS/CSI-RS occasions(s) was discussed in RAN1#103e and RAN1#104e with agreeing to consider following options:

Agreements: - Discuss further based on the following alternatives and down-select at RAN1#104-e: - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE. - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - The condition can be, e.g., existence of paging. - Alt 4. Combination of the above alternatives. - FFS for details - FFS for UE behavior when the availability is not informed. - Other techniques are not precluded. - Companies encourage to provide sufficient information for the proposal, e.g., - how to achieve power saving gain - how to minimize impact on NW how to minimize extra UE implementation complexity - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs - Proposals should be consistent with the WID objective. Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

One key aspect discussed in RAN1 on the TRS/CSI-RS occasion(s) for IDLE/INACTIVE mode UEs was the indication of the presence of the potential TRS/CSI-RS occasions versus UE autonomously detecting the presence of the TRS/CSI-RS occasions.

It was discussed that if UE does not have certainty that the RS are actually present in the informed occasions, UE would not be able to utilize them to the fullest extent. If the UE would not have a guarantee on the presence, it would need to evaluate the presence of the TRS/CSI-RS in the occasions so that it could fall-back to the use of the SSBs for synchronisation. Thus, there would be additional cost applied in the case that no TRS are present/detected.

From network perspective the problem is different. As per existing design, NR does not have any fixed LTE style of reference signals, e.g. CRS, but such are omitted to enable e.g. more flexible co-existence and forward compatibility. Enforcing, for the merit of the IDLE/INACTIVE UEs, the network to transmit such continuously, would affect the network overhead (albeit slightly if period is long) but more importantly to the network power consumption. It is also good to note, that as there are no fixed LTE style reference signals in earlier releases, legacy UEs for example receiving SI or paging in IDLE, do not assume such either and therefore gNB must avoid overlap of TRS with such PDSCH allocation.

Observation: Increasing frequency and/or duration of the always ON periodic signals may increase the network power consumption

At least system information based and paging based methods could be considered for providing the presence/availability indication. When considering presence/availability indication for the configured TRS/CSI-RS occasions it would be the lead time for changing the status (present/not present) is the key component for the network power consumption. Different presence/availability indication methods could of course have different lead times. Of course, introducing more frequent indication methods has the drawback that they will increase the ‘always-on’ overhead and thereby reduce the potential for energy efficiency of the network. On the other hand, providing the presence indication on channels that are transmitted by network could enable dynamic indication with decent overhead.

As there is no mandate for the UE to use the TRS/CSI-RS occasions, i.e. it is left completely for UE implementation whether to use the TRS/CSI-RS occasion or not, it would seem counter intuitive to mandate network to always provide presence/availability indication.

Proposal 1: Configuration for potential TRS/CSI-RS occasion(s) can be provided to IDLE/INACTIVE mode UEs in system information without presence/availability information.

Proposal 2: Benefits and drawbacks of dynamic presence/availability information needs to be studied further

In RAN2#113 meeting the following was agreed:

[041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - SIB signalling is the baseline; - Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - Option 2: Existing SIB, other than SIB1; - Option 3: New SIB type, e.g. SIB-x;

It can be assumed that TRS/CSI-RS configuration for IDLE/INACTIVE mode UEs is going to be rather large. Naturally legacy UEs are not able to utilize TRS/CSI-RS configuration for IDLE/INACTIVE and if existing SIB is used it will have impact on legacy e.g. due to system information update procedure in case TRS/CSI-RS configuration for IDLE/INACTIVE is changed. We also assume that TRS/CSI-RS configuration in SIB will not change often and legacy system information update procedure is sufficient. Therefore, we propose the following:

Proposal 3: New SIB is defined for providing TRS/CSI-RS occasion(s) configuration for IDLE/INACTIVE UE(s).

Proposal 4: The legacy SI update procedure is used for updating the TRS/CSI-RS configuration in SIB

Some network implementations may be aware of rough time occasions during the day/week when the TRS is transmitted on the cell for CONNECTED UEs. Such time information could be provided to the IDLE/INACTIVE UEs is system information. Such rough time information would not cause frequent presence information signalling. It would be also possible to provide some static information, informing the UE for example when the TRS can be assumed to be (or not to be) available. To obtain a trade-off between TRS presence indication signalling and actual TRS signalling, the network may provide a long-term time table of TRS presence to the UE. For example, the time table can define hours or days where TRS are present/not present and whether it is per beam/cell.

E.g. assuming that in the cell there are somewhat deterministic ‘peak traffic’ hours when there is large number UEs active in CONNECTED mode, resulting that TRS would be active, or in reverse deterministic ‘silent/night hours’ when the there are no UEs or infrequently UEs in CONNECTED mode, resulting no or sporadic TRS availability, TRS availability time table could be provided in SI. For example, the time table can define hours when TRS are present/not available in the cell. This would prevent the need to send any physical layer indications or changing the SI.

Proposal 5: Network can optionally provide rough TRS presence/availability time table information in system information. Details FFS.

# 3	Conclusion

In this contribution potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs was discussed, and the following is proposed:

Proposal 1: Configuration for potential TRS/CSI-RS occasion(s) can be provided to IDLE/INACTIVE mode UEs in system information without presence/availability information.

Proposal 2: Benefits and drawbacks of dynamic presence/availability information needs to be studied further

Proposal 3: New SIB is defined for providing TRS/CSI-RS occasion(s) configuration for IDLE/INACTIVE UE(s).

Proposal 4: The legacy SI update procedure is used for updating the TRS/CSI-RS configuration in SIB

Proposal 5: Network can optionally provide rough TRS presence/availability time table information in system information. Details FFS.