3GPP TSG RAN Meeting #113bis-e						R2-2103911

Online, 12 - 20 April, 2021

Agenda Item:		8.14.2.2

Source:			Huawei, HiSilicon

Title:				QoE measurement handling at RAN overload

WID/SID:			NR_QoE_enh-Core

Document for:		Discussion and Decision

### 1	Introduction

In RAN#91 meeting, a new WID on NR QoE management and optimizations for diverse services [1]. The objectives of this WID includes the followings:

- Specify the support for QoE measurement collection in NR standalone mode. [RAN2, RAN3] - Specify configuration, activation, and deactivation procedures for both signalling-based and management-based QoE measurement collection and reporting, taking LTE QoE solutions as baseline, as defined in TR 38.890. - Specify configuration and reporting for multiple simultaneous QoE measurements at a UE. - Specify QoE measurement handling at RAN overload, including pause and resume of QoE measurement reporting. - Specify QoE measurement handling in RRC_INACTIVE, i.e. keeping the QoE measurement configuration without measuring and reusing the same configuration upon transition from RRC_INACTIVE to RRC_CONNECTED. NOTE: RRC segmentation may be needed for transmission of QoE reports, and any potential solutions need detailed technical specification of the procedures (if time allows in RAN2).

In this contribution, we will discuss QoE handling at RAN overload, including reporting pause and resume mechanism, as well as QoE measurement handling in RRC INACTIVE state.

### 2	QoE measurement during RAN overload

#### 2.1	Release of QoE reporting

According to section 6.4 in the TR 38.890 [2], RAN3 and RAN2 agreed that RAN can release existing QoE measurement configuration when the session for which the QoE measurements are reported is completed or when the UE is handing over to a network that does not support the QoE measurement.

## 6.4 Release of QoE measurement configuration An NG-RAN node can issue a release of QoE measurement configuration for UEs previously configured for QoE measurement reporting, provided that the session for which the QoE measurements are reported is completed. RAN may need to release an ongoing QoE measurement configuration or QoE reporting configuration, e.g. if handing over to a network that does not support this.

According to section 6.5 in the TR 38.890 [2], RAN3 and RAN2 agreed that RAN can also release existing QoE measurement configuration in case of RAN overload.

## 6.5 QoE measurement handling at RAN overload In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting. RRC signaling is used by the gNB to indicate the UE to pause or resume the QoE reporting. Potential solutions of pause/resume mechanism need detailed technical specification of the procedures, e.g. pause/resume for all QoE reports or pause/resume per QoE configuration, how long can the UE store the reports, limit for stored reports size etc. of the details of QoE measurement handling at RAN overload in dual connectivity might be discussed in normative phase.

In our understanding, when RAN releases an existing QoE configuration, the UE will cease reporting for the associated QoE measurements. This is how it was handled in LTE, as per TS 36.331:

1> if the received otherConfig includes the measConfigAppLayer: 2> if measConfigAppLayer is set to setup: 3> forward measConfigAppLayerContainer to upper layers considering the serviceType; 3> consider itself to be configured to send application layer measurement report in accordance with 5.6.19; 2> else: 3> inform upper layers to clear the stored application layer measurement configuration; 3> discard received application layer measurement report information from upper layers; 3> consider itself not to be configured to send application layer measurement report.

In LTE, E-UTRAN can release the QoE measurement configuration from the UE at any time. The motivation is that the network may release the QoE measurement configuration when the load in the network is very high or the UE has moved out of the area of QoE measurement. In our understanding, the same motivation holds also for NR QoE as it is still the foremost RAN objective to ensure efficient transmission of user data at any time. The QoE measurement reporting is not the most important issue when the load of NR is very high and the QoS of user services is endangered. One way to avoid the issue of QoE reporting during RAN overload, is to release a QoE configuration from the UE, even if a related QoE measurement session is ongoing.

Furthermore, it should be noted that the management based QoE measurement is initiated without targeting a specific UE. Hence, releasing a QoE measurement triggered by management based QoE might not cause any negative impacts.  Quite the contrary, it may be beneficial for the network to do so in the case where the network configured multiple UEs for QoE reporting and it received a sufficient number of the reports from other UEs already. Even for the signaling based QoE where the QoE measurement is initiated towards a specific UE, missing of one specific QoE report is not a big issue, especially when the de-configuration occurs due to RAN overload. Also, the network can trigger the signalling based QoE measurement for the same UE in the future, if needed. Therefore we think releasing an ongoing signalling based QoE measurement will not cause large impacts and should be an option for the network.

Proposal 1: RAN is allowed to release a QoE configuration from a UE at any time including the time when the related QoE measurement session is ongoing.

Proposal 2: When RAN orders the UE to release a QoE configuration, a UE releases the QoE configuration and stops reporting for this QoE configuration (including any available and non-sent reports).

It should be noted that allowing to release a QoE configuration of an ongoing QoE session does not rule out having other mechanisms to deal with RAN overload such as QoE reporting pause/resume as also captured in the TR. We discuss this aspect further from RAN2 point of view in the following section.

#### 2.2	Pause/Resume at the RAN overload

According to the WID [1], RAN2 needs to discuss the QoE measurement handling at RAN overload, including pause and resume of QoE measurement reporting. In section 6.5 of the TR [2], it is indicated that RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting in the case of RAN overload. Therefore, RAN2 will have to design the pause and resume mechanism to handle RAN overload. In the last meeting, RAN2 has agreed to use the RRC signalling to indicate the UE to pause or resume the QoE report.

## 6.5 QoE measurement handling at RAN overload In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting. RRC signaling is used by the gNB to indicate the UE to pause or resume the QoE reporting. Potential solutions of pause/resume mechanism need detailed technical specification of the procedures, e.g. pause/resume for all QoE reports or pause/resume per QoE configuration, how long can the UE store the reports, limit for stored reports size etc.

In case the UE is configured with multiple QoE configurations, it would be beneficial to allow the network to pause reporting for only some of the configurations (similarly as it is possible to release only some of the configurations).

Proposal 3: In order to temporarily pause QoE reporting from a UE, e.g. during RAN overload, RAN can send the QoE reporting pause command to the UE in the DL RRC message, which indicates QoE configurations (one or more) for which the reporting should be paused.

According to the definitions in the TR 38.890, when the UE pauses the QoE report, the UE will continue the measurement collection, i.e. the UE will continue to generate the QoE measurement results.

QoE measurement reporting pause: an action resulting in temporary stopping of QoE measurement reporting, where the QoE measurement collection is not affected. QoE measurement reporting resume: an action resulting in resumption of a previously paused QoE measurement reporting. NOTE: The above definitions may need further adaptation, to align with SA5 and RAN2.

But according to the TS 28.405 of SA5 (although this TS is only for LTE and RAN2 and RAN3 agreed not to support the temporary stop in LTE ), in our understanding, the UE will stop recording further information, i.e. the UE will not continue to generate the QoE measurement results. Furthermore, based on TS 28.405 it can be concluded that the recorded data is stored in the application.

### 4.2.4 Temporary stop and restart of QoE information reporting during RAN overload in LTE In case of overload in RAN, the eNB may temporarily stop the reporting from the UE by send the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to temporarily stop application layer measurement reporting in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the temporary stop request. The application stops the reporting and stops recording further information when the data in the reporting container is used. Then the recorded data is kept until it is reported or when the UE request session is ended. When the overload situation in RAN is ended the eNB restart the reporting from the UE by send the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to restart application layer measurement reporting in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the restart request. The application restarts the reporting and recording if it was stopped.

If the above description can be assumed to be relevant also for NR, then AS layer does not have to handle any actions with respect to pausing and resuming the QoE measurement, other than forwarding this indication to application layer. It is the application layer which is responsible for storing the measurements collected so far and for ensuring that reporting is paused, i.e. no reports are forwarded to AS layer. When the UE receives resume command, it forwards it to the application layer. Afterwards, the application layer may provide reports to AS layer again, which are then handled in a usual way.

An alternative approach would be that “pause” indication is just used at the AS layer and is not forwarded to the application layer. In this case, application layer would continue operation as usual, i.e. continue to perform QoE measurements and provide reports to AS layer in case the measurement is complete. After receiving the measurement report, it would be AS layer’s duty to store it until the “resume” indication is received from the network. This would mean RAN2 has to discuss several additional issues, i.e.:

- Any memory constraints on the UE side for storing the reports

- How long should the UE store the QoE configurations before discarding them due to no “resume” command being received

- Reporting behavior after receiving “resume” command (as it would be a different case than just reporting upon having received the report from the application layer)

From RAN perspective, the first approach, where the pause/resume commands are forwarded to the application layer, allows to minimize the impacts to AS layer. At the same time, it seems this is what SA5 intended. Therefore, we propose to agree on this understanding and confirm it with SA5.

Proposal 4: RAN2 assumes that “pause” and “resume” commands for a QoE configuration are forwarded by the UE to application layer. After receiving a “pause” indication from the UE, application layer will stop sending reports to RRC layer and only continue to do so after receiving “resume” indication from the UE. An LS to SA5 should be sent to confirm this understanding.

### 3	Resume upon transition from RRC_INACTIVE

In order to decide how to handle QoE configurations when UE moves to RRC_INACTIVE state, we first discuss some general principles, which are applied to QoE handling during UE mobility in both LTE (according to normative specifications) and in NR (according to TR 38.890).

In LTE, the QoE measurement is activated by Trace function from the MDT framework.

## 23.16 Application Layer Measurement Collection This function enables collection of application layer measurements from the UE. The supported service types are QoE Measurement Collection for streaming services and QoE Measurement Collection for MTSI services. The feature is activated by Trace Function from the MDT framework (see clause 19.2.1.17 and TS 37.320 [43]). Both signalling based and management based initiation cases are allowed. For the signalling based case, the Application Layer Measurement Collection is initiated towards a specific UE from CN nodes using the MDT mechanism as described in clause 5.1.3 of TS 37.320 [43]; for the management based case, the Application Layer Measurement Collection is initiated from OAM targeting an area (without targeting a specific UE).

According to section 5.1.2.3 of 37.320, only the signalling based MDT configuration will propagate during handover.

In addition, MDT configuration handling during handover depends on MDT initiation from OAM defined in clause 5.1.3: - The MDT configuration configured by management based trace function will not propagate during handover. - For LTE, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, and may propagate during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. - For UMTS, the MDT configuration received by signalling based trace messages for a specific UE will continue during intra-PLMN handover, and may continue during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN, except for the case of SRNS relocation. - For NR, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, and may propagate during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. NOTE: In the case of SRNS relocation, MDT may be reactivated by the Core Network following a successful relocation.

Therefore, in LTE, the source eNB only explicitly forwards the signalling based QoE configuration to the target eNB via the X2 interface. On the other hand, according to TS 36.331, the source eNB also transfers the RRC configuration information as a container to the target eNB for the target eNB to determine the need to change the RRC configuration during the handover preparation phase. The RRC configuration information includes the QoE measurement configuration that has been sent to the UE in both signalling based QoE measurement and management based QoE measurement.

Observation 1: In LTE, only the QoE configuration received by the signalling based QoE will propagate during handover in the X2AP signalling. The RRC container in the handover request includes the QoE measurement configuration that has been sent to the UE in both signalling based QoE and management based QoE.

For the management based QoE measurement, considering a fact that the target gNB may also have received a management based QoE measurement request from OAM, during UE handover it is up to the target node to decide whether to keep the UE’s current QOE configuration or configure a new one based on target node’s local configuration. For example, the target eNB can compare whether the QoE measurement configuration in the UE is the same as the QoE measurement configuration received from the OAM. If they are different, the target eNB can release the QoE measurement of the source eNB and configure a new one. If they are the same, the target eNB can keep the original one. If the target eNB does not receive the configuration from the OAM, it can release the QoE configuration.

During SI phase of NR QoE work, RAN3 has agreed that the signalling based QoE measurement configuration transfer is supported on the Xn and NG interfaces. The transfer of management based QoE measurement configuration is still FFS.

## 6.6 Support for Mobility Seamless mobility is a key functionality in NR and its impacts should be measurable at the application layer. To enable measuring the impact of the mobility on the application and users’ QoE, it is required to support QoE measurement reporting continuity in intra-system intra-RAT intra-node and inter-node mobility scenarios: for intra-node mobility for both management-based and signalling-based QoE. At least signalling-based QoE supports this also in case of inter-node mobility. Support for management-based QoE will be discussed in normative phase, with respect to the "Requirements from SA WGs" below. In LTE, to support the QoE measurement in mobility scenarios, the QoE configuration is forwarded from the source eNB to the target eNB inside the Trace Activation IE over the X2 interface. The same IE is sent over the S1 interfaces for mobility scenarios when the X2 interface is not established between the source and the target. In NR, to support mobility for QoE measurements in RRC_CONNECTED state, the QoE measurement configuration transfer is supported on the Xn and NG interfaces, inside the Trace Activation IE as a part of UE Application Layer Measurement Configuration IE that may contain multiple QoE configurations for multiple service types. QoE measurements in RRC_IDLE and RRC_INACTIVE state can be supported for MBS. To support keeping QoE measurement configuration in RRC_INACTIVE state mobility, QoE measurement configuration for a UE can be fetched from the node hosting the UE Context. Whether UE stores its QoE configuration when going to RRC_INACTIVE state for potential use when the UE moves back to RRC_CONNECTED state will be decided in technical specification of the procedures.

In our understanding, the management based QoE measurement targets the UEs in a specified area and RAN should configure the management based QoE measurement according to the configuration received from the OAM. Therefore, alike the principle in LTE, the source RAN does not need to forward the management based QoE measurement configuration to the target RAN in the Trace Activation IEs. The target node decides whether to continue/release the original one or configure a new one based on its local configuration and the RRC configuration received from the source node.

Observation 2: Based on TR 38.890, it is likely the NR QoE configuration handling during UE mobility will be the same as in LTE (for signalling-based vs. management-based activation).

For QoE handling when UE is transitioning to RRC_INACTIVE, it should be first noted that QoE configuration is provided to the UE while it is in RRC CONNECTED state, but since the QoE measurement is triggered when the user executes the corresponding application on her/his device, before that happens the UE may transition to RRC INACTIVE state due to data inactivity. With respect to QoE configuration preservation for the UEs which are transitioning to RRC INACTIVE state, the following is captured in TR 38.890 [2]:

To support keeping QoE measurement configuration in RRC_INACTIVE state mobility, QoE measurement configuration for a UE can be fetched from the node hosting the UE Context. Whether UE stores its QoE configuration when going to RRC_INACTIVE state for potential use when the UE moves back to RRC_CONNECTED state will be decided in technical specification of the procedures.

From the above, it can be deduced that the QoE configuration is kept at least on the RAN side while whether it is kept at the UE as well needs to be further decided. Two options exist:

- QoE configuration is released by the UE when it transitions to RRC INACTIVE state. When the UE moves to RRC CONNECTED state, new gNB fetches UE’s QoE configuration from the node currently hosting UE context and if it is still valid, it provides it to the UE again. If QoE configuration is no longer valid in the new gNB, the gNB does not provide it to the UE.

- QoE configuration is kept by the UE when it transitions to RRC INACTIVE state. When the UE moves to RRC CONNECTED state, new gNB fetches UE’s QoE configuration from the node currently hosting UE context, but there is no need to provide it to the UE again in case it is valid. If it is invalid, the gNB should have a possibility to release the configuration from the UE during RRC connection resume procedure.

What is similar for both of these cases is that the new gNB needs to be aware whether the QoE configuration fetched from the old gNB is still valid or not (e.g. whether the new gNB is in the target QoE measurement area). Here, it is important to distinguish the case of management based QoE activation and signalling based QoE activation, similarly as for connected mode mobility as described above. For signalling-based activation, we assume that the QoE configuration will be forwarded from the source/anchor gNB to the target gNB within Xn signalling, in the same way as for connected mode mobility. That way the target gNB can verify whether the configuration should be kept or released. For management based activation on the other hand, if the target gNB receives the QoE measurement configuration directly from OAM, it can verify whether the UE is configured with the needed QoE configuration already by comparing the transparent containers from OAM and within the UEs current RRC configuration.

Proposal 5: Inform RAN3 that for signalling-based QoE activation, in order to support QoE configuration preservation when the UE transits to RRC_INACTIVE state, the information about the applicable QoE measurement should be provided to the new gNB when fetching UE’s QoE configuration from the old gNB during UE’s RRC connection resumption procedure.

Since Proposal 5 is applicable to both options outlined above, it has no impact on RAN2 decision about which option to select. However, we believe that the option where the UE keeps QoE configuration when going to RRC INACTIVE is advantageous, since it does not require the new gNB to resend the configuration in case it is still valid in the new gNB. If it is no longer valid, a simple indication can be used to inform the UE that it should release its QoE configuration. In general, this option imposes less signalling overhead than the other one.

Proposal 6: The UE should keep its QoE configuration when going to RRC INACTIVE state.

When the connection is resumed in another gNB, it will find out whether the UE is configured with QoE measurements based on the context it fetches from the anchor node. However, the new gNB may want to release the QoE measurement configuration, e.g. due to RAN overload or for other RAN-specific reasons. It should be then possible for the new gNB to decide whether to restore or release the QoE measurement configuration during RRC resume procedure. To achieve that, a similar approach can be used as for DCCA enhancements where the network can decide whether the UE should keep its SCells or SCG configuration when resuming the connection by using restoreMCG-SCells and restoreSCG parameters. Similarly, the gNB could include the flag in RRCResume message telling the UE whether the QoE measurements should be kept or released.

Proposal 7: The target node can indicate whether the UE should restore or release its QoE configuration during RRC resume procedure.

### 4	Conclusions

Based on the discussion in this contribution, the following is proposed for pause and resume at the RAN overload:

Proposal 1: RAN is allowed to release a QoE configuration from a UE at any time including the time when the related QoE measurement session is ongoing.

Proposal 2: When RAN orders the UE to release a QoE configuration, a UE releases the QoE configuration and stops reporting for this QoE configuration (including any available and non-sent reports).

Proposal 3: In order to temporarily pause QoE reporting from a UE, e.g. during RAN overload, RAN can send the QoE reporting pause command to the UE in the DL RRC message, which indicates QoE configurations (one or more) for which the reporting should be paused.

Proposal 4: RAN2 assumes that “pause” and “resume” commands for a QoE configuration are forwarded by the UE to application layer. After receiving a “pause” indication from the UE, application layer will stop sending reports to RRC layer and only continue to do so after receiving “resume” indication from the UE. An LS to SA5 should be sent to confirm this understanding.

The following is observed and proposed for the resume upon transition from RRC_INATIVE to RRC_CONNECTED:

Observation 1: In LTE, only the QoE configuration received by the signalling based QoE will propagate during handover in the X2AP signalling. The RRC container in the handover request includes the QoE measurement configuration that has been sent to the UE in both signalling based QoE and management based QoE.

Observation 2: Based on TR 38.890, it is likely the NR QoE configuration handling during UE mobility will be the same as in LTE (for signalling-based vs. management-based activation).

Proposal 5: Inform RAN3 that for signalling-based QoE activation, in order to support QoE configuration preservation when the UE transits to RRC_INACTIVE state, the information about the applicable QoE measurement should be provided to the new gNB when fetching UE’s QoE configuration from the old gNB during UE’s RRC connection resumption procedure.

Proposal 6: The UE should keep its QoE configuration when going to RRC INACTIVE state.

Proposal 7: The target node can indicate whether the UE should restore or release its QoE configuration during RRC resume procedure.

### 5	References

[1] RP-210913, NR QoE management and optimizations for diverse services

[2] 3GPP TR 38.890 V0.3.0, Study on NR QoE management and optimizations for diverse services