3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2104304

Online, April 12 – April 20, 2021

Source: 	Session Chair (InterDigital)

Title:	Report for Rel-17 Small data and URLLC/IIoT

Email discussions:

[AT113bis-e][500] Organizational Diana – URLLC/IIoT, Small data]

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to URLLC/IIoT, Small data and NR-U, 2-step RACH, and power saving

    - Share meetings notes and agreements for review and endorsement

[AT113bis-e][501][SDT] UP SDT open issues (LG)

Scope:

    - Discuss open UP SDT open issues AI 8.6.2

Intended outcome:

    - Agreeable Proposals in R2-2104395

Deadline for providing comments:

    - Companies inputs April 15th

    - Rapporteur Proposals – April 16th

    - Comments on Proposals and final proposals – April 19th

## 8.5	NR IIoT URLLC

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 0 threads

THIS FEATURE WILL NOT BE TREATED in 113bis-e online and offline (i.e. no in-meeting email discussions).   However, two post-meeting email discussions to get company views will be triggered for 8.5.3 and 8.5.4 (see below)

### 8.5.1	Organizational

Rapporteur input

No input expected

R2-2102631	LS on gNB-based propagation delay compensation (R3-211136; contact: Nokia)	RAN3	LS in	Rel-17	NR_IIOT_URLLC_enh	To:RAN1, RAN2	Cc:-

### 8.5.2	Enhancements for support of time synchronization

Including requirements and scope.

No input expected

This AI will not be treated in 113bis-e and no email discussion will be triggered on this topic during or post April meeting.

### 8.5.3	Uplink enhancements for URLLC in unlicensed controlled environments

RAN2 aspects related to URLLC in unlicensed controlled environments. Initial discussion on potential impacts, including requirements and scope

This AI will NOT be treated in 113bis-e and NO in meeting email discussions will be triggered.

Contributions on this topic can be submitted, but is not required, and a post April meeting email discussion is expected to be triggered to get company inputs on the remaining open issues.

R2-2102685	CG Harmonization for Unlicensed Controlled Environment	Qualcomm Incorporated	discussion	Rel-17

R2-2102725	URLLC in UCE	CATT	discussion	NR_IIOT_URLLC_enh-Core

R2-2102992	HARQ Process Prioritization of Configured Grant for IIoT in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103059	Remaining issues about uplink enhancements for URLLC in UCE	Huawei, HiSilicon	discussion	NR_IIOT_URLLC_enh-Core

R2-2103072	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103126	Issue of Prioritizing Initial Transmission over Retransmission on a CG	vivo	discussion

R2-2103211	Consideration on URLLC over NRU	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103297	Enhancements for URLLC in unlicensed controlled environments	Lenovo, Morotola Mobility	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103428	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson	discussion	Rel-17

R2-2103441	Further Consideration on the UL transmission in UCE	ZTE Corporation, Sanechips	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103492	CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments	III	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103566	Prioritization of UL transmissions in unlicensed URLLC	Sony Europe B.V.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103648	CG Harmonization for UCE	Samsung	discussion	Rel-17

R2-2103688	Discussion on the remaining issue for uplink enhancements for URLLC in UCE	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103797	IIoT operation in unlicensed controlled environments	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104103	Further details on harmonization	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

R2-2104224	Remaining issues of CG harmonization	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

=> Revised in R2-2104288

R2-2104288	Remaining issues of CG harmonization	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

### 8.5.4	RAN enhancements based on new QoS

RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

This AI will NOT be treated in 113bis-e and NO in meeting email discussions will be triggered.

Contributions on this topic can be submitted taking into account SA2 progress, but is not required, and a post April meeting email discussion is expected to be triggered to get company inputs on the remaining open issues.

R2-2102686	RAN Enhancement to support new QoS	Qualcomm Incorporated	discussion	Rel-17

R2-2102726	Handling of Survival Time	CATT	discussion	NR_IIOT_URLLC_enh-Core

R2-2102993	RAN Enhancement for New QoS Parameters	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103060	RAN enhancements based on new QoS related parameters	Huawei, HiSilicon	discussion	NR_IIOT_URLLC_enh-Core

R2-2103125	Discussion on RAN enhancement to support survival time	vivo	discussion

R2-2103196	Topics on new QoS handling	Fujitsu	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	R2-2000418

R2-2103212	RAN enhancement based on new QoS	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103329	Further considerations on new QoS	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd	discussion	NR_IIOT_URLLC_enh-Core	R2-2100328

R2-2103420	Discussion on RAN enhancements based on Survival Time	III	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	R2-2100449

R2-2103429	RAN enhancements based on new QoS related parameters	Ericsson	discussion	Rel-17

R2-2103432	Entering and operating in the Survival Time state	Samsung Electronics GmbH	discussion

R2-2103689	Discussion on the RAN support for new QoS parameters	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103735	RAN2 Enhancements for Support of QoS Parameters	Intel Corporation	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103798	Enhancements based on new QoS requirements	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103896	Discussion on entering and exiting survival time state	Futurewei Technologies	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104097	View on survival time mechanisms	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

R2-2104225	Clarification on the survival time	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104265	RAN enhancements based on new QoS	TCL Communication Ltd.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

## 8.6	Small Data enhancements

(NR_SmallData_INACTIVE-Core; leading WG: RAN2; REL-17; WID: RP-210870)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 4 threads

FFS whether RACH partitioning should be initially done as a common design for multiple WIs: RAN slicing, RedCap, Small Data Transmission, CovEnh? Or whether coordination should be attempted once each WI has produced CRs.

### 8.6.1	Organizational

In coming LSs, rapporteur input for email discussions summaires etc (tdocs in this don’t count towards tdoc limit).

Including [Post113-e][501][502][503][504]

R2-2102620	Reply LS on physical layer aspects of small data transmission (R1-2102125; contact: ZTE)	RAN1	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2

R2-2102634	Reply LS on small data transmission (R3-211280; contact: Ericsson)	RAN3	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2	Cc:-

R2-2103527	Stage-2 running CR Introduction of SDT	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	16.5.0	0357	-	B	NR_SmallData_INACTIVE-Core

R2-2102707	Report from email discussion [POST113-e][501][SDT] Selection criteria and overall Procedure	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103022	Summary of General and other control plane open issues for SDT (email: [Post 113-e][502])	Rapporteur (ZTE)	discussion

R2-2103897	DRAFT Reply LS on small data transmission	Ericsson	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN3

R2-2104490	DRAFT Reply LS on physical layer aspects of small data transmission (Reply to R2-2102620)	ZTE Corporation	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN1

### 8.6.2	User plane common aspects

Overall user plane procedure for SDT (including triggering and thresholds, HARQ, and MAC CEs), data volume computation,. suppression of PDCP status report, RSRP threshold for SDT selection, switching between CG/RA, and any other user aspects included in Post113-e][501][503]   which cannot be concluded as part of the email

Email discussion summary expected for this AI durin 113bis-e

R2-2104395	Summary of UP SDT open issues 		LG

[CB on Tuesday, April 20th]

R2-2102708	User Plane Common Aspects of RACH and CG based SDT	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102750	Discussion on user plane issues of SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102755	Discussion on User Plane Aspect of Small Data Transmission	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100139

R2-2102840	User plane aspects for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103018	User plane open issues for SDT	ZTE Corporation, Sanechips	discussion

R2-2103102	Analysis on UP common aspects of SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103197	Support of CA and PDCP CA duplication	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2100419

R2-2103319	The UP common issues for small data transmissions	Lenovo, Motorola Mobility	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103430	Discussion on user plane common aspects of NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101221

R2-2103444	Discussion on data volume threshold for small data transmission	PANASONIC R&D Center Germany	discussion

R2-2103454	Avoid triggering RA during subsequent SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103521	Common aspects for SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103528	User Plane common aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103531	User plane common aspects for SDT	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103583	Some aspects of User Plane for SDT in NR 	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103672	Discussion on small data transmission	Google Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103674	Discussion on beam operations for small data transmission	Google Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103714	Remaining issues on transmission type selection and overall procedure	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103870	User plane aspects on the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103990	Consideration on overall SDT procedure and criteria	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104206	On the overall and detailed procedure of SDT	China Telecommunications	discussion

R2-2104220	Discussion on data volume calculation	Xiaomi Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104263	Discussion on Small Data Transmission	TCL Communication Ltd.	discussion	Rel-17

### 8.6.3	Control plane common aspects

Cell reselection and failure handling, handling of subsequent data transmissins (including, how to indicate presence of subsequent data, etc) handling of non-SDT DRBs (including whether to resume or not non-SDT), CP data over SDT, SDT termination and data loss prevention and any other control plane aspects included in [Post113-e][501][502][503] which cannot be concluded as part of the email

R2-2103971	Report of [Post113-e][503][SDT] T319, cell reselection and re-establishment	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

Not treated

R2-2102709	Control Plane Common Aspects of RACH and CG based SDT	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102751	Discussion on control plane issues of SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102756	Discussion on RRC-Controlled Small Data Transmission	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100140

R2-2102841	Signalling and NAS-AS interaction for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102842	Fallback and failure handling for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102900	New timers for SDT failure detection	Langbo	discussion	Rel-17	NR_SmallData_INACTIVE-Core	Late

=> Withdrawn

R2-2102991	Handling of non-SDT traffic arrival	PANASONIC R&D Center Germany	discussion

R2-2103019	Control plane aspects of SDT	ZTE Corporation, Sanechips	discussion

R2-2103103	Considerations on Some Common Control Plane Issues	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103151	Handling of non-SDT data arrival	Potevio Company Limited	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103198	RAN paging reception and response during SDT	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103257	Handling of non-SDT data during SDT	ETRI	discussion

R2-2103299	Discuss on solutions for arriving of non-SDT data during SDT	NEC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103405	Consideration on CP issues for small data transmission	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103431	Discussion on control plane common aspects of NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101223

R2-2103455	Beam management in SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103497	SDT control plane aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE

R2-2103522	CP aspects for SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103568	Discussion on subsequent SDT in NR, timer handling, and support for SRB1/2	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103715	Non-SDT data transmission	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103796	Subsequent small data transmission	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103867	Non-SDB handling during the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103868	Control plane aspects on the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103904	Control plane common aspects for SDT	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103970	CP and configuration aspects for small data	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103972	[Draft] LS to SA WG3 on re-use of same NCC and I-RNTI value for RRC Resume procedure in different cells during small data transmission procedure.	InterDigital	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:SA3

R2-2103989	Discussion on switching to non-SDT procedure	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103991	Discussion on cell reselection during SDT	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104204	Resuming non-SDT RBs and indication	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core	Late

R2-2104221	Discussion on the support of the RRC-less SDT	Xiaomi Communications, Intel Corporation, ASUSTeK, Fujitsu, MediaTek, Apple, Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104222	Technical details of the RRC-less SDT	Xiaomi Communications, ASUSTeK, Fujitsu, Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

### 8.6.4	Aspects specific to RACH based schemes

RA resource configuration and selection, PDCCH monitoring after successful SDT RA completion, RAN2 specific details of context fetch/data forwarding with and without anchor relocation

R2-2102710	Details of RACH bsaed Small Data Transmission	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102752	Discussion on RACH based SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102757	Supporting Small Data Transmission via RA Procedure	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100141

R2-2102847	Fallback issue for 2-step RA based small data transmission	Sharp	discussion	NR_SmallData_INACTIVE-Core	R2-2100413

R2-2103020	Open issues for RACH based SDT	ZTE Corporation, Sanechips	discussion

R2-2103104	Considerations on Procedures without Anchor Relocation	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103105	Analysis on Search Space of RA-SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103252	Discussion on RACH-based SDT	Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103264	PDCCH monitoring in subsequent data transmission period	Asia Pacific Telecom co. Ltd, FGI	discussion

R2-2103403	Analysis on open issues of RA based SDT	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103433	Discussion on RACH based NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101231

R2-2103456	Discussion on RO configuration between SDT and non-SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103519	RACH based SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103529	Details of context fetch and data forwarding	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103580	Discussion on context fetch and anchor relocation 	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103716	Anchor relocation and context fetch	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103869	Subsequent data transmission for SDT	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103903	Small data transmission with RA-based schemes	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

### 8.6.5	Aspects specific to CG based schemes

This AI will not be treated in RAN2#113bis-e (only the email discussion [504] in AI 8.6.1 will be treated)

CG resources, configuration and selection, validity of CG resources, multiple CG configurations, handling of beam selection for CG (including association between CGs and SSBs) etc, any other aspects included in [Post113-e][504][SDT] which cannot be concluded as part of the email

R2-2103533	Report from [POST113-e][504][SDT] CG Open Issues	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

Not treated

R2-2102711	Details of Configured Grant based Small Data Transmission	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102753	Discussion on CG based SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102758	Supporting Small Data Transmission via CG configuration	vivo	discussion	NR_SmallData_INACTIVE-Core

R2-2102843	On Configured Grant aspects for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103021	Open issues for CG based SDT	ZTE Corporation, Sanechips	discussion

R2-2103199	PDCCH monitoring after TAT expiry	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2000420

R2-2103265	CG-SDT based on beam operation	Asia Pacific Telecom co. Ltd, FGI	discussion

R2-2103367	Aspects specific to CG based SDT	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE

R2-2103404	Consideration on CG based small data transmission	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103434	Discussion on CG based NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101233

R2-2103457	Beam selection for CG-SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101752

R2-2103520	Details of CG based SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103532	Small data transmission with CG-based scheme	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103581	Details of CG-based scheme for SDT in NR   	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103795	CG-based SDT	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104223	Remaining issues of CG SDT	Xiaomi Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104241	On CG Resource Configuration in Small Data enhancement	China Telecommunications	discussion