3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2104304

Online, April 12 – April 20, 2021

Source: 	Session Chair (InterDigital)

Title:	Report for Rel-17 Small data and URLLC/IIoT

Email discussions:

[AT113bis-e][500] Organizational Diana – URLLC/IIoT, Small data]

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to URLLC/IIoT, Small data and NR-U, 2-step RACH, and power saving

    - Share meetings notes and agreements for review and endorsement

[AT113bis-e][501][SDT] UP SDT open issues (LG)

Scope:

    - Discuss open UP SDT open issues AI 8.6.2

Intended outcome:

    - Agreeable Proposals in R2-2104395

Deadline for providing comments:

    - Companies inputs April 15th 2300 UTC

    - Rapporteur Proposals – April 16th 0800 UTC

    - Comments on Proposals and final proposals – April 19th 1200 UTC

[AT113bis-e][502][SDT] Reply LS to RAN3  (Ericsson)

Intended outcome

    - Agreeable Reply LS to RAN3

Deadline for providing comments:

    - Companies inputs April 19th

    - Final LS – April 20th

[AT113bis-e][503][SDT] LS to SA3 (InterDigital)

Intended outcome

    - Agreeable LS to SA3

Deadline for providing comments:

    - Companies inputs April 19th

    - Final LS – April 20th

[AT113bis-e][504][SDT] LS to CT1 (Intel)

Intended outcome

    - Agreeable LS to CT1

Deadline for providing comments:

    - Companies inputs April 19th

    - Final LS – April 20th

Post meeting long email discussions

[POST113bis-e][505][R17 IIoT]  URLLC in UCE (LG)

Scope:

    - Discuss remaining open issues related to URLLC in UCE based on inputs submitted to 8.5.3.

    - Agreeable Proposals

Deadline: as set per RAN2 chair

[POST113bis-e][506][R17 IIoT] Enhancements based on QoS (CATT)

Scope:

    - Discuss remaining open issues related to RAN enhancements on new QoS based on inputs submitted to AI 8.5.4.

    - Agreeable Proposals

Deadline: as set per RAN2 chair

[POST113bis-e][507][SDT]  Scope TBD and will be shared on the reflector

## 8.5	NR IIoT URLLC

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 0 threads

THIS FEATURE WILL NOT BE TREATED in 113bis-e online and offline (i.e. no in-meeting email discussions).   However, two post-meeting email discussions to get company views will be triggered for 8.5.3 and 8.5.4 (see below)

### 8.5.1	Organizational

Rapporteur input

No input expected

R2-2102631	LS on gNB-based propagation delay compensation (R3-211136; contact: Nokia)	RAN3	LS in	Rel-17	NR_IIOT_URLLC_enh	To:RAN1, RAN2	Cc:-

### 8.5.2	Enhancements for support of time synchronization

Including requirements and scope.

No input expected

This AI will not be treated in 113bis-e and no email discussion will be triggered on this topic during or post April meeting.

### 8.5.3	Uplink enhancements for URLLC in unlicensed controlled environments

RAN2 aspects related to URLLC in unlicensed controlled environments. Initial discussion on potential impacts, including requirements and scope

This AI will NOT be treated in 113bis-e and NO in meeting email discussions will be triggered.

Contributions on this topic can be submitted, but is not required, and a post April meeting email discussion is expected to be triggered to get company inputs on the remaining open issues.

R2-2102685	CG Harmonization for Unlicensed Controlled Environment	Qualcomm Incorporated	discussion	Rel-17

R2-2102725	URLLC in UCE	CATT	discussion	NR_IIOT_URLLC_enh-Core

R2-2102992	HARQ Process Prioritization of Configured Grant for IIoT in NR-U	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103059	Remaining issues about uplink enhancements for URLLC in UCE	Huawei, HiSilicon	discussion	NR_IIOT_URLLC_enh-Core

R2-2103072	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103126	Issue of Prioritizing Initial Transmission over Retransmission on a CG	vivo	discussion

R2-2103211	Consideration on URLLC over NRU	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103297	Enhancements for URLLC in unlicensed controlled environments	Lenovo, Morotola Mobility	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103428	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson	discussion	Rel-17

R2-2103441	Further Consideration on the UL transmission in UCE	ZTE Corporation, Sanechips	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103492	CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments	III	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103566	Prioritization of UL transmissions in unlicensed URLLC	Sony Europe B.V.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103648	CG Harmonization for UCE	Samsung	discussion	Rel-17

R2-2103688	Discussion on the remaining issue for uplink enhancements for URLLC in UCE	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103797	IIoT operation in unlicensed controlled environments	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104103	Further details on harmonization	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

R2-2104224	Remaining issues of CG harmonization	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

=> Revised in R2-2104288

R2-2104288	Remaining issues of CG harmonization	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

### 8.5.4	RAN enhancements based on new QoS

RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

This AI will NOT be treated in 113bis-e and NO in meeting email discussions will be triggered.

Contributions on this topic can be submitted taking into account SA2 progress, but is not required, and a post April meeting email discussion is expected to be triggered to get company inputs on the remaining open issues.

R2-2102686	RAN Enhancement to support new QoS	Qualcomm Incorporated	discussion	Rel-17

R2-2102726	Handling of Survival Time	CATT	discussion	NR_IIOT_URLLC_enh-Core

R2-2102993	RAN Enhancement for New QoS Parameters	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103060	RAN enhancements based on new QoS related parameters	Huawei, HiSilicon	discussion	NR_IIOT_URLLC_enh-Core

R2-2103125	Discussion on RAN enhancement to support survival time	vivo	discussion

R2-2103196	Topics on new QoS handling	Fujitsu	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	R2-2000418

R2-2103212	RAN enhancement based on new QoS	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103329	Further considerations on new QoS	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd	discussion	NR_IIOT_URLLC_enh-Core	R2-2100328

R2-2103420	Discussion on RAN enhancements based on Survival Time	III	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	R2-2100449

R2-2103429	RAN enhancements based on new QoS related parameters	Ericsson	discussion	Rel-17

R2-2103432	Entering and operating in the Survival Time state	Samsung Electronics GmbH	discussion

R2-2103689	Discussion on the RAN support for new QoS parameters	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

R2-2103735	RAN2 Enhancements for Support of QoS Parameters	Intel Corporation	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103798	Enhancements based on new QoS requirements	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2103896	Discussion on entering and exiting survival time state	Futurewei Technologies	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104097	View on survival time mechanisms	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

R2-2104225	Clarification on the survival time	Xiaomi Communications	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

R2-2104265	RAN enhancements based on new QoS	TCL Communication Ltd.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

## 8.6	Small Data enhancements

(NR_SmallData_INACTIVE-Core; leading WG: RAN2; REL-17; WID: RP-210870)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 4 threads

FFS whether RACH partitioning should be initially done as a common design for multiple WIs: RAN slicing, RedCap, Small Data Transmission, CovEnh? Or whether coordination should be attempted once each WI has produced CRs.

### 8.6.1	Organizational

In coming LSs, rapporteur input for email discussions summaires etc (tdocs in this don’t count towards tdoc limit).

Including [Post113-e][501][502][503][504]

R2-2102620	Reply LS on physical layer aspects of small data transmission (R1-2102125; contact: ZTE)	RAN1	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2

=>	Noted

R2-2102634	Reply LS on small data transmission (R3-211280; contact: Ericsson)	RAN3	LS in	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN2	Cc:-

-	Ericsson would like to reply to say that we haven’t discussed the processing and RAN3 would have to progress this further.

=>	Noted

R2-2103527	Stage-2 running CR Introduction of SDT	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	16.5.0	0357	-	B	NR_SmallData_INACTIVE-Core

=>	Noted and to be reviewed over email after the meeting

R2-2102707	Report from email discussion [POST113-e][501][SDT] Selection criteria and overall Procedure	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

=>	Noted

Proposal 1:

-	Sony is asking if this threshold is optional

-	Vivo asks if this is L1 RSRP or L3 RSRP.  Samsung thinks this is the same as carrier based selection, i.e. L1-based.  Xiaomi asks whether it is cell level or SS

Proposal 4 (20/6): RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT)

-	Nokia is not sure we need SDT specific RSRP and what the use case is.  Samsung explains that if SDT is configured only on one UL carrier then the network can direct the UE to that UL  carrier for SDT.

-	CATT doesn’t think there needs to be a link between carrier selection and SDT

-	Intel thinks that this point overlaps with 504 email discussion.   Intel would like to confirm that we do not modify the legacy cell selection mechanism.

-	Ericsson thinks that it would be good to control the carrier selection

-	Huawei agrees with Nokia and thinks that we need to discuss CG and RA with 504.

-	LG thinks that this depends whether we do carrier selection first or SDT selection

Proposal 6 (18/8): Data volume threshold is the same for CG-SDT and RA-SDT

-	Xiaomi thinks that CG configuration is UE specific so it should be aligned between common RA and CG configuration.

-	Nokia thinks that since we don’t do switching between CG and RA there is no reason to not keep them independent.  Panasonic agrees with Nokia. Huawei thinks that this targets different use cases and it’s simple to provide separate configuration.

-	Qualcomm sees little benefit and if we have different thresholds we’d have a problem on how to select.

-	ZTE explains that this is just an overall threshold on whether SDT is allowed or not and we would end up mixing the CG and RA selection.  This threshold will be larger than the CG TB or RA TB.  Apple, InterDigital, Lenovo, LG agrees with ZTE.   Lenovo also thinks that the SDT selection step will be simple.  LG thinks we need to keep it simple, we already have enough selection criteria.

-	Vivo also agrees with ZTE and this is also for the network to determine whether the UE should do subsequent transmissions.

Agreements: - RSRP threshold is used to select between SDT and non-SDT procedure, if configured (RSRP refers to the same RSRP measured for carrier selection). - RSRP threshold to select between SDT and non-SDT procedure is used for both CG-SDT and RA-SDT - RSRP threshold to select between SDT and non-SDT procedure is same for both CG-SDT and RA-SDT - RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network. - Confirm that cell selection mechanism is not modified - RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT) - Data volume threshold is the same for CG-SDT and RA-SDT (can be checked further in stage 3 if we obtain majority support) 8 FFS on the order and missing pieces (e.g. failure, fallback) of the high level procedure. The details of the procedures are left for stage 3. FFS on the procedure below, but copied for information. A. Upon arrival of data only for DRB/SRB(s) for which SDT is enabled, the high level procedure for selection between SDT and non SDT procedure is as follows: If CG-SDT criteria is met: UE selects CG-SDT. UE initiate SDT procedure Else if RA-SDT criteria is met: UE selects RA-SDT. UE initiate SDT procedure Else: UE initiate non SDT procedure. B. CG-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold FFS 3) CG-SDT resources are configured on the selected UL carrier and are valid C. RA-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold 3) 4 step RA-SDT resources are configured on the selected UL carrier and criteria to select 4 step RA SDT is met; or 2 step RA-SDT resources are configured on the selected UL carrier and criteria to select 2 step RA SDT is met - Switching from SDT to non-SDT is supported. - FFS Switching from CG-SDT to RA-SDT is not allowed 11 UE switches from SDT to non-SDT in following cases: - Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. - Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. - FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

R2-2103022	Summary of General and other control plane open issues for SDT (email: [Post 113-e][502])	Rapporteur (ZTE)	discussion

Proposal 2: When data arrives on non-SDT bearer during SDT, then the UE shall send a DCCH message. The detailed contents of the message are FFS (new message/using an existing message etc) (21/4)

-	LG thinks that there are cases where DCCH doesn’t work.

-	Huawei doesn’t think that the problems with CCCH are properly characterized.

Proposal 2: When data arrives on non-SDT bearer during SDT, then the UE shall send a DCCH message. The detailed contents of the message are FFS (new message/using an existing message etc) (21/4)

-	If non-SDT data arrives during SDT, then the RRC layer will generate a DCCH message and submit it to the lower layers. The MAC layer will include this in UL after contention resolution. If contention resolution fails a new RACH procedure will be triggered by MAC (same as today) and the DCCH message can be sent after RACH procedure is complete.

Proposal 3: gNB can only configure MN terminated MCG bearer type for SDT (25/0)

=>	Noted

Agreements

1	gNB can only configure MN terminated MCG bearer type for SDT

2	Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today)

3	Down-scope to two solutions (CCCH or DCCH) and ask SA3 about security issues (explain that CCCH message will be repeated in same cell and ask if there is a question)

R2-2103897	DRAFT Reply LS on small data transmission	Ericsson	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN3

-	Need to add that MAC and RLC need to be in the same entity and a different assumption would have impact.

=>	The LS is revised in R2-2104398

R2-2104398	DRAFT Reply LS on small data transmission	Ericsson	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN3

=>	add a comment that this is a RAN2 assumption.  It is up to RAN3 to make the final decisions, however, if RAN3 needs another solution to please let RAN2 know.

=>	The LS will be updated in R2-2104400

R2-2104490	DRAFT Reply LS on physical layer aspects of small data transmission (Reply to R2-2102620)	ZTE Corporation	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:RAN1

=>	Not treated

### 8.6.2	User plane common aspects

Overall user plane procedure for SDT (including triggering and thresholds, HARQ, and MAC CEs), data volume computation,. suppression of PDCP status report, RSRP threshold for SDT selection, switching between CG/RA, and any other user aspects included in Post113-e][501][503]   which cannot be concluded as part of the email

Email discussion summary expected for this AI durin 113bis-e

R2-2104395	Summary of UP SDT open issues 		LG

=>	Noted

Agreements

1	 The UE performs PDCP re-establishment implicitly, i.e. without explicit indication for PDCP re-establishment, when the UE initiates SDT procedure.

2	As in legacy, whether to support ROHC continuity is explicitly configured by the network.

3	PDCP duplication is not supported for SDT

4	connected mode DRX is not supported for SDT

5	PHR functionality is supported for SDT.   FFS on PHR procedure

6	SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA because SR resource is not available, same as legacy

For potential agreement:

Proposal 6: Data volume used for SDT selection criteria is calculated as the total sum of Buffer Size across SDT RBs. (15/2/5/5)

Proposal 7: PHR functionality is supported for SDT. (21/8)

-	LG explains then next step is to discuss the details.

Proposal 9: SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA-SDT because SR resource is not available, same as legacy (14/13).

For further discussion:

Proposal 2: RAN2 discuss further whether the UE can implicitly disable PDCP status report when the UE initiates SDT procedure. (13/14)

Proposal 5: RAN2 discuss further whether the RLC failure handling should be supported for SDT. (11/13)

Proposal 8: RAN2 discuss further whether and how the LCH restriction is used for SDT (12/12/12).

Left for RAN1 decision:

Proposal 11: Whether to support BFD and BFR for SDT is up to RAN1 decision.

Not treated

R2-2102708	User Plane Common Aspects of RACH and CG based SDT	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102750	Discussion on user plane issues of SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102755	Discussion on User Plane Aspect of Small Data Transmission	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100139

R2-2102840	User plane aspects for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103018	User plane open issues for SDT	ZTE Corporation, Sanechips	discussion

R2-2103102	Analysis on UP common aspects of SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103197	Support of CA and PDCP CA duplication	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2100419

R2-2103319	The UP common issues for small data transmissions	Lenovo, Motorola Mobility	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103430	Discussion on user plane common aspects of NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101221

R2-2103444	Discussion on data volume threshold for small data transmission	PANASONIC R&D Center Germany	discussion

R2-2103454	Avoid triggering RA during subsequent SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103521	Common aspects for SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103528	User Plane common aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103531	User plane common aspects for SDT	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103583	Some aspects of User Plane for SDT in NR 	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103672	Discussion on small data transmission	Google Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103674	Discussion on beam operations for small data transmission	Google Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103714	Remaining issues on transmission type selection and overall procedure	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103870	User plane aspects on the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103990	Consideration on overall SDT procedure and criteria	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104206	On the overall and detailed procedure of SDT	China Telecommunications	discussion

R2-2104220	Discussion on data volume calculation	Xiaomi Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104263	Discussion on Small Data Transmission	TCL Communication Ltd.	discussion	Rel-17

### 8.6.3	Control plane common aspects

Cell reselection and failure handling, handling of subsequent data transmissins (including, how to indicate presence of subsequent data, etc) handling of non-SDT DRBs (including whether to resume or not non-SDT), CP data over SDT, SDT termination and data loss prevention and any other control plane aspects included in [Post113-e][501][502][503] which cannot be concluded as part of the email

R2-2103971	Report of [Post113-e][503][SDT] T319, cell reselection and re-establishment	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

Proposal 7

-	Intel thinks that we should discuss further whether the UE goes to Idle or stays in INACTIVE.

-	LG thinks this is a corner case and we shouldn’t optimize behaviour.  Ericsson, Lenovo, vivo, oppo, QC, agrees with LG.

-	ZTE thinks that we shouldn’t use different mechanisms and using the same behaviour as 6.  Samsung doesn’t think proposal 7 depends on 6 as it depends on SA3.

=>	Noted

Agreements:

- SDT failure detection timer is started upon initiation of SDT procedure

- T319 legacy is not started if RRCResumeRequest or RRCResumeRequest1 is transmitted …

- T319 legacy stop conditions also apply to SDT failure detection timer

- RRC re-establishment procedure is not supported for SDT

5	An LS is sent to SA3 to verify feasibility/impacts of re-using same NCC/I-RNTI value temporarily for RRC Resume procedure in new cell during SDT procedure (include same cell question from 502]

6	FFS - RAN2 to select between the following options for cell re-selection during ongoing SDT procedure next meeting: 1) UE transitions to IDLE, possibly performing high-layer retransmission (8/25); or 2) UE remains in INACTIVE and sends RRC Resume to new cell

7	FFS Upon SDT failure detection timer expiry, the same procedure as T319 expiry is used (e.g. transition to IDLE as in the case of expiry of the T319 timer and attempts RRC connection setup)  (18/8)

Requires online discussion for next meeting

Proposal 3: 	RAN2 to decide whether SDT failure detection timer: 1) has an extended duration to accommodate subsequent SDT (13/25); or 2) is restarted upon (re)transmission or reception of small data (12/25) [CB next Tuesday or postpone it to next meeting]

R2-2104396		LS to SA3 on Small data transmissions – email discussion 503 (InterDigital)

-	Intel would have liked to add PDCP count as the CCCH solution would break the assumption that we shouldn’t do key reuse

=>	Update the following question: Can a CCCH message reusing the I-RNTI and resumeMAC-I be transmitted again in the same cell after SDT initiation, e.g. similar to legacy RRC Reject case (but without having received RRC Reject at the UE)?

=>	The LS is approved in R2-2104401 with the change above

R2-2104397	LS to CT1 on Small data transmission – email discussion 504 (Intel)

-	LG thinks that companies want to remove question 1 and question 2.

-	ZTE thins that we have no question but we are asking feedback.  We don’t need to explicitly ask about all three, we can solicit any inputs.   Vivo agrees with ZTE

-	Ericsson also had same feeling about question 1 and question 2.  Focus the question for 3.

-	Huawei and QC are fine to remove question 1 and question 2

=>	Remove question 1 and 2

=>	Email discussion to continue and to be approved Wednesday

R2-2104399	Report of email discussion 504 	Intel

=>	Noted

Not treated

R2-2102709	Control Plane Common Aspects of RACH and CG based SDT	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102751	Discussion on control plane issues of SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102756	Discussion on RRC-Controlled Small Data Transmission	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100140

R2-2102841	Signalling and NAS-AS interaction for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102842	Fallback and failure handling for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102900	New timers for SDT failure detection	Langbo	discussion	Rel-17	NR_SmallData_INACTIVE-Core	Late

=> Withdrawn

R2-2102991	Handling of non-SDT traffic arrival	PANASONIC R&D Center Germany	discussion

R2-2103019	Control plane aspects of SDT	ZTE Corporation, Sanechips	discussion

R2-2103103	Considerations on Some Common Control Plane Issues	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103151	Handling of non-SDT data arrival	Potevio Company Limited	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103198	RAN paging reception and response during SDT	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103257	Handling of non-SDT data during SDT	ETRI	discussion

R2-2103299	Discuss on solutions for arriving of non-SDT data during SDT	NEC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103405	Consideration on CP issues for small data transmission	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103431	Discussion on control plane common aspects of NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101223

R2-2103455	Beam management in SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103497	SDT control plane aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE

R2-2103522	CP aspects for SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103568	Discussion on subsequent SDT in NR, timer handling, and support for SRB1/2	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103715	Non-SDT data transmission	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103796	Subsequent small data transmission	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103867	Non-SDB handling during the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103868	Control plane aspects on the SDT procedure	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103904	Control plane common aspects for SDT	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103970	CP and configuration aspects for small data	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103972	[Draft] LS to SA WG3 on re-use of same NCC and I-RNTI value for RRC Resume procedure in different cells during small data transmission procedure.	InterDigital	LS out	Rel-17	NR_SmallData_INACTIVE-Core	To:SA3

R2-2103989	Discussion on switching to non-SDT procedure	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103991	Discussion on cell reselection during SDT	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104204	Resuming non-SDT RBs and indication	LG Electronics Inc.	discussion	Rel-17	NR_SmallData_INACTIVE-Core	Late

R2-2104221	Discussion on the support of the RRC-less SDT	Xiaomi Communications, Intel Corporation, ASUSTeK, Fujitsu, MediaTek, Apple, Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104222	Technical details of the RRC-less SDT	Xiaomi Communications, ASUSTeK, Fujitsu, Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

### 8.6.4	Aspects specific to RACH based schemes

RA resource configuration and selection, PDCCH monitoring after successful SDT RA completion, RAN2 specific details of context fetch/data forwarding with and without anchor relocation

Not treated

R2-2102710	Details of RACH bsaed Small Data Transmission	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102752	Discussion on RACH based SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102757	Supporting Small Data Transmission via RA Procedure	vivo	discussion	NR_SmallData_INACTIVE-Core	R2-2100141

R2-2102847	Fallback issue for 2-step RA based small data transmission	Sharp	discussion	NR_SmallData_INACTIVE-Core	R2-2100413

R2-2103020	Open issues for RACH based SDT	ZTE Corporation, Sanechips	discussion

R2-2103104	Considerations on Procedures without Anchor Relocation	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103105	Analysis on Search Space of RA-SDT	CATT	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103252	Discussion on RACH-based SDT	Spreadtrum Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103264	PDCCH monitoring in subsequent data transmission period	Asia Pacific Telecom co. Ltd, FGI	discussion

R2-2103403	Analysis on open issues of RA based SDT	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103433	Discussion on RACH based NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101231

R2-2103456	Discussion on RO configuration between SDT and non-SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103519	RACH based SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103529	Details of context fetch and data forwarding	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103580	Discussion on context fetch and anchor relocation 	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103716	Anchor relocation and context fetch	CMCC	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103869	Subsequent data transmission for SDT	Apple	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103903	Small data transmission with RA-based schemes	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

### 8.6.5	Aspects specific to CG based schemes

This AI will not be treated in RAN2#113bis-e (only the email discussion [504] in AI 8.6.1 will be treated)

CG resources, configuration and selection, validity of CG resources, multiple CG configurations, handling of beam selection for CG (including association between CGs and SSBs) etc, any other aspects included in [Post113-e][504][SDT] which cannot be concluded as part of the email

R2-2103533	Report from [POST113-e][504][SDT] CG Open Issues	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

The following proposals are potentially easily agreeable:

Proposal3: UE start a window after CG/DG transmission for CG-SDT (24/24). FFS whether to design a new timer or to reuse an existing timer.

-	InterDigital explains in the UP email discussion companies don’t think DRX is needed so we can agree with the new timer.   LG clarifies that we may not need new mechanism.

Proposal4: Support retransmission by dynamic grant for CG-SDT. (24/24)

Proposal5: Support multiple HARQ processes for uplink CG-SDT. (18/24)

Proposal6: CG resource availability delay is not considered as a criterion for CG validation. (18/20)

Proposal7: UL carrier selection is performed before CG-SDT transmission. (23/24)

Agreements:

1	CG-SDT resources can be configured at the same time on NUL and SUL

2	Implicit release of CG-SDT resource is not supported

3	UE start a window after CG/DG transmission for CG-SDT.   FFS whether to design a new timer or to reuse an existing timer.

4	Support retransmission by dynamic grant for CG-SDT.

5	Support multiple HARQ processes for uplink CG-SDT.

6	CG resource availability delay is not considered as a criterion for CG validation.

7	UL carrier selection is performed before CG-SDT selection

8	FFS CG-SDT resource can be configured on BWPs other than initial BWP

The following proposals need further discussion for next meeting:

Proposal8: RAN2 should further discuss whether to support CG configuration request.

Proposal9: Release of CG-SDT configuration by system information indication is not supported. (5/21)

Proposal10: CG-SDT resource can be configured on BWPs other than initial BWP configured by system information (17/24).

-	Nokia would like to discuss how it impacts the paging if it moves to another BWP.  Huawei explains that there is network implementation to solve this problem.  Qualcomm also sees some issues.

Proposal11: RAN2 should further discussion whether to support autonomous retransmission for CG-SDT.

Proposal12: Support L1-ACK feedback for CG-SDT. (14/24) Send an LS to RAN1 on this.

-	Nokia doesn’t support this and there has been no motivation.  Huawei explains that subsequent transmissions are supported with CG.  This is also linked with the agreement related to the multiple HARQ processes.  ZTE asks why this is different to CG-DFI but support asking RAN1.

Proposal13: UE does not select any SSB if none of the SSBs’ RSRP is above the RSRP threshold. (18/23) FFS the UE behavior when none of the SSB’s RSRP is above the threshold

Not treated

R2-2102711	Details of Configured Grant based Small Data Transmission	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102753	Discussion on CG based SDT	OPPO	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2102758	Supporting Small Data Transmission via CG configuration	vivo	discussion	NR_SmallData_INACTIVE-Core

R2-2102843	On Configured Grant aspects for SDT	Intel Corporation	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103021	Open issues for CG based SDT	ZTE Corporation, Sanechips	discussion

R2-2103199	PDCCH monitoring after TAT expiry	Fujitsu	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2000420

R2-2103265	CG-SDT based on beam operation	Asia Pacific Telecom co. Ltd, FGI	discussion

R2-2103367	Aspects specific to CG based SDT	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SmallData_INACTIVE

R2-2103404	Consideration on CG based small data transmission	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103434	Discussion on CG based NR small data transmission	Qualcomm Incorporated	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101233

R2-2103457	Beam selection for CG-SDT	ASUSTeK	discussion	Rel-17	NR_SmallData_INACTIVE-Core	R2-2101752

R2-2103520	Details of CG based SDT	Ericsson	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103532	Small data transmission with CG-based scheme	Huawei, HiSilicon	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103581	Details of CG-based scheme for SDT in NR   	Sony Europe B.V.	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2103795	CG-based SDT	InterDigital	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104223	Remaining issues of CG SDT	Xiaomi Communications	discussion	Rel-17	NR_SmallData_INACTIVE-Core

R2-2104241	On CG Resource Configuration in Small Data enhancement	China Telecommunications	discussion