3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2104301

Online, April 12 – April 20, 2021

Agenda item:	10.1

Source: 	Vice Chairman (Nokia)

Title:	Report on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

Document for:	Approval

# Organizational

List of offline email discussions:

NOTE: the email discussion deadlines are meant to allow at least all regions to have one day to comment (other than weekend) and also give rapporteurs time to update their proposals before the meeting)

Organizational

[AT113bis-e][200] Organizational Tero – LTE legacy, LTE Rel-16 and LTE/NR mobility

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs for presentation

Intended outcome (for LS discussion):

    - General information sharing about the sessions

Deadline for providing comments to LSs:

    - Deadline: 1st week Thu, UTC 0900

LTE Legacy

[AT113bis-e][201][LTE] LTE Miscellaneous R15/16 corrections (Ericsson)

Scope:

    - Discuss which CRs under AI 4.5 and 7.4 marked for this email discussion are agreeable

    - Provide final CRs

Intended outcome:

    - Discussion summary in R2-2104310 (by email rapporteur)

    - Agreeable CRs by proponents (if revised versions are required, proponents should obtain Tdoc numbers from session chair or RAN2 secretary to provide those)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

LTE Rel-17

[AT113bis-e][202][LTE] UPIP for LTE Rel-17 (Qualcomm)

Scope:

    - Discuss the UPIP contributions under AI 9.3 and determine whether there is consensus on what RAN2 could reply to SA3.

    - Can provide also draft LS reply to SA3

Intended outcome:

    - Discussion summary in R2-2104325 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary and draft LS):  1st week Fri, UTC 0900

LTE Legacy up to Rel-16 (kicked off after 1st week online session)

[AT113bis-e][203][LTE] One-shot configurations (Huawei)

Scope:

    - Discuss whether something needs to be done for one-shot configurations in 36.331

Intended outcome:

    - Discussion summary in R2-2104323 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary):  1st week Fri, UTC 0900

LTE/NR Mobility (to be kicked off on 1st week Monday)

[AT113bis-e][210][MOB] CHO/CPC corrections (Huawei)

Scope:

    - Discuss which CHO/CPC corrections (for LTE and NR) marked for this discussion are seen agreeable. CRs that are editorial can be merged together

Intended outcome:

    - Discussion summary in R2-2104311 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][211][MOB] DAPS corrections (Samsung)

Scope:

    - Discuss which DAPS corrections (for LTE and NR) marked for this discussion are seen agreeable. CRs that are editorial can be merged together.

Intended outcome:

    - Discussion summary in R2-2104312 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

LTE/NR Mobility (tentative - to be decided during 1st week Monday online session)

[AT113bis-e][212][MOB] CRs UAI/SUI after CHO completion (MediaTek)

Scope:

    - Finalize CRs for UAI/SUI repetition after CHO based on online decisions.

Intended outcome:

    - Agreeable CRs to 36.331 in R2-2104327 and to 38.331 in R2-2104328

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Mon, UTC 1000

[AT113bis-e][213][MOB] RRCReconfiguration with DAPS source release (Ericsson)

Scope:

    - Discuss how/whether to capture the agreements on what is allowed to be configured when daps-SourceRelease is sent to UE according to online agreements.

Intended outcome:

    - Discussion summary in R2-2104330 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

LTE/NR Rel-16 DCCA (to be kicked off on Monday August 17th)

[AT113bis-e][220][DCCA] Miscellaneous DCCA corrections (Ericsson)

Scope:

    - Discuss corrections under R16 DCCA WI marked for this discussion to see which CRs could be agreeable. CRs that are editorial or smal can be merged to rapporteur CRs.

Intended outcome:

    - Discussion summary in R2-2104313 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][221][DCCA] NR-DC power control signalling (Huawei)

Scope:

    - Discuss NR-DC PC signalling corrections (for FR2) under R16 DCCA WI marked for this discussion to understand best way forward for RAN2.

Intended outcome:

    - Discussion summary in R2-2104314 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

LTE/NR Rel-16 DCCA (only kicked off if RAN4 LS reply is received)

[AT113bis-e][222][DCCA] NR-DC Cell grouping (NN)

Scope:

    - Discuss RAN2 details of NR-DC cell grouping based on online agreements

Intended outcome:

    - Discussion summary in R2-2104324 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1200

NR Rel-17 DCCA (only started after 1st week online session)

NR Rel-17 Multi-SIM (only started after 1st week online session)

[AT113bis-e][230][MUSIM] Reply LS to SA2 on paging cause (Intel)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104331 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

[AT113bis-e][231][MUSIM] Impacts of NAS-based busy indication (RAN2 VC)

Scope:

    - Discuss whether the agreement to only support NAS-based busy indication creates issues with SA2/CT1 and determine whether LS needs to be sent to SA2/CT1.

    - If needed, provide draft LS to SA2/CT1 asking them for feedback

Intended outcome:

    - Discussion summary in R2-2104332 (by email rapporteur) and (if needed) draft LS in R2-2104333

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  2nd week Mon, UTC 1200

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1600

NR Rel-17 RAN Slicing (only started after 1st week online session)

[AT113bis-e][251][NR] Slice-specific cell reselection (Intel)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104321 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

[AT113bis-e][252][NR] Slice-specific RACH (CMCC)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104322 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

List of assigned pre-meeting AI summary document responsibles

Summary of AI 8.2.2.1: Deactivation of SCG (Huawei)

Summary of AI 8.2.2.2: UE measurements and reporting in deactivated SCG (OPPO)

Summary of AI 8.2.2.3: Activation of deactivated SCG (ZTE)

Summary of AI 8.3.2: Paging collision avoidance  (Ericsson)

Summary of AI 8.3.3: UE notification on network switching for multi-SIM (Samsung)

Summary of AI 8.3.4: Paging with service indication (vivo)

Dates and deadlines

April 1 23.59 PDT	(April 2 06.59 UTC) Tdoc number allocation deadline for all tdocs.
General Tdoc Submission Deadline, as usual. Kick off, summaries.

Late submission up until April 6 06.59 UTC is accepted for CRs (as TSes are late).

April 6		Emails are allowed, 3GPP silent period has ended.

April 8 0700 UTC	Tdocs submission deadline for Summaries (baseline version)

April 12 0700 UTC	e-Meeting Start (by email) (April 13 0700 UTC is first possible email deadline).

April 16 1000 UTC 	Suspend decision making in email discussions (= no deadlines etc)

It should be possible for a delegate to take the weekend off, rejoin and not miss decisions.

April 19 0800 UTC	Resume decision making in email discussions.

April 19 1800 UTC	For AT-meeting email discussions that doesn’t come back on-line: This is the Last Deadline for Technical/Functional Comments, non-agreeable parts are removed from proposed agreements. The last 24h until e-meeting Stop is for checking and during this time only minor wording changes, removals / simplifications are done.

April 20 1800 UTC	e-Meeting Stop, no more email comments for AT-meeting email discussions. Decision confirmations announced within 24h. Session notes for email checking.

April 27		Deadline Short Post113bis-e email discussions.

April 28 – May 5	3GPP silent period

May 10 23.59 PDT	Deadline long Post113bis-e email discussions and submission deadline next meeting.

Web Conference Schedule

Note that this schedule is indicative and can change. Changes to the schedule will be announced with notice of at least 24h. No Overtime, Hard stop at UTC 15.55 and UTC 05:10

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 12:15-13:05 | NR15 NR16 Main session (Johan) UP [6.1.3.1 MAC]: Email discussion [Post113-e][052][NR16], UP [6.1.3.1 MAC]: Intra-UE prio and UL-skip, LSin: R2-2102626, R2-2102628. CP [5.4.1.1] RLC bearer Full Config R2-2104140 etc. CP [5.4.3] BCS EN-DC at least R2-2104025, R2-2103061 CP [6.1.4.3] Transp TxD R2-2102646 CP [5.4.3] Email discussion [Post113-e][051][NR15] CP [5.4.1.2] MN SN Configuration Restrictions CP [5.4.2] Email discussion [Post113-e][005][NR15] | NR16 Pos (Nathan) | NR17 NTN (Sergio) [8.10.1] Organizational [8.10.2.1] - [Post113-e][106] outcome [8.10.2.2] [8.10.2.3] - [Post113-e][107] outcome |
| 13:05-14:25 |  | NR16 V2X (Kyeongin) | NR17 NTN (Sergio) [8.10.3.1] [8.10.2.3] - [Post113-e][108] outcome - CHO aspects |
| 14:25-15:45 | NR17 Multicast (Johan) [8.1.1][8.1.3][8.1.2.1] | NR16 DCCA (Tero) - [Post113-e][224] outcome - NR-DC cell grouping NRLTE16 MOB (Tero) - UAI/SUI for CHO - RRC reconfig with DAPS release - RLF/re-establishment and DAPS LTE16e (Tero) - [Post113e][206] outcome - LTE Rel-15 topics - LTE Rel-16 topics | LTE17 IoT (Brian) [9.1.1] Organizational [9.1.3] Carrier selection |
| Tuesday |  |  |  |
| 12:15-13:05 | NR17 eNPN (Johan) Briefly: [8.16.1], [8.16.3] [8.16.2] | NR17 RAN Slicing (Tero) - Cell reselection - RACH | NR17 SL Relay (Nathan) - Organisational - Discovery - Re/selection (if time) |
| 13:05-14:25 | NR17 ePowSav (Johan) | NR17 Multi-SIM (Tero) - Network switching - Paging collision | NR17 SL enh (Kyeongin) |
| 14:25-15:45 | R17 Other (Johan) NR15 NR 16 continuation (if needed) | NR17 SONMDT (HuNan) | NR17 Small Data Enh (Diana) - email discussions [501][502][503] |
| Wednesd |  |  |  |
| 04:00-05:00 | NR17 Multicast (Johan) [8.1.2.2][8.1.2.4] | NR17 RedCap (Sergio) [8.12.1] Organizational [8.12.3.1] - [At113-e][101] outcome - continue on eDRX aspects [8.12.3.2] - [At113-e][102] outcome - continue on RRM relaxations aspects | NR16 SONMDT (HuNan) |
| Thursday |  |  |  |
| 04:00-05:00 | NR17 QoE (Johan) | NR17 DCCA (Tero) - SCG deactivation - UE measurements in deactivated SCG - SCG activation | LTE17 IoT (Brian) [9.1.4] Other |
| Friday |  |  |  |
| 04:00-05:00 | NR17 eIAB (Johan) | NR17 Pos (Nathan) - Organisational - Latency enhancements - RRC_INACTIVE | LTE16e IoT (Emre) |

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 12:15-13:05 | NR17 IoT NTN SI (Johan) | NR16 SONMDT / NR17 SONMDT (HuNan) | NR16 V2X / NR17 SL enh (Kyeongin) |
| 13:05-14:25 | NR17 eIAB (Johan) NR15 NR16 NR17 Main session (Johan) e.g. [6.1.3.5 BAP]: R2-2103935 | LTE17 (Tero) - GSMA LS on Scell attack - Outcome of [202]: SA3 LS on UPIP for LTE NR16 DCCA (Tero) - Outcome of [220] - Outcome of [221] NRLTE16 MOB (Tero) - Outcome of [210] - Outcome of [211] LTE16e (Tero) - Outcome of [201] (if needed) | NR17 Pos (Nathan) - RRC_INACTIVE (cont.) - On-demand PRS - Integrity |
| 14:25-15:45 | NR15 NR16 NR17 Main session (Johan) | CB Sergio [NR-NTN] - Outcome of any offline discussion(s) [RedCap] - Outcome of any offline discussion(s) | NR17 SL Relay (Nathan) - Re/selection (cont.) - L2 specific topics |
| Tuesday |  |  |  |
| 12:15-13:05 | CB Johan | CB Diana [SDT] - Outcome of SDT User Plane offline discussion ([AT113bis-e][SDT][501] - outcome of any other offline discussions | TBD CB Sergio (if needed) |
| 13:05-14:25 | CB Johan | CB Tero NR17 DCCA - Outcome of [Post11e-e][234] - Outcome of any SCG deactivation offline discussion(s) Multi-SIM - Outcome of any offline discussion(s) RAN slicing - Outcome of any offline discussion(s) | CB Nathan |
| 14:25-15:45 | CB Johan | CB Kyeongin | CB Brian Emre [9.1.2] Treat RAN4 reply if available, email discussion scope. |

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.5	Other LTE corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

Purely editorial corrections should be avoided, text enhancements may be deprioritized. Corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

By Email [203] (1)

Ambiguity in Need ON for one-shot configurations:

R2-2104013	Discussion on one-shot configuration	Huawei, HiSilicon	discussion	Rel-15	TEI15

To be handled in offline discussion [203] (Huawei)

By Email [201] (3)

MDT logging for any cell selection (postponed during RAN2#113e to allow more time for checking):

R2-2103816	On the lack of PLMN identity check in case of anyCellSelected state related logging	Ericsson	discussion

R2-2103813	On the lack of PLMN identity check in case of anyCellSelected state related logging	Ericsson	CR	Rel-15	36.331	15.13.0	4624	-	F	TEI15

R2-2103814	On the lack of PLMN identity check in case of anyCellSelected state related logging	Ericsson	CR	Rel-16	36.331	16.4.0	4625	-	A	TEI15

By Email [201]

Supported UE category fallbacks:

R2-2104014	Correction on category dependency for DL Category 13	Huawei, HiSilicon	CR	Rel-12	36.306	12.13.0	1806	-	F	TEI12

By Email [201]

T325 (frequency deprioritization timer) handling at inter-RAT HO:

R2-2104248	Correction on T325	Google Inc.	CR	Rel-15	36.331	15.13.0	4640	-	F	LTE-L23, TEI11

R2-2104253	Correction on T325	Google Inc.	CR	Rel-16	36.331	16.4.0	4641	-	F	LTE-L23, TEI11

Email discussions ([201],  [203])

[AT113bis-e][201][LTE] LTE Miscellaneous R15/16 corrections (Ericsson)

Scope:

    - Discuss which CRs under AI 4.5 and 7.4 marked for this email discussion are agreeable

    - Provide final CRs

Intended outcome:

    - Discussion summary in R2-2104310 (by email rapporteur)

    - Agreeable CRs by proponents (if revised versions are required, proponents should obtain Tdoc numbers from session chair or RAN2 secretary to provide those)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][203][LTE] One-shot configurations (Huawei)

Scope:

    - Discuss whether something needs to be done for one-shot configurations in 36.331

Intended outcome:

    - Discussion summary in R2-2104323 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary):  1st week Fri, UTC 0900

By Email or By Web Conf 2nd week (summary of [201] and summary of [203])

R2-2104310	Summary of [AT113bis-e][201][LTE] LTE Miscellaneous R15/16 corrections (Ericsson)	Ericsson	discussion	Rel-16	LTE-L23, TEI15, TEI16

R2-2104323	Summary of [AT113bis-e][203][LTE] One-shot configurations (Huawei)	Huawei	discussion	Rel-15	TEI15

# 6	Rel-16 NR Work Items

Essential corrections. While high maintenance intensity is expected, Rel-16 corrections are treated separately per WI.

Tdoc Limitation: 30 tdocs in total for all sub agenda items, or the restriction for each sub-AI, whichever is more restrictive.

NOTE: FOR R2#113bis-e it is expected that ~30% of the input tdocs under this AI will be selected for initial postponement to the next meeting.

## 6.4	NR and LTE mobility enhancements

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921)

Documents in this agenda item will be handled in a break out session).

No documents should be submitted to 6.4. Please submit to 6.4.x

Purely editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

Tdoc Limitation: 8 tdocs, See also tdoc limitation for Agenda Item 6

### 6.4.1	CHO/CPC Corrections

Including incoming LSs related to CHO/CPC (if any).

This AI addresses NR CPC and corrections to NR/LTE CHO (i.e. both NR and LTE-specific corrections for CHO should be submitted here).

Including corrections to control and user plane specifications (e.g. 3x.331, 3x.323, 3x.321) for CHO and CPC.

Web Conf (Monday 1st week) (2+4)

UAI/SUI transmission after CHO completion:

R2-2103215	Conditional handover and UAI/SUI		MediaTek Inc., Ericsson, Sharp, LG Electronics, Qualcomm Incorporated	discussion	Rel-16

Proposal 1: After executing a conditional handover, the UE unconditionally sends updated UAI and/or SUI messages to the target cell based on the UE’s current status.

Proposal 2: Adopt the draft CR of section 5.

R2-2104001	Discussion on the re-transmission of UL message after CHO execution	Huawei, HiSilicon, China Telecom	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Proposal 1: If both of the following conditions are met, the UE shall re-transmit the latest transmitted UE information to the target cell after the successful CHO execution:

(1) during the time window “from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution”, the UE should initiate at least one transmission of UE information (i.e.UAI or SUI)

(2) after successful CHO to the target, the UE is still configured with the reporting of the UE information

Discussion

-	Apple supports the MediaTek proposal. Nokia wonders what "unconditionally" means - is it only about timing and not about configuration? MTK confirms this is only about the timing.

-	Huawei would also want to ensure the configuration is followed and (2) is very relevant.

-	OPPO thinks MTK proposal this will increase signalling overhead.

-	Samsung doesn't think the Huawei proposal is very complex. If the UE never sent any UAI/SUI before CHO, is it now required to send it? MTK clarifies that if the UE is configured with the assistance information, it shall provide the UAI/SUI after CHO based on UE status (i.e. if the UAI/SUI would not trigger, it will not trigger it after CHO either).

-	Huawei can accept the MTK proposal. Wonders what happens to previous agreements? MTK clarifies this is not reverting them but adding on top. Intel can also accept MTK proposal.

- Agreement

- 1	After executing a conditional handover,if the UE is still configured with the reporting of the UE information, the UE sends updated UAI and/or SUI messages to the target cell based on the UE’s current status.

Offline discussion [212] for drafting the CR on this (MTK) - CR for 36.331 provided in R2-2104327 and CR for 38.331 provided in R2-2104328.

R2-2104100	Discussion on UE information transmission in CHO case ZTE Corporation discussion NR_Mob_enh-Core, 5G_V2X_NRSL-Core

Noted

R2-2102875	CR on UE Information report for CHO (Option-1)	OPPO	CR	Rel-16	38.331	16.4.1	2479	-	F	5G_V2X_NRSL-Core, NR_Mob_enh-Core

Not pursued

R2-2102876	CR on UE Information report for CHO (Option-1)	OPPO	CR	Rel-16	36.331	16.4.0	4608	-	F	5G_V2X_NRSL-Core, LTE_feMob-Core

Not pursued

R2-2102877	CR on UE Information report for CHO (Option-2)	OPPO	CR	Rel-16	38.331	16.4.1	2480	-	F	5G_V2X_NRSL-Core, NR_Mob_enh-Core

Not pursued

R2-2102878	CR on UE Information report for CHO (Option-2)	OPPO	CR	Rel-16	36.331	16.4.0	4609	-	F	5G_V2X_NRSL-Core, LTE_feMob-Core

Not pursued

By Email [210] (1+1)

Ambiguity in WI agreements and captured text:

R2-2103114	Discussion on Applicable Cases for Failure Recovery via CHO	CATT	discussion	Rel-16	NR_Mob_enh-Core

Observation 1: The agreement made in RAN2#113e allows the UE to perform failure recovery via CHO upon inter-RAT handover failure.

Observation 2: Agreement made in RAN2#109e clarified that failure recovery via CHO in Rel-16 is applicable only to RLF, intra-RAT handover failure or intra-RAT Conditional handover failure which is not applicable for inter-RA handover failure.

Proposal 1:RAN2 to confirm which of the following agreement is valid:

-	Confirm the agreement made in #109e meeting is still valid, i.e. Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure. Inter-RAT handover failure can’t trigger failure recovery via CHO. TP option 2 should be adopted.

-	Confirm the agreement made in #113e is valid, Inter-RAT handover failure can trigger failure recovery via CHO. The agreement made in #109e meeting should be revised as following: Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure or inter-RAT handover failure. TP option 1 should be adopted.

Is CHO with SCG configuration allowed in (MR-)DC?

R2-2103332	Clarification on SCG configuration in CHO	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_Mob_enh-Core

Observation 1: CHO to a target PCell candidate configuring an SCG is allowed in Rel. 16 in case there is no RAN3 impact.

Observation 2: There might be RAN3 impact if CHO with SCG configuration is allowed in (MR-)DC.

Proposal 1: RAN2 to clarify the scenarios for which CHO with SCG configuration shall be supported in Rel. 16.

Proposal 2: RAN2 to send an LS describing the RAN2 understanding on the support of CHO with SCG configuration and asking RAN3 to work on the specification changes, if needed.

By Email [210] (1+2+1+1+1)

CPC configuration via SRB1 after initial SRB3 configuration:

R2-2104000	Discussion on cross-SRB CPC reconfiguration	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

CHO evaluation after DAPS fallback:

R2-2103046	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	36.331	16.4.0	4613	-	F	LTE_feMob-Core

R2-2103047	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	38.331	16.4.1	2497	-	F	NR_Mob_enh-Core

Procedural text for section on" Inability to comply with RRCReconfiguration":

R2-2103331	38.331 CR: Revised inability to comply with conditional reconfiguration	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.4.1	2507	-	F	NR_Mob_enh-Core

Full config support for CHO:

R2-2104261	Full configuration for CHO	Google Inc.	CR	Rel-16	38.331	16.4.1	2565	-	F	NR_Mob_enh-Core

Miscellaneous CHO corrections to Stage-2:

R2-2104074	Miscellaneous corrections to 37.340 on mobility enhancement	ZTE Corporation (Rapporteur), Sanechips	CR	Rel-16	37.340	16.5.0	0262	-	F	NR_Mob_enh-Core

Email discussions ([210], [212])

[AT113bis-e][210][MOB] CHO/CPC corrections (Huawei)

Scope:

    - Discuss which CHO/CPC corrections (for LTE and NR) marked for this discussion are seen agreeable. CRs that are editorial can be merged together

Intended outcome:

    - Discussion summary in R2-2104311 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][212][MOB] CRs UAI/SUI after CHO completion (MediaTek)

Scope:

    - Finalize CRs for UAI/SUI repetition after CHO based on online decisions.

Intended outcome:

    - Agreeable CRs to 36.331 in R2-2104327 and to 38.331 in R2-2104328

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Mon, UTC 1000

Web Conf 2nd week or By Email (summary of [210])

R2-2104311	Summary of [AT113bis-e][210][MOB] CHO/CPC corrections (Huawei)	Huawei	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Web Conf 2nd week or By Email (CRs from [212])

R2-2104327	CR on UE Information report for CHO 	MediaTek	CR	Rel-16	36.331	16.4.0	4644	-	F	5G_V2X_NRSL-Core, LTE_feMob-Core

R2-2104328	CR on UE Information report for CHO	MediaTek	CR	Rel-16	38.331	16.4.1	2569	-	F	5G_V2X_NRSL-Core, NR_Mob_enh-Core

### 6.4.2	DAPS handover Corrections

Including incoming LSs related to DAPS handover (if any).

This AI jointly addresses corrections to NR and LTE DAPS (i.e. both NR and LTE corrections for DAPS should be submitted here).

Including corrections to LTE/NR control and user plane specifications (e.g. 3x.331, 3x.323, 3x.321) for DAPS HO.

Web Conf (Monday 1st week) (1)

Handling of RRC reconfiguration that includes DAPS source cell release:

R2-2102820	Reconfiguration during DAPS HO	Ericsson	discussion	Rel-16	R2-2100488

Observation 1	The restriction to have UDC or EHC configured during a DAPS handover is missing in the Stage-2 specifications.

Observation 2	It is not clear from the LTE specifications when the target node can configure the UE with SCG, SCells, uplinkDataCompression, ethernetHeaderCompression and/or conditional handover at a DAPS handover.

Observation 3	It is not clear from the NR specifications when the target node can configure the UE with SCG, SCells, multi-TRP configuration, SUL, sidelink, ethernetHeaderCompression and/or conditional handover at a DAPS handover.

Observation 4	Since the daps-SourceRelease indication is handled in the beginning of the procedure in 5.3.5.3 (in both 36.331 and 38.331), it is possible to include configuration of features not supported together with DAPS HO in the same RRC Reconfiguration message.

Observation 5	The explicit source cell indication (daps-SourceRelease) in the RRC Reconfiguration message is included to allow the network to reconfigure the UE before completion of the DAPS HO.

Observation 6	If it would be required for the network to include the explicit daps-SourceRelease in the first RRC Reconfiguration message after successful DAPS HO, it would instead be an implicit release. The daps-SourceRelease indication would then only trigger a failure when not set correctly, which is not the intention.

Observation 7	Conditional reconfigurations are included within an RRC Reconfiguration message that is built by the serving node. They can thus be included in the same message that contains the daps-SourceRelease set by the serving node.

Proposal 1	Clarify in the specifications that the first possible addition of SCG or SCells and configuration of multi-TRP, UDC, EHC, SUL, sidelink or conditionalReconfiguration (CHO) in the target cell at a DAPS HO is in the RRC Reconfiguration message that includes daps-SourceRelease.

Proposal 2	Correct field descriptions for parameters that can be configured in the RRC Reconfiguration message with daps-SourceRelease but where it now says that they cannot be configured if there is a DAPS bearer configured.

Proposal 3	The Text Proposals in section 3 should be introduced in the specifications.

Discussion

-	MTK supports P1. Huawei also agrees but thinks this is already clear in our specifications as it's a normal reconfiguration. Nokia agrees with Huawei.

-	Intel agrees with P1 but thinks it would be good to clarify this in specifications to avoid ambiguities. QC agrees since order of UE implementations can differ.

- Agreement

- 1	RAN2 confirms that the first possible addition of SCG or SCells and configuration of multi-TRP, UDC, EHC, SUL, sidelink or conditionalReconfiguration (CHO) in the target cell at a DAPS HO is in the RRC Reconfiguration message that includes daps-SourceRelease.

Discuss in offline [213] how and whether to capture these in the specifications (Ericsson)

By Email [211] (1+1)

RLF and re-establishment after RA success to target cell but before source cell release:

R2-2103626	Clarification on RLF detection of source Pcell	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Observation 1: It was agreed for UE to stop all the RLF related detection of the source link after the successful completion of the RACH to the target cell.

Observation 2: According to current stage-2 specification, the UE only stops RLM detection after the successful completion of the RACH to the target cell.

Observation 3: According to current stage-3 specification, how to deal with RLF detection of source cell after successful RACH towards target cell is missing.

Proposal 1: Clarify that UE stops RLF detection of the source PCell after the successful completion of the RACH to the target cell in TS 38.331 and TS 36.331.

Proposal 2: Clarify the UE behaviour to only continue RA failure detection and RLF re-transmission failure detection of the source PCell until the successful completion of the RACH to the target cell in TS 38.300 and TS 36.300.

R2-2103625	Correction on RRC re-establishment for DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Proposal 1: Add the case for initialling RRC re-establishment upon detecting radio link failure of target MCG while source cell is not released during DAPS handover in TS 38.331 and TS 36.331.

By Email [211] (8)

LCP handling for source cell in DAPS HO:

R2-2103291	CR on LCP of the source MAC entity	Samsung	CR	Rel-16	38.321	16.4.0	1079	-	F	NR_Mob_enh-Core

R2-2103292	CR on LCP of the source MAC entity	Samsung	CR	Rel-16	36.321	16.4.0	1522	-	F	NR_Mob_enh-Core

Addition/release of bearers during DAPS:

R2-2102821	Addition and release of DRBs in DAPS HO Command	Ericsson	CR	Rel-16	36.331	16.4.0	4607	-	F	LTE_feMob-Core

R2-2102822	Addition and release of DRBs in DAPS HO Command	Ericsson	CR	Rel-16	38.331	16.4.0	2478	-	F	NR_Mob_enh-Core

Stage-2 Description of UL switching for DAPS:

R2-2103333	38.300 CR: Transmissions to the source that continue upon DAPS UL switching	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.300	16.5.0	0353	-	F	NR_Mob_enh-Core

Miscellaneous RRC corrections for DAPS:

R2-2104072	Handling of physicalCellGroupConfig in DAPS handover	MediaTek Inc.	CR	Rel-16	38.331	16.4.1	2544	-	F	NR_Mob_enh-Core

R2-2104075	CR on T312 handling in DAPS HO	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.4.0	4627	-	F	LTE_feMob-Core

R2-2104076	CR on configuration release in DAPS HO	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.4.0	4628	-	F	LTE_feMob-Core

R2-2104125	Configuration for UDCEHC and DAPS	LG Electronics Inc.	CR	Rel-16	36.331	16.4.0	4632	-	F	LTE_feMob-Core

R2-2104128	Configuration for EHC and DAPS	LG Electronics Inc.	CR	Rel-16	38.331	16.4.1	2554	-	F	NR_Mob_enh-Core

Email discussions ([211], [213])

[AT113bis-e][211][MOB] DAPS corrections (Samsung)

Scope:

    - Discuss which DAPS corrections (for LTE and NR) marked for this discussion are seen agreeable. CRs that are editorial can be merged together.

Intended outcome:

    - Discussion summary in R2-2104312 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][213][MOB] RRCReconfiguration with DAPS source release (Ericsson)

Scope:

    - Discuss how/whether to capture the agreements on what is allowed to be configured when daps-SourceRelease is sent to UE according to online agreements.

Intended outcome:

    - Discussion summary in R2-2104330 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

Web Conf 2nd week or By Email (summary of [211] and summary of [213])

R2-2104312	Summary of [AT113bis-e][211][MOB] DAPS corrections (Samsung)	Samsung	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2104330	Summary of [AT113bis-e][213][MOB] RRCReconfiguration with DAPS source release (Ericsson)	Ericsson	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

### 6.4.3	Other corrections

Including incoming LSs related to LTE/NR mobility capabilities (if any). Corrections related to CHO/CPC/DAPS inter-operability with other features should be submitted to 6.1.4.3.

Including corrections to UE capability aspects of LTE/NR mobility WI (i.e. corrections to 3x.331 and 3x.306).

## 6.5	DC and CA enhancements

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

No documents should be submitted to 6.5. Please submit to 6.5.x

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.Tdoc Limitation: 8 tdocs, See also tdoc limitation for Agenda Item 6

### 6.5.1	Corrections to Fast Scell activation and Early measurement reporting

Including corrections to TS38.331, 36.331, 38.306, 36.306 and 38.321 related to Fast SCell activation and Early measurement reporting.

By Email [220] (4)

Miscellaneous corrections:

R2-2103110	Addition of early measurement in idle/inactive UE behavior description in 38.331	CATT	CR	Rel-16	38.331	16.4.1	2509	-	F	LTE_NR_DC_CA_enh-Core

R2-2103111	Addition of early measurement in idle/inactive UE behavior description in 36.331	CATT	CR	Rel-16	36.331	16.4.0	4615	-	F	LTE_NR_DC_CA_enh-Core

R2-2103803	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	38.331	16.4.1	2534	-	F	LTE_NR_DC_CA_enh-Core

R2-2103804	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	36.331	16.4.0	4622	-	F	LTE_NR_DC_CA_enh-Core

Email discussions ([220])

[AT113bis-e][220][DCCA] Miscellaneous DCCA corrections (Ericsson)

Scope:

    - Discuss corrections under R16 DCCA WI marked for this discussion to see which CRs could be agreeable. CRs that are editorial or smal can be merged to rapporteur CRs.

Intended outcome:

    - Discussion summary in R2-2104313 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

Web Conf 2nd week (summary of [220])

R2-2104313	Summary of [AT113bis-e][220][DCCA] Miscellaneous DCCA corrections (Ericsson)	Ericsson	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

### 6.5.2	Other DCCA corrections

Including corrections to NR-NR DC, MCG SCell and SCG configuration with RRC resume, Fast MCG link recovery on all specifications.

Including outcome of [Post113-e][224][DCCA] TCI state indication at direct SCell activation (MediaTek)

Reply LS on NR-DC power control:

R2-2102648	Further Reply LS on power control for NR-DC (R4-2103373; contact: vivo)	RAN4	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN2, RAN1	Cc:-

(moved from 6.5.1)

Noted (input contributions handled in [221])

Reply LS on TCI stae indication for direct SCell activation:

R2-2102613	Reply LS on TCI state indication at Direct SCell activation (R1-2102015; contact: MediaTek)	RAN1	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN4, RAN2

(moved from 6.5.1)

Noted (already handled in email discussion [Post113-e][224][DCCA])

Web Conf (Monday 1st week) (2)

Outcome of [Post113-e][224][DCCA] TCI state indication at direct SCell activation (MediaTek)

R2-2104036	Report of [Post113-e][224][DCCA] TCI state indication at direct SCell activation (MediaTek)	MediaTek Inc.	discussion	LTE_NR_DC_CA_enh-Core

- Agreements

- 1 	Adding TCI state in RRC for direct SCell activation is not pursued in Rel-16.

- 2	Send reply LS to RAN4/RAN1 and inform them that RAN2 decides not to add TCI state in RRC for direct SCell activation in Rel-16.

Discussion

-	ZTE would like to understand what is the alternative to this: Only 1 TCI state? MTK confirms that this is the main possibility and will limit the usage of the feature. ZTE thinks if there is a way for network to configure a list of TCI states but the first one would be used and this wouldn't require ASN.1 change. MTK clarifies this was discussed in RAN1 but not agreed. QC prefers to add RRC but can accept the rapporteur proposal.

-	QC wonders how the TCI state activation latency work in this case? MTK clarifies RAN4 will discuss this after RAN2 replies and this will have no RAN2 impact.

R2-2104040	Reply LS on TCI state indication at Direct SCell activation	MediaTek Inc.	LS out	LTE_NR_DC_CA_enh-Core	To:RAN4, RAN1

Revised in R2-2104326 (with usual updates to source and removal of "draft" from title).

R2-2104326	Reply LS on TCI state indication at Direct SCell activation	RAN2	LS out	LTE_NR_DC_CA_enh-Core	To:RAN4, RAN1

Approved

Web Conf (Monday 1st week) (2)

NR-DC cell grouping:

R2-2103805	Cell grouping for asynchronous NR-DC	Ericsson	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

Observation 1	Including intra FR cell group NR-DC signalling will increase overhead for UE capability signalling.

Observation 2	The increased overhead in UE capability reporting is unnecessary in FR1-FR2 NR-DC network deployments and may restrict the UE from reporting relevant capabilities.

Proposal 1	Await RAN4 input before deciding the cell grouping granularity.

Proposal 2	Introduce a new field includeNRDC-SameFR in UE-CapabilityRequestFilterCommon for requesting NR-DC band combinations with cells within the same FR in both MCG and SCG.

Discussion

-	AT&T wonders if P2 allows more than 5 BCs? Ericsson confirms this is the case for FR1-FR2.

-	AT&T would like to ensure >5 band cases are covered.

-	Apple thinks we already discussed this. Should wait for RAN4 input. QC agrees with Apple even though likes the P2. Intel and Samsung also agree to wait. Nokia agrees that we need to wait for RAN4 for CR agreement but there seems to be demand from operators to support >5 bands. So would be good to consider in RAN2 how this is done.

-	ZTE thinks that for FR1-FR1 with >5 bands, there could be some problems.

R2-2103273	NR DC Cell Grouping	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

Observation: Increasing number of bands in the endorsed CR style of signaling is not feasible

Observation: From signaling point of view it is feasible to support more than 5 bands with carrier type of signaling (i.e. one used for two PUCCH group capability signaling)

Proposal: It is proposed to discuss from RAN2 point of view how to realize more than 5 bands support for capability signaling

- 	Chair wonders if we need to progress already now. AT&T thinks we do. Apple thinks we still need RAN4 input on e.g. grouping.

-	ZTE is open to discuss candidate solutions during the meeting. Does not agree with the first observation. LGE is also open to discuss before RAN4 input. MTK thinks it's better to wait for RAN4. PUCCH grouping is the only one under discussion and that could work. QC thinks RAN already decided that RAN4 starts the work and RAN2 follows.

If RAN4 LS arrives during the meeting, can start offline discussion. Otherwise we discuss this in post-meeting email discussion.

CB [2nd week Monday]: Check LS status and assign post-meeting email discussion based on that.

By Email [220] (2+2)

T316 handling:

R2-2103981	T316 handling when the split SRB1 or SRB3 is released	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2103270	Set-up and release of T316 in procedures	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.4.1	2503	-	F	LTE_NR_DC_CA_enh-Core

SCG handling with RRC resume:

R2-2104044	Clarification on NR SCG configuration within RRC Resume	MediaTek Inc.	CR	Rel-16	38.331	16.4.1	2543	-	F	LTE_NR_DC_CA_enh-Core

R2-2103031	CR on SCG release and suspend in EN-DC	ZTE Corporation, Sanechips	CR	Rel-16	37.340	16.5.0	0257	-	F	LTE_NR_DC_CA_enh-Core

By Email [221] (5)

R2-2104139	Clarification on intra-FR2 NR-DC power control	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh

R2-2103271	NR DC power control signaling	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2103272	NR DC power control signaling	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.4.1	2504	-	F	LTE_NR_DC_CA_enh-Core

R2-2102874	Correction on FR2 NR-DC power control parameter	vivo, MediaTek Inc.	draftCR	Rel-16	38.331	16.4.1	F	LTE_NR_DC_CA_enh-Core

R2-2103806	Correction on p-UE-FR2 and p-NR-FR2 for NR-DC power control	Ericsson	CR	Rel-16	38.331	16.4.1	2535	-	F	LTE_NR_DC_CA_enh-Core

Email discussions ([221])

[AT113bis-e][221][DCCA] NR-DC power control signalling (Huawei)

Scope:

    - Discuss NR-DC PC signalling corrections (for FR2) under R16 DCCA WI marked for this discussion to understand best way forward for RAN2.

Intended outcome:

    - Discussion summary in R2-2104314 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

[AT113bis-e][222][DCCA] NR-DC Cell grouping (NN)

Scope:

    - Discuss RAN2 details of NR-DC cell grouping based on online agreements

Intended outcome:

    - Discussion summary in R2-2104324 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1100

Web Conf 2nd week (summary of [221] and, if needed, [222])

R2-2104314	Summary of [AT113bis-e][221][DCCA] NR-DC power control signalling (Huawei)	Huawei	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2104324	Summary of [AT113bis-e][222][DCCA] NR-DC cell grouping (NN)	NN	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

# 7	Rel-16 EUTRA Work Items

Essential corrections

## 7.1    EUTRA Rel-16 General

No documents should be submitted to 7.1. Please submit to.7.1.x

Purely editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

### 7.1.1	Cross WI RRC corrections

### 7.1.2	Feature Lists and UE capabilities

Corrections to UE capabilities should be taken up with the 36.331 and 36.306 specification editors before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

## 7.4	LTE Other WIs

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning)

(Documents relating to Rel-16 LTE but for which there is no existing RAN WI/SI, e.g. LSs from CT/SA requesting RAN2 action)

Purely editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

Including TEI16 corrections and issues that do not fit under any other topic.

Including outcome of [Post113-e][206][LTE] Clarification to Fallback band combination definition (Nokia)

Web Conf (Monday 1st week) (2)

Outcome of [Post113-e][206][LTE] Clarification to Fallback band combination definition (Nokia):

R2-2103546	Report on [Post113-e][206][LTE] Clarification to Fallback band combination definition (Nokia)	Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

Proposal 1: RAN2 confirms that fallback band combination supports the carriers’ bandwidth(s) that are the same as the carriers’ bandwidth(s) of the signalled parent band combination.

Proposal 2: RAN2 to agree a change of the text in Fallback band combination definition in TS36.306,

from: “A fallback band combination and the parent band combination supports the same bandwidths for each band of the fallback band combination.”

to:	A fallback band combination supports the same channel bandwidths for each carrier as its parent band combination.

Proposal 3:  Only Rel-16 CR on Clarification on Fallback band combination definition is agreed.

Discussion

-	Lenovo is fine with the proposals but thinks magic sentence could be used. Nokia agrees. QC is not sure magic sentence is needed. Lenovo thinks this was introduced in Rel-13 so that could be the earliest release.

-	QC has editorial issue with P2 in the CR ("support" --> "supports").

- Agreements

- 1	RAN2 confirms that fallback band combination supports the carriers’ bandwidth(s) that are the same as the carriers’ bandwidth(s) of the signalled parent band combination.

- 2	RAN2 to agree a change of the text in Fallback band combination definition in TS36.306,

- from: “A fallback band combination and the parent band combination supports the same bandwidths for each band of the fallback band combination.”

- to:	A fallback band combination supports the same channel bandwidths for each carrier as its parent band combination.

- 3	Only Rel-16 CR on Clarification on Fallback band combination definition is agreed.

Make clear in CR cover page that this is a clarification and nothing changes in existing implementations.

R2-2103547	Clarification to Fallback band combination definition	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.306	16.4.0	1782	3	F	TEI16	R2-2100606	Late

- Lenovo think the inter-operabiility need changes as there is no impact to that.

Align with P2 ("support" --> "supports")

Update inter-operability to make it clear there are no impacts in any case.

With these changes, the CR is agreed in principle in R2-2104329

R2-2104329	Clarification to Fallback band combination definition	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.306	16.4.0	1782	4	F	TEI16	R2-2103547	Late

Agreed in principle

By Email [201] (1)

RLC SDU retransmissions:

R2-2102944	RETX_COUNT upon expiry of t-PollRetransmit	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.322	16.0.0	0146	-	F	LTE-L23, TEI16

# 8	Rel-17 NR Work Items

## 8.2	MR DC/CA further enhancements

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

Time budget: 0.5 TU

Tdoc Limitation: 3 tdocs

Email max expectation: 3 threads

No documents should be submitted to 8.2. Please submit to.8.2.x

### 8.2.1	Organizational, Requirements and Scope

Including LSs and any rapporteur inputs (which do not count against Tdoc limits).

Including outcome of [Post113-e][233][eDCCA] Running Stage-2 CR on eDCCA (Huawei)

Web Conf (Thursday 1st week) (1+1+1)

LS on inter-node RRC container design:

R2-2102642	Reply LS on Conditional PSCell Addition/Change agreements (R3-211338; contact: Huawei)	RAN3	LS in	Rel-17	LTE_NR_DC_enh2-Core	To:RAN2	Cc:-

CBT: Whether to have LS as short email discussion

Outcome of [Post113-e][233][eDCCA] Running Stage-2 CR on eDCCA (Huawei)

R2-2103037	TS 37.340 CR for SCG deactivation and activation	ZTE Corporation, Sanechips	draftCR	Rel-17	37.340	16.5.0	B	LTE_NR_DC_enh2-Core	Late

CBT

Stage-2 CR for eDCCA:

R2-2103980	Introduction of further MRDC enhancements	Huawei, HiSilicon	CR	Rel-17	38.300	16.5.0	0362	-	B	LTE_NR_DC_enh2-Core

Endorsed

### 8.2.2	Efficient activation / deactivation mechanism for one SCG and SCells

No documents should be submitted to 8.2.2. Please submit to.8.2.2.x

Withdrawn:

R2-2103982	SCG activation and deactivation procedure	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core	Withdrawn

#### 8.2.2.1	Deactivation of SCG

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussion on how MN/SN request for SCG deactivation works and whether the request can be rejected.

Summary document (pre-meeting effort)

R2-2104315	Summary of AI 8.2.2.1: Deactivation of SCG	Huawei	discussion	Rel-17	LTE_NR_DC_enh2-Core

Discussion

- 	vivo thinks this is OK for NW-initiated procedures but not for UE-initiated.

- 	LGE agrees with P1 and P5. OPPO agrees.

5	Only the MN can generate an RRC message with SCG (de)activation.

1	Indication of SCG deactivation to the UE via the SCG is not supported.

Discussion

-	OPPO supports P7-9. LGE agrees to P7-9 but wonders if NW can omit activation status? Huawei agrees that the message design has to be compatible with legacy. If it's not included, it's activated by default. Depends on signalling design. Samsung thinks we could use "set". ZTE thinks we can use "indicate".

-	Intel wonders what P8 means: Is the deactivation indication in MN message, or in SN-generated message? Huawei clarifies it's in MN-generated message as per P7.

-	Intel wonders why we don't allow SN to set deactivation state? Huawei clarifies that MN and SN coordinate anyway, so this can be just as well included in MN since MN will make the final decision. Intel thinks we could consider different bearer cases. Huawei thinks this is too complex.

7: During handover preparation, the target MN can indicate the SCG state in the RRCReconfiguration message to be sent to the UE by the source MN.

8: The MN RRC reconfiguration message used to deactivate SCG and the embedded SN RRC reconfiguration message can reconfigure any parameter (any restriction requires an explicit decision).

9: While the SCG is deactivated, the MN RRC reconfiguration message and the embedded SN RRC reconfiguration message can reconfigure any parameter (any restriction requires an explicit decision).

UE-initiated SCG deactivation

Proposal 2: Further discuss the 5 proposals for UE-initiated deactivation of the SCG.

RAN3 interaction

Proposal 3: Discuss explanations that could be provided to RAN3 so that RAN3 can make decisions on MN-SN interactions for deactivation of the currently activated SCG.

(e.g. whether MN and/or SN can determine that deactivating the currently activated SCG is acceptable from a QoS perspective)

Proposal 4: Include the cases of SN addition/change in the explanations to RAN3.

Proposal 6: During handover preparation, the source MN sends the current SCG activation state to the target MN. Whether the current SCG activation state is part of the inter-node container or in the XnAP part of the message needs also be discussed in RAN3.

Web Conf (Thursday 1st week) (2)

R2-2103977	SCG deactivation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

Discussion

-	QC is fine with P1 but wonders how QoS fits in with O2?

-	LGE agrees with P1.

-	Apple supports P1-3 but would like to indicate that UE indicates UAI to MN (i.e. not to SN).

-	ZTE disagrees with the second bullet of observation 1  and observation 4. SN doesn't need to care about all traffic but about the SN data. So activitity notification.

-	Nokia agrees with the observations.

Proposal 1: Inform RAN3 about the following observations.

Observation 1: In certain MR-DC configurations:

-	the MN cannot see (all) the traffic on certain radio bearers using the SCG;

-	the SN cannot see (all) the traffic on certain radio bearers using the SCG.

Observation 2: In such MR-DC configurations, neither the MN nor the SN can appreciate alone whether SCG deactivation is possible while respecting the QoS required for all ongoing services.

Observation 3: If SN initiated SN modification supports SCG deactivation, the request for SCG deactivation may be frequently rejected by the MN because the SN is not aware of traffic on the MCG leg of MN-terminated split bearers.

Observation 4: If the SN uses activity notification to inform the MN every time SCG deactivation becomes possible or not possible from SN perspective, the MN can initiate SCG deactivation only when it is possible from the MN and from the SN perspective so the SN is likely to accept it.

Proposal 2: The UE can indicate to the network that the UE would like the SCG to be deactivated.

Proposal 3: The network replies to the UE indication whether the UE request is accepted or rejected.

-	Nokia thinks the UE indication use case is not so clear and whether we define some network requirements is unclear. Is this about overheating? Huawei clarifies that UE might want to use resources for other SIM. DCM thinks UE power saving or S-RLF could be another use case.

-	CATT has concern with P3 since this would mean new message could be needed and this doesn't seem needed. NW can deactivate and UE sees that. Ericsson agrees to reuse UAI framework but the activation or deactivation is the action. NW needs to know why UE wants to deactivate. DCM thinks UAI is not useful and UE could autonomously deactivate.

-	BT is fine with P2 but not P3. Vodafone is not OK with P2. Is concerned that this could be rogue UE so this is a security problem. Has strong concerns. AT&T supports network configurability.

-	Samsung thinks we already have indications. OPPO thinks there could be duplication with overheating and power saving.

2	The UE can indicate to the MN that the UE would like the SCG to be deactivated. FFS on the details (e.g. reusing UAI or existing messages, information included, etc.). Network can configure whether UE is allowed to do the indication.

R2-2103913	UE assistance information use case for SCG deactivation	Convida Wireless	discussion	Rel-17	LTE_NR_DC_enh2-Core

Observation 1: Indications of overheating issues or power saving preferences do not necessarily provide any useful information to the network when to deactivate SCG even though deactivation of SCG generally saves power and resolves overheating problems.

Observation 2: Combining the traffic and application behaviour related information from the UE side with the (in)activity related information at the network side facilitates more accurate and faster SCG deactivation decisions than without the information from the UE.

Proposal 1: The UE may indicate preference for SCG deactivation in UEAssistanceInformation message.

Proposal 2: MN can route the SCG deactivation assistance information to SN.

Discusssion

-	QC thinks MN would anyway tell SN so could agree to P2, but thinks this is MN decision. Convida clarifies that it's MN choice but SN could have additional information. QC wonders if there would be some additional information from MN to SN? Convida thinks this would only be the information from UE.

R2-2103807	SCG deactivation procedures	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

Observation 1	During SN addition procedure, the MN can request the SCG to be deactivated and the SN has a possibility to confirm the setting of SCG activation state.

Observation 2	The SCG target state can be set to deactivated in those handover cases where reconfigurationWithSync for the SCG is required per legacy (e.g. at AS security key change).

Observation 3	Allowing both the MN and the SN to generate the RRC message with (de)activation of the SCG may lead to unnecessary complexity and inconsistency.

Observation 4	For an MN generated message, in case of an MN initiated SCG activation where there are no changes to be done to the SCG configuration, the MN can trigger SCG activation towards the UE before receiving the response message from SN.

Observation 5	As a result of UE power saving preference for SCG on reduced maximum number of CCs, the network may decide to deactivate the SCG or release the SCG.

Observation 6	The UE power saving preference for SCG on reduced maximum number of CCs cannot indicate a preference on deactivating the SCG or releasing the SCG.

Proposal 1	The SN may accept or reject the MN request to deactivate SCG.

Proposal 2	The MN may accept or reject the SN request to deactivate SCG.

Proposal 3	In the SN Addition procedure during PSCell addition/change, the target SN should be able to set the SCG activation state in the response message to the MN.

Proposal 4	At PSCell addition/change, if the SCG activation target state is SCG activated, the UE performs random access in target PSCell (as in legacy).

Proposal 5	At PSCell addition/change, if the SCG activation target state is SCG deactivated, the UE does not perform random access in target PSCell.

Proposal 6	During handover preparation, source MN sends the current SCG activation state to the target MN. Whether the current SCG activation state is part of the inter-node container or in the XnAP part of the message needs also be discussed in RAN3.

Proposal 7	During handover preparation, the target MN includes the SCG activation state in the RRCReconfiguration message to be sent to the UE by the source MN.

Proposal 8	In the SN Addition procedure during handover preparation, the (target) SN should be able to set the SCG activation state in the response message to the target MN.

Proposal 9	At handover when SCG target state is set to deactivated, the UE does not perfom a random access in the target PSCell. This applies also for the handover cases where reconfigurationWithSync for the SCG is required per legacy (e.g. at AS security key change).

Proposal 10	SCG deactivation is only transmitted to UE via MCG on SRB1.

Proposal 11	On Uu only RRC signalling is used to activate/deactivate the SCG in Rel-17.

Proposal 12	Only the MN can generate an RRC message with SCG (de)activation.

Proposal 13	UE can be configured to report a preference for SCG deactivation or SCG release.

Proposal 14	Only the MN can configure the UE to report a preference for SCG deactivation or SCG release.

R2-2102898	Open issues for SCG deactivation procedure	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1: only SCG RLC bearer is suspended for SCG deactivation i.e. the PDCP resource of SCG is not suspended.

Proposal 2: the SCG leg of split bearer can be suspended for SCG deactivation and only MCG leg of split bearer is used for data transmission if SCG is or will be suspended.

Proposal 3: If proposal 2 is not agreed, then for each split bearer, if data volume is lower than the configured threshold, i.e ul-DataSplitThreshold and the primary leg is not in SCG side, then then SCG can be suspended.

Proposal 4: if PDCP duplication is configured and when any one below condition is met, the SCG cannot be suspend:

-	If the primary leg is configured in SCG side;

-	If the primary leg is configured in MCG side and if at least one SCG RLC leg is activated;

Proposal 5: if PDCP duplication is configured and when all the below conditions are met, the SCG can be suspend:

-	If the primary leg is not configured in SCG side and all SCG RLC legs are deactivated.

-	If the primary leg is not configured in SCG side and PDCP duplication is deactivated.

Proposal 6: RAN2 is kindly asked to confirm that there is no PSCell change alone with SCG reactivation.

Proposal 7: No need to introduce extra assistance information for deactivation of SCG.

Proposal 8: only MN can make the SCG deactivation decision based on revised SN ACTIVITY NOTIFICATION message or new message from SN, BSR from UE and MN will send the SCG deactivation command to the UE and SN respectively.

Proposal 9: It is not supported for UE to request SCG deactivation and there is also no reject case from SN and UE due to SCG deactivation command.

Proposal 10: One step RRC signalling is defined for SCG deactivation from MN only.

Proposal 11: SCG deactivation command via MAC CE is supported.

Proposal 12: UE resume RRC connection from RRC_INACTIVE state without SCG activation is supported if the SCG deactivation condition is met.

Proposal 13: UE will inform MN about MCG resuming only.

R2-2103106	Discussion on Deactivation of SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103153	Access handling with TAT in SCG fast activation	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103274	Deactivation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103397	Discussion on SCG deactivation	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103503	Issues on SCG deactivation procedure	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103681	Activation and Deactivation of SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103722	Discussions on deactivation of SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103890	Deactivation of SCG	Qualcomm Incorporated	discussion	Rel-17

R2-2103931	SCG activation/ deactivation procedure	Samsung Telecommunications	discussion	LTE_NR_DC_enh2-Core

R2-2104159	NW-triggered SCG activation and deactivation	MediaTek Inc.	discussion

R2-2104237	Further consideration on SCG activation and deactivation	NTT DOCOMO INC.	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

(moved from 8.2.2)

#### 8.2.2.2	UE measurements and reporting in deactivated SCG

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussion on what UE does when the SCG is deactivated: Does UE do RRM/RLM measurements when the SCG is deactivated? If RLM is used, what is UE behaviour if SCG RLF occurs? How does UE handle TAT when SCG is deactivated? Does UE need to perform L1 measurement (as configured by CSI-MeasConfig) and/or beam monitoring (as configured by RadioLinkMonitoringConfig) when the SCG is deactivated, and is associated reporting needed?

Summary document (pre-meeting effort)

R2-2104316	Summary of AI 8.2.2.2: UE measurements and reporting in deactivated SCG	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 2: TA timer for sTAG should stop after SCG deactivation.

Proposal 10: RAN2 confirms that RRC signalling can reconfigure SCG RRM configuration if SCG deactivation or reactivation is delivered by RRC signalling.

Proposal 11: Send LS to RAN4 and ask RAN4 to study the RRM relax for SCG deactivation and RRM requirement for maintaining DL fine Sync.

-	OPPO thinks we should discuss RRM part to have LS to RAN4.

Offline [240] on P11: Do we support RRM relaxations and why? Can discuss draft LS to RAN4 if there is sufficient support.

Proposal 11: Send LS to RAN4 and ask RAN4 to study the RRM relax for SCG deactivation and RRM requirement for maintaining DL fine Sync.

TA timer and RACH

Proposal 1: TA timer of PSCell is keep running after SCG deactivation, if TA timer is running.

Proposal 3: UE does not need to perform any procedure, e.g. RACH, to maintain UL timing alignment with SN if TA timer expires.

Proposal 4a: Upon SCG activation, if TA timer of the PSCell has already expired or stop, UE performs RACH on the PSCell to obtain UL timing information.

Proposal 4b: RAN2 is kindly asked to discuss whether providing ReconfigurationWithSync by SN is a condition to trigger RACH even if TA timer is running.

Proposal 4c: RAN2 confirms that when deactivated SCG PSCell is changed UE does not initiate RACH until there is need to activate SCG.

RLM/BFD

Proposal 5: RLM is supported after SCG deactivation and legacy SCGFailureInformation message and reporting procedure can be reused after RLM is detected.

Proposal 6: RAN2 is kindly asked to discuss whether BFD is supported or not after SCG deactivation.

Proposal 7: If Proposal 6 is agreed, when BFD is declared, the UE will stop BFD and report BFD occurrence to SCG via MCG. FFS which RRC message and what will be included in the RRC message.

Proposal 8: RAN2 is kindly asked to discuss whether the TA timer will stop due to BFD/RLM detection.

Proposal 9: RAN2 is kindly asked to discuss whether CSI measurement on PSCell is supported or not for SCG deactivation. FFS how and when perform CSI report if RAN2 agree to support CSI measurement and send LS to RAN1.

Proposal 10: RAN2 confirms that RRC signalling can reconfigure SCG RRM configuration if SCG deactivation or reactivation is delivered by RRC signalling.

Proposal 11: Send LS to RAN4 and ask RAN4 to study the RRM relax for SCG deactivation and RRM requirement for maintaining DL fine Sync.

Other

Proposal 12: it is up to network to perform SCG SCell addition/release/modification while SCG is deactivated.

Proposal 13: RAN2 is kindly asked to discuss how to support fast MCG recovery during SCG deactivation.

Proposal 14: RAN2 is kindly asked to confirm PSCell state for SCG deactivation, i.e. deactivate state or active state with dormancy behaviour.

Web Conf (Thursday 1st week) (1)

R2-2103893	UE measurements and reporting in deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

RLM, BFD, and radio link failure recovery in SCG deactivated

Proposal 1. UE supports performing RLM on PSCell while in SCG deactivated.

Proposal 2. UE supports performing BFD on PSCell while in SCG deactivated.

Observation 1. Performing RLM and BFD on PSCell while in SCG deactivated can enable UE to determine upon SCG activation if it has a usable beam for performing RACH or SR, which has the potential to reduce SCG activation delay.

Observation 2. In SCG deactivated, UE performing RLM and BFD on the PSCell does not result in significant additional power consumption if RRM measurements on the PSCell are already being performed.

Proposal 3. UE transmits RRC SCGFailureInformation message to MN upon detecting RLF on PSCell while in SCG deactivated.

Proposal 4. Upon detecting BFD on the PSCell while in SCG deactivated, UE reports the occurrence of BFD to the network via the MCG.

Proposal 5. Upon detecting BFD on the PSCell in SCG deactivated UE reports BFD by transmitting SCGFailureInformation to the MN (a new failure type can be introduced in the message for this purpose).

Observation 3. Upon receiving SCGFailureInformation, in case MN decides to release the SN or change the PSCell, MN transmits RRC reconfiguration to the UE, and if PSCell is changed, the included SN RRC reconfiguration also indicates whether the activation state of new SCG is deactivated or activated.

Proposal 6. Upon receiving SCGFailureInformation, in case MN decides not to change the PSCell, MN and SN may optionally provide a reconfiguration to the UE in response. In particular, SN may provide in an RRC reconfiguration an updated set of beams, RLM RSs, BFD RSs, and additional RSs for UE to measure on the PSCell while in SCG deactivated.

Observation 4. In order to assist the SN to determine the configuration of updated beams and RSs as in Proposal 6, UE should report PSCell beam measurement results in SCGFailureInformation.

Proposal 7. UE should be configured to report PSCell beam measurement results in SCGFailureInformation while in SCG deactivated.

Proposal 8. Upon receiving the RRC reconfiguration message as discussed in Proposal 6, UE resumes performing RLM and BFD on the PSCell as per the provided configuration, while in SCG deactivated.

Observation 5. Upon UE detecting BFD or RLM, the option in which UE reports measurements via the MCG and waits for reconfiguration (Proposals 3-8 above) seems more preferable than the option where UE waits for SCG activation and performs RACH on activation, since it is possible and more likely in the former option for UE to have a usable beam upon SCG activation.

Handling Time Alignment timer (TAT) of the PSCell in SCG deactivated

Observation 6. In general, TA is considered valid when the TA timer is running, and this holds for the TA timer of the PSCell while UE is in SCG deactivated.

Proposal 9. Upon UE entering SCG deactivated, if the TA timer of the PSCell is running, UE should keep the timer running.

Proposal 10. While in SCG deactivated, UE should not stop the TA timer of the PSCell if it is running when BFD or RLM is detected.

Proposal 11. While in SCG deactivated, if TA timer of the PSCell expires, UE does not perform any procedure, e.g., RACH on PSCell, to regain or maintain UL timing alignment with the SN.

Proposal 12. Upon SCG activation, if TA timer of the PSCell has already expired, UE performs RACH on the PSCell to obtain UL timing information.

CSI-RS measurements and reporting in SCG deactivated

Observation 7. Transmission of CSI reports on PSCell UL impacts power savings and if TA timer of the PSCell expires, requires UE to maintain UL timing with SN while in SCG deactivated.

Observation 8. CSI-RS measurements and reporting after SCG activation do not contribute significantly to the delay for SN to begin scheduling the UE on the DL.

Proposal 13. In SCG deactivated, UE does not perform CSI-RS measurements on the PSCell and CSI reporting based on these measurements.

Beam management in SCG deactivated

Observation 9. DL and UL beam management procedures involve periodic beam (L1) measurement reports on the UL or SRS transmissions, which could result in increased UE power consumption in SCG deactivated.

Proposal 14. In SCG deactivated, UE does not support DL or UL beam management procedures.

R2-2102749	Considerations on RLM during SCG deactivation	KDDI Corporation	discussion	Rel-17

R2-2102872	UE behavior when SCG is deactivated	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2102897	UE measurements and reporting in deactivated SCG	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103036	Discussion on UE behaviour when SCG is deactivated	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103107	UE Behavior in Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103275	Measurements for deactivated SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103398	UE behavior when SCG is deactivated	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103569	UE Measurement Aspects in SCG Deactivation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103682	Measurements and maintenance of UL synch with a deactivated SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103808	UE measurements and reporting in deactivated SCG	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103885	TA Maintenance and other UE actions in SCG deactivated state	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103978	UE behaviour in deactivated SCG	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2104124	Discussion for UE behaviour in deactivated SCG	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2104160	UE behavior during SCG deactivation	MediaTek Inc.	discussion

R2-2103740	Discussion on UE behavior in deactivated SCG 	China Telecommunications	discussion	Rel-17

(moved from 8.2.2)

R2-2103505	Further considerations on SCG deactivation	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

(moved from 8.2.2.4)

R2-2103294	DC power sharing for deactivated SCG	Samsung	discussion	Rel-17	LTE_NR_DC_enh2-Core

(moved from 8.2.2.4

R2-2103777	Mobility for deactivated SCG	NTT DOCOMO, INC.	discussion

(moved from 8.2.2.4

#### 8.2.2.3	Activation of deactivated SCG

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussion on SCG activation details: How does MN/SN/UE request SCG activation and can the request be rejected? Is usage of random access at SCG activation UE or network decision?

Summary document (pre-meeting effort)

R2-2104317	Summary of AI 8.2.2.3: Activation of deactivated SCG	ZTE	discussion	Rel-17	LTE_NR_DC_enh2-Core

-	ZTE thinks offline may be too short to allowe further progress.

MN/SN triggered SCG activation

Proposal 1: SN can accept or reject the SCG activation request from MN. Detailed signaling is up to RAN3.

Proposal 1.1: Confirm MN initiated SCG activation without SN involvement is not supported.

Proposal 2: SN can trigger SCG activation. Inform RAN3 that it is possible to use SN-initiated SN modification procedure, and it is also possible to send Activity Notification if SN wants.

Proposal 3: MN can accept or reject the SCG activation request from SN. Detailed signaling is up to RAN3.

UE triggered SCG activation

Proposal 4: SCG bearer is supported while SCG is deactivated.

Proposal 5: UE can trigger SCG activation request in following cases.

•	Arrival of UL data for SCG bearer.

•	MCG failure while SCG is deactivated.

•	FFS on arrival of UL data for split bearer with SCG as primary path.

•	FFS on arrival of UL data for split bearer with total data volume exceeds the threshold.

Proposal 6: UE can trigger SCG activation by:

•	Solution 1: Triggering RACH towards PSCell (if configured by network).

o	FFS whether SR can be used instead of RACH (e.g. when TAT is running).

•	Solution 2: Sending indication to MN, and wait for SCG activation command from MN.

o	Solution 2 is applied when solution 1 is not configured by network.

o	FFS whether the indication is explicit RRC message, or UP based mechanism.

Proposal 7: For solution 1 in Proposal 6, network cannot reject the SCG activation request from UE.

Proposal 8: For solution 2 in Proposal 6, network can accept or reject the SCG activation request from UE, FFS on the signaling.

Form of SCG activation command

Proposal 9.1: Discuss in RAN2 whether to specify reduced RRC processing delay for SCG activation (in case there is no or limited change of SCG configuration).

Proposal 9.2: Discuss in RAN2 whether to send LS to RAN4 about the possibility of specifying reduced Tprocessing for SCG activation (in case the cell or frequency of PSCell does not change).

Proposal 10: Continue the discussion of MAC CE based approach.

Handling of SCG SCell

Proposal 11: Continue to discuss the handling of SCG SCell(s) upon SCG activation, e.g.

•	Solution 1: Network indicates (in RRC signalling) the SCell state and active BWP for SCG SCell(s) in activation command;

•	Solution 2: UE keeps SCG SCell(s) in deactivated state upon SCG activation (via MAC CE, if supported).

Web Conf (Thursday 1st week) (2)

R2-2103276	Activation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1: Both MN and SN can initiate SCG activation. SN can reject the activation request, but MN cannot. The signalling details of this are up to RAN3.

Proposal 2: MN sends the (MN- or SN-initiated) SCG activation command to the UE.

Proposal 3: UE can request SCG activation via UP mechanism (e.g. BSR sent via configured grant).

Proposal 4: First try to enhance RRC procedure before defining another way to activate SCG

Proposal 5: Specify more strict processing time for activating SCG with reduced set of changed parameters.

Proposal 6: Specify reduced physical layer processing delay for SCG activation use case together with RAN4

Proposal 7: UE should be able to keep downlink synchronization including SFN for deactivated SCG. Consult with RAN4 if they have concerns with this.

R2-2103399	Discussion on SCG activation	Lenovo, Motorola Mobility	discussion	Rel-17

Observation 1	At this stage, time alignment towards SCG when SCG is deactivated is not supported. Random access helps to achieve time alignment towards SCG when SCG is activated.

Observation 2	SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell change

Observation 3	PHR is triggered if PSCell is added or changed.

Observation 4	When activating the SCG, NW can also configure the SCells states, e.g. if SCells enters activated state, deactivated state or dormant state.

Observation 5	All SCells are in deactivated state when SCG is deactivated

Observation 6	When fast MCG link recovery is configured, UE sends a MCG failure information to MN via SCG.

Proposal 1	Upon SCG deactivation, the relevant TAT continues running.

Proposal 2	Upon SCG activation, UE performs random access towards PSCell if the relevant TAT expires.

Proposal 3	UE performs random access towards the target PSCell during PSCell change in case the target SCG is configured as deactivated state.

Proposal 4	UE enters SCG deactivated state after random access towards the target PSCell if the SCG state is set to be deactivated during PSCell change.

Proposal 5	Upon SCG activation, MCG PHR is triggered.

Proposal 6	It is upon RAN3 discussion whether MN/SN triggered SCG activation can be rejected by the peer SN/MN.

Proposal 7	UE does not reject the SCG activation triggered by NW.

Proposal 8	When activating the SCG, explicit SCG activation indicator is not needed if any SCell is configured to enter activated or dormant state.

Proposal 9	When UE receives a RRC message to configure a SCell with activated or dormant state while the associated SCG has been deactivated, UE follow the procedures to activate the associated SCG.

Proposal 10	UE may trigger SCG activation in the following two scenarios:

a.	Upon UL data arrival at SCG radio bearer and SCG is deactivated

b.	Upon MCG failure and fast MCG link recovery is configured

Proposal 11	RAN2 supports two ways for UE to trigger SCG activation

a.	UE provides assisting information to MN requesting SCG activation.

b.	UE triggers random access to the PSCell if SCG TAT expires, or sends SR to SCG if SCG TAT is still running.

R2-2102873	Activation of deactivated SCG	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2102899	Open issues for activation of deactivated SCG	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103035	Activation and deactivation of SCG	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103108	Considerations on Activation of Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103251	Discussion on UE behavior when SCG is deactivated	Spreadtrum Communications	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103504	Issues on SCG activation procedure	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103570	Acrivation and Deactivation on SCG	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103723	Discussions on activation of deactivated SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103809	SCG activation procedures	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103886	UE initiation of SCG re-activation request	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103895	Activation of deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

R2-2103979	SCG activation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2104164	UE behaviour upon SCG activation	MediaTek Inc.	discussion

R2-2104170	Discussion on SCG activation	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2104231	Considerations on reactivating SCG	Intel Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

Withdrawn:

R2-2103154	Measurement report and RLM handling for deactivated SCG	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core	Withdrawn

#### 8.2.2.4	Other aspects of SCG activation/deactivation

This agenda item will be deprioritized during this meeting .

### 8.2.3	Conditional PSCell change / addition

No documents should be submitted to 8.2.3. Please submit to.8.2.3.x

#### 8.2.3.1	CPAC procedures and signalling flows

This agenda item will be deprioritized in this meeting. The email discussion will be treated.

Including outcome of [Post113-e][234][eDCCA] CPAC procedures (CATT)

Including discussion on CPAC configuration and execution details.

Including discussion on signalling flows for Stage-2 specification.

Web Conf (Tuesday 2nd week)

R2-2103109	Summary of [Post113-e][234][eDCCA] CPAC procedures (CATT)	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

Candidate generation & execution conditions

Proposal 1 	Source SN provides the candidate cells and it sets the execution condition per candidate cell.

Proposal 2 	No specification impact is introduced to allow the source SN to trigger inter-SN CPC blindly‎.

Proposal 3 	FFS whether it is possible for the target SN to come up with alternative candidate cells other than what suggested by the ‎source SN. ‎

Source SN configuration update

Proposal 4 	RAN 2 discuss and determine whether/which of the following are valid/necessary scenarios for the source SN configuration update based on the accepted candidate cells by the target SN before the CPAC configuration is sent to UE ‎

-	gap is not needed according to the response from the target SN

-	measID related with CPC that are not linked with the selected candidate PSCells.

-	The target SN determines alternative candidate cells other than what suggested by the ‎source SN (subject to previous FFS) ‎

Proposal 5 	Baseline is no specification impact is needed for removal of the unrequired measurement configuration of the source SN depending on the accepted candidate ‎cells by the target SN.

Proposal 5a	FFS whether/how to specify that UE does not have to measure measId(s) that are not linked ‎in CPC by a candidate.

Procedure details, solution 1

Proposal 6 	Option 2 is taken as baseline, i.e., SgNB Change Confirm message is transmitted after reception of RRCReconfigurationComplete in response of the CPAC configuration. The reception of SgNB Change ‎Confirmation message does not trigger the source SN to stop data transmission to the UE. Also another message from the MN to the source SN is required upon the execution of ‎CPC to inform the source SN to stop data transmission to the UE. ‎ RAN2 informs this agreement to RAN3.

Procedure details, solution 2

No proposal is made, as no clear majority is observed from the views.

Inter-node message content

Proposal 7	The message contents required for step 1, 2 and 3 are：

-	SN Change Required:

o	The legacy content as baseline

o	Execution condition per candidate cell,

o	FFS whether an indication for CPC should be added.

-	SN Addition Request :

o	The legacy content of as baseline,

o	FFS whether the indication for CPC should be added.

-	SN Addition Request Acknowledge:

o	The legacy content as baseline ,

o	FFS whether accepted cell list should be added.

Conditional configuration update by the target SN

Proposal 8 	RAN2 understand cancellation and modification of conditional configuration initiated by the target ‎SN, source SN and MN are supported. RAN2 wait for RAN3 progress before further discussions on ‎remaining aspects.

Coexistence of CHO an CPAC

Proposal 9 	Baseline is that CHO and CPAC

Not treated in this meeting

R2-2102861	Discussion on the configuration of CPAC	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103155	Discussion on issues with SN initiated CPC	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103158	Remaining issues for SN initiated inter-SN CPC	China Telecommunication	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103354	Discussion on procedures in CPAC and conventional PSCell change	ITRI	discussion	LTE_NR_DC_enh2-Core

R2-2103883	Details in conditional PSCell change and addition	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103932	CPAC stage 2 flow, progressing remaining issues	Samsung Telecommunications	discussion	LTE_NR_DC_enh2-Core

R2-2103986	Discussion on the inter-node message design (RAN3 LS)	Huawei Technologies France	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2104073	Further consideration on CPAC	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

#### 8.2.3.2	CPAC coexistence with CHO and CPAC failure handling

This agenda item will not be treated in this meeting.

Including discussion on CPAC failure handling and co-existence with CHO

Not treated in this meeting

R2-2102950	Failure handling of Conditional PSCell Addition	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103355	Discussion on SCG RLF handling in case CPC is configured	ITRI	discussion	LTE_NR_DC_enh2-Core	R2-2100827

R2-2103571	Coexistence and other issues in CPAC	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2100728

R2-2103683	Coexistence of CHO and CPC	InterDigital, Nokia	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2103721	Combination of CPAC and CHO	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

#### 8.2.3.3	Other CPAC aspects

This agenda item will not be treated in this meeting.

Not treated in this meeting

R2-2103253	CPC configuration number restriction	Spreadtrum Communications	discussion	Rel-17	LTE_NR_DC_enh2-Core

## 8.3	Multi SIM

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-210316)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

Email max expectation: 3 threads

### 8.3.1	Organizational, Requirements and Scope

Including LSs and any rapporteur input.

Web Conf (Tuesday 1st week) (1)

R2-2102664	LS on System support for Multi-USIM devices (S2-2102039; contact: Intel)	SA2	LS in	Rel-17	LTE_NR_MUSIM-Core	To:RAN2	Cc:RAN3, SA3

Noted (handled in offline discussion [230]

Web Conf (Tuesday 1st week) (2)

R2-2103343	Running CR to 36300 for Multi-USIM devices support	vivo	draftCR	Rel-17	36.300	16.5.0	LTE_NR_MUSIM-Core

Endorsed as running CR

R2-2103344	Running CR to 38300 for Multi-USIM devices support	vivo	draftCR	Rel-17	38.300	16.5.0	LTE_NR_MUSIM-Core

Endorsed as running CR

### 8.3.2	Paging collision avoidance

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussion on enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

Inclduing discussion on RAN2 impacts of the paging collision solution (e.g. whether UE assistance information is needed, whether of solution 1+2b or solution 1+3 is supported for NR, etc.)

Including discussion on whether RAN2 can make the UE behaviour predictable for paging collision avoidance

Summary document (pre-meeting effort)

R2-2104318	Summary of AI 8.3.2: Paging collision avoidance	Ericsson	discussion	Rel-17	LTE_NR_MUSIM-Core

Simpler proposals (could be attempted to be confirmed by e-mail during RAN2#113bis-e)

Proposal 2	No additional modification is needed on the EPS solution for paging collision for the eDRX case.

Proposal 3	Paging repetition as a solution for paging collision issue (Option 3) is not considered.

Proposal 5	RAN2 to confirm that E-UTRAN connected to 5GC scenario is also in the WID scope for paging collision avoidance. NR solution is the baseline for this case.

Proposals that may require more discussion (roughly sorted based on possible controversy and/or priority).

Proposal 4	To select a baseline solution for paging collision for 5GS between 5G-GUTI reassignment (Solution 1) or inclusion of offset to PO formulas (Solution 2b).

Proposal 7	RAN2 to discuss whether a UE assistance information is introduced to avoid paging collision.

Proposal 6	The paging collision avoidance solution is equally applicable for IDLE and INACTIVE state UEs. Additional aspects related to only RRC_IDLE or RRC_INACTIVE are not precluded.

Proposal 8	RAN2 to decide whether the UE behaviour for paging collision avoidance should be predictable.

Proposal 1	Clarify which approach is used in EPS solution for paging collision:  an additional offset in the SFN and PO calculation or an addition offset in UE_ID calculation.

Web Conf (Tuesday 1st week) (2)

R2-2104242	Consideration on the Paging Collision	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

Observation 1: The below 2 options may lead to different outcomes of i_s.

	Option 1: Include the UE_offset to the SFN and i_s calculation formula;

	Option 2: Include the UE_offset to the UE_ID calculation formula.

Observation 2: At the network side, the PO/PF was calculated based on the Option 2 (Include the UE_offset to the UE_ID calculation formula).

Observation 3: Even without the assistance information, the network can select a proper new 5G-STMSI, e.g.  the AMF select a 5G-STMSI that can shift the original PO with about 16rf.

Observation 4: If the RAN paging has the same paging cycle as the CN paging, the Ran paging occasions would be completely overlapped with the CN paging.

Proposal 3: The paging collision is a very low probability issue and RAN2 shall make the solution as simple as possible.

Proposal 7: It’s hard to make the UE behaviour predictable. On the paging collision detection and reporting, it can be left to the UE implementation.

Discussion

P1/2

-	OPPO would like to clarify P1 is only for EPS but agrees both P1 and P2. ZTE confirms P1 is for EPS. Samsung also supports. Vodafone agrees with P1 and P2.

-	Huawei wonders if P2 is only for EPS?

- Agreements

- 1: For the EPS PO/PF calculation, include the UE_offset to the UE_ID calculation formula.

- 2: No additional modification for the EPS eDRX case.

P4/5

-	Ericsson indicates opinions were a bit split on these.

-	LGE agrees that NAS assistance information is needed but AS information could be included in the NAS signalling. Nokia agrees with LGE. vivo and OPPO agrees with LGE. Apple also agrees. QC thinks this allows UE to request certain paging occasion.

-	Xiaomi thinks assistance information is not needed since collision probability is low. Would be simpler to have no assistance information. MediaTek agrees with Xiaomi as this is not guaranteed to solve the problem. Google also agrees and thinks this doesn't really guarantee any power saving. It will need to be updated frequently.

Proposal 4: From RAN2 side, the AS level assistance information is not needed, whether NAS level Assistance information was needed shall be determined by SA2/CT1.

Proposal 5: From paging occasion perspective, if the NAS-based scheme can solve the Idle+Idle state collision, it can also be used for the Idle+Inactive/Inactive+Inactive paging collision scenario.

Proposal 6: For the NAS based procedure, the RAN can get the updated UE_ID for the RAN paging occasion calculation with the legacy signaling and procedure.

R2-2103830	MUSIM Page Collision Avoidance	Apple	discussion	LTE_NR_MUSIM-Core

Observation 1: There are some inherent differences in the way the PO determination happens in EPS and 5GS, primarily based on the input UE_ID being used.

Observation 2 : The UE_ID used for EPS case is based on a permanent subscription identifier which do not change over time.

Observation 3 : The UE_ID used for 5GS case is based on a temporary identifier which can potentially be reassigned over time by the Network.

Observation 4 : Irrespective of EPS or 5GS, the MUSIM paging collision avoidance problem finally maps to a requirement to achieve uniqueness of the computed PO index value based on the input values of UE_ID, N and Ns.

Proposal 1: MUSIM UE can provide suitable assistance information to its Network which can help the Network to provide a non-conflicting paging configuration.

Proposal 2: RAN2 to discuss the actual content of the assistance information that can be provided by UE to NW to help resolve MUSIM paging collision.

R2-2102792	Paging Collision Avoidance	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2102939	Considerations for Paging Collision Avoidance Solution	Samsung	discussion

R2-2102948	Further Consideration on Paging Collision Avoidance	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103160	Paging collision solution of Multi-SIM	China Telecommunication	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103185	RAN Impacts for paging collision avoidance solutions for Multi-SIM	Nokia, Nokia Shanghai Bells	discussion	Rel-17

R2-2103193	5G-S-TMSI re-assignment is enough for paging collision avoidance in 5GS	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103225	Options for paging collision avoidance	Qualcomm Incorporated	discussion

R2-2103345	Paging Collision Solution for 5GS	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103451	UE indication of paging collision for Multi-SIM	ASUSTeK	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2101749

R2-2103480	Paging Collision Avoidance Open Issues	Huawei, HiSilicon	discussion	Rel-17

R2-2103544	Discussion on paging collision avoidance in Multi-SIM, and proposal for response to SA2.	Sony Europe B.V.	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103572	Considerations on Paging Collision	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103677	Solutions for Paging Collision Avoidance for Multi-SIM	Charter Communications, Inc	discussion	Rel-17

R2-2103743	Definition and solution for paging collision, RRC Inactive, SI change	Lenovo, Motorola Mobility	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103757	Paging collision avoidance	Ericsson	discussion

R2-2104151	Paging collision avoidance for MUSIM device	MediaTek Inc.	discussion

R2-2104168	Discussion of the paging collision problem in 5GS	Xiaomi Communications	discussion

### 8.3.3	UE notification on network switching for multi-SIM

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussion on mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose)

Including details of signalling from UE to network for the network switching for MUSIM purpose.

Summary document (pre-meeting effort)

R2-2104319	Summary of AI 8.3.3: UE notification on network switching for multi-SIM	Samsung	discussion	Rel-17	LTE_NR_MUSIM-Core

Decision between two switching procedures

Proposal 1: RAN2 to discuss whether the UE should use the switching procedure for leaving RRC_CONNECTED state in network A in case the UE needs to establish the RRC connection in network B.

Proposal 2: RAN2 to discuss whether the decision between switching procedures for leaving and without leaving RRC_CONNECTED state is up to UE implementation or based on network configuration (e.g. a max gap duration).

Switching procedure without leaving RRC_CONNECTED

Proposal 3: RRC signalling is used for switching procedure without leaving RRC_CONNECTED state in network A for UE temporarily switching to network B as a baseline. FFS on additional need of MAC signalling.

Proposal 4: RAN2 to discuss whether common switching procedure is used for periodic and one-shot/aperiodic activities on network B without leaving RRC_CONNECTED state in network A.

Proposal 5: Introduce dedicated scheduling gap configuration to support switching procedure without leaving RRC_CONNECTED in network A for UE temporarily switching to network B.

Proposal 6: Configuration of one or multiple gap pattern(s) is supported. FFS on the details of gap pattern.

Proposal 7: Network configures one or multiple scheduling gap(s) based on reception of scheduling gap assistance information (e.g. preferred gap pattern(s)) from UE.

Switching procedure for leaving RRC_CONNECTED:

Proposal 8: RAN2 to discuss whether RRC signalling and/or NAS signalling is used for switching procedure for leaving RRC_CONNECED state in network A.

Proposal 9: During switching procedure for leaving RRC_CONNECTED state in network A, UE is allowed to enter RRC_IDLE state if it does not receive response message from network A within a certain configured time period. FFS for RRC_INACTIVE state.

Busy indication:

Proposal 10: Busy indication procedure is supported in RRC_INACTIVE state.

Proposal 11: RAN2 to discuss how to send busy indication in RRC_INACTIVE

-	Option 1: Send AS based busy indication via RRCResumeRequest/1. If agreed, send an LS to SA3 to check whether there is no security issue.

-	Option 2: Send NAS based busy indication via NAS message, carried by RRCResumeComplete.

Web Conf (Tuesday 1st week) (3)

R2-2103545	Discussion on Busy Indication and Leaving in Multi-SIM 	Sony Europe B.V.	discussion	Rel-17	LTE_NR_MUSIM-Core

Observation 1: In RRC_INACTIVE state in 5GS/NR, and in LTE, if it is agreed to make changes in 36.331, a RAN-based busy indication would optimize the time the UE is away from the other network. The latency would change from the estimated time of up to 100ms for the NAS based to a about 10 ms for the RAN based proposal [2]

Observation 2: There is no reason to have different solution for the MuSIM leaving procedures in 5GC compared to what is decided for EPC, where it is decided to be NAS based,. This signalling for leaving is not time critical.

Proposal 1: Specify a RAN based busy indication as a response to RAN paging when in RRC_ Inactive state.

-	QC thinks busy indication is not so useful to UE. SA2 agreed to use NAS but allowed UE to not send it to avoid disrupting e.g. emergency calls. vivo agrees with QC. Samsung also agrees and thinks the main importance is that we support busy indication in INACTIVE. Apple also supports RAN-based busy indication and agrees with QC that UE should not be required to do it. LGE agrees with P1. CATT agrees with others and thinsk we should have a similiar procedure for IDLE and INACTIVE. OPPO thinks RAN-based busy indication makes sense. Xiaomi agrees with QC. MediaTek thinks busy indication is not useful but if we do it, agree with QC clarification. Lenovo also agrees.

-	Ericsson thinks that if network cannot rely on it the busy indication is not useful. Huawei agrees and thinks busy indication is not useful at all. Nokia thinks we should specify cases when UE is not allowed to send it. ZTE thinks this is useful for network to reduce paging resources but prefers NAS-based procedure as for IDLE. Ericsson clarifies they would fine not to have a busy indication. Nokia thinks RAN paging comes from RAN so busy indication in AS makes more sense. Samsung thinks that SA2 agreed UE doesn't need to send busy always in IDLE.

- 	Google thinks we could just not do RAN-based busy indication at all. Apple thinks we agreed we should aim to have predictable behaviour. QC thinks it will be impossible to specify in which cases UE is allowed to skip so we could just not do it. Ericsson agrees with QC. Samsung thinks majority wants to support RAN-based busy indication. FutureWei thinks RAN-based approach only reduces latency but doesn't help otherwise. Could just use NAS-based approach. Huawei agrees with QC on specifying use cases being difficult. Nokia thinks that we need a procedure at RAN level since we have NAS level.

- Agreements

- 1	Only support NAS-based busy indication (for IDLE and INACTIVE)

[200] It was raised that this decision may have unforeseen impacts to SA2/CT1 so session chair declared email discussion [231] to attempt to clarify those.

[200] discuss over email [231] what are the consequences of this decision, and if there are issues to ask from SA2/CT1, provide a draft reply LS.

R2-2103756	Graceful leaving for a Multi-USIM device	Ericsson	discussion

Observation 1	If the Multi-USIM UE interrupts abruptly the connection with the current PLMN, the network KPI might be affected negatively.

Observation 2	Two types of UE absences in PLMN1 are defined, depending on how long the UE takes to perform the actions in PLMN2:

- Very short UE absence, required to perform “quick” actions in PLMN2 which do not need a connection setup/resume. The UE is kept in RRC_CONNECTED in PLMN1.

- Longer UE absence, relevant when the UE perform longer actions in PLMN2, which require the connection setup/resume. The UE is moved to RRC_IDLE/RRC_INACTIVE in PLMN1.

Observation 3	The “very short UE absence” case can be implemented by:

- Specific UE implementation (no standardization is required) or,

- Network involvement: the measurement gaps framework can be reused.

Observation 4	SA2 group agreed to use NAS-level leaving procedure, for the “longer UE absence” case in the E-UTRA/EPS scenario. Then it is reasonable to use the same procedure for the other scenarios as well (NR/5GS and E-UTRA/5GS), to keep the specification complexity on reasonable level.

Observation 5	No issue from timing/delay point of view is expected if NAS signaling is used, considering that delay sensitive services like “emergency fallback” are currently triggered via NAS signaling and that there are no delay requirements on Multi-USIM UEs.

Observation 6	NAS signaling is a better candidate to carry the assistance information sent by the UE at leaving, since they are used by the CN.

Observation 7	An existing timer (e.g. T3517 or dataInactivityTimer) can already be used to cover failure cases, when the UE does not receive the RRCRelease.

Proposal 1	In case of long UE absence, it is recommended to specify only a common procedure for the graceful leaving indication based on NAS signaling.

Proposal 2	It would be beneficial from a RAN2 point of view if the Multi-USIM UE includes the leaving information and the additional assistance information in the NAS Service Request message and that such information is signaled from CN to the gNB.

Proposal 3	The UE leaves RRC CONNECTED (e.g. to establish an RRC connection with another network) only when receiving the RRCRelease message from the current network.

Proposal 4: RAN2 to discuss whether common switching procedure is used for periodic and one-shot/aperiodic activities on network B without leaving RRC_CONNECTED state in network A.

Proposal 4	The Busy Indication procedure is not introduced for RRC_INACTIVE.

-	QC thinks we should talk about leaving CONNCTED and not short and long. Vodafone wonders if we have some priority between NW A and BW B and how long is the "long absence"? Thinks network priorities are equal.

-	Vivo thinks we need an RRC procedure for leaving the network regardless of leave length. For NR we can change RRC more than for LTE. Huawei also prefers RRC signallling.

-	Apple thinks we are restricting to NAS if we consider only a common procedure.

- Agreements

- 1	RRC signalling is used for switching procedure without leaving RRC_CONNECTED state in network A for UE temporarily switching to network B as a baseline. FFS on additional need of MAC signalling.

- 2	During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state.

R2-2103346	Handling leftovers from email discussion on Switching Notification	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

(moved from 8.3.1)

NAS vs. RRC signalling for network switching

Proposal 1:	(28/28) AS level signalling is used to support the switching procedure for keeping the UE in RRC_CONNECTED state.

Proposal 2:	(18/29) RRC based signaling is used to support switching procedure for leaving RRC_CONNECTED state to RRC_IDLE state. FFS if NAS based signalling is also used.

Network switching details

Proposal 3:	(20/26) The RRC Switching Notification Message for long-time switching includes preferred RRC state as baseline, FFS whether other information is needed, e.g. duration of switching, duration of switching.

Proposal 4:	(14/25) UE is allowed to perform switching without the reception of RRCRelease message and goes to RRC_IDLE. FFS for RRC_INACTIVE state. UE waits in network A for Response Message within a certain time.

Proposal 5:	(19/26) The periodic short-time switching procedure contains the switching notification message and RRC Reconfiguration procedure to configure gaps. the switching notification message is triggered if the existing gap cannot meet the Multi-SIM requirement.

Proposal 6:	(19/26) the RRC switching notification message for periodic short-time switching includes Gap pattern request. FFS other information, e.g.  Indication of Need for Gap.

Proposal 7:	(19/26) The switching notification message for one-shot short-time switching carries gap pattern request information. FFS use the common switching notification message for the one-shot and periodic short-time switching.

Proposal 8:	 (17/25) A Return message is not needed for one-shot short-time switching.

Proposal 9:	(19/26) the general RRC procedure of sending Busy Indication in RRC_INACTIVE state includes: UE sends busy indication in the RRC connection resume request message, and the network confirms the busy indication via RRCRelease

If Busy indication is supported:

Proposal 10:	(19/25) UE shall keep RRC_CONNECTED  in network A during sending busy indication in network B.

Proposal 11:	(19/25) Switching for receiving the paging and sending busy indication is up to UE implementation in one-step or two steps.

R2-2102793	UE Notification on Network Switching for Multi-SIM		OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2102811	Analysis on various scenarios of UE switching	China Telecommunications	discussion	Rel-17

R2-2102940	Signalling design on short time switching procedure	DENSO CORPORATION	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2102949	Further Consideration on Network Switching	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103017	Discussions on various ways to support various of leave scenarios and procedures for Multi-SIM UEs	CableLabs	discussion	Rel-17	Late

R2-2103184	Switching notification for basic scenario for Multi-SIM	Nokia, Nokia Shanghai Bells	discussion	Rel-17

R2-2103194	Multi-SIM busy indication signaling for INACTIVE	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103224	Network switching mechanisms for Multi-SIM	Qualcomm Incorporated	discussion

R2-2103247	Discussion on the transmission of busy indication	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103300	UE notification procedure for short time switching	NEC	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103347	Discussion on Switching Notification	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103417	Switching notification and busy indication	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103452	MUSIM Release Assistance Info for network switching	ASUSTeK	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2101748

R2-2103573	Considerations on SIM Swithcing	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2100731

R2-2103588	On coordinated switch from NW for MUSIM device	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103678	Network Switching Solutions for Multi-SIM	Charter Communications, Inc	discussion	Rel-17

R2-2103831	MUSIM Network Switching	Apple	discussion	LTE_NR_MUSIM-Core

R2-2103832	MUSIM Band Conflict and RRC Processing Delay Requirements	Apple	discussion	LTE_NR_MUSIM-Core

R2-2103957	Procedures for MSIM UE notification on network switching	Futurewei Technologies	discussion	R2-2101937

R2-2104154	Network switching behavior for MUSIM device	MediaTek Inc.	discussion

R2-2104169	Discussion of the UE notification on network switching for multi-SIM	Xiaomi Communications	discussion

R2-2104174	Open issues on network switching for Multi-USIM devices	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2104211	RNAU Handling in MUSIM	SHARP Corporation	discussion

R2-2104215	Switching Notification for leaving RRC_CONNECTED	SHARP Corporation	discussion

R2-2104243	Consideration on the Switching Notification Procedure	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2104244	Consideration on the Busy Indication	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

Email discussions ([231])

[AT113bis-e][231][MUSIM] Impacts of NAS-based busy indication (RAN2 VC)

Scope:

    - Discuss whether the agreement to only support NAS-basd busy indication creates issues with SA2/CT1 and determine whether LS needs to be sent to SA2/CT1.

    - If needed, provide draft LS to SA2/CT1 asking them for feedback

Intended outcome:

    - Discussion summary in R2-2104332 (by email rapporteur) and (if needed) draft LS in R2-2104333

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  2nd week Mon, UTC 1200

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1600

Web Conf 2nd week (summary of [231])

R2-2104332	Summary of [AT113bis-e][231][MUSIM] Impacts of NAS-based busy indication (RAN2 VC)	RAN2 VC	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2104333	Draft LS on NAS-based busy indication	Nokia (RAN2 VC)	LS out	LTE_NR_MUSIM-Core	To:SA2, CT1

### 8.3.4	Paging with service indication

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Including discussions on mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR (pending SA2 feedback).

This agenda item will not be treated in this meeting (unless urgent SA2 request is received).

Summary document (pre-meeting effort)

R2-2104320	Summary of AI 8.3.4: Paging with service indication	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

Proposal 1：	An IE containing the Paging cause is included in the paging message, only for intended MUSIM UEs.

Proposal 6：	UE behavior upon reception of the paging cause is specified in both NR and LTE specifications.

Proposal 7：	Do not define spare paging cause values in paging message.

Proposal 2：	RAN2 to discuss which solution should be used for MT service type discrimination.

-	Solution 1: Two values defined for pagingCause [

-	Solution 2: A single value for pagingCause IE + indication of network support of Paging Cause feature

-	Solution 3: Different paging ID indicates different paging causes

Proposal 4：	RAN2 to discuss which solution should be used for paging message extension.

-	Solution 1: Including the number of UEs paged for voice in paging message

-	Solution 2: Including a separate list of pagingRecords for MUSIM UEs paged for voice in paging message

-	Solution 3: Including a separate list of pagingRecords with pagingCause for MUSIM UEs in paging message

-	Solution 4: Including a parallel list of pagingCause in paging message

-	Solution 5: Different paging causes are indicated implicitly with different paging IDs

Proposal 5：	The discussion related to security/privacy issue for paging cause is postponed in RAN2 and can be triggered according to SA3 progress.

Proposal 3：	A reply LS to SA2 and RAN3 is needed to inform the RAN2 decision or preference.

Offline [230] to discuss what could be answered to SA2 on paging cause (Intel, R2-2104331)

TBD after Web Conf (Tuesday 1st week) whether anything is treated in this meeting

R2-2103195	Support for Multi-SIM paging cause from SA2 LS	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2102794	Paging with Service Indication	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2102913	Discussion on support of paging cause for multi-USIM devices	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103186	Further analysis on Service type indication in paging and signalling mechanism for BUSY indication	Nokia, Nokia Shanghai Bells	discussion	Rel-17

R2-2103226	Paging Cause and Busy Indication 	Qualcomm Incorporated	discussion

R2-2103246	Supporting of Paging Cause Solution detection	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103248	Discussion on the transmission of paging cause	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103304	Introduction of paging cause	China Telecommunications	discussion

R2-2103348	Introduction of Paging Cause	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103483	Discussion on the paging with service indication	Huawei, HiSilicon	discussion	Rel-17

R2-2103574	Support of Paging Cause	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2103758	Introduction of a Paging cause indication	Ericsson	discussion

R2-2103958	Discussion on paging service indication for MUSIM	Futurewei Technologies	discussion

R2-2104158	Paging with service indication	MediaTek Inc.	discussion

R2-2104171	Discussion of the paging cause support for MUSIM	Xiaomi Communications	discussion

Email discussions ([230])

[AT113bis-e][230][NR] Reply LS to SA2 on paging cause (Intel)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104331 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

Web Conf 2nd week (summary of [230])

R2-2104331	Summary of [AT113bis-e][230][MUSIM] Reply LS to SA2 on paging cause (Intel)	Intel	discussion	Rel-17	LTE_NR_MUSIM-Core

## 8.8	RAN slicing

(NR_XYZ_enh-Core; leading WG: RAN2; REL-17; WID: RP-210912)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 2 threads

### 8.8.1	Organizational

Rapporteur input

Web Conf (Tuesday 1st week) (1+1)

R2-2103694	Work Plan for RAN Slicing WI	CMCC	Work Plan	Rel-17

- 	Lenovo thinks that Q4 meeting decision in RAN2#92e may still impact this.

Endorsed

SMBR enforcement:

R2-2103647	SMBR enforcement in RAN	Ericsson	discussion	Rel-17

Postponed: contributions on this topic requested for May meeting as it may impact SA2 work

### 8.8.2	Cell reselection

To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message. To assist cell reselection, include slice info (with similar information as in SI message) in RRCRelease message. Take into account SA2 progress / coordinate with SA2 when/if applicable.

Web Conf (Tuesday 1st week) (2)

Basic solution direction:

R2-2102831	slice specific cell reselection	Intel Corporation	discussion	Rel-17

Observation#1: SA2 have concluded in their TR to assume that each cell in the tracking area supports the same S-NSSAI(s) for their Rel-17 SA2 solution for steering the UE based on dedicated priority configuration

Observation#2:  With slice info provided in the SIB and RRC Release, the steering of the UE to desired frequency layer and cell is based on the slice info in SIB or RRC Release.

Discussion

P1

-	Lenovo agrees with Intel on following SA2 agreement but thinks the scenario where same slices are provided in different frequencies needs to be addressed. UE may need to do some reselections for MO traffic. CMCC is also interested in the MO case. Google agrees with Lenovo. Nokia agrees with P1.

-	Intel clarifies that the intent was to avoid reselection due to slice unavailability (since whole TA supports same slices). Prioritization due to slice presence will cause delay in cell access as UE needs to do reselection and read SI after that.

-	CMCC thinks SA2 agreement did not cover homogeneous deployments fully. Should consider also heterogeneous deployments. Nokia thinks any SA2 changes will come only in later releases so can use only homegeneous in Rel-17. Apple thinks that SA2 agreed that homogeneous slice support is needed for legacy UEs but is considering other solutions for heterogeneous cases.

-	Intel thinks that legacy UEs were the reason for the decision and is valid for Rel-17.

P2

-	Xiaomi thinks cell reselection priorities should be taken into account but can be left up to UE implementation.

-	ZTE agrees with proposal. Apple also agrees but thinks we can leave some room for UE implementation. Vodafone also wants to specify UE behaviour to enable operators to manage the networks better. LGE also agrees. Samsung, CMCC and BT also agree.

-	Google wonders what happens if UE is specified with slice-specific priorities but it enters a cell where those are not broadcast.

- Agreements

- 1	RAN2 aligns with SA2 assumption that support of slices in a TA is homogenous also for Rel-17 (i.e. all cells within a TA supports the same slice availability). If SA2 decides to support heterogeneous deployments, RAN2 can revisit this.

- 2	The criteria for determining the cell reselection priority for inter-frequency cell reselection should not be left to UE implementation, but should be defined in the specification (just like cell reselection priorities currently). The details of slice info and how the UE determines its priority list from slice info is FFS.

- 2b	FFS how to define slice priorities for reselection and how to handle conflicts between different priorities (e.g. broadcast vs. dedicated slice-specific priorities)

- 5	UE is only configured with either the existing dedicated priority configuration or the slice info in RRC Release.

- 3	In the case that slice info is also provided to the UE in the RRC Release message while SIB also provides the slice info, UE follows the dedicated slice info from RRC Release while T320-like timer is running and only if it expires that it follows the slice info in the SIB

- 4	In the case that existing dedicated priority configuration is provided to the UE in the RRC Release message while SIB also provides the slice info, UE follows the dedicated priority configuration while T320 is running as per legacy and only if it expires that it follows the slice info in the SIB

- 6	 For UE supporting slice based cell reselection, the UE should use slice info in the SIB for cell reselection if both slice info and existing cell reselection priority is broadcast in the SIB.

Security issues:

R2-2103213	Consideration on slice-specific cell reselection	OPPO	discussion	Rel-17

Proposal 1	If RAN2 agrees to resolve security concern on S-NSSAI exposure, introduce a new parameter to represent slice identity as slice index or slice group index.

Proposal 2	The relationship between slice index/slice group index and slice identity can be indicated via NAS or dedicated RRC message.

Proposal 3	Slice related cell reselection info, including slice identity and per-slice frequency priority, indicated in RRCRelease message overrides the one associated with the same slice received in SI message from the same gNB.

Proposal 4	RAN2 considers to indicate the “restricted area”, e.g. cells, frequencies, for the usage of per-slice frequency priority indicated in RRCRelease message, i.e. per-slice frequency priority indicated in RRCRelease message is only valid in the restricted area.

Proposal 5	RAN2 considers the trade-off between the performance and the complexity to support slice in cell reselection. The following solutions can be considered.

•	If the intended slice is not supported by the candidate cell, the cell is not considered for cell reselection.

•	If per-slice frequency priority is indicated, the UE performs cell reselection based on per-slice frequency priority associated with the intended slice.

•	If the indented slice is supported on a specific frequency, the UE autonomously sets the frequency priority of that frequency as high.

By Email  [251] (20)

R2-2103668	Slice-based cell reselection information	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2103646	On solution for RAN slicing enhancement	Ericsson	discussion	Rel-17

R2-2102696	Slice specific cell reselection	Qualcomm Incorporated		discussion

R2-2102773	Considerations on contents of slice based reselection	KDDI Corporation	discussion	Rel-17

R2-2103159	Discussion on slice based cell reselection	China Telecommunication	discussion	Rel-17

R2-2103695	Discussion on slice based cell reselection	CMCC	discussion	Rel-17

R2-2103269	Cell (re)selection for RAN slicing	Asia Pacific Telecom co. Ltd, FGI	discussion

R2-2104004	Discussion on slice based cell reselection under network control	Huawei, HiSilicon	discussion	Rel-17

R2-2104032	Discussion on slice based Cell Reselection	CATT	discussion

R2-2104063	Discussion on slice-aware cell reselection	ZTE corporation, Sanechips	discussion	Rel-17

R2-2104176	Discussion on slice based cell reselection	Samsung Electronics Co., Ltd	discussion	Rel-17

R2-2102762	Considerations on slice based cell reselection	Beijing Xiaomi Software Tech	discussion	Rel-17

R2-2102988	Considerations on slice-based cell reselection	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103239	Discussion on slice based cell reselection	Spreadtrum Communications	discussion	Rel-17

R2-2103375	Slice based cell reselection	vivo	discussion	Rel-17

R2-2103589	Slice based Cell Reselection 	Sony Europe B.V.	discussion	Rel-17

R2-2103621	Discussion on slice based cell reselection	LG Electronics UK	discussion	Rel-17

R2-2103745	Slice-specific system information for cell reselection	Google Inc.	discussion	Rel-17

R2-2103881	Discussion on slice based cell reselection	Apple	discussion	Rel-17

R2-2103961	System information contents for slice-aware cell reselection	Sharp	discussion	Rel-17

Email discussions ([251]) - kicked off after online session

[AT113bis-e][251][NR] Slice-specific cell reselection (Intel)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104321 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

Web Conf 2nd week (summary of [251])

R2-2104321	Summary of [AT113bis-e][251][NR] Slice-specific cell reselection (Intel)	Intel	discussion	Rel-17	NR_Slice-Core

### 8.8.3	RACH

Configuration of separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group. RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group. Determine how this works with existing functionality. FFS whether RACH partitioning should be initially done as a common design for multiple WIs: RAN slicing, RedCap, Small Data Transmission, CovEnh? Or whether coordination should be attempted once each WI has produced CRs.

Web Conf (Tuesday 1st week) (3)

Basic solution direction:

R2-2103696	Discussion on slice based RACH configuration	CMCC	discussion	Rel-17

Proposal 2: It is feasible and beneficial to work out a unified solution for RACH partitioning to support Rel-17 WI, e.g., slicing, RedCap, coverage enhancement, SDT.

Discussion

P1/3

-	Lenovo thinks we agreed to this during SI but details were left open. Preamble partiotioning might not be needed. Also wonders why P3 parameters are needed in dedicated signallling? CMCC explains they might use both signalling types. QC is fine with P1 but thinks partitions should not overlap. For P3, thinks this is covered by existing case which allows both. Samsung is  fine with P1 but how to configure these can be discussed further and "slice group" is not yet clear. OPPO support only RO partition. SIB information is also OK.

Proposal 1: Both RO partition and preambles partition are supported.

Proposal 3: scalingFactorBI and powerRampingStepHighPriority can be configured per slice group in SIB and dedicated RRC signalling.

P4/5

-	Lenovo thinks this implies that network can configrue some slices with 2-step and some with 4-step RACH. But is not sure why this is needed - could support only one type of RACH with slices. QC thinks we can leave this up to network as long as we use legacy behaviour.

Proposal 4: Network can configure some slices with 2-step RA resources. Only if the MO slice is configured with 2-step RA resources and the measured RSRP is higher than threshold msgA-RSRP-Threshold, should the 2-step RA be selected.

Proposal 5: Legacy 2-step RA fallback mechanism is supported. And msgA-TransMax can be configured per slice or slice group.

Proposal 6: slice specific RA prioritization parameter should override MPS/MCS specific parameter.

Proposal 7: RAN2 confirm that Slice specific RA prioritization has no impact on RA prioritization for HO and beam failure recovery.

- Agreements

- 1	RAN2 aims to support both RO partition and preambles partition.

- 2	scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling).

- 3	Network can configure slices with 4-step or 2-step (or both) RA resources.

- 4	Legacy 2-step RA fallback mechanism is supported.

R2-2102761	Considerations on slice based RACH configuration	Beijing Xiaomi Software Tech	discussion	Rel-17

Proposal 4: The number of slice/slice group with dedicated RACH resource should be limited.

Proposal 1: For slice based RACH configuration, SST can be considered as slice group.

Proposal 2: Separated RACH resource can be configured per SST and RA prioritization can be further configured per SD sharing the same SST-specific RACH resource.

Proposal 3: 2-step and 4-step RACH resource can be configured to the same slice/slice group.

Proposal 5: For slice-based RACH type selection, UE can take slices type into consideration as well as RSRP, e.g. URLLC slice prefers 2-step RACH type.

Proposal 6: For slice specific RACH, the fallback mechanism should be supported for 2-step/4-step specific RACH and 2-step/4-step common RACH.

Proposal 7: To guarantee UE fast access to the slice, UE should not be prevent to initiate access attempt based on common resource if it failed to access based on slice-specific RACH resource.

Proposal 8: The legacy identity-specific RA prioritization can overrule slice-specific RA prioritization if configured at the same time unless the priority of the two sets of RA prioritization is configured by network.

Proposal 9: The collision of RA-RNTI need to be resolved if slice-based RACH resources are configured in addition to the existing common RACH resources.

Discussion

R2-2102697	Slice specific RACH	Qualcomm Incorporated	discussion

Observation 1: WID objective limits scoping of slice specific RACH is only triggered by MO traffic. However, it is not clear whether it includes MO signaling and/or data traffic

Observation 2: MO signaling (e.g. TAU) triggered Slice specific RACH may not be reasonable in some scenario when the new camping cell doesn’t support some UE’s supported slice

Observation 3: Section 5.2.2 of TR 38.832 has captured to introduce the slice grouping, and thereby the only FFS is whether to define a new grouping mechanism or reusing UAC access category

Observation 4: Reusing UAC access category to configure slice grouping is not a clean solution because some slice info may not be derived if they belong to same AC and not all slices in one AC can be supported by gNB

Observation 5: It is important that slice specific RACH shall not prevent access of Rel-15 / Rel-16 legacy UEs. In addition, Rel-17 UEs supporting RACH isolation should not switch to another BWP to trigger common RACH when non-urgent slice traffic arrival

Observation 6: Following Rel-16 legacy mechanism, if only 2-step slice RACH resource configured in the BWP, high priority slice may only trigger 2-step RACH to reduce latency

Observation 7: Considering RAN2 is introducing RACH prioritization for different scenarios / cases ever from Rel-15 to Rel-17 (BFR/HO → MPS/MCS → Slice), specifying a flexible / configurable way is more forward compatible way

Scenario:

Proposal 1: RAN2 confirm that only MO data arrival triggered RACH can apply slice specific RACH, i.e. MO signaling (e.g. mo-Signalling and mo-SMS) triggered RACH is not applied to slice-specific RACH

Proposal 2: If Proposal 1 is agreed, RAN2 is kindly suggested to discuss whether CONNECTED UE can also apply slice specific RACH when RACH is triggered by MO data arrival (i.e. when UL synchronisation status is "non-synchronised", or there are no PUCCH resources for SR available, or SR failure)

Signaling:

Proposal 3: For both slice specific cell reselection and slice specific RACH, introduce a common slice grouping via a configured mapping from a set of S-NSSAIs to a slice group. FFS detailed signaling for slice grouping

Proposal 4: Due to lack of SA2/CT1 TU, RAN2 conclude it is up to UE implementation to determine the slice priority in this release if its intended slices includes more than one S-NSSAI in this release.

Common aspects of RACH isolation and prioritization:

Proposal 5: RAN2 confirm that slice specific RACH (including RACH isolation and RACH prioritization) is only applied to CBRA rather than CFRA

Aspects of RACH isolation:

Proposal 6: RAN2 confirm for a slice or slice group, separated RO and/or preamble can be configured without overlapping with the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA. FFS shared RO and preamble

Proposal 7: To support legacy UE and non-urgent slice, if slice specific RACH resource is configured in one BWP, common RACH resource (i.e. legacy CBRA resource) is required to be configured in the same BWP

Proposal 8: Keep the below principle of Rel-16 RACH type selection and fallback mechanism for slice specific RACH:

•	If only 2-step RACH resource is configured in one BWP, the UE shall only perform 2-step RACH

•	If both 2-step and 4-step resource are configured in one BWP, the UE selects to perform 2-step RACH or 4-step RACH based on RSRP threshold. FFS whether to introduce a slice (group) specific RSRP

•	Reuse access attempt number as condition to fallback from 2-step RACH to 4-step RACH. FFS whether to introduce a slice (group) specific attempt number threshold

Proposal 9: RAN2 confirm the following 5 cases in the table are supported for RACH type selection and fallback of slice specific RACH, where “common RACH” means legacy cell specific CBRA resource:

| Cases | RACH resource configuration in one BWP | RACH type selection | Fallback after MSGA attempt number beyond threshold | Notes |
| --- | --- | --- | --- | --- |
| Case 1 | 2-step slice specific RACH 4-step common RACH | Always perform 2-step slice specific RACH | UE switch to MSG1 of 4-step common RACH | Via only configuring 2-step slice RACH resource, high priority slice may only trigger 2-step RACH to reduce latency |
| Case 2 | 2-step slice specific RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | UE can switch to MSG1 of 4-step slice specific RACH | No fallback from 4-step slice specific RACH to 4-step common RACH |
| Case 3 | 4-step slice specific RACH 2-step common RACH | Always perform 4-step slice specific RACH | No fallback |  |
| Case 4 | 4-step slice specific RACH 4-step common RACH | Always perform 4-step slice specific RACH | No fallback |  |
| Case 5 | 2-step slice specific RACH 2-step common RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | UE can switch to MSG1 of 4-step slice specific RACH | No fallback from 4-step slice specific RACH to 4-step common RACH. Not preferred due to large RACH resource usage |

Aspects of RACH prioritization:

Proposal 10: scalingFactorBI and powerRampingStepHighPriority are baseline of slice specific prioritized RACH parameters. Other parameters can be considered only if time allows

Proposal 11: For each RA prioritization parameters set (e.g. one set for MPS/MCS and another set for URLLC slice), a priority value can be configured by gNB or pre-configured via UE’s subscription. And the UE’s AS selects the set of RACH prioritization parameters with highest priority to perform RACH

By Email  [252] (13)

R2-2102832	Considerations of slice based RACH	Intel Corporation	discussion	Rel-17

R2-2102989	Considerations on slice-based PRACH configuration	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2103089	Slice based RACH configuration	Samsung	discussion	Rel-17

R2-2103214	Consideration on slice-specific RACH	OPPO	discussion	Rel-17

R2-2103240	Consideration on slice based RACH configuration	Spreadtrum Communications	discussion	Rel-17

R2-2103376	Slice based RACH configuration	vivo	discussion	Rel-17

R2-2103548	RACH prioritisation for slices	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_slice

R2-2103882	Discussion on slice based RACH	Apple	discussion	Rel-17

R2-2104005	Discussion on slice based RACH configuration	Huawei, HiSilicon	discussion	Rel-17

R2-2104019	Analysis on slice based RACH configuration	CATT	discussion

R2-2104064	Discussion on slice specific RACH resources and RACH prioritization	ZTE corporation, Sanechips	discussion	Rel-17

R2-2104099	Slice-specific RA procedure	LG Electronics UK	discussion

Email discussions ([252]) - kicked off after online session

[AT113bis-e][252][NR] Slice-specific RACH (CMCC)

Scope:

    - Summarize main open issues based on contributions and online agreements.

    - Highlight if there are topics that clearly require online discussion.

    - Identify topics that might benefit from email discussions.

Intended outcome:

    - Discussion summary in R2-2104322 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

Web Conf 2nd week (summary of [252])

R2-2104322	Summary of [AT113bis-e][252][NR] Slice-specific RACH (CMCC)	CMCC	discussion	Rel-17	NR_Slice-Core

# 9	Rel-17 EUTRA Work Items

## 9.3	EUTRA R17 Other

Time budget: 0 TU

Tdoc Limitation: X tdocs

Email max expectation: X threads

Including discussion on RAN2 actions for user location tracking attack based on GSMA LS R2-2100003.

No TEI17 documents will be handled in this meeting.

Web Conf (Monday 2nd week) (1+2)

GSMA LS on location tracking attack:

R2-2102607	User location identification from Carrier Aggregation secondary cell activation messages (FSAG Doc 88_009; contact: GSMA)	GSMA	LS in	To:SA3, RAN2

R2-2102819	UE location attack based on SCell activation	Ericsson	discussion	Rel-17	R2-2100483

R2-2104039	Discussion on user location identification from SCell Activation	Huawei, HiSilicon	discussion	Rel-17

Web Conf (Monday 2nd week) (3)

SA3 LS on UPIP for LTE/EPC:

R2-2102659	Reply LS on User Plane Integrity Protection for eUTRA connected to EPC (S2-2101306; contact: Qualcomm)	SA2	LS in	Rel-17	To:SA3, CT1	Cc:RAN2, RAN3, CT4

Noted (RAN2 in cc only without actions, LS reply handled by email [202])

R2-2102605	Reply LS on User Plane Integrity Protection for eUTRA connected to EPC (C1-211461; contact: Qualcomm)	CT1	LS in	Rel-17	UPIP_SEC	To:SA3	Cc:RAN2, RAN3, CT4, SA2

Noted (RAN2 in cc only without actions, LS reply handled by email [202])

R2-2102667	LS on User Plane Integrity Protection for eUTRA connected to EPC (S3-210563; contact: Vodafone)	SA3	LS in	To:RAN2, RAN3, CT4, SA2	Cc:CT1

Noted (handled by email [202])

By Email [202] (5)

R2-2103016	User Plane Integrity Protection for eUTRA connected to EPC	Qualcomm Incorporated	discussion	Rel-17	UPIP_SEC

R2-2103962	PDCP for Integrity protection for LTE EPC	Intel Corporation	discussion	Rel-17

R2-2103295	User Plane Integrity Protection for LTE	Samsung	discussion	Rel-17

(moved from 8.17)

R2-2103928	Discussion on Capturing PDCP Impacts for User Plane Integrity Protection	Ericsson	discussion	R2-2101477

(moved from 8.17)

R2-2103865	RAN impact on UP IP for eUTRA connected to EPC	Apple	discussion	Rel-17

(moved from 8.17)

Not treated in this meeting

TEI17 documents will not be handled in this meeting (as per RAN#91e decision, the event-based trigger for LTE MDT will be discussed in RAN2#115e when RAN2 has TU allocation for TEI17)

R2-2102703	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation	draftCR	Rel-17	37.320	16.4.0	B	TEI17	Late

R2-2102721	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation	draftCR	Rel-17	36.331	16.4.0	B	TEI17	Late

Email discussions ([202])

[AT113bis-e][202][LTE] UPIP for LTE Rel-17 (Qualcomm)

Scope:

    - Discuss the UPIP contributions under AI 9.3 and determine whether there is consensus on what RAN2 could reply to SA3.

    - Can provide also draft LS reply to SA3

Intended outcome:

    - Discussion summary in R2-2104325 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary and draft LS):  1st week Fri, UTC 0900

Web Conf (Monday 2nd week) (3)

R2-2104325	Summary of [AT113bis-e][202][LTE] UPIP for LTE Rel-17 (Qualcomm)	Qualcomm	discussion	Rel-17	UPIP_SEC

## 9.4	NR and EUTRA Inclusive language

Time budget: N/A

CRs were endorsed/agreed-in-principle at R2#112-e. Final approval is expected when R17 TSes are to be created and at that point CRs need to be updated towards latest TS version and submitted again. Meanwhile this AI can be used to cover missing part, if any, and for correction/modification of the endorsed/agreed-in-principle CRs e.g. for inter-group consistency, inter-group review etc.