3GPP TSG-RAN2 Meeting #113bis-e	R2-210xxxx

eMeeting, 12th – 20th April, 2021

Agenda Item:	5.4.3 UE capabilities

Source: 	Ericsson

Title:  	[AT113bis-e][010][NR15] UE caps DL scheduling slot offset

Document for:	Discussion and Decision

# Introduction

During RAN2#906 it was agreed to have an offline email discussion, after the online discussion on Monday, about:

[AT113bis-e][010][NR15] UE caps DL scheduling slot offset (Ericsson)

START ONLY AFTER ON-line Monday

Scope: Taking into account on-line agreements, Treat R2-2103768, R2-2103770, R2-2103771, R2-2103769, R2-2103799

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

The deadline for the first round comments is Wednesday April 14 1000 UTC.

This report gives a summary of this offline email discussion.

# Contact information

| Company | Email |
| --- | --- |
| Ericsson | martin.van.der.zee@ericsson.com |

# Introduction

There was no time for online discussion on monday, and no online agreements were reached, but in this first round we will look for agreeable parts in:

- R2-2103768, Summary of [Post113-e][051][NR15] DL scheduling slot offset, Ericsson	report, RAN2#113bis-e

- R2-2103770, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericsson, CR 38.331, Rel-15, RAN2#113bis-e

- R2-2103771, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericsson, CR 38.331, Rel-16, RAN2#113bis-e

- R2-2103769, Open issues K0 configuration and use, Ericsson, DISC, RAN2#113bis-e

- R2-2103799, Configuration of common fields in dedicated signalling, Ericsson, DISC, RAN2#113bis-e

# Discussion

## Add DL scheduling slot offset capabilities to UERadioPagingInformation message

The gNB currently does not know if the UE has IOT-tested K0 > 0 when receiving a Paging message from CN. Thus the gNB does not know if it can use K0 > 0 in the PDCCH scheduling of the Paging message on PDSCH, provided that only UE(s) supporting K0 > 0 are paged in the Paging Occasion (PO). When the gNB does not know if the UE supports K0 > 0, or if also legacy UEs are paged in the PO, then the gNB cannot use K0 > 0 in the PO.

Issue 1: Do companies agree to add SchedulingOffset-PDSCH-TypeA and dl-SchedulingOffset-PDSCH-TypeB capability to the UERadioPagingInformation message?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes | None |

Issue 2: Do companies agree with the draft CRs for Rel-15 and Rel-16 in [2,3]?

- R2-2103770, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericsson, CR 38.331, Rel-15, RAN2#113bis-e

- R2-2103771, Introduction of DL scheduling slot offset capabilities in UERadioPagingInformation, Ericsson, CR 38.331, Rel-16, RAN2#113bis-e

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes | None |

## Open issues with K0 configuration and use

In the email discussion #051 [1] companies agreed that the NW can configure K0>0 in pdsch-TimeDomainAllocationList in SIB1 which is a common configuration for all UEs in the cell i.e. for UEs supporting K0>0 and UEs not supporting K0>0:

A UE that does not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB capability does support pdsch-TimeDomainAllocationList configuration in PDSCH-ConfigCommon in SIB1 including K0 values larger than 0.

Issue 3: Do companies agree to clarify this in the chairman notes?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes | None |

In the email discussion #051 [1] it was also discussed whether it should be clarified that the NW cannot use K0>0 when the NW does not know if the UE has IOT-tested it:

The network cannot use K0>0 for PDCCH/PDSCH scheduling without possible IOT issues when the network does not know if the UE has IOT-tested K0>0.

Issue 4: Do companies agree to clarify this in the chairman notes?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes | None |

In the email discussion #051 [1] it was also discussed whether the NW should use the UE capabilities when configuring K0 via PDSCH-Config (not PDSCH-ConfigCommon) in dedicated signalling, i.e. not configure K0>0 when the UE has not IOT-tested it. It is the understanding of the rapporteur that the normal approach is to use the UE capabilities in dedicated configuration in dedicated signalling:

The network configures K0 in PDSCH-Config in dedicated signalling according to the UE capabilities.

Issue 5: Do companies agree to clarify this in the chairman notes?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes | None |

## Common configuration in dedicated signalling

The contribution [5] presents three types of RRC signaling in the dimension of common (cell specific) and dedicated (UE specific):

- Common configuration included in SI

- Dedicated configuration included in dedicated signaling

- Common configuration included in dedicated signaling

The issue at hand is the third type, and the paper argues that it should be clarified whether Type 3 configurations should comply with what the UE supports or not.

Issue 6: Do you think clarifications are needed (why/why not)?

| Company | Comments |
| --- | --- |

Issue 7: If clarifications are needed, what should be the intended behaviour (e.g. network adapts all type 3 signalling to UE capabilities, or network does not have to adapt all type 3 signalling and the UE has to comprehend it regardless of UE capabilities, or something else)?

| Company | Comments |
| --- | --- |

# Summary of email discussion

TBD

# Conclusions

TBD