3GPP TSG RAN WG2#113bis-e	R2-2102619

e-Meeting, 12th - 20th April, 2021

3GPP TSG-RAN WG1 Meeting #104-e	R1-2102088

e-Meeting, Jan 25 – Feb 5th, 2021

Title:	LS on Introduction of DL 1024QAM for NR

Release:	Rel-17

Work Item:	NR_DL1024QAM_FR1

Source:	RAN WG1

To:	RAN WG2, RAN WG4

Cc:

Attachments: RRC parameters for NR DL 1024QAM.xls

Contact Person:

#### Name:	Ajit Nimbalker

E-mail Address:	Ajit.Nimbalker@ericsson.com

1. Overall Description:

RAN WG1 made following agreements regarding RRC signalling needed for support of DL 1024-QAM for NR.

- Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1

  - FFS : support of 1024-QAM MCS table for DCI format 1_2

    - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively

  - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config

- RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

- Introduce separate RRC signaling to indicate use of 1024-QAM MCS table for DCI format 1_2.

The corresponding excel sheet with detailed RRC parameter description is in the attachment.

RAN WG1 also agreed to the CQI table and MCS table with 1024-QAM entries, which are included in Annex A and Annex B, respectively.

2. Actions:

To RAN WG 2 and RAN WG4 group

ACTION: 	RAN WG1 would like RAN WG2 and RAN WG4 to take the above into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104-bis-e	Apr 12th – Apr 20th, 2021	e-Meeting

TSG-RAN WG1 Meeting #105-e	May 19th – May 27th, 2021	e-Meeting

Annex A (CQI table with 1024QAM entries)

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

Annex B (MCS table with 1024QAM entries)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 449 | 0.8770 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |