3GPP TSG-RAN WG2 Meeting #113bis electronic                   R2-2103038

e-meeting, 12th – 20th April 2021

Source: 			ZTE Corporation, Sanechips,

Title: 	RRM relaxation for RedCap UE

Agenda item:		8.12.3.2

Document for: 	Discussion and Decision

- Introduction

After RAN_92e meeting, RAN2 objectives for RedCap devices are approved in [1]. For RRM relaxation, the work item objectives are pasted below:

- RRM relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected, considering the alternatives identified in the RedCap SI: - Study until RAN#92e, and, if agreed, specify RRM measurement relaxation criteria (where, for RRC_Idle/Inactive the Rel-16 mechanism is the baseline, and for RRC_Connected the mechanism reuses the Rel-16 RRM relaxation criteria from RRC_Idle/Inactive so as to maximize the commonality with Idle/Inactive UEs) [RAN2] - Enabling/disabling of RRM relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM relaxation. - After RAN#92e, if agreed in RAN2, specify RRM measurement relaxation [RAN4] - No RRM relaxations are specified for the serving cell. - Specify RAN4 core requirements for the above.

In this contribution, we further discussed the Rel-17 RRM relaxation mechanisms for RedCap UEs.

- RRM relaxation for RRC_IDLE/INACTIVE

In past meetings, RAN2 mainly discussed the RRM relaxation mechanism for stationary RedCap UEs according to the SID. And several enhancement solutions were analyzed. However, in the WID, the objective does not mention which kind of UEs will be considered for RRM relaxation. In our understanding, the discussion will continue focus on “stationary” UEs. And based on previous study, this “stationary” includes both “fixed/immobile UEs” and “slightly moving UEs”.

Proposal 1: RAN2 to confirm the scope of RRM relaxation for RedCap UEs still involves fixed/immobile and low mobility UEs.

As described in WID, Rel-16 RRM relaxation mechanism can be taken as a baseline for RedCap RRM relaxation in RRC_Idle/Inactive. In our previous contribution [3], we discussed the inter-operability between Rel-16 and Rel-17 RRM relaxation, e.g. whether RedCap can apply Rel-16 RRM relaxation mechanism if broadcasts. Although we preferred to define independent RRM relaxation mechanisms (at least separate parameters) for RedCap and non-RedCap, to allow network to configure different thresholds for RedCap and non-RedCap UEs. It is also ok if majority companies prefer to adopt the same Rel-16 parameters for all kind of UEs.

In our understanding, once network configures Rel-16 RRM relaxation evaluation parameters in SIB, and RedCap UE fulfills the Rel-16 RRM relaxation criteria (i.e. low mobility, not-at-cell-edge), then Rel-16 RRM relaxation methods should be applied, there is no need for RAN4 to study the enhanced RRM relaxation mechanism specifically for RedCap UEs.

Proposal 2: If RedCap UE fulfills Rel-16 RRM relaxation criteria, then Rel-16 RRM relaxation method is applied (without further enhancement).

- The existing lowMobilityEvaluation-r16, cellEdgeEvaluation-r16 parameters (defined in SIB2) are applicable to both RedCap and non-RedCap UEs.

Since Rel-16 mechanism is sufficient to identify low-mobility UEs, if any enhancement is needed for Rel-17 RedCap UEs, we should focus on identifying fixed/immobile UEs, and ask RAN4 to study more aggressive RRM relaxation method for the fixed/immobile UEs.

Observation 1:  For Rel-17, RAN2 should focus on how to identify fixed/immobile RedCap UEs.

During SI, the following enhancements of RRM relaxation criterion have been studied.

- Enhancement 1: Introduce additional SsearchDeltaP_stationary threshold to support 2-level speed evaluation (i.e. stationary and low mobility); - Enhancement 2: Introduce additional TSearchDeltaP_stationary to support 2-level speed evaluation (i.e. stationary and low mobility); - Enhancement 3: Take into account changes in beam measurements in serving cell when evaluating the mobility status of UE, e.g. based on the number of beam changes, the beam can be the best beam or all beams exceeding a threshold; - Enhancement 4: UE determines its stationary property based on subscription information (e.g. USIM); - Enhancement 5: Introduce an additional SsearchDeltaP_correction threshold and configure the UE to use it if only it detects that it observes higher received signal power variation that do not violate stationary property, i.e. rotating around itself, dynamically changing multipath. - Enhancement 6: UE determines its confined mobility property based on subscription information (e.g. USIM). This kind of device is expected to be moving (slowly in many cases) or stationary, but different from Enhancement 5, these UEs are not expected to move out of a localized area in its lifetime.

Among the solutions, we prefer Enhancement #4, because it does not rely on the quality of serving cell or serving beams. Generally, for fixed/immobile UEs, the UE will keep stationary for a long period of time. We understand the motivation is to also identify temporary stationary UEs. But we are not sure using stricter RSRP threshold can be helpful for identifying these UEs, because even if the UE does not move, the channel condition may also be slightly changed due to the movement of other objects nearby. Therefore, it seems difficult for network to provide accurate RSRP thresholds to differentiate fixed/immobile UEs from low mobility UEs.

For Enhancement #2, based on current definition of low-mobility criterion, if the relaxed measurement criterion has not been met for TSearchDeltaP, the UE will set the value of SrxlevRef to the current Srxlev value of the serving cell. Which means the UE may keep refreshing SrxlevRef in certain cases. Thus even by extending the timer length, it is hard to determine whether the UE is truly stable or keeps moving lowly.

But Enhancement #4 is based on UE subscription information, it can precisely indicate the fixed/immobile attribution. For Enhancement #6, we understand the “stationary” part is same as in Enhancement #4, so we propose:

Proposal 3: For identifying fixed/immobile UEs in RRC_IDLE/INACTIVE, to consider the following enhancement:

- UE determines its stationary property based on subscription information (e.g. USIM);

- RRM relaxation for RRC_CONNECTED

Based on the WI objective, if agreed, RRM relaxation for RRC_CONNECTED can be considered by reusing Rel-16 RRM relaxation mechanism. In our understanding, this means the network can provide the RRM relaxation criteria parameters via RRC dedicated signalling, that is exactly “Solution 2” mentioned in TR 38.375.

-	Solution 2: Network provides (e.g. low mobility, not-at-cell-edge) evaluation parameters to UE via dedicated signalling.

As mentioned in WID, the benefit of this solution is to maximize the commonality with Idle/Inactive UEs, and reduced the specification effort. However, one disadvantage is: after sending evaluation parameters to UE, network has no idea when the UE fulfills the criterion, so network is unclear when does UE enter or leave RRM relaxation mode. In general, it would be fine if network does not know this information, as long as the RRM relaxation has no negative impact to handover performance.

Observation 2:  Simply reusing Rel-16 mechanism for RedCap RRC_Connected, the network is unaware when does UE enter or leave RRM relaxation mode.

However, for RRC_Connected UEs, the network may configure RRM measurements for different reasons, e.g. mobility based handover, load balancing, CGI reporting (if supported for RedCap)…etc. When UE fulfills low-mobility criteria, it is ok to relax the RRM measurements that are configured for mobility based handover. However, it does not make sense to relax the RRM measurements that are configured for load balance, because network would expect UE to send measurement report as soon as possible, so it can move the UE to another cell with lower load.

Observation 3:  For RRC_Connected state, not all RRM measurements can be relaxed. For measurement with specific reasons (e.g. load balancing), network would expect UE to send MR as soon as possible.

So we suggest to introduce mechanism to inform UE the measured frequencies that can be relaxed. This is similar to disable the RRM relaxation for high priority frequencies, or early measurements (EMR) in Rel-16. Considering the UE performs measurements based on the frequency indicated in measObject, it is reasonable to associate the indication with measurement object instead of measIDs.

Proposal 4: For RRC_Connected RedCap UEs, when configuring RRM relaxation criteria parameters, network can also indicate which measured frequency (i.e. measObject) can be relaxed, which cannot.

In addition, for fixed/immobile UEs, if network can obtain the UE’s property information, the network can take more aggressive action for power saving, such as: does not configure any RRM measurements for mobility. To achieve this, either UE can report “stationary” indication to network (cover both fixed and temporary stationary UEs), or network can make decision based on the “stationary” indication from AMF (covers only fixed UEs). For latter, it is already supported in NG-C interface. While for temporary stationary UEs, we are open to let UE reports its temporary stationary information to network. How to determining the “temporary stationary” property can be up to UE implementation. And as we commented to section 2, it is better if UE determines its speed based on GPS instead of RSRP values.

Proposal 5: Network can determine fixed UEs based on the stationary information sent by AMF. For temporary stationary UEs, allow UE to report “temporary stationary” attribution via UE Assistance Information.

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation 1:  For Rel-17, RAN2 should focus on how to identify fixed/immobile RedCap UEs.

Observation 2:  Simply reusing Rel-16 mechanism for RedCap RRC_Connected, the network is unaware when does UE enter or leave RRM relaxation mode.

Observation 3:  For RRC_Connected state, not all RRM measurements can be relaxed. For measurement with specific reasons (e.g. load balancing), network would expect UE to send MR as soon as possible.

Proposal 1: RAN2 to confirm the scope of RRM relaxation for RedCap UEs still involves fixed/immobile and low mobility UEs.

Proposal 2: If RedCap UE fulfills Rel-16 RRM relaxation criteria, then Rel-16 RRM relaxation method is applied (without further enhancement).

- The existing lowMobilityEvaluation-r16, cellEdgeEvaluation-r16 parameters (defined in SIB2) are applicable to both RedCap and non-RedCap UEs.

Proposal 3: For identifying fixed/immobile UEs in RRC_IDLE/INACTIVE, to consider the following enhancement:

- UE determines its stationary property based on subscription information (e.g. USIM);

Proposal 4: For RRC_Connected RedCap UEs, when configuring RRM relaxation criteria parameters, network can also indicate which measured frequency (i.e. measObject) can be relaxed, which cannot.

Proposal 5: Network can determine fixed UEs based on the stationary information sent by AMF. For temporary stationary UEs, allow UE to report “temporary stationary” attribution via UE Assistance Information.

- Reference

[1] RP-210918  Revised WID on support of reduced capability NR devices.

[2] TR 38.875;

[3] R2-2100570 Consideration on interoperability between Rel-17 RedCap RRM relaxation and Rel-16 RRM relaxation   ZTE Corporation, Sanechips.