3GPP TSG RAN WG2 Meeting #113 bis electronic                                             R2- 2103041

Online, April 12 – April 20, 2021

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Inter-UE Coordination for Sidelink Mode 2 Resource Allocation

Agenda item:	8.15.3

Document for:	Discussion

# Introduction

The work item for NR sidelink enhancement was approved in RAN#86 and revised in RAN#90e [1]. The following objectives were identified in relation to resource allocation to enhance reliability and latency:

The objective of this work item is to specify radio solutions that can enhance NR sidelink for the V2X, public safety and commercial use cases. 2. Resource allocation enhancement: - Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

This contribution discusses resource allocation based on inter-UE coordination and its ability to enhance reliability, reduce latency as well as to reduce power consumption for sidelink mode 2 UEs.

# Discussion

The WID [1] defines inter-UE coordination as one of the possible resource allocation enhancement techniques to overcome the lack of inadequate sensing results. This is due to known issues such as missing sensing results because of the half-duplex constraint, partial sensing, or DRX in Rel-17. Furthermore, the well-known hidden node problem increases the chances of resource allocation collisions. Inter-UE coordination should increase reliability, reduce latency on the sidelink, and may also decrease the UEs power consumption.

In addition, RAN1 concluded in the Chairman’s notes of RAN1#104e [2], that “the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.” The detailed observations can be found in R1-2102166 [3].

Inter-UE coordination is achieved when a UE, e.g. UE-A, shares a set of resources with another UE, e.g. UE-B, as shown in Figure 1. This is done so that the UE-B can use this information in order to assist its process of resource selection. This was explored in depth during the study phase of Release 16, under the form of Mode 2(b) and Mode 2(d) [4].

Figure 1: Inter-UE coordination where UE-A sends assistance information to UE-B.

Figure 2 shows another example how inter-UE coordination can help when two UEs are outside each other’s range. Depicted here is the well-known hidden node problem, where UE-1 and UE-2 are too far from each other and thus not within the reception range of one another. Consequently, both UE-1 and UE-2 can select interfering resources for simultaneous transmission to UE-3, resulting in a collision. Coordination of resources among these UEs can prevent such collisions and the RAN2 requirements for such a mechanism will be highlighted in the following sections.

Figure 2: Depiction of resource collisions due to the hidden node problem.

## Set of Resources for Inter-UE Coordination

The WID [1] defines that Inter-UE coordination is based on a “set of resources” determined by UE-A, next sent to UE-B, so that UE-B can consider the set of resources for resource selection for its own transmission.

As a first step, the term “set of resources” should be defined in the context of RAN2. For assisting in resource allocation in [3], RAN1 identified 3 schemes for inter-UE coordination, categorized as:

- Type A: Preferred set of resources: UE-A sends to UE-B the set of resources preferred for UE-B’s transmission, e.g., based on its sensing result. This preferred set of resources are assumed as high-quality resources from UE-A's perspective.

- Type B: Not preferred set of resources: UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission, e.g., based on its sensing result and/or expected/potential resource conflict. This set of resources should be avoided, i.e. not be used by UE-B. For example, it may include resources used by UE-A for its own transmission.

- Type C: Conflicting candidate resources: UE-A sends to UE-B the set of resources where the resource conflict is detected. Also this set of resources should not be used by UE-B.

- Three types of “set of resources” are distinguished for inter-UE Coordination: A set of preferred resources, a set of not preferred resources, and a set of conflicting resources.

## Content of Assistance Information

One of the defining aspects of inter-UE coordination is the set of resources that are transmitted by UE-A in order to assist UE-B as shown in Figure 3.

In the case that the set of resources is preferred for UE-B’s transmission, UE-A sends a set of resources that it finds available during its sensing and resource selection process, as shown in Figure 3. To enable UE-B to identify the preferred set of resources, UE-A may share its sensing information or results of the sensing information with UE-B. UE-B can use the set of resources for its own intended transmissions.

Figure 3: Exemplary depiction of UE-B using a preferred set of resources from UE-A.

On the other hand, the set of resources that are preferred not to be used by UE-B are those resources that UE-A detected to be occupied by other UEs for their respective transmissions. If UE-A is the intended recipient for transmission from UE-B, UE-A can also indicate resources that UE-A already has scheduled and reserved for other transmissions. If UE-B transmits to UE-A using these resources, due to the half-duplex constraint, UE-A would not be able to receive the intended transmission from UE-B, and hence, these resources are not preferred, as shown in Figure 4.

Figure 4: Exemplary depiction of UE-B using a set of not preferred resources from UE-A.

The set of resources transmitted by UE-A can be combined with a candidate resource set or a set of selected resources on the PHY layer or the MAC layer of UE-B. If the assistance information is received in the MAC layer, it would be straightforward for the MAC layer to carry out the combination of the resource sets. For example, this can be done by either finding common resources in both sets, as shown in Figure 3 or excluding not preferred resources, as shown in Figure 4. The detailed procedures for the combination of the resource sets need to be discussed by RAN2.

Assistance information as a set of resources to use or avoid can be merged in the MAC layer with the candidate resource set given by the PHY.

RAN2 should evaluate the procedures for merging assistance information in the MAC layer.

## Methods for Transmitting Assistance Information

As sidelink transmissions are fixed in size across time and frequency, just transmitting assistance information by itself is very inefficient. Therefore, it is desirable to multiplex assistance information with other data transmissions or to multiplex several assistance information into a single transmission.

Sending assistance information in L2 is inefficient.

Adding assistance information to a unicast transmission as a MAC CE is a straight forward approach. However, message handling for groupcast and broadcast scenarios needs to be further studied, especially when assistance information to different group members needs to be differentiated at the receiver side.

Also, assistance information can be sent as a physical layer message, as pointed out in our accompanying RAN1 TDoc [5].

RAN2 to evaluate mechanisms to reduce the impact on resource utilization caused by transmissions of assistance information.

## Triggering Conditions for Sending Assistance Information

The PHY sensing procedure differs depending on the transmission parameters. Here among others, the priority and packet delay budget play a role within the selection process. When UE-A selects a set of resources for UE-B, it should also perform the appropriate sensing procedure depending on the transmission parameters required by UE-B. Thus, the parameters used by UE-A in order to determine the set of resources can be pre-configured as one option. Another option is that UE A can adapt its parameters based on an assistance message request by UE-B. Depending on the expected content, the request may indicate how UE-B will utilize the assistance information, so that UE-A may adapt the sensing procedures and generate the appropriate set of resources. This is because the parameters related to the sensing procedure changes with priority, packet delay budget and feedback mode.

To generate the assistance information, UE-A should consider the requirements transmitted by UE-B.

There are two main scenarios where assistance information can improve the performance. One is a periodic case, where a UE-A assists UE-B in resource selection and sends a selected set of resources periodically to UE-B. This case can be set up via PC-5 RRC. Another case is that UE-A is triggered to send assistance information to UE-B. An example of a trigger could be the CBR-level of a resource pool, or alternatively, UE-A could be triggered to assist UE-B when it has consistent failed receptions from UE-B, for example, due to packet collisions with transmissions from another UE.

The  following design aspects of inter-UE coordination should be considered:

- Additional parameters are needed to generate the assistance information,

- Events that trigger the assistance information procedure.

## Power Saving Aspect on Sharing a Set of Resources by Specific Type of UE

As the identification of the “set of resources” by a UE demands continuous sensing, it might not be appropriate for battery-based UEs. Continuous sensing and transmission of a set of resources is expected to increase power consumption. However, for some types of UE, e.g. Vehicular-UE (V-UE) or Roadside Unit (RSU) type UE the impact of the increased power consumption is negligible. Therefore, the decision on which UE should perform sensing in order to determine the “set of resources” and how often the “set of resources” should be shared can be based on the type of UE.

Continuous sensing and sharing of set of resources for inter-UE coordination may increase the power consumption of a UE.

The type of the UE (e.g. V-UE, RSU-type UE, battery-based UE) should be considered to decide whether the set of resource for inter-UE coordination should be shared and how often this information should be shared.

# Conclusion

The following observations have been made in this document:

Observation 1:	Three types of “set of resources” are distinguished for inter-UE Coordination: A set of preferred resources, a set of not preferred resources, and a set of conflicting resources.

Observation 2:	Assistance information as a set of resources to use or avoid can be merged in the MAC layer with the candidate resource set given by the PHY.

Observation 3:	Sending assistance information in L2 is inefficient.

Observation 4: To generate the assistance information, UE-A should consider the requirements transmitted by UE-B.

Observation 5:	Continuous sensing and sharing of set of resources for inter-UE coordination may increase the power consumption of a UE.

The following proposals have been made in this document:

Proposal 1:	RAN2 should evaluate the procedures for merging assistance information in the MAC layer.

Proposal 2:	RAN2 to evaluate mechanisms to reduce the impact on resource utilization caused by transmissions of assistance information.

Proposal 3:	The  following design aspects of inter-UE coordination should be considered:

- Additional parameters are needed to generate the assistance information,

- Events that trigger the assistance information procedure.

Proposal 4:	The type of the UE (e.g. V-UE, RSU-type UE, battery-based UE) should be considered to decide whether the set of resource for inter-UE coordination should be shared and how often this information should be shared.

# References

- RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP RAN#90e, December 2020.

- Chairman’s notes, RAN1#104e, January-February 2021.

- R1-2102166, “Detailed observations from evaluation results”, Moderator (LG Electronics), RAN1#104e, January-February 2021.

- TR 38.885 v16.0.0, “Study on NR Vehicle-to-Everything (V2X) (Release 16)”, June 2020.

- R1-2102812, “Resource Allocation Enhancements for Mode 2”, Fraunhofer HHI, Fraunhofer IIS, RAN1#104b-e, April 2021.