3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2103064

Electronic, 12th April – 20th April 2021

Source:	Qualcomm Incorporated

Title:	On the accuracy of M5 and M7 measurements in split-bearer

Agenda Item:	8.13.3.1 Immediate MDT enhancements

Document for:	Discussion and Decision

# Introduction & Background

Rel-17 SON/MDT [1] includes the following RAN2-led objectives in the context of MDT.

- Support of data collection for MDT features for identified use cases, including 2-step RACH optimization and leftovers of Rel-16 SON/MDT WI (MDT enhancements and MDT for MR-DC) [RAN2, RAN3, RAN4]

- Enhancement of logged and immediate MDT (including coexistence with IDC) [RAN2, RAN3]

- Enhancement of reporting e.g. RLF and accessibility measurements, Successful Handover reporting [RAN2, RAN3].

- Specification of MDT for MR-DC [RAN2, RAN3, RAN4]

Furthermore, in RAN2#111-emeeting [2], the following topics are prioritized:

- For M5/M6/M7, it is proposed to apply them for EN-DC/MR-DC cases with different bear types. FFS on details.

In this contribution draft, we will discuss the configuration and accuracy of M5, M6, and M7 measurements in the split-bearer scenario.

# Discussion

In the post RAN2-113-emeeting email, discussion [4], the M5 and M7 measurements were discussed, where we proposed and supported the methodologies to obtain accurate M5 and M7 measurements. The alternatives that different companies proposed are as follows:

- A small indication (whether the duplication or packet aggregation is used) may be sufficient.

- Small bursts are excluded from the measurements. Therefore, enhancements are not needed.

However, these arguments are not valid. For example, see the explanation, why the indication of whether « whether the duplication or packet aggregation is used » is not sufficient for achieving the M5 and M7 accurate measurements was presented in our previous paper [5]. Furthermore, when considering the small data burst is considered, the small data burst that can be transmitted in one slot is neglected not the data burst that takes multiple slots [6]. Therefore, data burst taking multiple slots needs to be accounted for.

Observation 1: Indication « whether the duplication or packet aggregation is used «  is not sufficient to obtain accurate M5 and M7 measurements.

Observation 2: The throughput of the data burst that takes a single slot for transmission is considered zero. However, the throughput of the data burst taking multiple slots needs to be considered for accurate M5 measurements.

Observation 3: A packet is considered lost only if it is lost over both MN and SN, therefore for the accurate M7 measurement, we need to know what sequence numbers are lost over both MN and SN.

Observation 4: The accurate measurements are extremely important for the network operation, for example, to determine duplication is needed to achieve a certain level of QoS. For example, what action needs to be adopted to achieve a certain throughput and packet loss rate.

We argue that the proposals are important for accurate M5 and M7 measurements. In this paper, we clarify that only those data bursts are reported from SN that takes more than one slot for the transmission. The modified proposal is as follows:

Proposal 1: Considering both M5 and M7 measurements, we argue to introduce an X2/Xn message to obtain the following measurement at the end of the measurement period:

- Burst Size of data transmitted over SN (taking multiple transmission slots)

- The point in time when the transmission is started for the first data in the data burst over SN

- The point in time when the data until the second last piece of data burst TX over SN has been successfully received at the UE

- The RLC SDU sequence number of packets lost over the Uu interface

- The RLC SDU sequence number of the packet discard at the RLC or MAC for traffic management for which part is transmitted over the air.

# Conclusion

Observation 1: Indication « whether the duplication or packet aggregation is used «  is not sufficient to obtain accurate M5 and M7 measurements.

Observation 2: The throughput of the data burst that takes a single slot for transmission is considered zero. However, the throughput of the data burst taking multiple slots needs to be considered for accurate M5 measurements.

Observation 3: A packet is considered lost only if it is lost over both MN and SN, therefore for the accurate M7 measurement, we need to know what sequence number lost over both MN and SN.

Observation 4: The accurate measurements are extremely important for the network operation, for example, to determine duplication is needed to achieve a certain level of QoS. For example, what action needs to be adopted to achieve a certain throughput and packet loss rate.

Proposal 1: Considering both M5 and M7 measurements, we argue to introduce an X2/Xn message to obtain the following measurement at the end of the measurement period:

- Burst Size of data transmitted over SN (taking multiple transmission slots)

- The point in time when the transmission is started for the first data in the data burst over SN

- The point in time when the data until the second last piece of data burst TX over SN has been successfully received at the UE

- The RLC SDU sequence number of packets lost over the Uu interface

- The RLC SDU sequence number of the packet discard at the RLC or MAC for traffic management for which part is transmitted over the air.

# Reference

[1] RP-201281, “Revised WID on the enhancement of data collection for SON/MDT in NR and EN-DC”

[2] RAN2#111e meeting report

[3] RAN2#108 meeting report

[4] Post RAN2-113-emeeting [853] email discussion

[5] R2-2101342,” R2-2101342 - On the configuration and accuracy of M5, M6, and M7 measurements in split-bearer”

[6] TS 28.552