3GPP TSG-RAN WG2 Meeting #113bis electronic							    R2-2103118

E-Meeting, Apr 12th – Apr 20th, 2021

Source: 	vivo

Title:  	Consideration on SA2 Questions about Session Activation

Agenda Item:	8.1.1

Document for:	Discussion and Decision

# 1.Introduction

In the previous RAN2 and RAN3 meetings, agreements relevant to session activation were as follows.

RAN2 agreements

- When there is no data ongoing for the multicast session, the UE can stay in RRC_CONNECTED . Other cases FFS

- It is up to SA2 to decide whether the multicast session activation/deactivation mechanism is supported or not, and RAN2 will discuss if there is any RAN2 impacts based on SA2 inputs.

- Request a clarification from SA2 on whether and what the difference is between session start and session activation and between the session stop and session deactivation.

- If the UE which joined the multicast session is in RR CONNECTED state when the session is started, the gNB sends RRC Reconfiguration message with relevant MBS configuration to the UE and there is no need for separate session start notification for this UE. FFS for session activation.

RAN3 agreements

- When an MBS session is (re-)activated, group paging may be used toward supporting nodes (to be checked against RAN2 progress).

- Support 5GC triggered MBS Session Stop/Deactivation  (pending SA2 progress).

- Support of all activation scenarios, i.e. for UEs in all CM/RRC states and served by both, MBS supporting and MBS non-supporting RAN with UEs having Registration Areas covering supporting/non supporting RAN nodes; whether non-supporting RAN nodes should receive this information is FFS.

- NG functions support all NG-RAN signaling resulting from NAS Session Joining:  i.e. joining during an activated MBS Session and joining during a deactivated MBS Session, joining while served by a RAN supporting 5MBS and joining while served by a RAN not supporting 5MBS.

Besides, SA2 and RAN2/3 have sent several LSes to each other for achieving common understandings of session activation and deactivation issues. Hence, in this contribution, we would like to provide our considerations on SA2 LS questions [1].

# 2.Discussion

## 2.1 NG-RAN node not supporting MBS

Regarding the activation notification on the NG-RAN node not supporting MBS, SA2 has given feedback on the question proposed by RAN3 and proposed a new question to RAN2/3.

Further, RAN3 would like to ask for confirmation from SA2 whether a UE is supposed to receive the MBS Session activation notification also when served by a non-supporting NG-RAN node.

SA2 would like to confirm that it is necessary for UE to receive the MBS Session activation notification (e.g., legacy paging) when it is served by a non-supporting NG-RAN node.

SA2 follow-up question: SA2 asks RAN2/RAN3 for feedback on whether UEs camping on non-supporting NG-RAN nodes can be notified using MBS session ID or the 5GC is required to fallback to regular paging for UEs that have not connected during MBS session activation.

As for SA2’s question, in our views, regular paging can be better for the NG-RAN node not supporting MBS to notify session activation based on the following considerations.

Firstly, the consumption for updating legacy NG-RAN nodes to apply the new session activation mechanism using MBS session ID is huge, especially when the number of legacy NG-RAN nodes is large.

Secondly, more power consumption is required for UEs to monitor the session activation notification based on MBS session ID, especially when UEs are interested in various MBS services. Because in addition to the legacy paging monitoring for other services, UE has to monitor the multicast activation notification based on MBS session ID for MBS services. What is more, UE will perform different monitoring for different MBS services based on different MBS session IDs. On the contrary, UE only needs to monitor the legacy paging for multicast services and other services simultaneously.

Therefore, we have the following observations and proposal,

Observation 1: The consumption for updating legacy NG-RAN nodes to apply the new session activation mechanism using MBS session ID is huge, especially when the number of legacy NG-RAN nodes is large.

Observation 2: More power consumption is required for UE to monitor the session activation notification based on MBS session ID, especially when UE is interested in various MBS services.

Proposal 1: 5GC is required to fallback to regular paging for UEs camping on non-supporting MBS NG-RAN nodes during multicast session activation.

## 2.2 NG-RAN node supporting MBS

Regarding the activation notification on the NG-RAN node supporting MBS, SA2 has also given feedback on the question proposed by RAN3 and proposed a new question to RAN2.

RAN3 would like to ask RAN2 and SA2 to consider how to page the UEs which are not in RRC_CONNECTED state, whether a group notification towards NG-RAN supporting MBS and, if applicable, to NG-RAN nodes not supporting MBS is needed/feasible/beneficial comparing to the legacy paging methods.

SA2 concludes that it is beneficial, e.g. for signalling efficiency, to support 5GC requesting NG-RAN nodes to notify session activation of an MBS session to UEs based on MBS session ID, at least to NG-RAN nodes supporting MBS.

SA2 follow-up question: SA2 requests RAN2 for confirmation whether NG-RAN node can notify session activation to UEs based on MBS session ID. SA2 normative work on this aspect will be pending RAN2 conclusion.

In our understanding, it is better for the NG-RAN node supporting MBS to notify the session activation based on MBS session ID. Because compared with the NG-RAN node not supporting MBS, the NG-RAN node supporting MBS can identify that the core network’s notification is for MBS service via MBS session ID. Besides, given the signalling efficiency, the NG-RAN node supporting MBS can use the group notification mechanism to notify all UEs belonging to the same multicast group rather than notifying them individually. Especially when the number of UEs belonging to the same multicast group is very large, the signalling efficiency will be very high. Hence, we have the following observation and proposal,

Observation 3: More power consumption is required for UE to monitor the session activation notification based on MBS session ID, especially when UE is interested in various MBS services.

Proposal 2: RAN2 to confirm that NG-RAN node supporting MBS can notify session activation to UEs based on MBS session ID.

Regarding the specific methods of group activation notification based on MBS session ID, several methods exist. For example, 5GC or NG-RAN nodes can initiate a group paging activation notification associated with MBS session ID to all UEs belonging to the same multicast group. However, in our understanding, more standardization efforts and power consumption is required for the group paging activation notification, especially when UEs are interested in various MBS services. The related figure is as below,

Figure 1: Paging occasions for a MBS UE

Firstly, we need to define a new kind of paging mechanism in which the paging occasions may be associated with the MBS session ID or MBS session ID group rather than UE ID to achieve the group notification for all related UEs. Especially when the MBS services and MBS service groups are various, the standardization effort is huge and complex. Secondly, when UE is interested in various MBS services, UE has to monitor various paging occasions for different MBS services individually( e.g., in Figure 1, UE needs to monitor 4 POs in a PF). In other words, more power consumption for UE and the network is required.

Except for the group paging activation notification, the MCCH group activation notification based on MBS session ID can also be useful. Referring to the LTE SC-MCCH message in TS36.331 [2] below, the MBS session ID list can be indicated in MCCH for the multicast session activation group notification in NR. After detecting the corresponding MBS session IDs in MCCH including the MBS session ID list, UE should enter the RRC-CONNECTED state to receive the activated multicast session which is provided by delivery mode 1.

SCPTMConfiguration message

-- ASN1START

SCPTMConfiguration-r13 ::=		SEQUENCE {

sc-mtch-InfoList-r13			SC-MTCH-InfoList-r13,

scptm-NeighbourCellList-r13		SCPTM-NeighbourCellList-r13			OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SCPTMConfiguration-v1340			OPTIONAL

}

SCPTMConfiguration-v1340 ::= SEQUENCE {

p-b-r13								INTEGER (0..3)			OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

In our understanding, the MCCH group activation notification is better than the group paging activation notification. Firstly, the standardization effort is less. Because we have to design the MCCH mechanism to support delivery mode 2, the optimization work of MCCH for supporting delivery mode 2 and delivery mode 1 simultaneously is less. Secondly, different activated MBS session IDs included in the MBS session ID list can be notified together in the MCCH to save the power of UE and network. Thirdly, except for the activated multicast service, UE may also need to receive the broadcast service by the MCCH. So it will be simpler for this kind of UE to monitor the group activation notification by MCCH. Hence, we propose the following proposals,

Proposal 3: For NG-RAN nodes supporting MBS, the MCCH message including the MBS session ID list can be used for multicast session activation group notification.

Proposal 4: After detecting the corresponding MBS session IDs in MCCH including the MBS session ID list, UE should enter the RRC-CONNECTED state to receive the activated multicast session which is provided by delivery mode 1.

# 3.Conclusions

Based on the analysis given above, we have the following observations and proposals:

-NG-RAN node not supporting MBS

Observation 1: The consumption for updating legacy NG-RAN nodes to apply the new session activation mechanism using MBS session ID is huge, especially when the number of legacy NG-RAN nodes is large.

Observation 2: More power consumption is required for UE to monitor the session activation notification based on MBS session ID, especially when UE is interested in various MBS services.

Proposal 1: 5GC is required to fallback to regular paging for UEs camping on non-supporting MBS NG-RAN nodes during multicast session activation.

-NG-RAN node supporting MBS

Observation 3: More power consumption is required for UE to monitor the session activation notification based on MBS session ID, especially when UE is interested in various MBS services.

Proposal 2: RAN2 to confirm that NG-RAN node supporting MBS can notify session activation to UEs based on MBS session ID.

Proposal 3: For NG-RAN nodes supporting MBS, the MCCH message including the MBS session ID list can be used for multicast session activation group notification.

Proposal 4: After detecting the corresponding MBS session IDs in MCCH including the MBS session ID list, UE should enter the RRC-CONNECTED state to receive the activated multicast session which is provided by delivery mode 1.

# 4.Reference

[1] S2-2102077, “Reply LS on 5MBS progress and issues to address”.

[2] TS36.331-g30, " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC);

Protocol specification ".