3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2103552

Online, April 12th – April 20th, 2021

Agenda item:	8.13.2.3	Other WID related SON features

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on other SON aspects

Document for:	Discussion and Decision

# Introduction

In the RAN#88-e meeting, it was agreed that the WID includes the following SON aspects [1]:

- Leftovers of Rel-16 SON/MDT WI including MRO for SN change failure,

- Leftovers of Rel-16 SON /MDT WI including specification of MDT for MR-DC.

Further, the email discussions [5] considered related enhancements to MCG / SCG failure. The email discussion conclusion proposed to include the enhancements for SN change failure in Rel-17, while remained inconclusive for fast MCG recovery.

The email discussion addressed also UL/DL coverage imbalance aspects with proposal to introduce a list of CEF reports to identify and resolve the problem.

In this paper, we provide explanations, observations and proposals for the above open aspects. We also provide our view on the LS sent by RAN3 on information needed for MRO in SCG Failure Report.

# Discussion

### MRO for SN Change Failure

MRO for SN change failure was discussed in detail by RAN3 in several previous RAN3 meetings and RAN2 has been waiting for the progress in RAN3 before starting the discussion. RAN3 has sent an LS at RAN3#111-e meeting to RAN2 to confirm whether the information can be reported from the UE and if so, define the corresponding reporting mechanisms [2]:

“RAN3 discussed the solution for the optimization of PScell change failure for MRO in case of MR-DC. RAN3 agreed it is beneficial for the NG-RAN node to receive the list of information as shown below for the purpose of PSCell failure analysis:

- CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell.

- CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed PScell change. The Failed PSCell could be E-UTRA cell or NR cell.

- timeSCGFailure: the time elapsed since the last PSCell change initialization until SCG failure.

- connectionFailureType: radio link failure or SN change failure.

5)     random-access related information set by the PSCell.”

In our opinion, the Rel-17 UEs can provide the items above in the SCG Failure Information sent by the UE to the gNB in case of PSCell failure. On the other hand, we understand that the item (4) is already present in the SCG Failure Information (i.e., failureType), therefore a repetition of this information is not needed.

Observation 1: connectionFailureType already exists at SCG Failure Information as failureType.

For item (5), RAN3 has already sent an LS to RAN2 in [3] at RAN3#109-e meeting with the following contents, and the discussions in RAN2 has been triggered:

RAN3 discussed the use cases of RACH report for SgNBs, and observed that there was no means for the SgNB to retrieve from UE in MR-DC any information on RACH access procedure at SgNB, and thus there was no input for SON algorithm to adjust the RA related parameters in SgNBs.

RAN3 respectfully asks RAN2 to consider UE RACH report for SgNBs and provide feedback to RAN3.

Since the RACH information would be included in the UE RACH report for PSCell, we do not see any use case, where the random-access related information set by PSCell additionally in the SCG Failure Information would be beneficial. Thus, we do not need (5) in the SCG Failure Information.

Observation 2: Random-access related information set by the PSCell can be retrieved via RACH report for PSCell, whose details are in discussion in RAN2.

Thus, RAN2 should confirm the items (1), (2) and (3), whereas (4) and (5) are not needed.

Proposal 1: RAN2 confirms the items (1), (2) and (3), whereas (4) and (5) are not needed to be added in the SCG Failure Information sent by the UE to the gNB in case of PSCell failure.

### Further Issues on MDT for MR-DC

RLF Triggered By MCG/SCG Failure

Several companies have proposed in their contributions to RAN#2-113-e meeting and during Post113-e e-mail discussions to include a container of SCGFailureInformation in the RLF report. The aim is for the UE to be able to log the SCGFailureInformation, when the MCG transmission is suspended, so that the SCGFailureInformation can be sent even in case the UE cannot immediately report the SCG failure to the MN.

In our view, if both MN and SN have a radio link problem, the RLF report should indicate this case. Currently, RLF report does not contain the SCGFailureInformation. By addition of a container of SCGFailureInformation, MRO can be successfully performed even in case MCG transmission is suspended.

Proposal 2: A container of SCGFailureInformation is included to the RLF report for MCG/SCG failure enhancement.

In addition, we have observed from the contributions and post-meeting e-mail discussions that a few companies are interested in enhancements for Fast MCG Recovery. However, Fast MCG Recovery is out of the scope of the WID, therefore should not be treated.

Proposal 3: Fast MCG Recovery enhancement is not treated, since the topic is out of the scope of the current WID.

### UL/DL coverage imbalance

As described in [4], the problem starts with RLF caused by UL coverage issue (expressed by failureType “maximum number of RLC retransmission reached”, and this failureType occurs only if DL is visible for the, otherwise UE would lose sync with gNB anyway and UE would not need to react with RLC retransmissions.

Provided that DL is still measurable, the next step in this failure scenario is a re-establishment attempt, but RRC message or already RACH will not reach gNB because of weak UL, and UE goes to IDLE with T311 expiry. Next, RRC connection establishment attempt might follow, if DL is still measurable. Multiple contiguous CEFs after RLF are indicating good DL but UL coverage issue. But CEFs are send independent from RLF. With likely scenario of giving up the connection establishment attempts would indicate that no DL cell is measurable, and this is not expressed in CEF, it is just missing or failed attempt.

Following the Rel-17 enhancement that supports multiple CEF reports, the node receiving RLF report and CEF reports including list of failed RRC attempts simultaneously from one and the same UE can make use of both types of failure information together for more detailed root cause analysis provided that node can identify and verify that the problems are linked. But a more detailed information about DL coverage during the outage period, e.g. whether DL synchronization in IDLE mode gets lost for some time, is not really derivable from these two reports. It is an UL issue triggered RLF report and a CEF report including a series of failed CE attempts without knowing when they happened.

Proposal 4: RAN2 should proof if UE can record DL absence and to add this information about DL availability to the RLF report.

# Conclusion

In this contribution, we have discussed various aspects of this WI. From those discussions we have the following observations and proposals:

Observation 1: connectionFailureType already exists at SCG Failure Information as failureType.

Observation 2: Random-access related information set by the PSCell can be retrieved via RACH report for PSCell, whose details are in discussion in RAN2.

Proposal 1: RAN2 confirms the items (1), (2) and (3), whereas (4) and (5) are not needed to be added in the SCG Failure Information sent by the UE to the gNB in case of PSCell failure.

Proposal 2: A container of SCGFailureInformation is included to the RLF report for MCG/SCG failure enhancement.

Proposal 3: Fast MCG Recovery enhancement is not treated, since the topic is out of the scope of the current WID.

Proposal 4: RAN2 should proof if UE can record DL absence and to add this information about DL availability to the RLF report.

# Reference

- RP-201281 “Revised WID on enhancement of data collection for SON/MDT in NR and EN-DC”, 3GPP TSG RAN meeting #88-e, 26th June – 3rd July, 2020.

- R3-211332 “LS on information needed for MRO in SCG Failure Report”, 3GPP TSG RAN3 meeting #111-e, 25th January – 5th February, 2021.

- R3-205662 “LS to RAN2 on RACH report for SgNB”, 3GPP TSG RAN3 meeting #109-e, 17th – 27th August, 2020.

- R2-2100602 “Refined UL coverage outage detection”, Nokia, Nokia Shanghai Bell

- R2-2103093 “[Post113-e][852][NR R17 SON/MDT] 2 step RA and other SON changes (CATT)”, 3GPP TSG RAN2 post-meeting e-mail discussions, 26th March, 2021.

# Annex A: Text Proposal for TS 38.331

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                   SEQUENCE {

criticalExtensions                           CHOICE {

scgFailureInformation                        SCGFailureInformation-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=       SEQUENCE {

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                         OPTIONAL

}

FailureReportSCG ::=                       SEQUENCE {

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                          MeasResultFreqList                                                      OPTIONAL,

measResultSCG-Failure                      OCTET STRING (CONTAINING MeasResultSCG-Failure)                OPTIONAL,

...,

[[

locationInfo-r16                            LocationInfo-r16            OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL

]],

[[

sourcePSCell-CGI                               CGI-Info-Logging-r16,

targetPSCell-CGI                               CGI-Info-Logging-r16 OPTIONAL,

timeSinceLastPSCellChange                      INTEGER (0..172800)

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

sourcePSCell-CGI This field indicates the CGI of the cell that is the source PSCell of the last SN change.

targetPSCell-CGI This field indicates the CGI of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change.

timeSinceLastPSCellChange This field is used to indicate the time that elapsed since the last PSCell change initialization until SCG failure.

#### –	SCGFailureInformationEUTRA

The SCGFailureInformationEUTRA message is used to provide information regarding E-UTRA SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformationEUTRA message

-- ASN1START

-- TAG-SCGFAILUREINFORMATIONEUTRA-START

SCGFailureInformationEUTRA ::=                SEQUENCE {

criticalExtensions                                CHOICE {

scgFailureInformationEUTRA                       SCGFailureInformationEUTRA-IEs,

criticalExtensionsFuture                          SEQUENCE {}

}

}

SCGFailureInformationEUTRA-IEs ::=           SEQUENCE {

failureReportSCG-EUTRA                           FailureReportSCG-EUTRA                      OPTIONAL,

nonCriticalExtension                              SCGFailureInformationEUTRA-v1590-IEs                                    OPTIONAL

}

SCGFailureInformationEUTRA-v1590-IEs ::=  SEQUENCE {

lateNonCriticalExtension                  OCTET STRING            OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                  OPTIONAL

}

FailureReportSCG-EUTRA ::=                       SEQUENCE {

failureType                                           ENUMERATED {

t313-Expiry, randomAccessProblem,rlc-MaxNumRetx,

scg-ChangeFailure,scg-lbtFailure-r16,

beamFailureRecoveryFailure-r16, t312-Expiry-r16, spare},

measResultFreqListMRDC                             MeasResultFreqListFailMRDC                                                   OPTIONAL,

measResultSCG-FailureMRDC                         OCTET STRING                                                         OPTIONAL,

...,

[[

locationInfo-r16                                    LocationInfo-r16                                                         OPTIONAL

]],

[[

sourcePSCell-CGI                               CGI-InfoEUTRALogging,

targetPSCell-CGI                               CGI-InfoEUTRALogging OPTIONAL,

timeSinceLastPSCellChange                      INTEGER (0..172800)

]]

}

MeasResultFreqListFailMRDC ::=      SEQUENCE (SIZE (1.. maxFreq)) OF MeasResult2EUTRA

-- TAG-SCGFAILUREINFORMATIONEUTRA-STOP

-- ASN1STOP

SCGFailureInformationEUTRA field descriptions

measResultFreqListMRDC The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig.

measResultSCG-FailureMRDC Includes the E-UTRA MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message.

sourcePSCell-CGI This field indicates the CGI of the cell that is the source PSCell of the last SN change.

targetPSCell-CGI This field indicates the CGI of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change.

timeSinceLastPSCellChange This field is used to indicate the time that elapsed since the last PSCell change initialization until SCG failure.