3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2103577

Online, April 12 – April 20, 2021

Agenda Item:	8.15.2

Source:	MediaTek Inc.

Title:	On coordination between Uu DRX and SL DRX

Document for:     Discussion and Decision

# Introduction

In this paper, We propose our view on alignment/coordination between Uu DRX and SL DRX.

# Discussion

Based on online discussion in RAN2#113, we think the Uu/SL DRX alignment in both Tx UE side and Rx UE side are reasonable:

- TX UE side coordination: The gNB in TX UE side needs to ensure that for mode-1 scheduling, Uu DRX is in active time when Rx UE is in SL active time. Otherwise, TX UE cannot send data to RX UE in RX UE’s SL active time since TX UE cannot get timely SL grant during  Uu DRX off time.

- RX UE side coordination: The gNB in RX UE side may want to save UE reception power by aligning Uu DRX and SL DRX, i.e. make Uu active time and SL active as aligned as possible.

Obseravtion 1: TX UE side coordination is necessary for mode-1 scheduling.

Observation 2: RX UE side coordination is optional for power saving enhancement.

Since TX UE side coordination is necessary for normal mode-1 scheduling, and Rx UE side is aligned with the WI intention for power saving, we think both side coordnaiton should be supported. Further, we don’t think supporting both side coordination would have big spec impact because whther/how to perform coordination is up to gNB implementation. What UE should do is just to forward the required information from peer UE (e.g. assistance information from peer UE or SL DRX configuration provided by peer UE ).

Proposal 1: Uu/SL DRX coordination in both TX UE and RX UE side are supported.

Proposal 2: Tx/Rx UE forwards the assistance information or SL DRX configuration from peer UE to its gNB upon rececption from peer UE.

Proposal 3: Whether gNB reconfigure Uu/SL DRX for DRX alignment is up to network implementation.

# Conclusion

In this paper, we discuss the coordination between Uu DRX and SL DRX. We have observations and proposals as follows:

Obseravtion 1: TX UE side coordination is necessary for mode-1 scheduling.

Observation 2: RX UE side coordination is optional for power saving enhancement.

Proposal 1: Uu/SL DRX coordination in both TX UE and RX UE side are supported.

Proposal 2: Tx/Rx UE forwards the assistance information or SL DRX configuration from peer UE to its gNB upon rececption from peer UE.

Proposal 3: Whether gNB reconfigure Uu/SL DRX for DRX alignment is up to network implementation.

# Reference