3GPP TSG-RAN WG2 Meeting #113 bis electronic	                                   R2-2103707
Online, April 12 – April 20, 2021

Agenda Item:	8.12.3.1

Source: 	CMCC

Title:  	Discussion on eDRX for RedCap

WID/SID:	         NR_redcap

Document for:	Discussion and Decision

Introduction

In the RAN#92e meeting, the objectives of Extended DRX have been discussed and the following items have been achieved [1]:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4. - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.

Based on the related objective and discussion in SI phase, we provide our view on eDRX related issues in this contribution.

Discussion

As described in TS 23.501[2], the UE and the network may negotiate over non-access stratum signaling the use of extended idle mode DRX for reducing its power consumption, and the eDRX related information is transmitted to UE by NAS signaling. Besides RRC_Idle, eDRX could  also be supported in RRC_Inactive in some RAT types.

For RAT types that support extended DRX for CM-CONNECTED with RRC Inactive state, the AMF passes the UE's accepted idle mode eDRX cycle length value to NG-RAN. If the UE supports eDRX in RRC_Inactive, based on its UE radio capabilities, NG-RAN configures the UE with an eDRX cycle in RRC-INACTIVE up to the value for the UE's idle mode eDRX cycle as provided by the AMF in "RRC Inactive Assistance Information" or up to 10.24 seconds (whichever is lower).

Observation 1: In LTE, CN could pass UE idle mode eDRX cycle length value to RAN node.

Observation 2: In LTE, it’s RAN node to decide the eDRX cycle in RRC_Inactive, which is up to the value for the UE's idle mode eDRX cycle or 10.24 seconds.

For RedCap UEs, the same mechanism that CN passes UE idle mode eDRX cycle length value to RAN node, which may be used for the eDRX cycle in RRC_Inactive decision by gNB could be reused. In this way, the specification workload could be decreased, and from RAN point of view, it may provide more flexibility for RAN nodes’ configuration.

Proposal 1：The current mechanism should be reused for RedCap UE’s eDRX configuration decision, i.e.,

- gNB is responsible for the eDRX cycle configuration in RRC_Inactive, based on the information provided by CN in "RRC Inactive Assistance Information".

Besides, another issue is whether same parameters should be configured for RRC_Idle and RRC_Inactive.

In the discussion of SI phase, four options were captured in TR 38.875[3]:

-	A common PTW and eDRX cycle.

-	A common PTW but with different eDRX cycle.

-	A common eDRX cycle but with different PTW length.

-	Different eDRX cycle and different PTW length.

It is obvious that the first one with a common PTW and eDRX cycle is a simple way to minimize the complexity, and it could be used as a start point for the discussion.

Proposal 2:  A common PTW and eDRX cycle for RRC_Idle and RRC_Inactive could be used as a start point for RedCap eDRX mechanism design.

However, if Proposal 1 is adopted, as the description in [2], gNB could configure different eDRX cycle length in RRC_Inactive based on the UE capability. Besides, considering the difference between RRC_Idle and RRC_Inactive, maybe a different eDRX cycle length is more suitable, for example, a shorter eDRX cycle for small data transmission.

Observation 3: Shorter eDRX cycle may be more suitable to small data transmission.

Regarding to PTW, considering the UE power saving, it’s better for PTWs for RAN paging and CN paging to overlap as much as possible.

Proposal 3：Overlapping PTW but with different eDRX cycle could be considered.

Conclusion

In this contribution, the discussion on the eDRX for RedCap is provided. Following are our observations:

Observation 1: In LTE, CN may pass UE idle mode eDRX cycle length value to RAN node.

Observation 2: In LTE, it’s RAN node to decide the eDRX cycle in RRC_Inactive, which is up to the value for the UE's idle mode eDRX cycle or 10.24 seconds.

Observation 3: Shorter eDRX cycle may be more suitable to small data transmission.

Based on the observations, we propose:

Proposal 1：The current mechanism should be reused for RedCap UE’s eDRX configuration decision, i.e.,

- gNB could be responsible for the eDRX cycle configuration in RRC_Inactive, based on the information provided by CN in "RRC Inactive Assistance Information".

Proposal 2:  A common PTW and eDRX cycle for RRC_Idle and RRC_Inactive could be used as a start point for RedCap eDRX mechanism design.

Proposal 3：Overlapping PTW but with different eDRX cycle could be considered.

Reference

[1] RP-210918 Revised WID on support of reduced capability NR devices, Nokia, Ericsson

[2] 3GPP TS 23.501 V16.4.0 (2020-03) System architecture for the 5G System (5GS); stage 2; Release 16

[3] R2-2102056 3GPP TR 38.875 V1.0.0 (2020-12) 3GPP TR 38.875 V1.0.0 (2020-12); Release 17