3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2103726

Electronic Meeting, 12th Apr – 20th Apr 2021

Agenda item:	8.16.2

Source:	CMCC

Title:	Left Issues and Analysis on LS from SA2 on Supporting SNPN with Credentials by a Separate Entity

Document for:	Discussion

Introduction

In last SA2#143-e meeting, SA2 sent a reply LS to RAN in S2-2101076 LS on clarification request for eNPN features [1] to provide the answers on the exact questions from that RAN2. And the answers to questions related to SNPNs with subscription or credentials by a separate entity are as follows:

Questions related to SNPNs with subscription or credentials by a separate entity: Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area? [SA2 answer] Yes, these parameters should be set uniformly per SNPN. Question 2: Shall Group IDs be broadcasted per SNPN or per cell? [SA2 answer] Yes, It is assumed that that the Group IDs will be broadcast per SNPN

And in last RAN2#113-E meeting, the following conclusion on support SNPN along with credentials owned by an entity separate from the SNPN are as follows:

A new indicator that "access using credentials from a separate entity is supported" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenarios. RAN2 assumes that the new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1. The supported Group IDs are broadcasted A new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenario. RAN2 assumes that the new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1. In the UE, AS reports to NAS about the following broadcasted new parameters: - Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN - Supported Group IDs - Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN. Send an LS to SA2 (CC: RAN3 and CT1) with the following questions: - Can RAN2 assume uniform support of GID(s) across a network or a registration area? - Is the GID selected by NAS given to AS after registration to assist UE subsequence cell selection and reselection? - Should AS support the (IDLE/INACTIVE/CONNECTED mode) mobility scenarios between different SNPNs or SNPN and PLMN when the same credentials can be used on the source and the target networks? E.g. Can a UE move from SNPN#1 to SNPN#2 when the GID used to access SNPN#1 is supported by SNPN#2? Can a UE move between SNPN#1 to PLMN#a when the credential of PLMN#a is used to access SNPN#1? - Shall Group IDs be broadcasted per SNPN? (or per cell?)

This paper discusses the left issues from RAN2 aspects for support of SNPN along with subscription / credentials owned by an entity separate from the SNPN including:

- System information broadcast,

- Associated SNPN selection/cell selection/ cell reselection

Discussion

System Information Broadcast

The enhanced SNPN information with credentials owned by an entity separate from the SNPN is required for cell selection and reselection, and hence it should be included in SIB. In this section, we focus on System information design which allows including the enhanced SNPN information in SI.

As shown above, RAN2 had agreed that the SIB will be enhanced as follows:

- A new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1.

- The supported Group IDs are broadcasted.

- A new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenario in SIB1.

Then the following are the left issues on the system information:

- FFS is the maximum number of Group IDs, and the SIB to be used for broadcasting GIDs.

- Whether there is a need to broadcast Human Readable Network Names for GIDs.

Regarding issue 1:

As we know, in Rel-16, we limited the maximum number of SNPN as 12, as follows:

- Up to 12 different SNPNs can be broadcasted in a cell. - If “mixed” network sharing is allowed (i.e. a cell can contain both PLMNs and NPNs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN) shall not exceed 12.

This restriction discussed in Rel-16 comes from the fact that there are several places in the RRC signalling where the maximum number of networks is set to 12. Hence, we can start from the same restriction applied to the Rel-16 SNPN number was applied to the SNPN. That is, it is proposed that it is up to 12 different Group IDs can be broadcasted in a cell and If “mixed” network sharing is allowed (i.e. a cell can contain PLMNs, SNPNs/Group IDs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN+ #Group IDs) shall not exceed 12. If the future Requirements/market need more size of Group IDs and cause the increase of the overhead substantially, the extension can be performed in the future release.

Proposal 1: Up to 12 different Group IDs can be broadcasted in a cell.

Proposal 2: If “mixed” network sharing is allowed (i.e. a cell can contain PLMNs, SNPNs/Group IDs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN+ #Group IDs) shall not exceed 12.

Based on above number limitation, and considering that this indicate will affect the network/ cell selection, we prefer to include it in the SIB1.

Proposal 3: it is prefer to add the Group IDs in the SIB1.

Below is a SIB structure which allows to include enhanced SNPN information inside Rel-16 IE NPN-IdentityInfoList-r16 of SIB1. Since the new additional IEs are in SNPN granularity, it is proposed to add the new IEs inside NPN-IdentityInfo-r16 as below. It requires introduction of Rel-17 IEs which are shown in red.

Regarding the issue #2:

Additionally, supported Group IDs (GIDs)’s Human Readable Network Name is also needed in SIB for cell manual selection.

Proposal 4: it is proposed to add the new IEs inside SIB10 for cell manual selection:

-	Optionally, supported Group IDs (GIDs)’s Human Readable Network Name

SNPN selection/ Cell selection/ Cell re-selection

SNPN selection in case of SNPN along with credentials owned by an entity separate from the SNPN is similar to normal SNPN selection and both manual and automatic network selection is supported, as showed in the SA2 conclusions.

Observation 1: Both automatic and manual PLMN selection in relation to SNPN along with credentials owned by an entity separate from the SNPN are agreed in SA2.

As shown above, RAN2 had agreed that the SIB will be enhanced as follows:

- In the UE, AS reports to NAS about the following broadcasted new parameters:

  - Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN

  - Supported Group IDs

Moreover, in case of manual network selection, the human readable network name (if broadcasted) of the SNPN will also be provided from AS to NAS to assist the user.

Proposal 5: In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS, on top of the agreed content.

In Rel-16, since a UE can only register to an SNPN if the UE has SUPI and credentials for the given SNPN, i.e., a UE cannot register to an SNPN with a credential of another (roaming partner's) credential, there is no roaming use case. This means the selected and registered SNPN is always the "home" SNPN. Therefore, there is no need for the UE to search for higher priority SNPNs.

However, in Rel-17, in case of SNPN along with credentials owned by an entity separate from the SNPN, the SNPN will host a function (e.g. enhanced AUSF or new NF) supporting primary authentication and authorization of SNPN UEs that use credentials from the AAA Server. In the scenario, it is similar as in roaming use case, where the SNPN for which the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination) and which supports access using credentials (Home SP credentials) owned by an entity separate from the SNPN can be regarded as Visited SNPN. Hence, there is a need for the UE to search for higher priority SNPNs according to order indicated in TR 23.700 section 8.1.

# Figure 1: Architecture in case separate entity that owns the subscription

Observation 2: since roaming use case( visited SNPN) exists in case of SNPN along with credentials owned by an entity separate from the SNPN, there is a need for the UE to search for higher priority SNPNs according to order indicated in TR 23.700 section 8.1.

For manual network selection UEs operating in visited SNPN access mode provide to the user the list of NIDs and related human-readable names (if available) of the available visited SNPNs the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination), and optionally the name of the respective entity separate from the SNPN owning the credentials (e.g. Home SP credentials) which can be used to access the visited SNPN if exists. This requires the AS to provide the advertised human-readable names of the available visited SNPNs the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination), and optionally the name of the respective entity separate from the SNPN owning the credentials (e.g. Home SP credentials) which can be used to access the visited SNPN if exists to NAS.

Observation 3: The AS shall provide human-readable names of the available visited SNPNs the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination), and optionally the name of the respective entity separate from the SNPN owning the credentials (e.g. Home SP credentials) which can be used to access the visited SNPN if exists to NAS for manual network selection whenever it is requested by NAS.

Proposal 6: Besides the human-readable names , it is proposed that the AS shall provide the name of the respective entity separate from the SNPN owning the credentials (e.g. Home SP credentials) which can be used to access the visited SNPN if exists to NAS for manual network selection whenever it is requested by NAS, if exits.

In Rel-16 NR, the suitability criteria are as follow:

- suitable cell: … For UE operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled: - The cell is part of either the selected SNPN or the registered SNPN of the UE; - The cell selection criteria are fulfilled, see clause 5.2.3.2; According to the latest information provided by NAS: - The cell is not barred, see clause 5.3.1; - The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas for Roaming" which belongs to either the selected SNPN or the registered SNPN of the UE.

The table below shows the suitability of a cell based on the conclusion from SA2:

|  | SNPN cell | SNPN cell where the UE’s credentials owned by an entity separate from the SNPN (Visited SNPN cell) |
| --- | --- | --- |
| Rel-16 UE that is operating in SNPN access mode (SNPN UE) | Suitable if SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials | Not Suitable if the SNPN for which the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination) |
| Rel-17 UE that configured: - User-controlled prioritized list of preferred SNPNs - Separate entity controlled prioritized list of preferred SNPNs - Separate entity-controlled prioritized list of Group IDs (GIDs) | Suitable if SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials | Suitable if where the SNPN for which the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination) and which supports access using credentials (Home SP credentials) owned by an entity separate from the SNPN |
| Rel-17 UE that UEs with a PLMN subscription | Not Suitable | Suitable if where the SNPN for which the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination) and which supports access using credentials (Home SP credentials) owned by an entity in PLMN separate from the SNPN |
| UE that can access PLMN and CAG cell | Not suitable | Not suitable |

Based on above analysis, for Rel-17 UE which supports accessing SNPN cell where the UE’s credentials owned by an entity separate from the SNPN, it also requires the following condition to consider a cell as suitable during SNPN selection in NAS layer:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

- The cell is broadcasting "access using credentials from a separate entity is supported" indication and the broadcasted SNPN ID(s) which is part of the SNPN ID(s) contained in the user-controlled list  of the UE;

- The cell is broadcasting "access using credentials from a separate entity is supported" indication and the broadcasted SNPN ID(s) which is part of the SNPN ID(s) contained in the separate entity-controlled list  of the UE;

- The cell is broadcasting "access using credentials from a separate entity is supported" indication and the broadcasted Group IDs (GIDs) which is part of the Group IDs (GIDs) contained in the separate entity-controlled list  of the UE;

- The cell is broadcasting " the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN " indication;

- The cell selection criteria are fulfilled, see clause 5.2.3.2 in TS 38.304;

Proposal 7: supporting SNPN with subscription or credentials by a separate entity has impacts on SNPN selection in NAS layer, but will impact on the definition of suitable cell.

Conclusions

Observation 1: Both automatic and manual PLMN selection in relation to SNPN along with credentials owned by an entity separate from the SNPN are agreed in SA2.

Observation 2: since roaming use case( visited SNPN) exists in case of SNPN along with credentials owned by an entity separate from the SNPN, there is a need for the UE to search for higher priority SNPNs according to order indicated in TR 23.700 section 8.1.

Observation 3: The AS shall provide human-readable names of the available visited SNPNs the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination), and optionally the name of the respective entity separate from the SNPN owning the credentials (e.g. Home SP credentials) which can be used to access the visited SNPN if exists to NAS for manual network selection whenever it is requested by NAS.

Proposal 1: Up to 12 different Group IDs can be broadcasted in a cell.

Proposal 2: If “mixed” network sharing is allowed (i.e. a cell can contain PLMNs, SNPNs/Group IDs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN+ #Group IDs) shall not exceed 12.

Proposal 3: it is prefer to add the Group IDs in the SIB1.

Proposal 4: it is proposed to add the new IEs inside SIB10 for cell manual selection:

-	Optionally, supported Group IDs (GIDs)’s Human Readable Network Name

Proposal 5: In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS, on top of the agreed content.

Proposal 6: Besides the human-readable names of a visited SNPN, it is proposed that the AS shall provide the name of the respective entity separate from the SNPN owning the credentials if exists to NAS for manual network selection.

Proposal 7: supporting SNPN with subscription or credentials by a separate entity has impacts on SNPN selection in NAS layer, but will impact on the definition of suitable cell.

References

- S2-2101076 LS OUT	Approval	[DRAFT] Reply LS on clarification request for eNPN features	Nokia, Nokia Shanghai Bell