3GPP TSG-RAN WG2#113-e	R2-2103765

Electronic meeting, 12th– 20th April 2021

Agenda Item:	5.4.3

Source:	Ericsson

Title:	Transparent TxD Capability and Signaling

Document for:	Discussion, Decision

# 1	Introduction

RAN2 received an LS in [1] on a new RAN4 UE capability for transparent TxD. The LS informs RAN2:

RAN4 has agreed to introduce a new per-band capability signaling in Rel-16 for FR1 UEs supporting transparent TxD.

RAN4 would also like to ask RAN2 to enable release-independent support of this new capability from Rel-15 for PC2, if possible.

In this paper, we consider how this capability relates to other capabilities, and additional information needed to develop capability signaling for transparent TxD.

Since RAN4 is asking for the new capability starting from Rel-15 for power class 2, the discussion should start with Rel-15.

# 2	Discussion

When power class with two 2 layer UL MIMO was introduced in RAN4 specifications, it was agreed that two 23 dBm (half power) PAs could be assumed as a UE implementation.  When such a UE transmits a MIMO layer on each Tx chain at half power, the rated 26 dBm power is achieved.  However, when this UE transmits one layer, if it only transmits on one Tx chain, it will have only half the rated power.  One way to reach full power is to instead transmit one layer on both Tx chains, virtualizing the two Tx chains to one antenna port.  If such a virtualization is done so that the resulting antenna port is indistinguishable from a port transmitted on a single Tx chain, it is ‘transparent’ to the gNB.  Furthermore, transmitting on two Tx chains may have diversity gain, in which case a transparent diversity scheme results.

Since it was assumed in codebook based UL MIMO that the UE transmits with less than full power for certain PMIs or on when the UE is configured with fewer than its maximum number of SRS ports, uplink full power MIMO transmission was defined in Rel-16.  These full power modes can also use the virtualization that transparent TxD uses to form a DMRS port.  Moreover, these full power modes can be seen as a more formal approach to obtaining full power as compared to transparent TxD, since they can be configured, and have their behavior defined in the RAN1 specifications.   Without signaling to configure the use of transparent TxD, its use is left to UE implementation, and the network is unaware of if the UE will use transparent TxD.

UEs that support UL full power are likely to be able to support transparent TxD, and the converse is also possible.  When such a UE that supports transparent TxD and full power operation is not configured for full power operation in a band that it supports transparent TxD, it is not clear if it will use transparent TxD operation in that band.

UL MIMO UEs may also support transmission of SRS for DL CSI acquisition (SRS ‘antenna switching’).  If a UE that supports SRS antenna switching and transparent TxD is configured for e.g. 1T2R antenna switching, it should not virtualize the SRS, but instead transmit on each 'receive’ chain.  If a transparent TxD UE supports full power on both Tx chains, it will not need to virtualize to reach full power.  However, if the UE has half power Tx chains, it may not be able to deliver full power, and so could not support SRS antenna switching in a band where transparent TxD is supported.  Therefore, it is not clear if indicating that transparent TxD in a band precludes support for SRS antenna switching in that band or not.

In a band where transparent TxD is supported, it is unclear if the UE can also support multi-antenna related features, such as UL full power operation (mode 0, 1, or 2) and antenna switching SRS transmission

We expect such issues as interdependence of transparent TxD with other multi-antenna features will be resolved as the discussions progress in RAN4.  However, it is difficult to define UE capability for transparent TxD until such issues are resolved.

As RAN4 may not be aware of all RAN2 perspectives on UE capability, it may be beneficial to send an LS reply asking for further information.  On the other hand, such an LS may not be critical, as RAN4 anyway is still developing the feature, and interaction among UE capabilities is a natural issue to resolve when developing a new feature.   Therefore, we tend to favor sending a reply LS, but do not have a strong view.  Regardless of if the LS is to be sent, work on transparent TxD capability should be started only when it is clear what to specify.

Wait for further input from RAN4 prior to specifying UE capability for transparent TxD.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In a band where transparent TxD is supported, it is unclear if the UE can also support multi-antenna related features, such as UL full power operation (mode 0, 1, or 2) and antenna switching SRS transmission

Based on the discussion in the previous sections we propose the following:

Proposal 1	Wait for further input from RAN4 prior to specifying UE capability for transparent TxD.

# 4	References

R4-2103360  	LS on Signalling scheme of Transparent TxD