3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103783

Online, April 12 – April 20, 2021

Agenda Item:	8.12.3.1

Souce:	MediaTek Inc.

Title:	Further considerations for eDRX

Document for:	Discussion and decision

# 1 Introduction

The work item description for support of reduced capability NR devices has captured the following objectives for Extended DRX enhancements [1]:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4.

In this contribution, we discuss some further considerations for the support of eDRX in RedCap aligned with the above objectives.

# 2 Discussion

## 2.1 PH/PTW calculations for eDRX

It was agreed during the RedCap study item that, if the eDRX cycle is larger than 10.24 s, the eDRX mechanism will include the use of H-SFN, PH and PTW, similar to LTE [2].

In LTE and NB-IoT, the paging hyper-frame (PH) is the H-SFN satisfying the following equation [3]:

| H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H), where | Eq 1 |
| --- | --- |

-	UE_ID_H:

- 10 most significant bits of the Hashed ID, if P-RNTI is monitored on PDCCH or MPDCCH

- 12 most significant bits of the Hashed ID, if P-RNTI is monitored on NPDCCH

-	T eDRX,H : eDRX cycle of the UE in Hyper-frames, (TeDRX,H =1, 2, …, 256 Hyper-frames) (for NB-IoT, TeDRX,H =2, …, 1024 Hyper-frames) and configured by upper layers.

PTW_start, the first radio frame of the PH that is part of the PTW, has SFN satisfying the following equation:

| SFN = 256* ieDRX, where | Eq 2 |
| --- | --- |

-	ieDRX = floor(UE_ID_H /TeDRX,H) mod 4

PTW_end, the last radio frame of the PTW, has SFN satisfying the following equation:

| SFN = (PTW_start + L*100 - 1) mod 1024, where | Eq 3 |
| --- | --- |

-	L = Paging Time Window length (in seconds) configured by upper layers

It can be observed from the formulas that, from the network point of view, an eDRX cycle consists of a number of PTWs offset from each other by 256 SFNs. The UEs are then distributed uniformly between the PTWs. However note that this is based on the assumption that the same eDRX configuration is applied to all UEs. In reality, there can be differences between the eDRX configurations of the UEs, which means a completely uniform distribution of UEs may not always be achieved.

An example configuration with TeDRX=20.48 s and L=5.12 s can be illustrated in Figure 1 below.

Figure 1 Example with TeDRX=20.48 s and L=5.12 s for different UEs

The maximum value for the eDRX cycle and the number of possible PTW_start offsets in a hyper-frame dictate the number of bits of the UE identity (e.g. Hashed ID) that is used in the formulas. For example, to support a maximum eDRX cycle of 10485.76 s 10 bits are needed (to cover 1024 hyper-frames), and additionally to support 4 starting offsets 2 more bits are needed, therefore in total 12 bits of UE identity are used in the eDRX formulas for NB-IoT (i.e. NPDCCH case).

The range for the PTW length (L) should also be defined for RedCap. In LTE, PTW length can take values between 1.28 s and 20.48 s for eMTC and 2.56 s and 40.96 s for NB-IoT. For RedCap, due to the wide range of scenarios and use cases, we can use the combined range of LTE/NB-IoT, i.e. 1.28 s - 40.96 s. This can cover the use cases where minimizing the delay for the paging response is important, such as wearables, as well as the use cases where the power savings is a critical factor, such as industrial sensors. The exact values within this range can be discussed further.

Proposal 1: The minimum and maximum PTW length in RedCap is 1.28 s and 40.96 s respectively. The exact values within this range are FFS.

Fixing the number of possible PTW_start positions in a hyper-frame to 4 (i.e. 256 SFNs between each other) introduces some limitations for the network configuration of eDRX for multiple UEs. For example, for small values of L (1.28 s), there may be time gaps where no UE can be paged by the network. This can be illustrated in Figure 2 below.

Figure 2 Example with TeDRX=20.48 s and L=1.28 s for different UEs

As an enhancement, the offset between PTW_start positions can be configurable by the network, and the formulas above can be updated accordingly. This can enable the network to schedule the paging cycles for multiple UEs in a more flexible and efficient manner. Note that this will have an effect on the number of UE_ID_H bits used in the formulas (for example, if the offset is 128 SFN instead of 256 SFN, one more bit of UE_ID_H will be used in the formulas).

Alternatively, the offset between the starting positions for PTW_start could be reduced to 128 SFNs. This will require one more bit of UE_ID_H to be used in the PH/PTW calculations.

Proposal 2: RAN2 to select from the following options regarding the PTW_start positions:

1) Configurable by the network (i.e. not fixed to multiples of 256 SFNs as in LTE)

2) Fixed to multiples of 128 SFNs.

3) Fixed to multiples of 256 SFNs as in LTE (at a potential cost of gaps between PTWs).

For determining the UE specific eDRX cycle in LTE, when the UE is connected to 5GC, a hashed UE ID based on 5G-S-TMSI is used. The main reasons for using a Hashed ID rather than the S-TMSI itself are randomness and security. The UE ID used for determining the PFs/POs during the PTW should be different from the UE ID used for determining the PH/PTW_start. Otherwise, only a subset of PFs will be used in a hyper-frame. On the other hand, using too many bits of S-TMSI or IMSI was considered as a security risk. Therefore, a Hashed ID that is based on an FCS generated from S-TMSI was used.

We think that the Hashed UE ID calculation in LTE should be re-used for RedCap, which is already supported by 5GC.

Proposal 3: The Hashed UE ID calculation from LTE (TS 36.304) is re-used in eDRX calculations.

## 2.2 Other operational aspects

### 2.2.1 Indication of eDRX allowed in the cell

In LTE, whether the eDRX is allowed in a cell or not is indicated by the eDRX-Allowed flag carried in SIB1. The UE may operate in extended DRX only if the UE is configured by upper layers and the cell indicates support for eDRX in System Information. The UE stops using extended DRX if eDRX-Allowed is not present after cell reselection. On the other hand, in NB-IoT eDRX support is mandatory in the network and therefore a separate flag was not necessary.

For RedCap, we think that not all NR cells (deployments) may support eDRX. Forcing all RedCap cells to support eDRX would limit the deployment options. Therefore a flag in system information should be used to indicate whether a cell supports eDRX or not.

Proposal 4: Whether a RedCap cell supports eDRX operation or not is indicated by a flag (e.g. eDRX-Allowed) in SIB1.

### 2.2.2 System information modification

In LTE and NR, when there is a change in system information, an SI change indication is transmitted using a short message addressed to the P-RNTI. Updated SI message (other than SI message for ETWS, CMAS and positioning assistance data) is broadcast in the modification period following the indication.

However, if the eDRX cycle is longer than the modification period, the UE can miss the indication and may not use the updated SI. In LTE, some enhancements were done to resolve this issue. A new eDRX acquisition period was introduced. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. If the eDRX cycle is greater than the modification period, a new SI modification for eDRX indication is transmitted in the Short Message and the UE acquires the updated SI during the next eDRX acquisition period.

We think that this mechanism can be re-used for eDRX in RedCap and it is relatively straight forward to apply the necessary specification changes using LTE as baseline.

Proposal 5: SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX).

# 3 Conclusion

In this contribution, we discuss further considerations for eDRX support in RedCap and have the following proposals:

Proposal 1: The minimum and maximum PTW length in RedCap is 1.28 s and 40.96 s respectively. The exact values within this range are FFS.

Proposal 2: RAN2 to select from the following options regarding the PTW_start positions:

1) Configurable by the network (i.e. not fixed to multiples of 256 SFNs as in LTE)

2) Fixed to multiples of 128 SFNs.

3) Fixed to multiples of 256 SFNs as in LTE (at a potential cost of gaps between PTWs).

Proposal 3: The Hashed UE ID calculation from LTE (TS 36.304) is re-used in eDRX calculations.

Proposal 4: Whether a RedCap cell supports eDRX operation or not is indicated by a flag (e.g. eDRX-Allowed) in SIB1.

Proposal 5: SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX).

# 4 References

- RP-210918 - Revised WID on support of reduced capability NR devices

- TR 38.875 - Study on support of reduced capability NR devices

- TS 36.304 User Equipment (UE) procedures in idle mode