3GPP TSG-RAN WG2 #113bis-e	R2-2103790

Electronic meeting, Apr 12 – Apr 20, 2021

Agenda Item:	5.4.1.1

Source:	ZTE Corporation, Sanechips

Title:	Discussion on the release of active BWP

Document for:	Discussion, Decision

# 1	Introduction

In the last e-meeting, the following proposals were postponed:

------------------

Postpone P9 P11

- P9: whether the NW can release the active BWP for SpCell using RRC, and if allowed, whether the NW should always provide the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in the same RRC message.

- 11: The active BWP of an SCell cannot be released by RRC message.

--------------------

In this paper, we discuss the above issues and provide proposals.

# 2	Discussion

First, when the netwok decides whether or not to provide the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in the RRCReconfiguration message, the network knows which BWP is the UE’s current active BWP even the BWP has been switched via DCI before, because the field masterCellGroup including firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is provided by DU, and DCI-based BWP switch is also performed by DU.

Second, it is noted that when the netwok includes elements of ToAddModList and ToReleaseList in a single RRC message, the UE shall perform the ReleaseList prior to the AddModList. It is specified in 38331 as below.

=========================

If no procedural text is provided for a set of ToAddModList and ToReleaseList, the following generic procedure applies:

The UE shall:

1>	for each ElementId in the elementsToReleaseList,:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	release the Element from the current UE configuration;

1>	for each Element in the elementsToAddModList:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	modify the configured Element in accordance with the received Element;

2>	else:

3>	add received Element to the UE configuration.

==========================

We think the network can release the active BWP for SpCell using RRC, because the action is allowed by ASN code. But whether the NW should provide the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in the same RRC message in this case, the following cases are considered:

Case 1: The UE is configured with two DL dedicated BWPs (such as BWP-ID 1 and BWP-ID 2), and the current active DL BWP is the BWP with ID 2. If the network includes downlinkBWP-ToReleaseList to release BWP-ID 2, and simultaneously includes downlinkBWP-ToAddModList to add a new DL BWP identified by BWP-ID 2 in the same RRCReconfiguration message, the network may not provide the firstActiveDownlinkBWP-Id in the same RRC message. In this case, if the network do not provide the firstActiveDownlinkBWP-Id, the UE can continue to use the DL BWP with ID 2 after processing the RRCReconfiguration message. And the same actions are applicable to UL BWP. There is no ambiguity. So we give the following proposal.

Proposal 1: For SpCell, if network release the UE’s active BWP, and add a new BWP with the same BWP ID in a single RRC reconfiguration message, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is optionally present in the same RRC message.

Case 2: The UE is configured with two DL dedicated BWPs (such as BWP-ID 1 and BWP-ID 2), and the current active DL BWP is the BWP with ID 2. If the network includes downlinkBWP-ToReleaseList to release BWP-ID 2, and not includes downlinkBWP-ToAddModList in the same RRCReconfiguration message, the network must provide the firstActiveDownlinkBWP-Id in the same RRC message if the BWP-ID 2 was the firstActiveDownlinkBWP-ID or the firstActiveUplinkBWP-ID prior to the reconfiguration, otherwise (i.e. BWP ID 1 was the first active BWP) the network need not provide the first active BWP ID as it is not changed. Based on this, we propose the following:

Proposal 2: For SpCell, if network releases the UE’s active BWP, and does not add a new BWP with the same BWP ID in a single RRC reconfiguration message, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is mandatory present if the released BWP ID corresponds to the firstActiveDownlinkBWP-ID and/or the firstActiveUplinkBWP-ID prior to the reconfiguration. Otherwise, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id are optional in the same RRC message.

Case 3: The UE is configured with two DL dedicated BWPs (such as BWP-ID 1 and BWP-ID 2), and the current active DL BWP is the BWP with ID 1. If the network only want to release the DL BWP with ID 1, it is unallowed to only include downlinkBWP-ToReleaseList to release BWP-ID 1, because it is specified in spec 38331 that ‘The network configures the BWPs with consecutive IDs from 1’. So the correct actions are that the network includes downlinkBWP-ToReleaseList to release BWP-ID 1 and BWP-ID 2, and simultaneously includes downlinkBWP-ToAddModList to add a new BWP identified by BWP-ID 1 (i.e. which can be understood as the network copied parameters from the old BWP with ID 2 to the new BWP with ID 1) in the same RRCReconfiguration message. In this case, the network may not provide the firstActiveDownlinkBWP-Id in the same RRC message, because the UE’s active DL BWP is still the BWP with ID 1 after processing the RRCReconfiguration message. And the same actions are applicable to UL BWP. There is no ambiguity, similarly as Case 1.

For SCell, because the field firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is mandatory present upon SCell addition, and absent in other cases, the network cann’t include firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in RRC message except SCell additon case. So when the network release the UE’s current active BWP, the network must add a new BWP with the same BWP-ID in a single RRC message for SCell. we give the following proposal.

Proposal 3: For SCell, if network release the UE’s active BWP, the network must add a new BWP with the same BWP ID in a single RRC reconfiguration message.

# 3	Summary

Based on the discussion above we propose the following proposals:

Proposal 1: For SpCell, if network release the UE’s active BWP, and add a new BWP with the same BWP ID in a single RRC reconfiguration message, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is optionally present in the same RRC message.

Proposal 2: For SpCell, if network releases the UE’s active BWP, and does not add a new BWP with the same BWP ID in a single RRC reconfiguration message, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is mandatory present if the released BWP ID corresponds to the firstActiveDownlinkBWP-ID and/or the firstActiveUplinkBWP-ID prior to the reconfiguration. Otherwise, the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id are optional in the same RRC message.

Proposal 3: For SCell, if network release the UE’s active BWP, the network must add a new BWP with the same BWP ID in a single RRC reconfiguration message.

# 4 Reference

[1] Draft_R2-113-e_Meeting_Report_v1

[2] R2-2102293-[AT113-e][005][NR15] Connection Control II (Apple)_Phase-1-summary