3GPP TSG RAN WG2 #113bis-e			R2-2103844

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.16.3

Source:	Apple

Title:	On the need for additional on-boarding options in eNPN

Document for:	Discussion/Decision

# Introduction

As part of Rel-17 eNPN WI, the following agreements regarding the on-boarding of UEs were made in RAN2#113-e [2].

Broadcast a 1-bit indication for onboarding per O-SNPN.

R2 assumes that the 1-bit indication for onboarding is in SIB1.

The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

Additionally, for a 3rd party authentication server, we note the relevant agreements for this discussion.

A new indicator that "access using credentials from a separate entity is supported" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenarios.

RAN2 assumes that the new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1.

A new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenario.

RAN2 assumes that the new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1.

In the UE, AS reports to NAS about the following broadcasted new parameters: Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN

- Supported Group IDs

- Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN.

In this discussion, we request RAN2 to look at additional indications that are needed in order for such an SNPN on-boarding scenario and solution to be successful.

# Discussion

From on-boarding perspective additional use cases such as – if on-boarding is supported for all UEs or only a subset of UEs is needed similar to the 3rd party authentication mechanism options of group IDs. An additional option needed is for UEs which want to on-board using an authentication mechanism of a specific vendor and not with any 3rd party vendor authentication servers. Also, solutions related to SIM modifications with O-SNPN tags are also being discussed in CT1. For all these additional situations, the 1-bit agreements in [1] are severely restrictive. Given such additional criteria we have the following proposals -

Proposal 1: RAN2 to add additional tags to O-SNPNs if on-boarding is supported for all or selected UEs.

Proposal 2: RAN2 to discuss with an LS to CT1 if UE can on-board using a vendor specific 3rd party authentication server.

Proposal 3: RAN2 to discuss additional indication mechanism beyond just the 1-bit SIB for the new SNPN scenarios proposed.

# Conclusion

In this contribution, we provided our views on the eNPN on-boarding procedures. proposals are as follows:

Proposal 1: RAN2 to add additional tags to O-SNPNs if on-boarding is supported for all or selected UEs.

Proposal 2: RAN2 to discuss with an LS to CT1 if UE can on-board using a vendor specific 3rd party authentication server.

Proposal 3: RAN2 to discuss additional indication mechanism beyond just the 1-bit SIB for the new SNPN scenarios proposed..

# References

- RAN3#113-e, Chairman’s notes, Jan.. 2021.