3GPP TSG-RAN WG2 #113bise	R2-2103920

Online Meeting, 12th – 20th April 2021

Agenda Item:	6.3.2

Source:	Ericsson

Title:	Summary for RRC Corrections for Positioning

Document for:	Discussion, Decision

# Introduction

The below papers have been submitted in the RRC AI 6.3.2

R2-2102924 Corrections on the description of SRS-Config CATT CR Rel-16 38.331 16.4.1 2490 - F NR_pos-Core R2-2103849 Correction on the SI offset usage of posSI Scheduling Apple CR Rel-16 38.331 16.4.1 2539 - F NR_pos-Core R2-2103919 Same posSIB-Type in multiple SI messages Ericsson discussion R2-2104175 Correction on posSI-RequestConfig and posSI-RequestConfigSUL field description Samsung R&D Institute UK CR Rel-16 38.331 16.4.1 2559 - F NR_pos-Core

# Discussion

## R2-2102924	Corrections on the description of SRS-Config

SRS-Config in TS 38.331 lacks the description of positioning reference signal. The CR provides the SRS-Config IE names and appends the impacted fields and other description which has also positioning impacts.

From rapporteur point of view, the CR is needed. One editorial suggestion would be to use “comma” instead of slash “/”. Example below.

SRS-ResourceSet, SRS-PosResourceSet field descriptions

RAN2 to agree CR R2-2102924 for the corrections of description of SRS-Config to reflect positioning impacts.

## R2-2103849	Correction on the SI offset usage of posSI Scheduling

The CR aims to say that the current SI offset is applicable for all posSI and not applicable per SI. Further, it has been mentioned that in LTE the design does not allow to use it per posSI and hence it should be same also in NR.

From previous discussions in LTE and from rapporteur point of view, yes it was intened to have the offset to be applciable for the whole list. Thus, the clarification is needed. However, it is not needed to specify that ”If SI offset is used, this field needs to be present in each of the SI messages in the posSchedulingInfoList. “ This is clear from the addition of “all”: offsetToSI-Used

This field, if present indicates that all the SI messages.

Hence, the flag can be set for the first SI message and if it is set, it would imply it is applicable for the whole list.

RAN2 to discuss CR R2-2103849 and agree to clarify that SI offset is applicable for all the SI in the posSISchedulingInfoList.

## R2-2103919	Same posSIB-Type in multiple SI messages

Currently in RRC specification, the below description has been captured for SIBs.

-	The mapping of SIBs to SI messages is configured in schedulingInfoList, while the mapping of posSIBs to SI messages is configured in pos-SchedulingInfoList. Each SIB is contained only in a single SI message and each SIB and posSIB is contained at most once in that SI message;

There has been questions as why posSIBs have not been included. Thus, some clarification is needed as why posSIBs has this exception. The discussion paper provides the reason as why it is so and provides changes in this part to illustrate that a single posSIB type might be used several times in the system;

RAN2 to discuss and agree to clarify as why each posSIB cannot be contained only in a single SI message.

## R2-2104175	Correction on posSI-RequestConfig and posSI-RequestConfigSUL

The CR adds below clarification:

posSI-RequestConfig Configuration of Msg1 resources that the UE uses on normal uplink for requesting SI-messages for which posSI-BroadcastStatus is set to notBroadcasting.

posSI-RequestConfigSUL Configuration of Msg1 resources that the UE uses on supplementary uplink for requesting SI-messages for which posSI-BroadcastStatus is set to notBroadcasting.

However, this has been taken from the existing field description as below

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

If any change is done for positioning than the same change should be also done for the legacy case.

The CR to clarify posSI-RequesConfig is for normal UL or Supplementary Uplink is not agreed as it requires similar change also for legacy field description

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to agree CR R2-2102924 for the corrections of description of SRS-Config to reflect positioning impacts.

Proposal 2	RAN2 to discuss CR R2-2103849 and agree to clarify that SI offset is applicable for all the SI in the posSISchedulingInfoList.

Proposal 3	RAN2 to discuss and agree to clarify as why each posSIB cannot be contained only in a single SI message.

Proposal 4	The CR to clarify posSI-RequesConfig is for normal UL or Supplementary Uplink is not agreed as it requires similar change also for legacy field description

# References

[1] AI 6.3.2