3GPP TSG-RAN WG2 #113e	R2-2103928

Electronic meeting, 2021-01-25 - 2021-02-05

Agenda Item:	8.17

Source:	Ericsson

Title:	Discussion on Capturing PDCP Impacts for User Plane Integrity Protection

Document for:	Discussion, Decision

# 1	Introduction

SA3 are discussing addition of UP IP support for option 1 (LTE connected to EPC) and option 3 (EN-DC) [1]. In this contribution we highlight the RAN2 aspects and impact of this topic.

# 2	Discussion

Integrity protection is supported in LTE for control plane, but today it is not supported for user plane traffic. On the NR- however, integrity protection is supported both for Control Plane and User Plane.

SA3 are working on enabling integrity protection also for user plane in LTE, i.e. in architecture option 1 and option 3. One aspect of this work is: which PDCP version should be used? We see these two options:

- use LTE PDCP

- use NR PDCP

With option 1, the RAN2 impact would be that we have to add support of integrity protection for user plane. That would require at least updates to LTE PDCP specification, and updates to LTE RRC to allow configurability of integrity protection in the LTE PDCP configuration.

With option 2, user plane integrity protection is already supported for NR PDCP meaning that no major change to the NR PDCP specification is foreseen. However, there is a restriction NR RRC specification as to when integrity protection can be enabled for user plane, see below. To enable user plane integrity protection in LTE using NR PDCP would hence at least impact the below part of NR RRC specification.

PDCP-Config ::= SEQUENCE { drb SEQUENCE { discardTimer ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} OPTIONAL, -- Cond Setup pdcp-SN-SizeUL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup2 pdcp-SN-SizeDL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup2 headerCompression CHOICE { notUsed NULL, rohc SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15, profiles SEQUENCE { profile0x0001 BOOLEAN, profile0x0002 BOOLEAN, profile0x0003 BOOLEAN, profile0x0004 BOOLEAN, profile0x0006 BOOLEAN, profile0x0101 BOOLEAN, profile0x0102 BOOLEAN, profile0x0103 BOOLEAN, profile0x0104 BOOLEAN }, drb-ContinueROHC ENUMERATED { true } OPTIONAL -- Need N }, uplinkOnlyROHC SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15, profiles SEQUENCE { profile0x0006 BOOLEAN }, drb-ContinueROHC ENUMERATED { true } OPTIONAL -- Need N }, ... }, integrityProtection ENUMERATED { enabled } OPTIONAL, -- Cond ConnectedTo5GC1 statusReportRequired ENUMERATED { true } OPTIONAL, -- Cond Rlc-AM-UM outOfOrderDelivery ENUMERATED { true } OPTIONAL -- Need R } OPTIONAL, -- Cond DRB moreThanOneRLC SEQUENCE { primaryPath SEQUENCE { cellGroup CellGroupId OPTIONAL, -- Need R logicalChannel LogicalChannelIdentity OPTIONAL -- Need R }, ul-DataSplitThreshold UL-DataSplitThreshold OPTIONAL, -- Cond SplitBearer pdcp-Duplication BOOLEAN OPTIONAL -- Need R } OPTIONAL, -- Cond MoreThanOneRLC t-Reordering ENUMERATED { ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20, spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09, spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 } OPTIONAL, -- Need S ..., [[ cipheringDisabled ENUMERATED {true} OPTIONAL -- Cond ConnectedTo5GC ]], [[ discardTimerExt-r16 SetupRelease { DiscardTimerExt-r16 } OPTIONAL, -- Cond DRB2 moreThanTwoRLC-DRB-r16 SEQUENCE { splitSecondaryPath-r16 LogicalChannelIdentity OPTIONAL, -- Cond SplitBearer2 duplicationState-r16 SEQUENCE (SIZE (3)) OF BOOLEAN OPTIONAL -- Need S } OPTIONAL, -- Cond MoreThanTwoRLC-DRB ethernetHeaderCompression-r16 SetupRelease { EthernetHeaderCompression-r16 } OPTIONAL -- Need M ]] } […] Conditional presence; Explanation. ConnectedTo5GC1; The field is optionally present, need R, if the UE is connected to NR/5GC. Otherwise the field is absent..

Regardless of if option 1 or option 2 is adopted, stage 2 descriptions are likely needed as well

We believe that it is a RAN2 decision which of the two options is to be adopted. We therefore suggest that RAN2 should evaluate which of the two options is preferred.

RAN2 should evaluate how to implement user plane integrity protection for option 1 and option 3.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should evaluate how to implement user plane integrity protection for option 1 and option 3.

# References

3GPP TR 33.853 "Integrity protection of the User Plane".