3GPP TSG-RAN WG2 Meeting #113b-e electronic	R2-2104001

# Online, 12 - 20 April 2021

Agenda Item:	6.4.1	CHO/CPC Corrections

Source:	Huawei, HiSilicon, China Telecom

Title:	Discussion on the re-transmission of UL message after CHO execution

Document for:	Discussion and Decision

# 1. Introduction

In this paper, we would like to discuss the following issue:

There are some FFSs for the issue of re-transmission of UAI/SUI message after CHO execution [1].

2. Discussion

In the last RAN2#113-e meeting email discussion [1], we have discussed the mobility issues related to coexistence of CHO and UAI/SUI message, some conclusions have been reached but there are still leftovers some FFS.

R2-2102334 Summary of [AT113-e][015][NR16 V2X MOB DCCA] RRC II (OPPO) OPPO DISCUSSION P4 - CHO preparation is not required to be re-triggered due to the UE information procedure. - If CHO is configured, the UE can assume that the target cell gNB knows about UE information status previously provided in the source cell that was provided there by the UE up to 1s before the UE reception of ConditionalReconfiguration - If the UE information status in the UE has changed since the above point in time, the UE may need to update the UE information to the target cell after HO execution, FFS if the UE can unconditionally always update the target in this case. FFS how to / if to specify.

Based on the above highlighted in yellow, if the UE information (i.e.UAI/SUI) had changed before the last 1 second preceding reception of the RRCReconfiguration message including conditionalReconfiguration, the UE does not need to re-transmit the UE information to the target cell after the CHO execution, because the UE may assume that the target cell gNB knows the information state of the UE at that time.

However, if the UE information status changes during the period from 1 second before the UE receives conditonalReconfiguration to the CHO execution, i.e., the UE initiated the transmission of the UE information during this period, and the UE is still configured to provide the concerned UE information for the corresponding cell group, then the UE needs to re-transmit the UE information to the target cell gNB after the CHO execution. This is because there is no signalling interaction between the source gNB and the target gNB during this period of time. Therefore, the target gNB cannot know the information status change of the UE during this period.

In addition, if the UE initiated one or more UE information transmissions within the time period from 1 second before receiving conditionalReconfiguration to CHO execution, the UE shall re-transmit the latest transmitted UE information to the target cell after the CHO execution. So that the information (i.e. UAI/SUI) about the UE and the target gNB is synchronized.

Proposal 1: If both of the following conditions are met, the UE shall re-transmit the latest transmitted UE information to the target cell after the successful CHO execution:

(1) during the time window “from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution”, the UE should initiate at least one transmission of UE information (i.e.UAI or SUI)

(2) after successful CHO to the target, the UE is still configured with the reporting of the UE information

3. Conclusions

In this paper, we discuss the re-transmission of UL message after CHO execution and it is proposed:

Proposal 1: If both of the following conditions are met, the UE shall re-transmit the latest transmitted UE information to the target cell after the successful CHO execution:

(1) during the time window “from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution”, the UE should initiate at least one transmission of UE information (i.e.UAI or SUI)

(2) after successful CHO to the target, the UE is still configured with the reporting of the UE information

For proposal 1, Draft CRs have been prepared in section 5.

4. References

[1] R2-2102334, Summary of [AT113-e][015][NR16 V2X MOB DCCA] RRC II (OPPO)

5. Appendix

5.1 Draft CR for TS 38.331

3GPP TSG-RAN WG2 Meeting #113 electronic	R2-21xxxxx

Online, Jan 25 – Feb 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the re-transmission of UL message after CHO execution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current text describes the UE should initiate transmission of the following message if the UE transmitted it during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG. - SidelinkUEInformationNR - UEAssistanceInformation For CHO, the UE execues a CHO command in a time period after receiving it. Therefore, to ensure the UE can successfully transmit these messages to the network, if the UE initiated the transmission of the UE information (i.e.UAI or SUI) one or more times during the period from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution, and the UE is still configured to provide the concerned UE information for the corresponding cell group, the UE shall re-transmit the lastest transmitted UE information to the target cell after the CHO execution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.5.3, clarify that the SUI/UAI triggering, if the UE initiated the transmission of the UE information (i.e.UAI or SUI) one or more times during the period from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution, and the UE is still configured to provide the concerned UE information for the corresponding cell group, the UE shall re-transmit the lastest transmitted UE information to the target cell after the CHO execution. Impact analysis Impacted 5G architecutre options: NR SA Impacted functionality: Conditional Handover Inter-operability: The change only impacts the UE. Therefore, no inter-operability issue between the UE and network is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not be able to provide assistance information for the impacted functionalities to the network after CHO execution. In the case that the UE fails to send the assistance information due to CHO execution, the UE may not get its preferred configuration from the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection while timer T311 is running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfig, if any;

<Partially omitted>

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above:

2>	stop timer T304 for that cell group;

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or:

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPC was configured

3>	remove all the entries within VarConditionalReconfig, if any;

3>	for each measId of the source SpCell configuration, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	for the associated reportConfigId:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup; and

2>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second if the RRCReconfiguration message is not applied due to a conditional reconfiguration, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group: or

2>if the UE transmitted one or more times UEAssistanceInformation message for the corresponding cell group in the period between the last 1 second preceding reception of ConditionalReconfiguration and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group:

3>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

3>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

2>	if SIB12 is provided by the target PCell; and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG if the RRCReconfiguration message is not applied due to a conditional reconfiguration: or

2> 	if SIB12 is provided by the target PCell; and the UE transmitted the SidelinkUEInformationNR message one or more times indicating a change of NR sidelink communication related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) in the period between the last 1 second preceding reception of ConditionalReconfiguration of an MCG and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group:

3>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

5.2 Draft CR for TS 36.331

3GPP TSG-RAN WG2 Meeting #113 electronic	R2-21xxxxx

Online, Jan 25 – Feb 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the re-transmission of UL message after CHO execution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current text describes the UE should initiate transmission of the following messages if the UE transmitted it during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo. - SidelinkUEInformation - UEAssistanceInformation - MBMSInterestIndication - InDeviceCoexIndication The current text ensures the UE can successfully transmit these messages above to the network for the case that the UE fails to transmit these during the last 1 second before reception of the RRCConnectionReconfiguration message including mobilityControlInfo. For CHO, the UE execues a CHO command in a time period after receiving it. Therefore, to ensure the UE can successfully transmit these messages to the network,if the UE initiated the transmission of the UE information (i.e.UAI or SUI) one or more times during the period from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution, and the UE is still configured to provide the concerned UE information for the corresponding cell group, the UE shall re-transmit the lastest transmitted UE information to the target cell after the CHO execution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.5.4, clarify that the SUI/UAI triggering, if the UE initiated the transmission of the UE information (i.e.UAI or SUI) one or more times during the period from the last 1 second preceding reception of conditionalReconfiguration to the conditional reconfiguration execution, and the UE is still configured to provide the concerned UE information for the corresponding cell group, the UE shall re-transmit the lastest transmitted UE information to the target cell after the CHO execution. Impact analysis Impacted functionality: Sidelink communication, MBMS service continuity, IDC indication, power preference indication, overheating assistance information Inter-operability: The change only impacts the UE. Therefore, no inter-operability issue between the UE and network is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not be able to provide assistance information for the impacted functionalities to the network after CHO execution. In the case that the UE fails to send the assistance information due to CHO execution, the UE may not get its preferred configuration from the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if daps-HO is not configured for any DRB:

2>	stop timer T310, if running;

2>	stop timer T312, if running;

2>	if timer T316 is running:

3>	stop timer T316;

3>	clear the information included in VarRLF-Report, if any;

2>	resume MCG transmission, if suspended;

<Partially omitted>

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	if daps-HO is configured for any DRB:

3>	stop timer T310, if running;

3>	stop timer T312, if running;

3>	for each DAPS bearer trigger UL data switching, as specified in TS 36.323 [8];

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	if the UE is configured to provide IDC indications:

3>	if the UE has initiated the transmission of an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo if the RRCConnectionReconfiguration message is not applied due to a conditional reconfiguration; or

3>if the UE transmitted one or more times InDeviceCoexIndication message in the period between the last 1 second preceding reception of ConditionalReconfiguration and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group :

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has initiated the transmission of a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has initiated the transmission of a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo if the RRCConnectionReconfiguration message is not applied due to a conditional reconfiguration; or

3>if the UE transmitted one or more times MBMSInterestIndication message in the period between the last 1 second preceding reception of ConditionalReconfiguration and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group :

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo if the RRCConnectionReconfiguration message is not applied due to a conditional reconfiguration; or

2> if SystemInformationBlockType18 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message one or more times indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the period between the last 1 second preceding reception of ConditionalReconfiguration message and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group:

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2> if SystemInformationBlockType19 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message one or more times indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the period between the last 1 second preceding reception of ConditionalReconfiguration message and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo if the RRCConnectionReconfiguration message is not applied due to a conditional reconfiguration: or

2>if SystemInformationBlockType21 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation one or more times indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the period between the last 1 second preceding reception of ConditionalReconfiguration message and the conditional reconfiguration execution if the RRCReconfiguration message is applied due to a conditional reconfiguration, and the UE is still configured to provide the latest concerned UE assistance information for the target cell group;:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

2>	remove all the entries within VarConditionalReconfiguration, if any;

2>	for each measId of the source SpCell configuration, if the associated reportConfig is condReconfigurationTriggerEUTRA:

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the measObjectId is only included in a MeasIdToAddMod:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.