3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104093

Electronic, 12th – 20th April, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2547 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Radio bearer handling upon SCG RLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In sub-clause 5.7.3.2 in TS 38.331, upon SCG failure reporting, Upon initiating the procedure, the UE shall: 1> suspend SCG transmission for all SRBs and DRBs; 1> reset SCG MAC; 1> stop T304 for the SCG, if running; 1> if the UE is in (NG)EN-DC: 2> initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a. 1> else: 2> initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5. The UE will suspend SCG transmission for all SRBs and DRBs upon SCG failure, but will not suspend radio bearers. After receiving SCG failure reporting, the network may perform PSCell change to recover SCG, via reconfigurationWithSync. In sub-clause 5.3.5.5.1 in TS 38.331, upon receiving a CellGroupConfig containing the spCellConfig with reconfigurationWithSync: The UE performs the following actions based on a received CellGroupConfig IE: 1> if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync: 2> perform Reconfiguration with sync according to 5.3.5.5.2; 2> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended; The part of “resume SCG transmission for all radio bearers” correponds to the action “suspend SCG transmission for all SRBs and DRBs” upon SCG failure. As the radio bearers were not suspended, there is actually no case to “resume all suspended radio bearers”. Note that this action was introduced at the beginning of Rel-15 for EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.5.5.1, remove “resume all suspended radio bearers”. Impact analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC,NR-DC, NR SA Impacted functionality Reconfiguration with sync Inter-operability: If the UE is implemented according to this CR while the network is not, there is no inter-operability issue forseen. If the network is implemented according to this CR while the UE is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some redundant actions in the specification which the UE will never execute, and cause confusions to implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==================================START OF CHANGES==================================

##### 5.3.5.5.1	General

The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). In (NG)EN-DC, the MCG is configured as specified in TS 36.331 [10], and for NE-DC, the SCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2>	perform Reconfiguration with sync according to 5.3.5.5.2;

2>	resume SCG transmission for all radio bearers, if suspended;

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2>	perform SCell addition/modification as specified in 5.3.5.5.9.

===================================END OF CHANGES===================================