3GPP TSG-RAN WG2 Meeting #113bis electronic          	          R2-2104095

Online, 12th- 20th April, 2021

Agenda Item:	5.4.1.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on active BWP release

Document for:	Discussion and decision

Introduction

In RAN2#113e meeting, there was a discussion in [1] on how to release an active BWP for SpCells as well as SCells, and the discussion was postponed.

Postpone P9 P11 - P9: whether the NW can release the active BWP for SpCell using RRC, and if allowed, whether the NW should always provide the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in the same RRC message. - 11: The active BWP of an SCell cannot be released by RRC message.

In this paper, we will discuss if anything needs to be clarified or specified for this issue.

Discussion

The BWP configuration is specified in TS 38.331 as below:

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                   OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                       OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink            OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

...

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

...

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                         OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink              OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

...

}

There can be one or several DL/UL BWPs configured, and firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id can be signalled to activate a BWP via RRC. The condition to include firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id is defined below:

This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for a SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M.

A BWP (with ID 1 to 4) can be added or modified in downlinkBWP-ToAddModList or uplinkBWP-ToAddModList, and can be released in downlinkBWP-ToReleaseList or uplinkBWP-ToReleaseList. The initial BWP (BWP#0) can be added as the dedicated BWP by including any dedicated configuration in initialDownlinkBWP, and can be released by releasing all dedicated configuration in initialDownlinkBWP (even if an empty initialDownlinkBWP is still configured).

Observation 1: The network can add/modify/release any BWP via RRC reconfiguration.

On the other hand, supporting more than 1 dedicated BWP configuration is optional to UEs. The UE only supporting 6-1 capability can only support 1 dedicated DL/UL BWP and does not support DCI based BWP switch.

6-1; Basic BWP operation with restriction; 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell.

The non-6-1 UE (i.e. 6-2/6-3/6-4) would be able to support DCI based BWP switch.

If the UE doesn’t support DCI based BWP switch, the UE will ignore the Bandwidth part indicator in the received DCI if any, as specified in TS 38.212. For example, for DCI Format 0-1 it is specified below.

- Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where - if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id; - otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1; If a UE does not support active BWP change via DCI, the UE ignores this bit field.

For the 6-1 UE which doesn’t support DCI based BWP change, the only case that there is a non-zero bit Bandwidth part indicator can be that the network configures a BWP other than the initial BWP for the 6-1 UE but doesn’t configure dedicated configuration for the initial BWP, in which case  =1 and there is only one dedicated BWP.

Observation 2: The 6-1 UEs only support configuration of a single dedicated BWP and do not support DCI based BWP switch, and non-6-1 UEs can support more than one dedicated BWPs and DCI/timer based BWP switch.

Observation 3: The 6-1 UEs will ignore the Bandwidth part indicator signalled in DCI if any, and non-6-1 UEs shall switch BWP based on the Bandwidth part indicator signalled in DCI.

UEs not supporting DCI based BWP switch (i.e. 6-1 UEs)

For the UE not supporting DCI based BWP switch (i.e. 6-1 UEs), since there is only one dedicated BWP which is also active, the only case the network may release the active BWP is that the network switches the BWP, e.g. from BWP#0 to BWP#0. There are only two methods to switch BWP: DCI based and RRC based. Since the UE doesn’t support DCI based BWP switch, the gNB has to perform RRC based BWP switch, by releasing a BWP and adding another BWP in the same RRC reconfiguration message and including firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id. Note that in this case there is a possibility the UE can be aware of the BWP switch even without firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id included as there is only one dedicated BWP left to be configured. However, given that there is no such an implicit BWP switch defined in specifications, we assume the network should perform BWP switch based on RRC.

For SpCell, according to the presence condition, the fields of firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id can be signalled in RRC reconfiguration without reconfigurationWithSync.

The field is optionally present for a SpCell, Need N, upon reconfiguration without reconfigurationWithSync.

However, for SCells, the fields of firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id can only be included upon SCell addition. Therefore, RRC based BWP switch for SCell in this case can only be done by SCell release/addition.

The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M.

Observation 4: For the UE not supporting DCI based BWP switch (i.e. 6-1 UEs), the network has to release the active BWP and add another dedicated BWP in a same message in order to change the BWP, and the network has to perform RRC based BWP switch, i.e. by including firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id in the RRC Reconfiguration message. For SCells, this procedure can be done via SCell release/addition in a single message.

UEs supporting DCI based BWP switch (i.e. non-6-1 UEs)

For the UE supporting DCI based switch (i.e. non-6-1 UEs), there can be more than one dedicated BWPs configured and one of them is activated. The BWP change/activation can be done by either RRC or DCI. There can be a case the network may release a BWP which is active, and there are several options for such procedure:

- Option 1: In the RRC Reconfiguration message which release a BWP, the network may include firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id to change to/activate another dedicated BWP. For SCells, this option can only happen with SCell release/addition.

- Option 2: In the RRC Reconfiguration message which release a BWP, the network may not include firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id, and only change/activate the BWP via DCI. Note that the bandwidth part indicator in the DCI format is not the BWP-ID as used in RRC, and there is a mapping rule between the bandwidth part indicator and the BWP-ID as below, which basically means that how to interpret the value of the bandwidth part indicator also depends on the RRC configuration. The network should schedule the DCI properly to avoid the UE using the wrong BWP, and this can be basically a network implementation. For SCells, this option does not need to involve SCell release/addition.

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

Note that in the original proposals in [1], the wording “release the active BWP” is only applicable when DCI based BWP switch is not considered. Otherwise, it would not be clear what the action is to release the active BWP, given that “release” is done at RRC and the “active BWP” can be changed at PHY. It is not preferable to agree or capture this kind of ambiguous text.

Observation 5: For the UE supporting DCI based BWP switch (i.e. 6-1 UEs), the network may release the active BWP and change the active BWP, either via RRC based BWP switch by including firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id in the RRC Reconfiguration message, or via DCI based BWP switch. The network implementation should ensure the DCI scheduled properly to avoid the UE using the wrong BWP.

Proposal: RAN2 can discuss if to capture anything in Observation 4 and 5 into RAN2 chairman notes or specifications.

Conclusion

In this paper we discussed how to release an active BWP, and have observed/proposed:

Observation 1: The network can add/modify/release any BWP via RRC reconfiguration.

Observation 2: The 6-1 UEs only support configuration of a single dedicated BWP and do not support DCI based BWP switch, and non-6-1 UEs can support more than one dedicated BWPs and DCI/timer based BWP switch.

Observation 3: The 6-1 UEs will ignore the Bandwidth part indicator signalled in DCI if any, and non-6-1 UEs shall switch BWP based on the Bandwidth part indicator signalled in DCI.

Observation 4: For the UE not supporting DCI based BWP switch (i.e. 6-1 UEs), the network has to release the active BWP and add another dedicated BWP in a same message in order to change the BWP, and the network has to perform RRC based BWP switch, i.e. by including firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id in the RRC Reconfiguration message. For SCells, this procedure can be done via SCell release/addition in a single message.

Observation 5: For the UE supporting DCI based BWP switch (i.e. 6-1 UEs), the network may release the active BWP and change the active BWP, either via RRC based BWP switch by including firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id in the RRC Reconfiguration message, or via DCI based BWP switch. The network implementation should ensure the DCI scheduled properly to avoid the UE using the wrong BWP.

Proposal: RAN2 can discuss if to capture anything in Observation 4 and 5 into RAN2 chairman notes or specifications.

References

- R2-2102293  Summary of [AT113-e][005][NR15] Connection Control II (Apple).