3GPP TSG-RAN2 Meeting 113-bis-e	R2-2104238

# on-line, 12th -20th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4639 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RRC full config for intra-SN PSCell change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTTDOCOMO, INC. , Ericsson, Fujitsu, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-3-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R2-1804100, RAN2 requests RAN3 to create a RRC full config indication in X2 for SN to indicate the full config. After receving this RRC full config indication in SGNB ADDITION REQUEST ACKNOWLEDGE in X2, MN may release and add the NR SCG part of the configuration and generates drb-ToRleaseList for the SN terminated RBs towards the UE. After that, RAN3 introduced the RRC full config indication in SgNB modificaiton required msg. and SgNB modificaiton request Ack msg. for intra CU inter-DU full config case (R3-183923, R3-183924). In current TS36.331, for (NG)EN-DC, it is specified that if the target SgNB cannot comprehend the NR SCG configuration provided by the source SgNB, the MN may release and add the NR SCG part of the configuration and generates drb-ToRleaseList for the SN terminated RBs towards the UE. However, it is unclear whether the rule above applies only for inter-SN PSCell change or both inter-SN and intra-SN PSCell change. Our understanding is that it applies only to the inter-SN PSCell change case. For intra-SN PSCell change, since the PDCP termination point is not changed, it is not necessary for eNB to generate drb-ToReleaseList for SN terminated RBs. Therefore the corresponding part needs to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “for inter-SN PSCell change” in the following sentence in 5.3.1.3 - Clarify for intra-SN PSCell change, E-UTRA does not include drb-ToReleaseList for SN terminated RBs. In case of (NG)EN-DC, for inter-SN PSCell change, the target SgNB may be unable to comprehend the NR SCG configuration provided by the source SgNB. In such a case, release and addition may be applied for the NR SCG part of the configuration. NOTE 1: For inter-SN PSCell change, when using release and addition for the NR SCG configuration, E-UTRAN includes drb-ToReleaseList for the SN terminated RBs. For intra-SN PSCell change, E-UTRA does not include drb-ToReleaseList for SN terminated RBs. Impact Analysis Impacted 5G architecture option: (NG)EN-DC Impacted functionality - PSCell change Inter-Operability: - If network is implemented according to the CR and UE is not, there is no inter-opterability problem, since the network behavior is not seen by UE. - If UE is implemented according to the CR and network is not, there is no inter-operability problem, since the network behavior is not seen by UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of RRC full config for intra-SN PSCell change, It remains unclear whethere MN takes the same action as the case of inter-SN PSCell change or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Of Change

Unchanged part are omitted

5.3.1.3	Connected mode mobility

Unchanged part are omitted

The same behavior applies in (NG)EN-DC, if upon handover the target eNB is unable to comprehend the MCG part of the UE configuration i.e. the target eNB uses the full configuration option which involves release and configuration of (most of the) MCG and NR SCG configuration. In case of (NG)EN-DC, for inter-SN PSCell change, the target SgNB may be unable to comprehend the NR SCG configuration provided by the source SgNB. In such a case, release and addition may be applied for the NR SCG part of the configuration.

NOTE 1:	For inter-SN PSCell change, when using release and addition for the NR SCG configuration, E-UTRAN includes drb-ToReleaseList for the SN terminated RBs. For intra-SN PSCell change, E-UTRA does not include drb-ToReleaseList for SN terminated RBs.

Unchanged part are omitted

End Of Change