3GPP TSG-RAN2 Meeting #113bis-e 	R2-2104260

# E-Meeting, 12 – 20 April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1809 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS video capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Currently, there is no way for the UE to indicate whether the UE supports IMS video in the UE-ETURA-Capability. The existing capability indications for IMS voice to indicate capabilities of IMS video. - The UE should not be reqired to support IMS video over split bearer for (NG)EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - If the UE supports IMS video, the capabilities for IMS voice apply to the IMS video. - IMS video over split bearer is not supported for (NG)EN-DC. Impact analysis Impacted 5G architecture options: E-UTRA/5GC, (NG)EN-DC Impacted functionality: IMS video over NR PDCP with MCG bearer IMS video over NR PDCP with SCG bearer IMS video over FR1/FR2 IMS video voer EUTRA/5GC Inter-operability: If the network does not implement the CR, the UE fails to establish the IMS video call due to an invalid configuration. The UE triggers the RRC connection re-establishment procedure due to the invalid coniguration. Otherwise, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE may fail to establish an IMS video call. - The UE is required to support IMS video call over the split bearer, which imposes an implementation burden on the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1A, 4.3.34.13, 4.3.34.14, 4.3.36.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.1A	NR PDCP Parameters

NR PDCP capabilities: the definition of rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15 and sn-SizeLo-r15 are the same as supportedROHC-Profiles, maxNumberROHC-ContextSessions, uplinkOnlyROHC-Profiles, continueROHC-Context, outOfOrderDelivery and shortSN defined in TS 38.306 [32].

ims-VoiceOverNR-PDCP-MCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR PDCP with only MCG RLC bearer.

ims-VoiceOverNR-PDCP-SCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with EN-DC. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR PDCP with only SCG RLC bearer when configured with EN-DC.

ims-VoNR-PDCP-SCG-NGENDC-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with NGEN-DC. If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR PDCP with only SCG RLC bearer when configured with NGEN-DC.

NOTE:	In this release of specification, neither IMS voice over split bearer nor IMS video over split bearer is supported for (NG)EN-DC.

#### 4.3.34.13	ims-VoiceOverNR-FR1-r15

This field indicates whether the UE supports IMS voice over NR FR1.  If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR FR1.

#### 4.3.34.14	ims-VoiceOverNR-FR2-r15

This field indicates whether the UE supports IMS voice over NR FR2.  If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR FR2.

#### 4.3.36.6	IMS-VoiceOverMCG-BearerEUTRA-5GC-r15

This field indicates whether the UE supports IMS voice over NR PDCP for MCG bearer for E-UTRA/5GC. It is mandated to the IMS voice capable UE if the UE supports eutra-5GC-r15.  If the UE supports IMS video, this field also indicates whether the UE supports IMS video over NR PDCP for MCG bearer for E-UTRA/5GC.