3GPP TSG-RAN WG2 meeting #113bis-e		R2- 2104279

E-Meeting,12th– 20th April 2021

Source:	vivo

Title:	Discussion on ambiguity of cell ID in RAN sharing

Agenda Item:	5.4.1.3

Document for:	Discussion

- Introduction

In TS 38.331, it is described that using the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo corresponding to the SIB for SI validity check. However, we find there is ambiguity of cellIdentity in RAN sharing case.

In this contribution, the issue is discussed and the corresponding solution is proposed.

- Discussion

According to the following description in TS 38.331 ver 15.13.0:

5.2.2.2.1  SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification; and whenever the UE does not have a valid version of a stored SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

- NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

To reduce the coordination between operators in RAN sharing cell, it has been agreed that each operator could set its own cellIdentity values for such shared cells.

Observation: The Cell-ID is configured per PLMN.

When UE performs SI validity checking, it needs to compare the cellIdentity received from the serving cell with the cellIdentity of the stored version of the SIB in UE. However, there may be many cellIdentities for RAN sharing cells since there are more than one PLMNs in the cell and one PLMN may correspond to one cellIdentity. However, it doesn’t make clear which cellIdentity will be used for SI validity check in TS 38.331.

Generally speaking, the cellIdentity corresponding to the selected PLMN will be used for SI validity checking.  Besides, in order to solve the issue on ambiguity of SI area ID, using the first PLMN-Identity in the PLMN-IdentityInfoList to check the SI validity. Firstly, when UE determines to store a SIB in a cell, it will store the first PLMN-Identity and the cellIdentity corresponding to the selected PLMN by the UE. Then, when UE checks whether the stored SIB is valid, it will check  the first PLMN-Identity and the cellIdentity corresponding to the selected PLMN in latest SIB1 received from current serving cell.

However, in RAN sharing case, we find there may occur a mistake when performing the SI validity checking procedure. The issue is illustrated in the following figure.

Figure 1 the issue of SI validity check

In the figure, cell1 is shared by operator A and B and operator A is the primary PLMN (the first PLMN in cell’s PLMN list). For a user served by operator B, it will select the PLMN controlled by operator B during PLMN selection, i.e. PLMN=2. When the UE camps on cell1, it may store the SIB(s) received from cell1. And the cell related informations it stores for SI vadility for cell-specific SIB in cell1 are PLMN=1 and cellIdentity =2 base on TS 38.331.  If the UE moves to cell2 within 3 hours (SI validity duration) of storing the SIB(s) from cell1, meanwhile the cell2 is only controlled by operator A and its PLMN=1. Besides the cellIdentity of PLMN1 configured in cell2 is equal to the cellIdentity of operator B configured in cell1 unfortunately. Then it seems very likely that the SIB valueTag in cell2 is just same as the valueTag of the SIB stored in the UE. In this case, the UE will consider the stored SIB from cell1 will be same as the SIB in cell2 by mistake even if the SIB is different between cell1 and cell2 actually. Consequently, it will not receive the corresponding SIB(s) any longer until the SIB(s) validity expired, which is not expected to occur.

With careful network implementation, the above issue will not cause error in UE. Operator A just ensures the cells which a user may reach in 3 hours will not be configured with the same cellIdentity with other operators sharing the same cells with itself. It seems a little inconvenient for operator A since there may be a lot of cells like cell2 in above examples. And the more operators share one cell, the more complexity in the setting of cellIdentity.

Taking the above into account, we suggest stating clearly in TS 38.331 using the first cellIdentity in the PLMN-IdentityInfoList to check the SI validity, i.e. cellIdentity corresponding to the first PLMN-Identity in the PLMN-IdentityInfoList. Since the cellIdentity is used to unambiguously identify a cell within a PLMN and the cellIdentity in cell1 and cell2 can't be same for the same PLMN. Then the issue can be easily solved, just illustrated in the following figure.

Figure 2 using the first Cell-ID in PLMN list for SI validity check

Proposal: The cellIdentity corresponding to the first PLMN in a cell’s PLMN list combination should  be taken into account while judging the validity of stored cell-specific SIBs.

- Summary

Based on the discussion above we propose the following observations and proposals:

Observation 1: The Cell-ID is configured per PLMN.

Proposal: The cellIdentity corresponding to the first PLMN in a cell’s PLMN list combination should to be taken into account while judging the validity of stored cell-specific SIBs, samely for NPN-only cell.

- References

[1] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, V15.13.0.

[2] R2-1701515. EUTRAN Sharing enhancement. Ericsson, Spidercloud, Qualcomm.

[3] R2-1804884. Ambiguity of SI Area ID in RAN Sharing.vivo

# Annex A

- Text proposal for Rel-15 TS 38.331 ver 15.13.0:

START OF the CHANGE

5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification; and whenever the UE does not have a valid version of a stored SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity and the first cellIdentity in the PLMN-IdentityInfoList, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.

- NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the first PLMN-Identity and the first cellIdentity in the PLMN-IdentityInfoList, the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

End OF the CHANGE