3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2104512

Online, 12–20 April 2021

# Agenda item:	5.5

# Source:	Samsung

# Title:	Report of [AT113b-e][602][POS] Positioning corrections for NR Rel-15 (Samsung)

# Document for:	Discussion and Agreement

1	Introduction

This is to report the result of the following email discussion in RAN2#113-bis-e Meeting [1].

[AT113b-e][602][POS] Positioning corrections for NR Rel-15 (Samsung)

Scope: Discuss and conclude on R2-2103219/R2-2103220 on SUPL support

Intended outcome: Agreed CRs

Deadline:  Friday 2021-04-16 1000 UTC

2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung | Taeseop LEE (taeseop.lee@samsung.com) |
| Intel | Yi Guo (yi.guo@intel.com) |
| Huawei,HiSilicon | Yinghao Guo (yinghaoguo@huawei.com) |
| CATT | Jianxiang Li(lijianxiang@datangmobile.cn) |
| ZTE | Liu.yansheng@zte.com.cn |
| Qualcomm | sfischer@qti.qualcomm.com |

3	Discussion

3.1	Correction on SUPL support of positioning method

(The following two contributions are discussed together here.)

R2-2103219	Correction on SUPL support of positioning methods	Samsung 	CR	Rel-15	38.305	15.8.0 	0070	-	F	UTRA_LTE_iPos_enh2-Core

R2-2103220	Correction on SUPL support of positioning methods	Samsung 	CR	Rel-16	38.305	16.4.0 	0071	-	A	UTRA_LTE_iPos_enh2-Core

These two papers contain the same correction on SUPL support of positioning methods. In TS 38.305, two tables (i.e., Table 4.3.1-1 and Table B.1-1) show SUPL support of positioning method. However, there is a contradiction between the two, regarding whether positioning methods using Bluetooth/sensors are supported in SUPL.

For your convenience, Table 4.3.1-1 and Table B.1-1 in TS 38.305 are copied below:

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| Bluetooth | No | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |

Table A.1-1: SUPL support of positioning methods

| Positioning Protocol: | RRLP (GSM/GPRS/WCDMA/ LTE/WLAN/WiMAX) | RRC (WCDMA) | LPP (NR/LTE) |
| --- | --- | --- | --- |
| Positioning Method: |  |  |  |
| A-GPS (A-GANSS) SET Assisted |  |  |  |
| A-GPS (A-GANSS) SET Based |  |  |  |
| Autonomous GPS/GANSS |  |  |  |
| Enhanced Cell ID NOTE 1 |  |  | NOTE 2 |
| Enhanced Observed Time Difference (E-OTD) | (GSM only) | NA | NA |
| Observed Time Difference of Arrival (OTDOA) NOTE 1, NOTE 3 | NA |  |  |
| Sensors | NA | NOTE 5 |  |
| WLAN | NA |  |  |
| Bluetooth | NA |  |  |
| TBS NOTE 4 | NA |  |  |
| NR DL-TDOA | NA | NA |  |
| NR DL-AoD | NA | NA |  |
| NR E-CID (downlink) | NA | NA |  |
| NR Multi-RTT | NA | NA |  |
| NR UL-TDOA | NA | NA |  |
| NR UL-AoA | NA | NA |  |

From the latest version of OMA SUPL v2.0.6 [2], we can find that Bluetooth and Sensors are not shown in PosMethod field like below.

PosMethod ::= ENUMERATED { agpsSETassisted(0), agpsSETbased(1), agpsSETassistedpref(2), agpsSETbasedpref(3), autonomousGPS(4), aflt(5), ecid(6), eotd(7), otdoa(8), noPosition(9), ..., ver2-historicalDataRetrieval(10), ver2-agnssSETassisted(11), ver2-agnssSETbased(12), ver2-agnssSETassistedpref(13), ver2-agnssSETbasedpref(14), ver2-autonomousGNSS(15), ver2-sessioninfoquery(16), ver2-mbs(17), ver2-NR-DL-TDOA(18), ver2-NR-DL-AoD(19), ver2-NR-Multi-RTT(20), ver2-NR-DL-E-CID(21), ver2-NR-UL-TDOA(22), ver2-NR-UL-AoA(23)}

Thus, rapporteur thinks that the check marks (green-highlighted) in Table B.1-1 need to be changed to ‘NA’.

- Note:  Other 3 CRs with the same changes are submitted to TS 36.305 and will be discussed in another e-mail discussion ‘[Offline-601][POS] Positioning Corrections for R15 and earlier’. Rapporteur expects that we can have consistent results in both discussions.

Q1. Do you agree that the changes in R2-2103219/R2-2103220 are needed? In the comment field please indicate if you request some changes in the CR.

| Company | Agree as it is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Agree as it is. | Note that we have submitted the CRs to TS 36.305 under AI 4.4 for the same changes to the legacy LPP from Rel-14, so the WI code UTRA_LTE_iPos_enh2-Core is used. We are fine to add the WI code NR_newRAT_Core to the TS 38.305 CRs but have no strong view. |
| Intel | Agree as it is |  |
| Huawei, HiSIlicon | Agree as it is |  |
| CATT | Disagree | The column SUPL in table 4.3.1-1 “Supported versions of UE positioning methods” means what SUPL itself supports. In fact SUPL 2.0 doesn’t support Bluetooth and sensor while SUPL 3.0 supports. According to the References in TS38.305, only SUPL 2.0 is referred. So the table 4.3.1-1 truly specifies what SUPL supports. [15] OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0". [16] OMA-TS-ULP-V2_0_4: "UserPlane Location Protocol Approved Version 2.0.4". However the LPP(NR/LTE) column (green marked) in table A.1-1 ” SUPL support of positioning methods” means what positioning methods supported by 3GPP LPP. Of course, LPP(NR/LTE) already supports Bluetooth and sensor. The two tables are talking about two different protocols. So from my perspective, the CR is not needed and the two tables in TS 38.305 are correct. |
| ZTE | Agree as it is |  |
| Qualcomm | Rel-16 only | Not essential. This is an informative Annex; a Rel-16 CR (as it is) should be sufficient. |
| Qualcomm-2 | Disagree | After further checking, we agree with the general comment from CATT: The Table 4.3.1-1 and Table A.1-1 have a different meaning. Table A.1-1 lists the positioning methods supported using a particular protocol such as LPP/LPPe etc., rather than the positioning methods indicated by the SLP/SET on SUPL ULP. So although WLAN, Sensor and Bluetooth are not indicated as supported positioning methods in ULP, the positioning methods can be supported in SUPL via tunnelling e.g., LPP as noted at the end of section A.1 for ECID (which has been omitted in the CR). Additionally, with the now added NR Location ID supported in SUPL ULP, the text following Table A.1-1 is no longer correct and needs modification. Table 4.3.1-1 seems in need of cleanup or clarification as well, since there is no exact alignment between UE-assisted and UE-based for control-plane positioning and SUPL – which however, should be present due to common use of LPP. Some notes might need to be added to clarify the distinction between a SUPL ULP position method and an LPP position method. |

Conclusion:

From the phase 1 discussion, the majority of companies (5/7) agree with the changes. However, 2 companies (CATT, Qualcomm) disagree and pointed out that Table 4.3.1-1 and Table A.1-1 have a different meaning, which rapporteur also agrees based on the additional inputs during phase 2. Qualcomm also suggested other 2 corrections regarding the modification of text following Table A.1-1 and the additional notes for Table 4.3.1-1. Rapporteur agrees with the motivation and expects the corrections can be discussed at the following RAN2#114-e meeting, and thus propose:

Proposal 1: The CRs R2-2103219 and R2-2103220 are not pursued.

4	Conclusion

Proposal 1: The CRs R2-2103219 and R2-2103220 are not pursued.

5	References

[1]	R2-113bise Chairman notes 2021-04-11.docx

[2]	OMA-TS-ULP-V2_0_6-20200804-A