3GPP TSG-RAN WG2  Meeting # 113bis electronic	R2-2104589

# Online, April 12 – April 20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Failure Recovery via CHO for Inter-RAT Handover Failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussion in R2-2103114, agreement made In RAN2#113e was as following: The CR on the release of VarConditionalReconfig for inter-RAT handover case in R2-2100585 is not pursued for both NR and LTE; Clarify in meeting notes that the CR is correct i.e. that UE release VarConditionalReconfig only if the inter-RAT handover was successful (but not in case of failure)'. However it is incosistent with the agreement made in RAN2#109e:  2: Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure. Procedure for attemptCHO in 5.3.7.3 of TS 38.331 is updated with a condition as shown below. Then in RAN2#113bis meeting, it is concluded that inter-RAT handover failure can trigger failure recovery via CHO, which means Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure or inter-RAT handover failure. |
| Summary of change: |  | - Modify the 5.3.7.3 to allow UE perform failure recovery via CHO upon inter-RAT handover failure. Impact analysis Architecture options SA Impacted functionality: CHO Inter-operability: If only the network is implemented according to the CR and the UE is not, no interoperability problems are foreseen. If only the UE is implemented according to the CR and the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can’t perform failure recovery via CHO upon inter-RAT handover failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	if the UE is connected to 5GC and the selected cell is only connected to EPC; or

1>	if the UE is connected to EPC and the selected cell is only connected to 5GC:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

1>	else:

2>	stop timer T311;

2>	if the cell selection is triggered by detecting radio link failure of the MCG or handover failure (including intra-EUTRA handover and mobility from E-UTRA); and

2>	if attemptCondReconf is configured; and

2>	if the selected cell is one of the target candidate cells in VarConditionalReconfiguration:

3>	apply the stored condReconfigurationToApply of the selected cell and perform the actions as specified in 5.3.5.4;

2> else:

3>	if the UE is configured with conditionalReconfiguration:

4>	release uplinkDataCompression, if configured;

4>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

4>	reset MAC;

4>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

4>	release the SCell group(s), if configured, in accordance with 5.3.10.3d;

4>	apply the default physical channel configuration as specified in 9.2.4;

4>	for the MCG, apply the default semi-persistent scheduling configuration as specified in 9.2.3;

4>	for the MCG, apply the default MAC main configuration as specified in 9.2.2;

4>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

4>	release reportProximityConfig, if configured and clear any associated proximity status reporting timer;

4>	release obtainLocationConfig, if configured;

4>	release idc-Config, if configured;

4>	release sps-AssistanceInfoReport, if configured;

4>	release measSubframePatternPCell, if configured;

4>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

4>	if (NG)EN-DC is configured:

5>	perform MR-DC release, as specified in TS 38.331[82], clause 5.3.5.10;

5>	release p-MaxEUTRA, if configured;

5>	release p-MaxUE-FR1, if configured;

5>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

4>	release naics-Info for the PCell, if configured;

4>	if connected as an RN and configured with an RN subframe configuration:

5>	release the RN subframe configuration;

4>	release the LWA configuration, if configured, as described in 5.6.14.3;

4>	release the LWIP configuration, if configured, as described in 5.6.17.3;

4>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

4>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

4>	release overheatingAssistanceConfig and overheatingAssistanceConfigForSCG, if configured and stop timer T345, if running;

4>	release ailc-BitConfig, if configured;

3>	remove all the entries within VarConditionalReconfiguration, if any;

3>	for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA configured:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

3>	start timer T301;

3>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

3>	if the UE is a NB-IoT UE connected to EPC, the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation and AS security has not been activated; and

3>	if cp-reestablishment is not included in SystemInformationBlockType2-NB:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3>	else:

4>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	if the selected cell is a UTRA cell, and if the UE supports Radio Link Failure Report for Inter-RAT MRO, include selectedUTRA-CellId in the VarRLF-Report and set it to the physical cell identity and carrier frequency of the selected UTRA cell;

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

End of Change