3GPP TSG-RAN WG2 Meeting #114 electronic		R2-2105197

Online, May 19 – May 27, 2021

Agenda item:	8.13.2.1

Source:	China Telecommunication

Title:	Further discussion on SON of CHO

Document for:	Discussion

- Introduction

The RAN2#113bis-e meeting made a few conclusions about the CHO related SON aspects with some FFS issues left. In this paper, we would like to give our views on these FFS issues. On the other hand, we also would like to update our analysis given by our previous paper [1] in this paper based on the latest agreements.

- Discussion

2.1 views on CHO related SON left issues

According to the chair notes [2], the following CHO related SON issues were marked as FFS:

RAN2 to focus on the following CHO scenarios at least:

a.	Scenario 1 (too late HO): 1a, 1b, 1c, 1d

b.	Scenario 2 (too early HO): 2a, 2b

c.	Scenario 3 (HO to wrong cell): 3a, 3b, 3c, 3e, 3f

FFS the need to merge certain scenarios, e.g. 1b/1c, 2a/2b

……

The following information in the RLF report for CHO are needed:

b.	CHOCellId, to indicate the selected CHO cell after the first connection failure and before the reestablishment

c.	CellID to indicate the cell in which the UE attempted the second reestablishment after failure of the first reestablishment following an HOF/RLF.

How to provide these information is FFS.

Regarding scenario 1b/1c, we think it’s better to keep them as they are, i.e. do not merge scenario 1b with 1c. Because scenario 1b/1c are caused by different root causes and need different optimization actions, i.e. scenario 1b is caused by wrong CHO candidate cells configured by the network while scenario 1c is caused by coverage problem or capacity problem. When the network knows the UE was configured CHO before the failure through the CHO indication in RLF report, it need to further differentiate the cases to find the root cause.

Similarly, looking at scenario 2a/2b, we also think no need to merge them. Because the root causes of scenario 2a/2b are not totally the same, i.e. scenario 2a is caused by too loose CHO execution condition configuration (i.e. too early CHO) while scenario 2b is probably caused by imperfect configuration for the additional CHO attempt after CHOF besides too loose CHO execution condition configuration. Thus it’s better to make the network aware of the two different cases.

Proposal 1: no need to merge scenarios 1b/1c, and no need to merge scenario 2a/2b.

Regarding how to provide the information in RLF report for CHO, from our point of view, we see benefits of using one RLF report for successive failure cases when the UE is configured with CHO configuration as mentioned by some companies. Since it’s possible that a successive failure case includes ordinary HOF and CHO reestablishment failure, it is easier for the network to identify successive failure cases by putting the failure information of multiple failures in one RLF report, i.e. just adding new IEs or new fields in legacy RLF report.

Proposal 2: In case the UE is configured with CHO configuration, it is easier for the network to identify successive failure cases by putting the failure information of multiple failures in one RLF report, i.e. just adding new IEs or new fields in legacy RLF report.

2.2 other information needed in RLF report

According to the current specification, if multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select. Then look at the “CHO to wrong cell” scenario 3a, i.e. the UE experiences a CHO failure in one of the candidate CHO target cell (named as cell A in this paper) and successfully reestablishes in another candidate target cell (named as cell B in this paper), it is possible that the two candidate CHO target cells (i.e. cell A and cell B) selected by the UE are both included in the list of candidate CHO cells which meet the CHO execution conditions configured by the source cell. In this case, it seems like an invalid “CHO to wrong cell” case, and it is just the matter with the UE implementation. However, when only the first candidate CHO target cell (i.e. cell A) meets the CHO execution conditions configured by the source cell, and only the second candidate CHO target cell (i.e. cell B) meets the CHO execution conditions configured by the source cell, it seems like a valid “CHO to wrong cell” case. Thus it is useful to differentiate valid “CHO to wrong cell” case and invalid “CHO to wrong cell” case.

Since RAN2 has agreed to include the following information in RLF report for CHO, or the network can fetch these information if the network keeps UE’s context, the network can identify whether the target cell (i.e. cell B) in which it reestablished successfully also meets the conditions for the first CHO execution. Thus it can differentiate valid or invalid “CHO to wrong cell” case.

- Configured CHO execution condition(s)

- Latest radio measurement results of the candidate target cells

- List of candidate cells IDs

- CHOCellId, to indicate the selected CHO cell after the first connection failure and before the reestablishment

- CellID to indicate the cell in which the UE attempted the second reestablishment after failure of the first reestablishment following an HOF/RLF

Observation 1: The network can identify whether the CHO candidate target cell in which the UE successfully reestablishes after a CHO failure to another candidate CHO target cell also meets CHO execution conditions or not, based on the information in RLF report for CHO agreed by RAN2#113bis-e, in order to confirm the problem is improper CHO configuration or just UE implementation.

On the other hand, according to the current specification, the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure only when attemptCondReconfig-r16 is configured. The field is optional present if the UE is configured with at least a candidate SpCell for CHO, otherwise the field is not present. Then take the “too late CHO” scenario 1d as an example, the UE experiences RLF in the source cell before CHO execution conditions for any of the candidate cells are fulfilled and subsequently (un)successful reestablishment in non-candidate CHO cell. It is not clear the reason for the UE selecting a non-candidate CHO cell is that none of the candidate CHO cells fulfill the CHO execution conditions or that the UE was NOT configured attemptCondReconfig-r16. Furthermore, the network would be not sure the optimization direction is adjust the CHO execution conditions, the candidate CHO cell list or just configure the attemptCondReconfig-r16 to allow CHO recovery.

Thus an indication information whether attemptCondReconfig-r16 is configured or not can be reported to the network to help the network to deduce the optimization direction. However, since the attemptCondReconfig-r16 is optionally included in the CHO configuration configured by the source cell, the network can also obtain this information from the UE context stored in source cell. In addition, RAN2 has sent LS to RAN3 to confirm whether the source cell will keep the UE context, at least until the RLF-report is received by the source cell. We can wait for RAN3’s response and then decide whether the indication information is needed to be reported by the UE.

Proposal 3: the network need to confirm whether attemptCondReconfig-r16 is configured or not to help deducing the optimization direction. Whether an explicit indication is needed in RLF report depends on RAN3’s response to LS R2-2102149.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: no need to merge scenarios 1b/1c, and no need to merge scenario 2a/2b.

Proposal 2: In case the UE is configured with CHO configuration, it is easier for the network to identify successive failure cases by putting the failure information of multiple failures in one RLF report, i.e. just adding new IEs or new fields in legacy RLF report.

Observation 1: The network can identify whether the CHO candidate target cell in which the UE successfully reestablishes after a CHO failure to another candidate CHO target cell also meets CHO execution conditions or not, based on the information in RLF report for CHO agreed by RAN2#113bis-e, in order to confirm the problem is improper CHO configuration or just UE implementation.

Proposal 3: the network need to confirm whether attemptCondReconfig-r16 is configured or not to help deducing the optimization direction. Whether an explicit indication is needed in RLF report depends on RAN3’s response to LS R2-2102149.

4	References

R2-2103157, Discussion on CHO configuration optimization aspects.

R2-113b-e SONMDT HuNan 2021-04-20-1500 UTC.