3GPP TSG-RAN WG2 #114-e	R2-21xxxxx

Electronic, 19th – 27th May, 2021

Agenda Item:	5.4.1

Source:	ZTE Corporation

Title:	[AT114-e][007][NR15] Connection Control IV (ZTE)

Document for:	Discussion, Decision

# Introduction

This document is to kick off the following email discussion:

[AT114-e][007][NR15] Connection Control IV (ZTE)

Scope: Treat R2-2105392, R2-2105403, R2-2104827, R2-2104828, R2-2105404, R2-2105405, R2-2104905, R2-2104906, R2-2106264, R2-2106265

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

The plan for Schedule A is below:

A first round with Deadline for comments Friday May 21 1000 UTC to settle scope what is agreeable etc (phase 1).

A pre-final round with Deadline for any functional and/or scope comments Wednesday May 26 1200 UTC. At this point, non-agreeable parts shall be removed/excluded. (phase 2)

A final round (last 24h) for checking and smaller simplification / removal comments only including agreeable parts, with Deadline EOM (at this point all outcome documents need to be available in inbox with tdoc numbers).

Additional check-points etc if needed are defined by the Rapporteur. Offline discussion rapporteur must notify chairman / session chair if on-line comeback discussion is needed, if discussion doesn’t converge etc.

# Contact Information

| Company | Email address |
| --- | --- |
| ZTE | liu.yu3@zte.com.cn |
| Apple | naveen.palle@apple.com |
| QCOM | mambriss@qti.qualcomm.com |
| MediaTek | Chun-fan.tsai@mediatek.com |
| Samsung | jack.jang@samsung.com |

# Discussion

Companies are requested to add their comments on each of the CRs of this email discussion in the questionnaires below.

## First Active Downlink BWP

Was agreeable last meeting, avoid repeat discussion if possible

R2-2106460	Correction on firstActiveDownlinkBWP-Id 	ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-15	38.331	15.13.0	2530	2	F	NR_newRAT-Core

R2-2106461	Correction on firstActiveDownlinkBWP-Id(R16)	ZTE Corporation, Sanechips, Ericsson, Nokia	CR	Rel-16	38.331	16.4.1	2531	2	A	NR_newRAT-Core

The reason for changes is:

This CR was discussed at RAN2#113bis (R2-2103793) and the contents were concluded to be agreeable (see offline [005] report in R2-2104633). However, the CR was marked as postponed by mistake and hence we resubmit this for approval. The actual proposed change is same as the one in R2-2103793. In the current RRC spec the following description highlighted below, is not accurate because the intention of this is to indicate that the network shall set the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value when performing RA. firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon MAC-activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon PCell change and PSCell addition/change, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.. That is the UE expects to use the active DL BWP with the same bwp-Id as the active UL BWP when performing RA.

Q1: Do you agree with the two CRs R2-2106460 and R2-2106461?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | ok |  |
| QCOM | Yes |  |
| MediaTek | Yes |  |
| Huawei, HiSilicon | Yes | Should be merged to rapporteur CR according to the consensus of the last meeting in R2-2104633: For Active BWP change (R2-2103793 and R2-2103794): Proposal 2: Agree the changes in R2-2103793 and R2-2103794 and merge these into the rapporteur’s CRs. |
| Samsung | Yes | Fine with Huawei's suggestion too. |

## Default configuration

R2-2104827	CR on default configuration	OPPO	CR	Rel-15	38.331	15.13.0	2583	-	F	NR_newRAT-Core

R2-2104828	CR on default configuration	OPPO	CR	Rel-16	38.331	16.4.1	2584	-	A	NR_newRAT-Core

The reason for changes is:

In 5.3.7.4, it is specified that - apply the specified configuration defined in 9.2.1 for SRB1; However, 9.2.1 is for “default” configuration, of which the definition is different from “specified” configuration defined in 9.1.x, i.e., default configuration can be overriden but specified configuration cannot be.

Q2: Do you agree with the problem identified and the changes in R2-2104827, R2-2104828?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | No strong view, the intention has not changed. |  |
| QCOM | Yes |  |
| MediaTek | Yes | Looks like editroial. Suggest to put in rapporteur’s CR |
| Huawei, HiSilicon | Yes | Can be merged to rapporteur CR. |
| Samsung | - | Fine but minor/editorial, so prefer merging it into rapprteur CR. |

## AperiodicSRS-Resource

R2-2105404	Correction on aperiodicSRS-Resource	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.13.0	2624	-	D	NR_newRAT-Core

R2-2105405	Correction on aperiodicSRS-Resource(R16)	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.4.1	2625	-	A	NR_newRAT-Core

The reason for changes is:

- In the RAN2#111-e meeting, the CR R2-2007504 was agreed and merged to rapporteur CR, and the following agreement was made: [003] Both Partly merged, The reference clause in the field description of aperiodicSRS-ResourceTriggerList needs to be updated: change “6.1.1.2” to “6.1”. This change is merged into Rapporteur’s CR for Rel-15/16 specification. But the agreement is not correct, the intention of the CR R2-2007504 is to change “6.1.1.2” to “6”. In addition, the field description of aperiodicSRS-ResourceTrigger should be modified simultaneously. 2. The field description of rateMatchPatternToAddModList in ServingCellConfig is not correct, and the correct quote is 5.1.4.1, not 5.1.2.2.3, same as the field description of rateMatchPatternToAddModList in ServingCellConfigCommon.

Q3: Do you agree with the problem identified and the changes in R2-2105404, R2-2105405?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| QCOM | Yes |  |
| MediaTek | Yes | Looks like editroial. Suggest to put in rapporteur’s CR |
| Huawei, HiSilicon | Yes | Can be merged to rapporteur CR. |
| Samsung | - | Fine but minor/editorial, so prefer merging it into rapprteur CR. |

## CSI-RS configuration

R2-2104905	Correction on CSI-RS configuration	vivo	CR	Rel-15	38.331	15.13.0	2587	-	F	NR_newRAT-Core

R2-2104906	Correction on CSI-RS configuration	vivo	CR	Rel-16	38.331	16.4.1	2588	-	A	NR_newRAT-Core

The reason for changes is:

- In current description of parameter maxNrofNZP-CSI-RS-ResourceSets, it is mentioned that: Maximum number of NZP CSI-RS resources per cell. Actually, more than one resources could be configured in one resource set. This parameter should be used to define the maximum number of NZP CSI-RS resource sets per cell. Thus, the corresponding description should be updated. Similar issue exists for parameter maxNrofCSI-IM-ResourceSets. - In the description of parameter maxNrofCSI-IM-Resources and maxNrofCSI-IM-ResourcesPerSet, TS 38.214 is referred. Actually, in TS 38.214, there is no such parameters. Only the csi-IM-ResourceId or the entry number (i.e. csi-IM-ResourcesForInterference) in csi-IM-ResourceSetList is referred. Thus, the corresponding description should be updated.

Q4: Do you agree with the problem identified and the changes in R2-2104905, R2-2104906?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes | rapporteur CR of editorial changes? |
| QCOM | Yes | editorial change ... may be a rapporteur CR |
| MediaTek | Yes | Looks like editroial. Suggest to put in rapporteur’s CR |
| Huawei, HiSilicon | Yes | Can be merged to rapporteur CR. |
| Samsung | - | Fine but minor/editorial, so prefer merging it into rapprteur CR. |

## A-CSI trigger state configuration

R2-2106264	Correction on A-CSI trigger state configuration	vivo	CR	Rel-15	38.331	15.13.0	2685	-	F	NR_newRAT-Core

R2-2106265	Correction on A-CSI trigger state configuration	vivo	CR	Rel-16	38.331	16.4.1	2686	-	A	NR_newRAT-Core

The reason for changes is:

Parameters nzp-CSI-RS-ResourcesforChannel and qcl-info-forChannel are now being referred in the field descriptions of csi-IM-ResourcesForInterference and qcl-info , however, it is not correct since these two parameters are not defined in the current specification. Actually, nzp-CSI-RS-ResourcesforChannel should be replaced to the resourceSet IE in CSI-AssociatedReportConfigInfo. And qcl-info-forChannel should be replaced to qcl-info.

Q5: Do you agree with the problem identified and the changes in R2-2106264, R2-2106265?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Apple | Yes |  |
| QCOM | Yes |  |
| MediaTek | Yes with comment | We think the term „above“ is a little bit strange. Instaed of „resourceSet above“, we suggest to use the term „resourceSet within nzp-CSI-RS“ Also this kind of change is more suitable in rapporteur’s CR. |
| Huawei, HiSilicon | Yes | MTK’s update is fine to us. |
| Samsung | - | Fine but minor/editorial, so prefer merging it into rapprteur CR. We are also fine with suggestion from MediaTek. |

# Conclusion

TBD

# References

[1]