3GPP TSG RAN WG2 Meeting #114-e		R2-21xxxxx

e-Meeting, 19th – 27th May, 2021

Title:	Draft Reply LS on overlapped data and SR with equal L1 priority

Response to:	R1-2102244

Release:	Rel-16

Work Item:	NR_IIOT-Core

Source:	RAN2

To:	RAN1

Cc:

Contact person:

#### Name:	Sangkyu Baek

E-mail Address:	sangkyu.baek@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN2 would like to appreciate the LS on overlapped data and SR are of equal L1 priority (R1-2102244). RAN2 has discussed and concluded the following.

For case 2-1 and case 4, RAN2 has made the following agreement in RAN2#114-e:

We go with Understanding 1: MAC does not use knowledge of UCI multiplexing when MAC executes LCH based prioritization and deciding when to transmit SR

For case 2-2 and case 3, RAN2 has made the following working assumption in RAN2#113-e:

Working assumption: The MAC entity does not generate a MAC PDU for a deprioritized uplink grant even when its associated PUSCH is overlapping with PUCCH. This working assumption is not agreed until confirmed by RAN1.

It was further confirmed in RAN2#113bis-e:

Confirm the WA that LCH based prio has higher priority than UL skipping still applies, and we expect that if there are issues, RAN1 will come-back.

The intended MAC layer behaviour of the working assumption is Understanding 2.

# 2	Actions

To RAN1

ACTION:  RAN2 respectfully asks RAN1 to take the above into account.

# 3	Dates of next TSG RAN WG2 Meetings

TSG-RAN WG2 Meeting #115-e		        16th – 27th August 2021		        E-meeting

TSG-RAN WG2 Meeting #116-e		        1st – 12th Novemebr 2021		        E-meeting